# SYSC-3006	Computer Organisation

# Study Exercises : Subroutines & Parameter Passing.

References in the Text :

Section 5.4  and 5.5

Chapter 9 	(Beyond the course, but great stuff)

#### Selected Exercises from the Text

Section Review, page 158	Questions 1-8

Section Review, page 168	Questions 1-3, 5-7

#### Additional Exercises

- For each of the following C-prototypes, write the code for calling and implementing the subroutine. For the implementation, we don’t know (nor care) what it is supposed to do.  Simply show you would access and manipulate each of the parameters and return type.

  - void sub ()

  - void sub (byte a)

  - void sub (byte a, byte b)

  - void sub (word a)

  - void sub (word a, int b)

  - void sub (boolean &a)

  - void sub (word &b)

  - void sub (byte a, word &b)

  - byte sub ()

  - int sub (byte a)

  - boolean sub (char a, byte b)

  - long int sub (char a, int b, int &c)

- For each of the following code fragments, reverse-engineer the C-like prototype for the subroutine.

  - sub PROC

MOV AL, xxx

RET

sub ENDP

  - sub PROC

PUSH BP

MOV BP, SP

PUSH AX

PUSH DX

MOV BX, [BP+6]

MOV [BX], DX

MOV AL, [BP+4]

POP DX

POP AX

POP BP

RET

sub ENDP

  - sub PROC

PUSH BP

MOV BP, SP

PUSH BX

PUSH DX

MOV BX, [BP+8]

MOV BX, xxx

MOV AX, [BP+4]

MOV DX, [BP+6]

MOV AL, xxx

POP DX

POP BX

POP BP

RET

sub ENDP