# SYSC3601 LABORATORY #4 (Summer 2009)	Date:___________

# FLIGHT Motorola 68EC020 EVB and Application Board

| Group: _______ | Student #1 | Student #2 |
| --- | --- | --- |
| Name: |  |  |
| Student ID: |  |  |
| Pre-lab (/5): |  |  |
| In-Lab (/5): |  |  |
| TOTAL: |  |  |

Fill in the blanks on these answer sheets (One per group). These sheets constitute your lab report. The T.A. will ask you questions about your work. You will have to demo your work to the T.A.

## Part A – Getting to know the EVB

Table A1:

| Slot ID | Chip type/Label | Description |
| --- | --- | --- |
|  |  | The main processor |
|  | MC68881RC16B |  |
| U5 (=IC5) |  |  |
|  |  | Peripheral interface/timer chip |
| U29 |  |  |
|  |  | Dual asynchronous receiver/transmitter (DUART) |
| U14 |  |  |
|  |  | EPROM (64kB) which contains monitor and assembler/disassembler firmware. |

A2: ________________________________________________________________________________

___________________________________________________________________________________

A3: ________________________________________________________________________________

___________________________________________________________________________________

A4: ________________________________________________________________________________

___________________________________________________________________________________

A5: ________________________________________________________________________________

___________________________________________________________________________________

## Part B - Running a simple program

B1: ________________________________________________________________________________

___________________________________________________________________________________

Table B2:

| - Instructions | Machine Code |
| --- | --- |
| - MOVEQ #66,D5 |  |
| - MOVEQ #$72,D0 |  |
| - SUBQ.L #4,D5 |  |
| - ADDQ.B #6,D0 |  |
| BRA $400604 |  |
| - END |  |

B3: ________________________________________________________________________________

___________________________________________________________________________________

B4:

F> PR

- BEGIN: 00000000 400600

END  : 00000000 400608

00400600  ____  ____  ____  ____  ____  ____  ____   ____

B5: ________________________________________________________________________________

___________________________________________________________________________________

B6: ________________________________________________________________________________

___________________________________________________________________________________

___________________________________________________________________________________

___________________________________________________________________________________

___________________________________________________________________________________

## Part C – Compiling on the host PC using the cross-compiler

C1: ________________________________________________________________________________

___________________________________________________________________________________

Part C, TA Demo: ______________________________ (add by 4, subtract by 1)

## Part D – The application board

Lab4d.asm, fill in the blanks below:

ORG $400600

INIT	MOVEA.L #$800001,A0	* Load A0 with base address for PI/T registers

MOVE.B  #$80,(___,A0)* Set PortA control register to mode 1X

MOVE.B	 #$0,(___,A0)	* Set all bits of PortA to be input using PortA DDR

MOVE.B	 #$80,(___,A0)	* Set PortB control register to mode 1X

MOVE.B  #$FF,(___,A0)* Set all bits of PortB to be output using PortB DDR

MOVE.B  #0,(___,A0)	* Clear the PortB Data Register to clear LEDs

READ	MOVE.B	 (___,A0),D0	* Read from the PortA Data Register into D0

MOVE.B	 D0,(___,A0)	* Echo the data from D0 out to PortB Data Register

BRA	 READ

Part D, TA Demo: ______________________________ (quits for $AA bit pattern)

## Part E – Controlling LEDs for traffic intersection simulator

Part E, TA Demo: ______________________________ (Get program working. Add walk signal)

## Final Remarks:

Do not leave any of your files on the local hard drives (i.e. C:). Keep all your files on the H: drive, on a USB memory key, or email them to yourself.