# SYSC3601 LABORATORY #4 (Summer 2009) – Home Preparation

In this lab you will be using a Motorola 68EC020 EVB and Application board.  The application board contains red, yellow and green LEDs as shown below:

Each LED can be turned ON if its corresponding bit is set to 1, and OFF if it is set to 0.  Bit 0 corresponds to the yellow LED on the right and bit 7 corresponds to the red LED on the left. The LEDs are found at address $800013.

- The program listed below mimics traffic lights at an intersection. Bits 3-1 control north-south traffic, while bits 7-5 control east-west traffic. For example, while the north-south light is GREEN (i.e. LED 1 is ON), the east-west light is RED (i.e. LED7 is ON).

Comment the Motorola assembler code below to explain what the program is doing at each line.  A couple of lines are filled in for you as an example.

| Line | CODE | DESCRIPTION |
| --- | --- | --- |
| 0 | TABST MOVEA.L #TABLE,A1 |  |
| 1 | MOVE.B #6,D3 | Move byte value 610 into register d3 (Hint: there are 6 entries in TABLE) |
| 2 |  |  |
| 3 | * Main program |  |
| 4 | LOOP MOVE.B (A1)+,( $800013) |  |
| 5 | MOVE.B (A1)+,D2 |  |
| 6 | BSR DELAY |  |
| 7 | SUBQ.L #1,D3 |  |
| 8 | BNE.S LOOP |  |
| 9 | BRA.S TABST |  |
| 10 |  |  |
| 11 | * Delay subroutine (delay for <d2> seconds) |  |
| 12 | DELAY MOVE.L #551800,D5 |  |
| 13 | DEL1 SUBQ.L #1,D5 |  |
| 14 | BNE DEL1 |  |
| 15 | SUBQ.L #1,D2 |  |
| 16 | BNE DELAY |  |
| 17 | RTS |  |
| 18 |  |  |
| 19 | * TABLE holds bit pattern followed by seconds to display that pattern for LEDs |  |
| 20 | TABLE DC.B $82,15 | Display for 15 seconds, E-W=RED (LED 7) and N-S=GREEN (LED 1) |
| 21 | DC.B $84,4 |  |
| 22 | DC.B $88,2 |  |
| 23 | DC.B $28,15 |  |
| 24 | DC.B $48,4 |  |
| 25 | DC.B $88,2 |  |

- What is the purpose of TABST (lines 0 and 1)?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

- The DELAY subroutine should delay for d2 seconds (i.e. if you load d2 with 5, then call DELAY, it should not return for 5 seconds). Given the constant value in line 12, how many seconds should each iteration of the DEL1 loop (lines 13-14) take? (ignore the time taken by the other lines in DELAY and the JSR/RTS time). Assuming that the P is running at 8MHz, how many cycles is that?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

- In the lab you will need to modify this program to add a walk/don’t walk signal using the extra yellow LEDs (bits 0 and 4) to represent pedestrian crossings (LED on when it is safe to cross, and flash during last 3 seconds of green). Modify the LED pattern/time TABLE shown above to include the pedestrian signal.  Note: It is only safe to walk when the light is green.

Write your new TABLE in the space below. Your new TABLE will be slightly longer than the old TABLE -- what one other line in your program needs to change to accommodate this growth?

One line change:______________________________________________________________________

____________________________________________________________________________________

New Table:

| 19 | * TABLE holds bit pattern followed by seconds to display that pattern for LEDs |  |
| --- | --- | --- |
| 20 | TABLE DC.B |  |
| 21 |  |  |
| 22 |  |  |
| 23 |  |  |
| 24 |  |  |
| 25 |  |  |
| 26 |  |  |
| 27 |  |  |
| 28 |  |  |
| 29 |  |  |
| 30 |  |  |
| 31 |  |  |