#### Modeling a Robot Arm with Two Degrees of Freedom Using DEVS in CD++

#### Fall 2011

#### Assignment1

Saeed Ahmadi 100787872

Carleton University

Part I

This proposed system models the behaviour of a robot in performing a homing stage and then reacts to any command inputs entered by the user. The robot has two arms (joints). In this model user can request the robot to move by entering the amount of movement for each joints in degrees. In 5% of cases, the robot may not achieve the desired point exactly, but still trying to get to destination and does not return from moving state. For this reason there would be a time out timer which would be equal to longest possible movement of the robot in order to force a transition from the moving state. This will increase the idle time of robot. I will simulate this idle time and efficiency of the robot by this model. Figure 1shows a brief sketch of the original proposed of the model structure.

Robot

Motion Controller

Homing

User Input (X)

Homing

Command

Done

Decision

Joint1 Homing (JH1)

Done

Joint2 Homing (JH2)

Motion

(Y)

When we turn on the robot it does nothing. I will present this state with “IDLE” state. The sensors of the robot need to be zeroed (called “homing”) before it start accepting the move commands so the robot will be able to calculate its position from that point. Then it will accept the move commands which are like (MOVE, J1, J2). J1 and J2 are numbers in degrees that joint 1 and joint 2 of the robot should move. These commands will be coded in simulation.

The overall component at the top is the “Robot” which consists of two submodels: “Motion Controller” and “Homing”. Each of these submodels in turn consists of two atomic submodels.  The Motion Controller submodel includes “Decision” atomic submodel and “Motion” atomic submodel. The Decision model will receive the input from the user and will define the state of robot which is S= {INIT, IDLE, MOVING}. This is the set of state variables. When robot is in INIT or MOVING, it is in active state, and when it is in IDLE mode, it is in passive state. As the first command, the robot will accept only the INIT command and will ignore the others. The states of the Motion are chosen from {IDLE, MOVING} (active, passive)

The homing submodel is decomposed into two atomic submodels “Joint1 Homing” and “Joint2 Homing”. When the robot receives the homing command, it will do homing first for joint1 and then for joint2. Both “Joint1 Homing” and “Joint2 Homing” submodels are acting in the same way and can be in one of the {active, passive} states. The “Init” state variable will be used to show that the robot is homed.

The following list is a summary of assumptions and conditions:

- A robotic arm with two degrees of freedom (DOF)

- When the robot is turned on, it is in IDLE mode (passive).

- The sensors need to be zeroed through a process called homing

- The average speed of robot motion is 10 degrees/second

- Two joints can run at the same time so the max time would be the time of the joint with the greater movement. (while in MOVING mode and not for homing)

- User requests can be one of the following:

  - Homing  (command to home the robot)

  - Move j1 j2 (command to move to a new position defined by j1 and j2 joint angles)

  - Stop (command to stop a motion and be ready to receive new commands)

- Robot will ignore all move commands as long as it is not homed.

- As long as it is moving, the robot will ignore new move commands.

- The longest motion takes 36 seconds (moving 360 degrees)

- If the motion takes more than 40 seconds, then it assumes the motion is completed.

Using the model for time efficiency testing

At the end to test the model, I will provide a number of commands to the model, representing the user input and evaluate the correctness of the behaviour in general. Every time that the robot (5% chance), will waste some time till it gets out of the moving state resulting in lower efficiency. The chance of not reaching the goal will be modeled by a random number generator and the overall time efficiency of the robot in operation will be evaluated using this model.

Part II

A user can enters the command from the User_Input input port to move the joints of robot. The command has to be coded for the model. I used the following convention for coding the commands:

- The command consist of 7 digits

- First digit defines the type of command

  - Number 1 indicates the Init command (the next 6 digit are not important but should enter)

  - Number 2 indicates the Move command

  - Number 3 indicates the Stop command (the next 6 digit are not important but should be entered)

- The three digits after the first digit (command) indicate the angle that the first joint should move

- The last three digits indicate the angle that the second joint should move

A command looks like: 2175095, that means move the first joint 175 degrees and the second joint 95 degrees. Since the direction of the movement is not important and just the time of movement is intresting for our simulation,  I didn’t simulate the sign of the degrees.

After user inputs the command, the first digit will be extracted and analyse in Decision atomic model. The Decision model then will decide what to do with command based on the condition discussed in Part I.

There are variables to keep track of the current state of the system. The Current_State variable can take the IDLE, INIT, or MOVING values which are 0, 1 and 2 in order. At first the Current_State is IDLE and it will get the other values based on the entered command. This variable will be check when implementing the conditions in Decision model. The other variables, Move_Command, Init_Command, Stop_Command, and STOPPING are boolean and show what command have been entered. They will be used for checking conditions. The Homed variable is a static boolean variable that when it is set, it means that the robot is initialized. Before it is homed, the robot won’t accept other commands except initializing command. Aslo if it is in the initializing process and other Stop command arrives it should accept the Stop command. This will stop the homing process and cause the Homed variable stay to false. After processing each command, Decision model changes the state to IDLE to accept the new commands because the next command may be a Stop command to stop the current process.

I added some other output ports to have a more readable output file form the original proposal in Part I.  Figure 2 shows the diagram of model with more details. This is a system that has been implemented.

Figure 2: Diagram of Robot Model

In Motion model, a random function will generate a random number between zero to 100. The Random_Number variable hold this value. If this number is less than 5 (5%), then the Motion uses a time out value instead of motion. The time out is 40 seconds, slightly more than the maximum time of robot movement whis is 36 second. This will simulate the idle time and efficiency of the robot. The “total” variable keeps the total number of move command that have been entered. The eCounter variable holds the total number of times that the Random_Number has been less than 5. The efficiency of robot then will be calculated as follow then:

eTime = 100 * (total – eCounter)/total

Here the eTime will be sent out to Efficiency port in Motion model and can be seen on RobotEfficiency port of the robot model each time that the Random_Number is less than 5.

When the Random_Number variable is more than 5, then the command will be processed and the degrees of two joints will be extracted from the coded command as explained before. Since both arms can move at the same time, the bigger degree will be used as the time of movement. The degree1 and degree2 variables keep these values. The bigger number will be assigned as the sigma value. After sigma time expires, the number 2 will be sent out on Done output port to Decision model to tell that the move has done. This number would be 3 for stopping process. For stopping commands the sigma is zero. At the same time, the new position of the robot will be sent out to robot model through J1_Position and J2_Position output ports (not for stopping).

The same variable names as in Decision model, Current_State, IDLE, INIT, and MOVING have been used here as well to show the current state of the model. Other variables were used to hold the temp values.

Joint1 and Joint2 models are very similar. When a homing command arrives at the H_Homing_Command port of the Homing coupled model, it will forward it to the Homing port of the Joint1 model. In Joint1 model, the average time of the movement of the robot for homing process is considered 18 seconds. After this time is consumed, the Done signal (1) will be sent to Joint2, and Joint2 does the homing in the same way. If number 3 arrives at Homing port, Joint1 uses the sigma=0 and send 3 to Joint2 from Done output port. Joint2 also uses sigma=18 for homing and sigma=0 for stopping. Joint2 then sends the value 1 to the Done port to the Decision model after it is done homing, or value 3 if it stopped the process.

Formal Specifications of Model

Based on the formal definition of atomic models:  <S, X, Y, δint, δext, λ, ta> , below are formal description of robot arm atomic models:

Decision Modle:

Variables:

IDLE, INIT, MOVING, Current_State, Init_Command, Move_Command, Stop_Command, STOPPING, Homed

S = {passive, active}

X = {userCommand, Done_Homing, Done_Motion}

Y = {Home_Command, Motion_Command}

δint (active) = Set the command variables to false and passivate().

δext (UserCommand, passive)

{

Extract the first digit of the command and set the command variables,

If (robot is not homed) then

{

If (command is init) then

{

Initialization: do the homing

Current_State=INIT,

}

Else If (command is stopping and Current_State is INIT) then

{

stop the homing,

STOPPIN=true,

}

else ignore the command (passivate)

}

Else If (robot is homed)

{

If (command is move and robot is IDLE) then

{

Send the command to Motion model for moving,

Current_State=MOVE,

}

Else if (command is move but robot is not IDLE) then

Ignore the command (passivate)

Else if (command is stop and robot is moving) then

{

Stop movement (send 3 to Motion)

STOPPING=true,

}

Else if (command is stop and robot is IDLE) then

Ignore the command.

}

}

δext (Done_Motion)

{

If (value of Done_Homing is 1) then

{

Set Homed to true,

Reset all the variables and passivate,

}

Else if (value of Dome_Homing is 3)  then

{

Set Homed to false,

Reset all the variables and passivate,

}

}

δext (Done_Homing)

{

If (value of Done_Motion is 2 or 3) then

Reset all the variables and passivate,

}

λ()  // output function

{

if (Init_Command) then

Send Comand_Digit to  Home_Command output port,

If (Move_Command) then

Send Comand to  Motion_Command output port,

if (Stop_Command) then

{

if (Current_State=INIT and Homed=false) then

Send Comand_Digit to  Home_Command output port,

if (Current_State=MOVING)  then

Send Comand to  Motion_Command output port,

}

}

Motion Model:

Variables:

IDLE, INIT, MOVING, Current_State, Random_Number, eTime, eCounter, total, degree1, degree2 (and other common temporary variables)

S = {passive, active}

X = {command}

Y = {Efficiency, J1_Position, J2_Position, Done}

δint (active) =

{

Set the current state to IDLE

Passivate(),

}

δext (command)

{

Extract the first digit of the command form the input,

If (command is a move command (Command_Digit == 2)) then

{

Random_Number=Generate a random number between 0 to 100,

If (Random_Number <5) then

{

Five_Percent=true,

eCounter = eCounter +1,

eTime = (100 * (total-eCounter))/total;

ta =TimeOUT (40 second),

}

Else if (Random_Number >=5) then

{

degree1= extract three digits after first digit,

degree2= extract the last three digits of the command,

if (degree1>degree2) then

ta= degree1

else   ta=degree2

}

}

If (command is a Stop command (Command_Digit == 3)) then

Stop the movement,

} // end of δext (command)

λ()  // output function

{

if (Command_Digit=2 and Current_State=MOVING) then

{

Send Command_Digit to Done output port,

Send degree1 to J1_Position output port,

Send degree2 to J2_Position output port,

}

if (Command_Digit=3 and Current_State=STOPPED) then

Send Command_Digit to Done output port,

if (Five_Percent  and Current_State=MOVING) then

{

send eTime to Efficiency output port,

send Comand_Digit to Done output port,

}

}

Joint1  and Joint2 Models:

Variables:

Homed, Init, Stopped, Command

S = {passive, active}

X = {Homing}

Y = {Done}

δint (active)

{

If (Robot is not Homed yet (Init=0)) then

{

If (Command == 3) then

if (Homed=false and Stopped= true)  then  // homing process stopped

Stopped=false,

If (Command=1 and Homed=true) then   // robot done the homing

{

Init=1, 	 // will stay one until robot turn off

Homed=true,

}

} else

{

Homed=true,

Stopped=false,

}

passivate(),

}

δext (Homing)

{

If (Robot is not initialized yet (Init=0)) then

{

If (Command =1 and Init=0) then

{

Homed=true,

ta= Homing time (18 seconds),

}

Else if (Command=3 and Homed is true) then

{

Homed=false,

Stopped =true,

ta=0,

}

Else  ta=0,

} else ta=0,

}

λ()  // output function

{

If (Init=1) then

Send Init to Done output port,

Else   send Command to Done output port,  // homing or stopping command to Joint2

}

MotionController Coupled Model

X = {Input_Command, Done};

Y = {OutJ1_Pos, OutJ2_Pos, OutEfficiency, M_Homing_Command};

D = {Decision, Motion};

I(Decision) = {Motion, self};

I(Motion) = {Decision, self};

Z(Motion) = self;

Z(Motion) = Decision;

Z(Decision) = Motion;

Z(Decision) =self;

SELECT:  ({Motion, Decision}) = Motion;

Homing Coupled Model

X = {H_Homing_Command};

Y = {Homed};

D = {Joint1, Joint2};

I(Joint1) = {Joint2};

I(Joint2) =  self;

Z(Joint1) = Joint2;

Z(Joint2) = self;

SELECT:  ({Joint1, Joint2}) = Joint1;

RobotArm Model

X = {User_Input};

Y = {RoboJ1, RobotJ2, RobotEfficiency};

D = {MotionController, Homing};

I(MotionController) = {Homing, self};

I(Homing) = {MotionController};

Z(MotiontionController) = self;

Z(MotiontionController) = Homing;

Z(Homing) = MotiontionController;

SELECT:  ({MotiontionController, Homing}) = MotiontionController;

Testing Strategies

In order to test the robot model, different event files (.ev) will be used with different values. At the first step, the conditions will be tested to make sure that the model can check all the conditions that discussed in Part I. I will test all of atomic and coupled model individually with first simple input to check the algorithm, and then with more complex conditions to see how the model reacts to those conditions. At the second step, the file with more inputs will be used to examine the efficiency of robot. More input will give more accurate results. I have used and event file with 100 inputs for this reason.

Part III

As I mentioned in Part II, I have added more output ports to the Motion atomic model as well as Motion Controller and Robot coupled model to separate the information in the output file for analysis. In order to faster simulation, I have divided the times by 100. For example, instead of 40 seconds, I use 0.4 second and 0.36 instead of 36 seconds and so on.

Test Cases and analysis of the results

The Decision Atomic Model:

I made two event files for the Decision atomic file. The first commands in first event file will initiate the robot (Homing) and then there move a move commands in the next lines. One of the move commands will be stop right after it starts by a Stop command.  In the second even file, the reaction of robot to commands while it is not homed yet will be examined. Here I have discussed some of the input entries and their associated outputs by the model. Below are first entries from the first event file:

00:00:01:00 UserCommand 1034334

00:00:01:36 Done_Homing 1

00:00:02:00 UserCommand 2340165

00:00:02:20 UserCommand 2099056

00:00:02:25 UserCommand 2224143

00:00:02:30 Done_Motion 2

00:00:03:00 UserCommand 2196006

00:00:03:30 Done_Motion 2

00:00:04:00 UserCommand 3167245

00:00:05:00 UserCommand 2356334

00:00:05:10 UserCommand 3124221

00:00:05:20 Done_Motion 3

00:00:06:00 UserCommand 2306244

The Decision model should send the initialization command to the Homing coupled model to Home_Command output port at the time 00:00:01:00 and after receiving the done homing signal form the Done_Homing input port, it should accept the move commands. Below are the correspondence outputs for these entries:

00:00:01:000 home_command 1

00:00:02:000 motion_command 2.34016e+06

00:00:03:000 motion_command 2.19601e+06

00:00:05:000 motion_command 2.35633e+06

00:00:05:010 motion_command 3.12422e+06

00:00:06:000 motion_command 2.30624e+06

As we see, it accepts the motion commands after homing, and sends the motion commands to the Motion_Command output port. The entries that we will discuss are bold in the samples. At the time 00:00:04:00, there is Stop command as the external event, but since the Decision model has already received the Done_Motion for the previous command and is in IDLE state, it will ignore the stop command, and so we don’t see this command at the output. At the time 00:00:05:10 there is another Stop command. This time the model is in active (MOVING) state and we can see in the output that the Stop command has passed to Motion_Command output port.

In the second event file, I entered more over lapped command to check if the Decision model works as desire.

Below are some entries from the second event file:

00:00:00:001 UserCommand 2122095

00:00:00:005 UserCommand 3112055

00:00:00:010 UserCommand 1098324

00:00:00:011 UserCommand 3214002

00:00:00:013 Done_Homing 3

00:00:00:020 UserCommand 2005009

00:00:00:025 UserCommand 3165298

00:00:01:000 UserCommand 1149099

00:00:01:036 Done_Homing 1

00:00:02:000 UserCommand 2045354

00:00:02:010 Done_Motion	2

00:00:02:020 UserCommand 2121044

00:00:02:030 Done_Motion	2

00:00:02:045 UserCommand 2242242

00:00:02:057 Done_Motion	2

In this example, the first two commands should be ignored by the model since it is not homed yet (bold lines).  A homing command should send to Homing model as in the third line, but it should stop later because there is a Stop command after this command and before the model receives the done signal form the Homing model shows the homing is done. Then the next two commands again should be ignored since the homing process by previous command stopped and didn’t complete. Then at the time 00:00:01:00 model receives another INIT command the robot initializes itself this time. It should accept the move commands after this time. The correspondence outputs below confirm this simulation:

00:00:00:010 home_command 1

00:00:00:011 home_command 3

00:00:01:000 home_command 1

00:00:02:000 motion_command 2.04535e+06

00:00:02:020 motion_command 2.12104e+06

00:00:02:045 motion_command 2.24224e+06

00:00:03:000 motion_command 2.14405e+06…

As we see the first two commands at 00:00:00:001 and 00:00:00:005  have been ignored, and the first command is at 00:00:00:010 appears in the output. Also the commands at times 00:00:00:020 and 00:00:00:025 have been ignored and have not been sent to output ports because it is not homed yet.

There are other conditions checks in other part of this event file. Below are the other entries:

…

00:00:03:00 UserCommand 2144046

00:00:03:10 UserCommand 2045086

00:00:03:20 UserCommand 2165003

00:00:03:30 Done_Motion 2

00:00:03:50 UserCommand 2005003

00:00:03:56 Done_Motion 2

…

While is in active state (MOVING here), the model should ignore other move commands (bold). As we see there are two other move commands after the one at 00:00:03:00 that should be ignored since model is still waiting for the first move to be completed. Then it should accept the move command at 00:00:03:50. The lines below are the output for these events:

…

00:00:02:045 motion_command 2.24224e+06

00:00:03:000 motion_command 2.14405e+06

00:00:03:050 motion_command 2.005e+06

….

The output shows that the model has worked as desired. The value of 2005003 has been sent to Motion_Command output port at 00:00:03:50 after the move command at 00:00:03:000 with the value of 2144046 and the other two move commands with the value of 2045086 and 2165003 have been ignored.

The Motion Model:

Similar to Decision model, different test cases were used to test the Motion model. I confirmed that this model works as desire and have discussed some of the results here. Below are some entries for the event file of the test case:

00:00:01:00 Command 2121121

00:00:05:00 Command 2145111

00:00:05:11 Command 3154115

00:00:10:00 Command 2032090

…

The correspondence output for these events are as follow:

00:00:01:121 done 2

00:00:01:121 done 41

00:00:01:121 j1_position 121

00:00:01:121 j2_position 121

00:00:05:011 done 3

00:00:10:090 done 2

00:00:10:090 done 60

00:00:10:090 j1_position 32

00:00:10:090 j2_position 90

…

The extra lines in the output for the port “done” (00:00:01:121 done 41) show the random number and are for the test of Motion model only. As we see in bold the move command at 00:00:05:00 has been stopped by user in the next command. The output shows the result. At the time 00:00:05:011 we only see the done 3 output to Decision model and not the output for position of the joints. The italic/bold lines in the output show the random numbers (41 and 60) that have been generated for these events. As shown in the output and entries, the larger degree will be used to calculate the movement time. The last line of the list shows that it has taken 90 mili second to finish the movement that is the degree for second joint.

I used a longer event file with 100 entry to make sure that I have some random numbers lower than 5 to test the efficiency of robot. Below are the output lines that show the last efficiency number of the robot in the output:

…

00:01:32:260 done 2

00:01:32:260 done 70

00:01:32:260 j1_position 117

00:01:32:260 j2_position 260

00:01:33:360 efficiency 4   // this line shows the random number which is less than 5. This output used

just in individual test of Motion model and it is not part of Robot model

00:01:33:360 efficiency 91

00:01:34:265 done 2

00:01:34:265 done 89

00:01:34:265 j1_position 117

00:01:34:265 j2_position 265

…

The bold line shows the efficiency of the robot.

The Motion Controller Coupled Model

I used simple and long event file to test the Motion Controller coupled model for conditions and for efficiency test. I have tested the conditions at the beginning of the long file as well and discussed them here. Below are the first lines from the event file:

00:00:00:01 Input_Command 2034033

00:00:00:15 Input_Command 2034033

00:00:00:25 Input_Command 3034033

00:00:00:35 Input_Command 1034345

00:00:00:50 Done 1

00:00:01:00 Input_Command 2034033

…

Again the commands before initialization should be ignored (bold lines). Then after homing at 00:00:00:35 it should accept the commands starting at 00:00:01:00.

The following are the output of these events:

00:00:00:035 m_homing_command 1

00:00:01:034 outj1_pos 34

00:00:01:034 outj2_pos 33

…

As we see the first three commands have been ignored. The initial command has been sent to Homing model at 00:00:00:035 and the model accepts the move commands after homing as expected. Also notice that the time for moving is equal to the larger angel that has been entered for moving which is 034 in here. The lines below also show the efficiency calculated by model:

…

00:01:26:273 outj1_pos 273

00:01:26:273 outj2_pos 227

00:01:27:400 outefficiency 91

00:01:28:168 outj1_pos 53

00:01:28:168 outj2_pos 168

…

The bold line shows the last line that the efficiency has been calculated. The time out for entries that have random number lower than 5% is .400 that has appeared in its time. Below are the correspondence entries for the same output:

…

00:00:86:00 Input_Command 2273227

00:00:87:00 Input_Command 2014278

00:00:88:00 Input_Command 2053168

…

And also we can confirm that the movement time is equal to the bigger degree in value (273 and 168 in these events).

Joint1 and Joint2 Models

Because these two models work very similar in sequence, I discuss their test results together. Similar to other atomic model, I tested the conditions with event files. Below show the lines for the event file:

00:00:01:00 Homing 1

00:00:01:01 Homing 3

00:00:03:00 Homing 3

00:00:04:00 Homing 3

00:00:05:00 Homing 1

00:00:06:00 Homing 1

00:00:07:00 Homing 3

00:00:08:00 Homing 1

00:00:09:00 Homing 3

The output of this event file is as follow:

00:00:01:001 done 3

00:00:03:000 done 3

00:00:04:000 done 3

00:00:05:180 done 1

00:00:06:000 done 1

00:00:07:000 done 1

00:00:08:000 done 1

00:00:09:000 done 1

The output shows that the first INIT command has been stopped after the stopped command. Then while the model is not homed yet, it sends back all the stop commands to Joint2 that act similarly and sends the output to Decision telling that the homing has been stopped.

Then at 00:00:05:00 Homing 1, the joint will home itself and send the signal to the Joint2. The time is .18 second. After Homing process, the Init variable in the model get the value of 1 and so model will send this value for all of the requests after this that should never happen from the Decision. This has shown in bold lines.

Homing Coupled Model

I used the very similar event file as for Joint1 and Joint2 and the result s are similar. The only different is the time of homing. Because there are two joints, it takes .360 second for homing process. This has been shown below in the third line:

…

00:00:01:001 homed 3

00:00:03:000 homed 3

00:00:04:360 homed 1

00:00:05:000 homed 1

00:00:06:000 homed 1

00:00:07:000 homed 1

00:00:08:000 homed1

00:00:09:000 homed 1

The RobotArm Model

The combinations of different event files from the testing scenarios of atomic and coupled models were used to test the RobotArm model. The results confirmed that the model works as expected. I tested the model with at least one event file for simple test, one for more complex conditions (as discussed in atomic models such as Decision model) and one for efficiency with more events. The following are the entries from the first event file, RobotArm.ev:

00:00:00:010 User_Input 1214345

00:00:00:040 User_Input 2119209

00:00:03:000 User_Input 2102090

00:00:04:000 User_Input 3174252

00:00:05:000 User_Input 2210120

00:00:06:000 User_Input 2122144

00:00:07:005 User_Input 3111111

00:00:08:000 User_Input 2316001

00:00:09:000 User_Input 2138357

The following are the output file from this event file:

00:00:03:102 robotj1 102

00:00:03:102 robotj2 90

00:00:05:210 robotj1 210

00:00:05:210 robotj2 120

00:00:06:144 robotj1 122

00:00:06:144 robotj2 144

00:00:08:316 robotj1 316

00:00:08:316 robotj2 1

00:00:09:400 robotefficiency 80

As we see after the robot model initialized with the first command, it will accept the move commands. The first move command (bold) ignored since it has arrived before the Homing is done which takes .360 second. Also Stop commands (bold/italic) have been ignored since the model is in Idle mode. The last line of the output shows the efficiency of the model which is low here due to the limited number of the events. The outputs of the robot simulator are not always the same for the same event file since the time out value and efficiency are based on random numbers. Here we have had 5 valid move commands that one of them has a timeout, so efficiency is 80%.

In the second event file, RobotArm_2.ev, more conditions have been tested. Below are the entries of the file:

00:00:00:001 User_Input 2122095

00:00:00:005 User_Input 3112055

00:00:00:010 User_Input 1098324

00:00:00:011 User_Input 3214002

00:00:00:020 User_Input 2005009

00:00:00:025 User_Input 3165298

00:00:01:000 User_Input 1149099

00:00:02:000 User_Input 2045354

00:00:02:020 User_Input 2121044

00:00:02:045 User_Input 2242242

00:00:03:000 User_Input 2144046

00:00:04:000 User_Input 3167252

00:00:05:000 User_Input 2333053

00:00:05:010 User_Input 3124223    // this stops the previous command and so does not appear in output.

00:00:06:000 User_Input 2344214

00:00:07:000 User_Input 3119121

00:00:08:000 User_Input 2347003

00:00:09:000 User_Input 3032314

This is a very similar test case to cases that we saw for other atomic models and the results are similar. The output of the file is as follow:

00:00:02:354 robotj1 45

00:00:02:354 robotj2 354

00:00:03:400 robotefficiency 50

00:00:06:344 robotj1 344

00:00:06:344 robotj2 214

00:00:08:347 robotj1 347

00:00:08:347 robotj2 3

The first initialization has happened at 00:00:01:000 so the first valid move command accepted at 00:00:02:000. The commands before 00:00:01:000 are either ignored or stopped in similar way as discussed before. Also the two bold move commands in the list after the first accepted move command were ignored since the model has not finished the previous movement yet. It needed .354 second to finish the move command arrived at 00:00:02:000. The Stop commands while the model is idle have been ignored (italic/underlined).

Also we see an efficiency output in third line with the timeout of .400 showing that the random number is working fine.

For testing the efficiency, I used the same event file that used for testing Motion model and MotionContrroller coupled model. The output is similar and the last line of the output file that shows the robot efficiency is as below:

…

00:01:34:265 robotj2 265

00:01:35:400 robotefficiency 91

00:01:36:235 robotj1 235

00:01:36:235 robotj2 207

…

The simulator was tested against all possible conditions and responded as expected. This model can be expanded for the robot arms with several joints (more than two).