Simulating Hydraulic Fracturing Using Cell-DEVS

|  | Christopher Burt christopherdburt@gmail.com |  |
| --- | --- | --- |

ABSTRACT

Permission to make digital or hard copies of all or part of this work for personal or classroom use is granted without fee provided that copies are not made or distributed for profit or commercial advantage and that copies

bear this notice and the full citation on the first page. To copy otherwise, or republish, to post on servers or to redistribute to lists, requires prior specific permission and/or a fee.

This project has produced a rudimentary simulation of a hydraulic fracturing event, based on work in progress on a paper that presents a simple formula to consider the dynamics of such an event. The specification of the solution is presented. The reasoning and development of the solution is elaborated and discussed. Although the completed simulation fulfills the basic vision of the work upon which it is based, there are remaining aspirations for the hydrofracking simulation. These are outlined, and a possible solution is discussed.

## Author Keywords

Cellular automata; hydraulic fracturing, hydrofracking; simulation.

## ACM Classification Keywords

I.6.8 Simulation and Modelling; Discrete Event.

# INTRODUCTION

The goal of this project is a model and simulation to implement the formula for considering underground stresses put forth by Sergio Zlotnik in the unpublished paper known (in its current draft) as “Simulation of artificial hydraulic fracturing of rocks in boreholes using a cell-devs approach”. That paper serves as the primary source material for this project, and is referred to using this term throughout this paper.

# Problem Description

The domain of the model (as outlined in the source material) is a 2-dimensional cross section of an underground hydrofracking event, located immediately next to the well bore wall. Some simplifying assumption are made, such as ignoring the pressure exerted by the well bore wall itself, accounting mainly for the fluid pressure introduced to the region at an injection point referred hereafter as the ‘well spout’.

Fig. 1 The simulation domain illustration, 
taken from the source material.

# Implementation

The simulation is implemented in basic Cell-DEVS using CD++.

## Conceptual Outline

The local state of the cell represents the hydraulic pressure on the region of the space. The value is interpreted by the rules to determine the type of cell.

- A value of zero means that the cell is “rock”. It is occupied by solid material that is not yet fractured, but can be affected by pressure exerted by fractured neighbors.

- A value greater than zero means that the cell is “fracture”. The rock material has broken and admits hydraulic pressure.

There is one special cell called the ‘well spout’ which behaves like a fracture, but continuously introduces hydraulic pressure into the system. Hydraulic pressure tends to dissipate across adjoining fracture cells. In this way, fractures transmit hydraulic pressure from the well spout throughout the space.

Rock cells have a threshold at which they can break and become fracture cells. This threshold is called the maximum supported pressure. Calculating this value is outlined in detail in the source material. An abbreviated specification will be given later in this paper. The formula accounts for depth under the earth’s surface, which in the cell space equates to the vertical position of the cell within the space.

As the system progresses, the maximum supported pressure of a rock cell is multiplied by its resistance factor. The factor increases with the number of neighboring fractures. This represents the rock getting stronger as it is compressed by the fractures opening nearby.

## Formal Definition

The simulation uses one atomic Cell-DEVS model defined as follows:

CD = <X, Y , I ,S, θ, N, D , δint , δext , τ, λ, D >

A zone “wellspout” is defined.

X = {}

Y = {}

I = <η, μ, Px, Py>

η = 9

μ = 0

Px = {all in neighborhood}

Py = {all in neighborhood}

Θ = {

0 : rock

Greater than 0 : fracture with this value as hydraulic pressure in Pascals

}

N = {O}

D = inertial delay 100ms

δext = {O}

δint : described as pseudocode as follows:

Fracture rules:

If this cell’s value is greater than zero, become a value that is the sum of all neighboring cells including the self, divided by (9 minus ZeroUndefCount).

ZeroUndefCount is the sum of the results of a series of if statements that check if each neighbor is zero or undefined. These statements return 1 if true, zero if false).

Rock rules:

For each neighbor, if this cell’s value is zero AND the neighbor’s value exceeds total maximum supported pressure, become a value sigma, ie. a very small positive number.

The total maximum supported pressure is the result of the base max pressure multiplied by the resistance factor.

The base max pressure is the result of the formula outlined in the source material. Having replaced all variables with their constant (for our purposes) values, it proceeds as follows:

2 * ( [Vertical cell coordinate] / 1000 + 3000 ) * 9.8 * 2200 - ( [Vertical cell coordinate] / 1000 + 3000 ) * 10500 + 15000000

The resistance factor is the result of the following formula:

[Resistance multiplier minimum] + [resistance multiplier range span] * ([percentage of fractured non-self neighbors])

This formula proceeds as follows:

1 + 10 * (8 - ([number of cells which are zero or undefined])) / 8

The number of cells which are zero or undefined is counted with a series of checks that return 1 or zero, similar to the fracture rules.

In any other case, the cell keeps its current value.

Zone “wellspout” rules:

In all cases, the cell designated as wellspout follows the fracture rules, and then adds a constant value.

## Results

The simulation begins with the entire cell space at the default value of zero. The wellspout cell introduces pressure, itself become a fracture as a result.

It continues to do so, and pressure builds. Eventually, adjacent rock cells detect that their pressure threshold is exceeded, and they become fractures as well.

Rock cells continuously recalculate their own thresholds.  Because of the resistance factor, as more fractures emerge, rocks with more adjacent fractures become harder to break.

But eventually some do, forming an intricate pattern as the survivors become more resistant.

# Development

By collaborating with and getting feedback from the domain expert, the simulation was developed over several interations.

In addition to the interations outlined in this section, the collaboration resulted in aspirations for the simulation that remain unfulfilled. These remaining ideas came late in the project and exceeded the time frame. They are outlined and discussed later in this paper, in the section on going further.

## Version 1 (Basic)

The initial version of the simulation was purposed to visualize the basic reasoning of the model. It did not use the source material’s complete formula, but rather some practical stand-in values to test the core procedure. As pseudocode, the cells followed the following rules:

If fracture (greater than zero) become the averaged value of neighbors.

If rock, for each neighbor, check if the neighbor value exceeds a constant threshold (10). If so, become sigma (1).

For wellspout, follow the fracture rule in all cases, and introduce new pressure by adding a constant value to each new state (value of 2).

Version 1 can be tested with the accompanying file “hf-v1.ma”. The results are visible using the log file “hf-v1.log” and the palette file “hf-2.pal”. It is viewable at http://youtu.be/XeGtBbpGmCo

## Version 2 (Pressure Formula)

The second version of the simulation was purposed to introduce the complete formula from the source material. Essentially, this modified the threshold values of the rock cells. Accordingly, to continue producing visible, confirmable results, the wellspout rate was also vastly increased.

Version 2 can be tested with the accompanying file “hf-v2.ma”. The results are visible using the log file “hf-v2.log” and the palette file “hf-1.pal”. It is viewable at http://youtu.be/KkFBe21a_mI

## Version 3 (Current)

Past versions saw a mostly-round pattern emerge as the simulation progressed. Version 3 was purposed to approach a non-round pattern, with more of a cracking pattern. In real life, a fracture emerges in a somewhat linear way, progressing through material as if travelling along a course. The source material’s formula for supported pressure is specifically meant for the “tip” of such a fracture. To account for this, the resistance factor was introduced.

As the simulation progress, the intricate pattern emerges and grows. This pattern is particularly the result of the resistance factor. It represents a desire for fractures to develop in a somewhat linear way (instead of just growing outward in a round pattern).

Version 3 can be tested with the accompanying files “hf-v3.ma”. The results are visible using the log file “hf-v3.log” and the palette file “hf-1.pal”. It is viewable at http://youtu.be/-c5FO8JjwfY

# Going Further

Several aspirations for the simulation remain which could not be accomplished due to time constraints at the juncture at which they emerged. The ideas are outlined as follows:

- When a fracture opens, the solid walls around it are compressed, making the walls of the fracture stronger. This was approximated in Version 3 using the resistance factor.

- The wider the fracture, the stronger the wall becomes. This has not been approximated in existing versions; the resistance factor accounts only for immediate neighbors. In the short term, further developments could experiment with expanding the cell neighborhood to enlarge the effect of the resistance factor, but this does not truly approximate the idea as described.

- The “tip” of the fracture (the leading point of the linear pattern of expansion), does not see its resistance effected by the expansion of the whole fracture. That is to say, it behaves normally, without any augmenting resistance factor. This would encourage a more expected pattern of linear growth.

## Possible Solution

Speculating on a solution, it seems this can be accomplished using several variables per cell.

- A flag to indicate whether this is a “new” fracture that was rock in the previous timestep.

- A code to stand for the direction from which this fracture was fractured. In other words, an index of which neighbor was detected as having sufficient pressure to break this cell when it was rock.

- The resistance multiplier, stored for persistence and readability by other cells, instead of just being recalculated at each step.

When a rock looks at a neighbor, and that neighbor's direction is aligned with this current rock, then this rock is a 'tip'. It will use no augmenting resistance factor when calculating its supported pressure. Rocks that are not the 'tip' have a multiplier of greater than one. They are thus more resistant to break. This will encourage the linear growth.

If a rock has a “new” fracture neighbor, and this rock is not the tip, this rock reads the resistance multiplier of the new fracture neighbor (ie. how strong that rock was before it fractured), and increments its own resistance multiplier to be even greater. In this way, the walls of the whole fractured region will grow stronger as they are “pushed out”.

## Exploring Implementation of this Solution

There are two ways of encoding more variables with basic Cell-DEVS (which uses only one state value per cell).

The first way is to encode the other values in higher digits, and then extract them uses the modulus operator. For example, in the existing simulations, hydraulic pressure goes into the hundreds of millions. If the "new" flag is stored by adding it to the cell value in the billions, the pressure of a cell can still be read by reading (cell value MODULUS 1 billion).

The other way of storing more information for a cell is to add another layer of cells. For example, The dimensions of the space would be 30x30x4 instead of just 30x30. The additional layers are where the other variables can be stored.

In order to utilize either of these methods, we must avoid using the CD++ functions stateCount() and undefCount. These are built-in functions that are useful for quickly evaluating neighbor states, which is necessary for the fracture rules for dissipating pressure by averaging the value of neighbors. However, if more information is encoded into cell values (the first solution above), or if there are more neighbors than cells which are relevant to the rule (the second solution above), these function become problematic. Both functions can, and in the existing simulations have, been replaced by a series of calls to the if() function.

For instance, the number of non-self neighbors that are either zero or undefined can be found as follows:

(if((-1,-1, 0) = 0 OR (-1,-1, 0) = ?, 1, 0) + if(( 0,-1, 0) = 0 OR ( 0,-1, 0) = ?, 1, 0) + if(( 1,-1, 0) = 0 OR ( 1,-1, 0) = ?, 1, 0) + if((-1, 0, 0) = 0 OR (-1, 0, 0) = ?, 1, 0) + if(( 1, 0, 0) = 0 OR ( 1, 0, 0) = ?, 1, 0) + if((-1, 1, 0) = 0 OR (-1, 1, 0) = ?, 1, 0) + if(( 0, 1, 0) = 0 OR ( 0, 1, 0) = ?, 1, 0) + if(( 1, 1, 0) = 0 OR ( 1, 1, 0) = ?, 1, 0))

This avoids issues because the value to be evaluated can be explicity controlled (eg. by performing modulus operations on the value of each neighbor), and because the exact neighbors to be evaluated can be controlled (instead of simply all of them).

There is an additional solution that is not familiar to me, but is worth looking into: there is an advanced version of CD++ Cell-DEVS that allows more than one state variable per cell. Considering the requirement for more variables that can be independently managed, this might serve to create the cleanest, most maintainable solution. I recommend exploring this option.

## Realistic Values

In the course of this project, several values were noted as reasonable to approximate real events. Some of these values were used in the existing simulations, while others were not used because they did not contribute to development or realism at the current level of sophistication. They are compiled here for reference.

- The input rate from the well spout has two estimates for reasonable values:

  - Fast: 4.16e+05 Pa/sec (or 41600 Pa at each time step of 100ms)

  - Slow: 1.39e+04 Pa/sec (or 1390 Pa at each time step of 100ms)

- At these rates, it will take a long virtual time for the scenario to develop. A scale of several hours can be assumed. The timestep, currently 100ms, could probably be much longer, if only for the sake of development expediency.

- The scale of each cell is currently one meter. Meters are the units divised in the source material’s formula, however, the formula was developed to work on a scale of millimeters.

# Conclusion

This project has produced a rudimentary simulation of a hydraulic fracturing event, based on work in progress on a paper that presents a simple formula to consider the dynamics of such an event. The specification of the solution was presented. The reasoning and development of the solution was elaborated and discussed. Although the completed simulation fulfills the basic vision of the work upon which it is based, there are remaining aspirations for the hydrofracking simulation. These were outlined, and a possible solution was discussed.

# ACKNOWLEDGMENTS

This project was accomplished in collaboration with, and based on work by, Sergio Zlotnik. I was privileged to work with Meagan Leflar, who focused on a separate visualization component of the project. I thank Gabriel Wainer for his guidance.