SYSC 5104. METODOLOGIES FOR DISCRETE EVENT MODELLING AND SIMULATION

Assignment 1

Conceptual Model Description

Red-light Running Detection System

Submitted by: Natalia Gorbenko

Student ID: 100961984

# Introduction

Driving through an intersection after the light has turned red is probably one of the most dangerous traffic violation. According to the Insurance Institute for Highway Safety, every year, these accidents kill about 800 people and rack up an estimated $7 billion dollars in property damage, medical bills, lost productivity and insurance hikes[1].

Collisions resulting from red-light running tend to be more severe than other intersection collisions because they usually involve at least one vehicle travelling very quickly. In the most red-light running collisions, the vehicles hit each other at right angles causing severe injuries and often leading to death.

City of Ottawa Road Safety estimated frequency of drivers run red lights. The data represented in Table 1 was recorded during a five-hour period and applies to only one direction [2]:

Table 1-1 Red-light running violation statistic

| Intersection | Number of violations |
| --- | --- |
| Albert Street & Kent Streets (northbound) | 37 |
| Carling Avenue & Richmond Road (westbound) | 29 |
| Coventry/Ogilvie Road & St. Laurent Boulevard (northbound) | 78 |
| Heron Road & Riverside Drive (eastbound | 55 |

Public awareness of red-light cameras improves driving behavior. Studies have shown that red-light violation rates can be decreased by 42 percent within a few months of camera installation [2].

In the scope of this project the model of Red-light Running Detection System for one road intersection will be build using DEVS formalism. In order to conduct deep investigation of the model behavior, discrete-event simulation will be executed by using CD++ simulation tool.

# Red-light Running Detection System Description

Red-light Running Detection System consists of three types of elements: electromagnetic detectors, embedded in the pavement just before the limit line, cameras and controller. Electromagnetic detectors, triggered by speeding vehicle, send signal to the controller that (if the traffic light is red) activates Camera which catches red-light runner. By using the speed measurements, the system predicts if a particular vehicle will not be able to stop before entering the intersection, and activates camera, which takes photographs of the event. The first photo shows the rear view of the vehicle just before it enters the intersection, with the light showing red, and the second photo, taken a second or two later, shows the vehicle when it is in the intersection. Intersection equipped by the Red Light Running Detection System is illustrated on the Figure 2-1.

Figure 2-1 Intersection equipped by the Red-Light Running Detection System

Next details may be recorded by the camera system: the vehicle license plate, the date and time, the location, the vehicle speed, and the amount of time elapsed since the light turned red and the vehicle passed into the intersection. The events can be captured as a series of photographs or a video clip, depending on the technology used. Video evidence shows the vehicle before it enters the intersection on a red light signal and its progress through the intersection. Then, the data and images are sent wirelessly to the relevant law enforcement agency. The information is typically reviewed by police department clerk, who determines if a violation occurred and, if so, approves issuing a ticket to the vehicle owner. [4]

# Conceptual Model Description

The structure of the model is introduced on the Figure 3-1.

Figure 3-2 Structured model of the Red-Light Running Detection System

The top-level model is a coupled model that is composed of six basic components: Traffic Lights model (TL), four identical Red Light Runner Detection (RLRD) models (one for each direction) and Counter. Traffic Lights (TL) model is a coupled model, which consist of two connected identical atomic models that create “RED On” signals and send them through two output ports to all RLRDs. Four RLRDs represent four directions of intersection (NORTH, SOUTH, EAST, and WEST) respectively. All RLRDs have a same structure which consists of two main components: an atomic Electromagnetic Detector (ED) and coupled Camera System (CS). ED is in charge of detection of speeding vehicle through the input port (In_1 to In_4) and sending activating signal to the CS. CS model consists of two simple atomic models – Camera Controller (CC) and Camera (C). CC is responsible for detection input signals “RED On” and “ED activated” and producing “Camera On” output signal when both input signals are detected. C activated by input signal from CC, captures video evidence and then sends it to Data Center (Outputs 2 to 5). “Violation Detected” signal produced by C is send to the Counter. Counter is an atomic model, responsible for counting number of violations from all RLRDs and computes statistics, which is also sent to Data Center through Output 1.

# The DEVS Formalism

According to Discrete-event modeling and simulation, G.A. Wainer [4] each real system can be described by using DEVS as a bundle of atomic and coupled models. The next coupled model definition suggested in [4] is used for description of Red Light Detection System:

CM = <X, Y, D, {MdD}, EIC, EOC, IC, select>

where

X = {(p,v)p IPorts, v Xp} is the set of input events, where IPorts represents the set of input ports and Xp represents the set of values for the input ports;

Y = {(p,v)|p OPorts, v Yp} is the set of output events, where OPorts represents the set of input ports and Yp represents the set of values for the output ports;

D is the set of the component names and for each d D;

Md is a DEVS basic (i.e., atomic or coupled) model;

EIC is the set of external input couplings, EIC  { ( (Self, inSelf), (j, inj) )inSelf IPorts, j D, inj Iportsj};

EOC is the set of external output couplings, EOC  { ((i,outi), (Self,outSelf))outSelf OPorts, i D, out i OPortsi};

IC is the set of internal couplings, IC 􀂍 { ((i,outi), (j,inj) )i,j D, outi OPortsi , inj IPortsj };

And select is the tiebreaker function, where select  D D, such that, for any nonempty subset E, select (E) E. [4]

## Top Model – Red light Detection System (RDS)

The top-level model RDS is a structured model that is composed of six basic components: Traffic lights atomic model, four CamWithDetector coupling models and another atomic model – counter. Each of CamWithDetector models consists of  two main atomic components: a Electromagnetic detector that detects speeding vehicle and sends activated signal to Camera; and Camera system, which makes shot of speeding vehicle when it has two input signals – red and EMD. When a new violation detected, CamWithDetector sends signal to counter and sends data to Data Center for analysis.

As we can see structure of top level model has been redefined in comparison with conceptual model. The Traffic lights coupled model have been simplified into one atomic model as well as Camera system coupled model. New model definition simplify and clarify model behavior.

MRDS = <X, Y, D, {Md D}, EIC, EOC, IC, select>

X={svN, svS, svW, svE};

Y={countOut, dataN, dataS, dataW, dataE};

D={trafficlights, CamWithDetectorN, CamWithDetectorS, CamWithDetectorW, CamWithDetectorE, counter};

Md={MTRAFFICLIGHTS, MCamWithDetectorN, MCamWithDetectorS, MCamWithDetectorW, MCamWithDetectorE, Mcounter}, where

Mtrafficlights – atomic model for Traffic Lights;

MCamWithDetectorN…. MCamWithDetectorE, - coupled models for Camera with Electromagnetic Detector System for North, South, West and East directions respectively;

MCamWithDetectorE – atomic model for counter.

EIC={((self, svN), (CamWith DetectgorN, sv));

((self, svS), (CamWith DetectgorS, sv));

((self, svW), (CamWith DetectgorW, sv));

((self, svE), (CamWith DetectgorE, sv));}

EOC={ ((counter, countOut), (self, countOut));

((CamWithDetectorN, data),(self, dataN));

((CamWithDetectorS, data),(self, dataS));

((CamWithDetectorW, data),(self, dataW));

((CamWithDetectorE, data),(self, dataE))};

IC={ ((trafficlights, redNS) (CamWithDetectorN, red));

((trafficlights, redNS) (CamWithDetectorS, red));

((trafficlights, redEW) (CamWithDetectorW, red));

((trafficlights, redEW) (CamWithDetectorE, red));

((CamWithDetectorN, counter), (counter, cam1));

((CamWithDetectorS, counter), (counter, cam2));

((CamWithDetectorW, counter), (counter, cam3));

((CamWithDetectorE, counter), (counter, cam4));}

Select=<trafficlights, CamWithDetector, counter>

## Testing proposal.

In order to complete proper testing of this model next sequence of even is proposed. Vehicles are heading from different directions during the different phases of Traffic Lights. Simultaneous violation of two vehicles heading in different directions (e.g North - South) should be detected as two different violations. Vehicles speeding on Green and Yellow phases of Traffic Lights shouldn’t be detected. Workable event sequence is presented in Table 4.1.

Table 4-2 Test strategy for RDS Top level Model

| Time | Phases of Traffic Lights |  | Direction of Speeding Vehicle | Detected YES/NO |
| --- | --- | --- | --- | --- |
|  | EW | NS |  |  |
| 00:00:01:200 | Yellow | Red | svE | No |
| 00:00:01:100 | Yellow | Red | svS | Yes |
| 00:00:02:100 | Yellow | Red | svW | No |
| 00:00:02:200 | Yellow | Red | svE | No |
| 00:00:03:200 | Red | Green | svE | Yes |
| 00:00:03:300 | Red | Green | svS | No |
| 00:00:03:700 | Red | Green | svN | No |
| 00:00:04:200 | Red | Green | svW | Yes |
| 00:00:10:300 | Red | Yellow | svN | No |
| 00:00:13:200 | Green | Red | svE | No |
| 00:00:14:200 | Green | Red | svE | No |
| 00:00:14:600 | Green | Red | svS | Yes |
| 00:00:15:200 | Green | Red | svE | No |
| 00:00:15:600 | Green | Red | svS | Yes |
| 00:00:16:200 | Green | Red | svE | No |
| 00:10:10:300 | Red | Yellow | svN | No |
| 00:10:13:200 | Green | Red | svE | No |
| 00:10:14:200 | Green | Red | svE | No |
| 00:10:14:600 | Green | Red | svS | Yes |
| 00:10:15:200 | Green | Red | svE | No |
| 00:10:15:600 | Green | Red | svS | Yes |
| 00:10:16:200 | Green | Red | svE | No |

Sequence presented below should generate next Counter outputs:

First 10 min counter should show 5 violations.

Next 10 min counter should show 2 violations.

## CamWithDetector Coupling Model

The CamWithDetector is a coupled model that is consisted of two basic components: Electromagnetic Detector atomic model and Camera System atomic model. Electromagnetic detector detects speeding vehicle and sends activated signal to Camera; and Camera system, which makes shot of speeding vehicle when it has two input signals – RED from Traffic Lights and EMD from Electromagnetic Detector. When a new violation detected, CamWithDetector sends signal to counter and sends data to Data Center for analysis.

Figure 4-3

MRDS = <X, Y, D, {Md D}, EIC, EOC, IC, select>

X={red, sv};

Y={counter, data,};

D={detector, camsystem};

Md={Mdetector, Mcamsystem, }, where

Mdetector – atomic model for Electromagnetic detector;

Mcamsystem, - atomic model for Camera System;

EIC={((self, red), (camsysrem, red));

((self, sv), (EMD, sv))};

EOC={ ((camsysrem, counter), (self, counter));

((camsysrem, data), (self, data))};

IC={ (detector, EMD) (camsystem, EMD)};

Select=<detector,camsystem>

    - Testing proposal.

In order to complete proper testing of this model next sequence of red signal and speeding vehicle events is proposed. Workable event sequence is presented in Table 4.2.

Table 4-3 Testing strategy for CamWithDetector Model

| Time | RED signal | Speeding vehicle | Detection Yes/No |
| --- | --- | --- | --- |
| 00:00:00:000 | 0 | 0 | No |
| 00:00:00:001 | 0 | 1 | No |
| 00:00:03:000 | 1 | 0 | No |
| 00:00:03:100 | 1 | 1 | Yes |
| 00:00:04:100 | 1 | 1 | Yes |
| 00:00:07:999 | 1 | 1 | Yes |
| 00:00:08:000 | 0 | 0 | No |
| 00:00:08:300 | 0 | 1 | No |

According to this sequence 3 violations should be detected.

## Traffic Lights Atomic Model

Traffic Lights Atomic Model is an atomic model which represents two traffic lights working together on the intersection. Combination of TL cycles and signal phase duration represented in the Table 4.3.

Table 4-4 Traffic Lights Cycles

| State number | Direction |  | Time, sec | Outputs |  |
| --- | --- | --- | --- | --- | --- |
|  | EW | NS |  | redEW | redNS |
| 1 | Green | Red | 7 | 0 | 1 |
| 2 | Yellow | Red | 3 | 0 | 1 |
| 3 | Red | Green | 7 | 1 | 0 |
| 4 | Red | Yellow | 3 | 1 | 0 |

As we can see from the table duration of the Green signal is 7 sec, duration of the Yellow signal – 3 sec. Duration of the Red signal equal to the sum of Green and Red, 10 sec.

This model does not have any external inputs and changes its states due to internal transition function. Output function executes before internal transition function and sends outputs to the output ports according Table 4.3

Trafficlights=<X, S, Y, >

X=n.a;

Y={redEW, redNS,};

S={ EW_GREEN_NS_RED_STATE, EW_YELLOW_NS_RED_STATE, EW_RED_NS_GREEN_STATE, EW_RED_NS_YELLOW_STATE };

case EW_GREEN_NS_RED_STATE:

this->holdIn(active, greenTime);

break;

case EW_YELLOW_NS_RED_STATE:

this->holdIn(active, yellowTime);

break;

case EW_RED_NS_GREEN_STATE:

this->holdIn(active, greenTime);

break;

case EW_RED_NS_YELLOW_STATE:

this->holdIn(active, yellowTime);

if (curLightState == EW_RED_NS_YELLOW_STATE)

{

curLightState = EW_GREEN_NS_RED_STATE;

}

else

{

curLightState++;

}

switch(curLightState)

{

case EW_GREEN_NS_RED_STATE:

sendOutput( msg.time(), redEW, 0 );

sendOutput( msg.time(), redNS, 1 );

break;

case EW_YELLOW_NS_RED_STATE:

sendOutput( msg.time(), redEW, 0 );

sendOutput( msg.time(), redNS, 1 );

break;

case EW_RED_NS_GREEN_STATE:

sendOutput( msg.time(), redEW, 1 );

sendOutput( msg.time(), redNS, 0 );

break;

case EW_RED_NS_YELLOW_STATE:

sendOutput( msg.time(), redEW, 1 );

sendOutput( msg.time(), redNS, 0 );

0" << "\n";

break;

}

ta (GreenTime) = 7 sec.

ta (YellowTime) = 3 sec

    - Testing proposal.

This model testing technique consists of running simulation for sufficient period of time and chequing state changes and outputs values according Table 4-4.

Table 4-5 Testing proposal for TrafficLights

| State number | Direction |  | Outputs |  |
| --- | --- | --- | --- | --- |
|  | EW | NS | redEW | redNS |
| 00:00:00:000 | Yellow | Red | 0 | 1 |
| 00:00:03:000 | Red | Green | 1 | 0 |
| 00:00:10:000 | Red | Yellow | 1 | 0 |
| 00:00:13:000 | Green | Red | 0 | 1 |
| 00:00:20:000 | Yellow | Red | 0 | 1 |
| 00:00:23:000 | Red | Green | 1 | 0 |

## Counter Atomic Model

Purpose of this model is to count all violations detected on all for directions of intersections. Since two simultaneous violation from different directions (e.g. from North and from South) could happen, Counter has input port for each of four direction. Each 10 min interval Counter sends amount of violations to the Data Center.

Trafficlights=<X, S, Y, >

X={cam1, cam2, cam3, cam4};

Y={countOut};

S={count, send};

{

if( msg.port() == cam1 OR		// then signal on input ports detected

msg.port() == cam2 OR

msg.port() == cam3 OR

msg.port() == cam4 )

{

counter++; 		//increment counter

}

}

{

counter=0;		//clear counter

this->holdIn(active, Time(0,10,0,0));		//keep state count 10 min.

}

{

sendOutput( msg.time(), countOut, counter )		//send value of counter for //previous 10 min

}

ta(preparation time) = 10ms.

Testing of this model will be implemented by applying sequence of external events (also simultaneously on different ports) and checking the value of Counter.

## Atomic Model of Electromagnetic Detector

Purpose of this model is to generate output for Camera System when signal of Speeding Vehicle is detected on input port. Detector preparation time 10 ms.

Detector=<X, S, Y, >

X={sv };

Y={emd};

S={active, passive}, preparationTime 10 ms.;

{

if( msg.port() == sv )

{

holdIn( active, preparationTime );

}

}

passivate();

{

sendOutput( msg.time(), emd, 1 ) ;

}

ta(preparation time) = 10ms.

Testing of this model will be conducted by applying sequence of external events on input port and checking the output.

## Atomic Model of Camera System

Purpose of this model is to generate outputs for Counter and Data Center when signal of emd and red are detected on input ports. Detector preparation time 10 ms.

CamSystem=<X, S, Y, >

X={red, emd };

Y={counter, data};

S={active, passive}, preparationTime 10 ms.;

if( msg.port() == red )

{

isRed = msg.value();

passivate();

}

else if (msg.port() == emd)

{

if (isRed == 1)

{

holdIn(active, preparationTime);

}

else

{

passivate();

}

passivate();

{

sendOutput( msg.time(), counter, 1 ) ;

sendOutput( msg.time(), data, 1 ) ;

}

ta(preparation time) = 10ms.

Testing of this model will be conducted by applying sequence of external events on input ports and checking the outputs according to sequence in Table 4-5.

Table 4-6 Testing sequence for CamSys Atomic Model

| Time | Input Red | Input emd | Output Counter | Output Data |
| --- | --- | --- | --- | --- |
| 00:00:00:001 emd 1 | 0 | 1 | 0 | 0 |
| 00:00:00:100 red 1 | 1 | 0 | 0 | 0 |
| 00:00:00:200 emd 1 | 1 | 1 | 1 | 1 |
| 00:00:03:100 red 0 | 0 | 0 | 0 | 0 |
| 00:00:03:300 emd 1 | 0 | 1 | 0 | 0 |

# Simulation and verification

## Top Model – Red light Detection System (RDS) Simulation

Table 5-7 Test strategy for RDS Top level Model

| Time | Phases of Traffic Lights |  | Direction of Speeding Vehicle | Detected YES/NO |
| --- | --- | --- | --- | --- |
|  | EW | NS |  |  |
| 00:00:01:200 | Yellow | Red | svE | No |
| 00:00:01:100 | Yellow | Red | svS | Yes |
| 00:00:02:100 | Yellow | Red | svW | No |
| 00:00:02:200 | Yellow | Red | svE | No |
| 00:00:03:200 | Red | Green | svE | Yes |
| 00:00:03:300 | Red | Green | svS | No |
| 00:00:03:700 | Red | Green | svN | No |
| 00:00:04:200 | Red | Green | svW | Yes |
| 00:00:10:300 | Red | Yellow | svN | No |
| 00:00:13:200 | Green | Red | svE | No |
| 00:00:14:200 | Green | Red | svE | No |
| 00:00:14:600 | Green | Red | svS | Yes |
| 00:00:15:200 | Green | Red | svE | No |
| 00:00:15:600 | Green | Red | svS | Yes |
| 00:00:16:200 | Green | Red | svE | No |
| 00:10:10:300 | Red | Yellow | svN | No |
| 00:10:13:200 | Green | Red | svE | No |
| 00:10:14:200 | Green | Red | svE | No |
| 00:10:14:600 | Green | Red | svS | Yes |
| 00:10:15:200 | Green | Red | svE | No |
| 00:10:15:600 | Green | Red | svS | Yes |
| 00:10:16:200 | Green | Red | svE | No |

Counter outputs:

Counter internalFunction

Timestamp: 00:10:00:000

.

00:10:00:000 countout 5

Counter internalFunction

Timestamp: 00:20:00:000

.

00:20:00:000 countout 2

We got results exactly as expected.

## CamWithDetector Coupling Model Simulation

Table 5-8 CamWithDetector Model Simulation

| Time | RED signal | Speeding vehicle | Detection Yes/No |
| --- | --- | --- | --- |
| 00:00:00:000 | 0 | 0 | No |
| 00:00:00:001 | 0 | 1 | No |
| 00:00:03:000 | 1 | 0 | No |
| 00:00:03:100 | 1 | 1 | Yes |
| 00:00:04:100 | 1 | 1 | Yes |
| 00:00:07:999 | 1 | 1 | Yes |
| 00:00:08:000 | 0 | 0 | No |
| 00:00:08:300 | 0 | 1 | No |

CamSystem internalFunction

Timestamp: 00:00:03:120

.

00:00:03:120 counter 1

00:00:03:120 data 1

00:00:04:120 counter 1

00:00:04:120 data 1

etector externalFunction

Timestamp: 00:00:07:999

Port: sv

.

.

CamSystem externalFunction

Timestamp: 00:00:08:000

Port: red

.

According to this sequence 3 violations should be detected. But after simulation only two of them have been detected. Since third violation happened on 00:00:07:999 and on 00:00:08:000 signal red changed state to 0 detection of violation didn’t happen. Because of preparation time 10ms which represents system delays, model was not able to detect this violation.

## Traffic Lights Atomic Model Simulation

Table 5-9 Testing proposal for TrafficLights

| State number | Direction |  | Outputs |  |
| --- | --- | --- | --- | --- |
|  | EW | NS | redEW | redNS |
| 00:00:00:000 | Yellow | Red | 0 | 1 |
| 00:00:03:000 | Red | Green | 1 | 0 |
| 00:00:10:000 | Red | Yellow | 1 | 0 |
| 00:00:13:000 | Green | Red | 0 | 1 |
| 00:00:20:000 | Yellow | Red | 0 | 1 |
| 00:00:23:000 | Red | Green | 1 | 0 |

TrafficLights outputFunction

Timestamp: 00:00:10:000

curLightState: 3

EW_RED_NS_YELLOW_STATE, redEW: 1, redNS 0

.

.

TrafficLights internalFunction

Timestamp: 00:00:10:000

00:00:10:000 redew 1

00:00:10:000 redns 0

.

TrafficLights outputFunction

Timestamp: 00:00:13:000

curLightState: 0

EW_GREEN_NS_RED_STATE, redEW: 0, redNS 1

.

.

TrafficLights internalFunction

Timestamp: 00:00:13:000

00:00:13:000 redew 0

00:00:13:000 redns 1

.

TrafficLights outputFunction

Timestamp: 00:00:20:000

curLightState: 1

EW_YELLOW_NS_RED_STATE, redEW: 0, redNS 1

.

.

TrafficLights internalFunction

Timestamp: 00:00:20:000

00:00:20:000 redew 0

00:00:20:000 redns 1

.

TrafficLights outputFunction

Timestamp: 00:00:23:000

curLightState: 2

EW_RED_NS_GREEN_STATE, redEW: 1, redNS 0

.

.

TrafficLights internalFunction

Timestamp: 00:00:23:000

00:00:23:000 redew 1

00:00:23:000 redns 0

Simulation shows results exactly as expected.

## Counter Atomic Model Simulation

According to assigned sequence of events Counter should detect 6 violation on the first 10 min and 0 violations on the second 10 min. Two vehicles are approaching simultaneously on the ports cam2 and cam3 on the 5th second.

00:00:00:001 cam1 1

00:00:01:100 cam2 1

00:00:02:200 cam3 1

00:00:03:100 cam4 1

00:00:05:300 cam2 1

00:00:05:300 cam3 1

Counter constructor

Preparation Time=00:00:00:010

.

.

Counter initFunction

.

.

Counter externalFunction

Timestamp: 00:00:00:001

Port: cam1, cam2, cam3, or cam4

.

.

Counter externalFunction

Timestamp: 00:00:01:100

Port: cam1, cam2, cam3, or cam4

.

.

Counter externalFunction

Timestamp: 00:00:02:200

Port: cam1, cam2, cam3, or cam4

.

.

Counter externalFunction

Timestamp: 00:00:03:100

Port: cam1, cam2, cam3, or cam4

.

.

Counter externalFunction

Timestamp: 00:00:05:300

Port: cam1, cam2, cam3, or cam4

.

.

Counter externalFunction

Timestamp: 00:00:05:300

Port: cam1, cam2, cam3, or cam4

.

.

Counter outputFunction

Timestamp: 00:10:00:000

.

.

Counter internalFunction

Timestamp: 00:10:00:000

.

00:10:00:000 countout 6

.

Counter outputFunction

Timestamp: 00:20:00:000

.

.

Counter internalFunction

Timestamp: 00:20:00:000

.

00:20:00:000 countout 0

We got results exactly as expected.

## Atomic Model of Electromagnetic Detector

00:00:00:001 sv 1

00:00:00:100 sv 1

00:00:00:200 sv 1

00:00:00:300 sv 1

According to assigned sequence of events detector should generate output signals 4 times.

Detector constructor

Preparation Time=00:00:00:010

.

.

Detector initFunction

.

.

Detector externalFunction

Timestamp: 00:00:00:001

Port: sv

.

.

Detector outputFunction

Timestamp: 00:00:00:011

.

.

Detector internalFunction

Timestamp: 00:00:00:011

.

00:00:00:011 emd 1

.

Detector externalFunction

Timestamp: 00:00:00:100

Port: sv

.

.

Detector outputFunction

Timestamp: 00:00:00:110

.

.

Detector internalFunction

Timestamp: 00:00:00:110

.

00:00:00:110 emd 1

.

Detector externalFunction

Timestamp: 00:00:00:200

Port: sv

.

.

Detector outputFunction

Timestamp: 00:00:00:210

.

.

Detector internalFunction

Timestamp: 00:00:00:210

.

00:00:00:210 emd 1

.

Detector externalFunction

Timestamp: 00:00:00:300

Port: sv

.

.

Detector outputFunction

Timestamp: 00:00:00:310

.

.

Detector internalFunction

Timestamp: 00:00:00:310

.

00:00:00:310 emd 1

We got results exactly as expected.

## Atomic Model of Camera System

Table 5-10 Testing sequence for CamSys Atomic Model

| Time | Input Red | Input emd | Output Counter | Output Data |
| --- | --- | --- | --- | --- |
| 00:00:00:001 emd 1 | 0 | 1 | 0 | 0 |
| 00:00:00:100 red 1 | 1 | 0 | 0 | 0 |
| 00:00:00:200 emd 1 | 1 | 1 | 1 | 1 |
| 00:00:03:100 red 0 | 0 | 0 | 0 | 0 |
| 00:00:03:300 emd 1 | 0 | 1 | 0 | 0 |

CamSystem constructor

Preparation Time=00:00:00:010

.

.

CamSystem initFunction

.

.

CamSystem externalFunction

Timestamp: 00:00:00:001

Port: emd

isRed: 0

.

.

CamSystem externalFunction

Timestamp: 00:00:00:100

Port: red

.

.

CamSystem externalFunction

Timestamp: 00:00:00:200

Port: emd

isRed: 1

.

.

CamSystem outputFunction

Timestamp: 00:00:00:210

.

.

CamSystem internalFunction

Timestamp: 00:00:00:210

.

00:00:00:210 counter 1

00:00:00:210 data 1

.

CamSystem externalFunction

Timestamp: 00:00:03:100

Port: red

We got results exactly as expected.

# Conclusions

Red light Running Detection System was presented as discret-event model using the DEVS formalism. Discrete-event simulation of top model was executed as well as all sub models simulation. DEVS formal specification of each coupled and atomic model was included. Verification of simulation results showed feasibility of this model. Precision of model detection is equal system preparation time  - 10 ms.

References

- T. Harris. How Red-light Cameras Work [Online]. Available:

http://auto.howstuffworks.com

- City of Ottawa. Transportation and parking. Road Safety [Online]. Available: http://ottawa.ca.

- Priority, Market-Ready Technologies and Innovations, Red Light Cameras. FHWA.gov. Retrieved 22 March 2011.

- Wainer, G. A. (2009). Discrete-event modeling and simulation: A practitioner's approach. Boca Raton: CRC Press