MKS Simulation

Cell-Devs Model

Sysc5807

Assignment 2

Nov 12/03

2003-11-19

Prepared

by art gresham

100640035

part 1 Conceptual Model

A Cell-Devs Model of an MKS coupled mass system has been created.

An MKS system is a Mass, Spring, Damper system model.

The MKS Model is first a 2-dimensional model, capable of deriving the incremental velocity of a mass under the influence of a spring and viscous damper. The initial implementation does not simulate the motion in real time - this will be done as part of the term project. The MKS elementary Cell-DEVS model will be used to establish a workable incremental dynamic model which can be mapped to the Cell-DEVS architecture. These can be coupled together to simulate the action of a surface under a number of different influences, coupling each MKS zone on the Force developed in the spring, pinioned on both the x and y boundaries. Eventually, a 3 dimensional implementation of the evolved CELL-DEVS formalism will be made to simulate the action of a solid under a number of external influences using an ARRAY of MKS Cell-Devs groups or Zones.

The Atomic Cell model diagram is as follows:

Physical properties of the model, ie the linear spring force coefficient Ko, the mass Mo, and the linear viscous damping factor So are specified as values where used in the mks include macro MKS.inc. The initial value of velocity will be specified in the cell initial values file MKS.val. The emitted mks.LOG file is converted to DRW format using DrawLog from the simu tool. The simulation pallete MKS.pal has been chosen to show the motion of the mass using the CD++ \GUI modeler tool. This model has not been tried in the vrml viewer as of yet - the intention is to run it there once real-time inertial delays have been worked on to the model's real-time simulation behaviour..

The model  has been tested under a number of Ko, Mo, So parameter values, along with a variety of length increments along the x axis. The attached demo has been chosen to best demonstrate the effects of the spring and viscous damping on a free moving mass Mo .

Equations of Motion

There are a number of standard equations of motion which describe the motion of a mass on a damped spring. We should review these first, then look at the way they have been represented to allow cell-devs to calculate incremental changes in the state of the system.

- Newton:

Newton says that Force = mass * acceleration, or dF = mdv/dt

Newton also says that the position x2 of a particle P under constrained kinematic influence of initial position x1, velocity v1, and acceleration a  for period t is

x2 = 1/2 a t * * 2 + v1 * t + x1.

- Components

Component elementary properties are:

For a linear spring, Force = spring constant * spring displacement, or Fk = Ko dx.

For a linear viscous damper, Force = Damping coefficient * velocity, or dFs = So v dx.

- Evaluation:

Using the elementary dynamic components and their kinetic and kinematic properties could probably be used and configured some how, but for cell-devs, we should look at this from the point of view of Energy.

- Energy

The Energy of the system, minus the Energy dispersed through damping is a constant. So we'll establish the energy equation for each component and see what happens over a small incremental delta-x (dx)

- Mass in kg : the energy change due to the Mass is

1/2 Mo (v2 * * 2 - v1 * *  2)

- Spring in Newtons/meter : the energy change due to the Spring is

1/2 Ko (x2 * * 2 - x1 * *  2)

- Damper in Newton . seconds per meter: the energy change due to the Damper is 1/2 So(v2 - v1) (x2 - x1).

SUM of Energy for MKS

The Sum of Energies for these components is zero 0 along a path x2 - x1.

Stated, we get:

1/2 Mo(v2**2 - v1**2) + 1/2Ko(x2**2 - x1**2) + 1/2So(v2 - v1)(x2 - x1) = 0

Simplifying and completing the square on the v2 components gives:

(v2 + So(x2-x1)v1/2Mo) =

+/- sqrt(v1**2 - Ko(x2**2 - x1**2)/Mo - So(v1)(x2 -x1)/Mo + So**2(x2-x1)**2v1**2/4Mo**2)

Note that we can discard with minimal error that last term under the square root, as it is 2 orders of magnitude smaller than each of the other components. This gives us a means of deriving the incremental new velocity v2, given v1, x1, x2, Ko, Mo, So. Each remaining component, if you do a units balance, works out to units of velocity .

An intuitive balance can be performed to establish the sense for each component:

The mass will have either Left Hand Motion lh-motion Right Hand Motion rh-motion.

V2 is the new velocity, the sign will indicate direction of motion.

So(x2-x1)v1/2Mo under the root, is the part of the effect of the damping slowing down the mass no matter which direction it has at present, at velocity v1

So(x2-x1)/2Mo, derived from completing the square on the other side, is the part of the effect of the damping slowing down the mass no matter which direction it has at present, at velocity v2.

Ko/Mo(x2**2 - x1**2) will act to slow down the mass, if the motion of the mass is currently acting against the spring; speeding it up, if the motion of the mass is in the same direction as the decompressing spring.

The above let's you calculate the next velocity v2, which you can set as a Cell-DEVS state.

We still need the inertial time difference if we want to simulate in Real time.

For this, we can use the conservation of momentum for the mass in the system, and apply the sum of the forces incrementally over an impulse time dt, and we will solve for dt.

(Fk + Fm + Fs) dt = Mo(v2 - v1)

Fk(ave) is (Ko(x2-x1) / 2 , is the average force over x2 - x1 by the spring

Fm(ave) is Mo(dv/dt)average, is also Mo (dv/dx)(dx/dt), is Mo(v2-v1)**2/2/(x2-x1)

Fs(ave) is So(v2-v1)/2

In order to calculate dt-inertial, we will have to configure Cell-Devs to save v2 and v1 from iteration to iteration . . .

Cell-DEVS Components

The Cell-DEVS formalism for MKS is defined as follows:

TDC = <X, Y, I, S, N, delay, d, sigma(int) , sigma(ext) , tau, lamda, D>

Where:

X = external input events		{null}

Y = output external events		{null}

I  = model interface			{null}

S = the set of States			{vel = R = -oo - +oo}

N = the set of input events		{Vinitial of nominal mass Mo}

Delay = inertial

d.= length of delay			{calculated}

sigma(int)=internal transition		{Vn -> 0}

sigma(ext)=external transition		{0->Vn where Vn is next velocity of arriving mass Mo}

tau is local computing function   	{tau=v2 of <Mo,Ko,So,x2,x1,v1>

lamda output function			{}

D = duration function			{dt(inertial) of <Mo,Ko,So,x2,x1,v2,v1>

Cell-DEVS Implementation

Implementation is best defined in terms of what's in the actual mks.MA definition, and the macro MKS.inc which define the behaviour of the cell.

MKS.ma

The array is 2 by 51, each 'x' increment is 0.01 meters

The second row will be for keeping the previous velocity

A third row  will be added to keep spring tension for coupling with adjacent models.

Configuration is for Inertial delay - to be calculated for realtime simulation.

The motion can live in any of 4 virtual quadrants. We do this to make sensible evaluation of the determinate of the motion equation:

(v2 + So(x2-x1)v1/2Mo) =

+/- sqrt(v1**2 - Ko(x2**2 - x1**2)/Mo - So(v1)(x2 -x1)/Mo + So**2(x2-x1)**2v1**2/4Mo**2)

| rh-motion > spring acting with > | Rh-motion > spring acting against < |
| --- | --- |
| Lh-motion < spring acting against > | Lh motion < spring acting with < |

The damper So always acts against velocity.

Neighbors:

There are special end rules to 'bounce' off the ends with no losses

Each cell has 2 neighbors, 1 left, 1 right.

Zone rules:

If the right neighbor is a negative number, the mass is moving left, and will occupy my space with new velocity V2 under positive or negative influence of the spring Ko, depending on which quadrant above.

If the left neighbor is a positive number, the mass is moving right . . ., again So the damper always works against the velocity.

The actual calculations are part from the .MA, part from the .INC as follows,

For lh motion, from the .MA note that the values for the physical constants are simply keyed in their numeric forms in the macro at a convenient location.

So here, the spring Ko is 25 N/m, So is 2 N.s/m, mass Mo is 1 kg

Delta x ie x2 - x1 is the cellpos - the center / 100 (m) ie 1 centimeter.

[cntr-rule]

rule : { -1.0 * #macro(rh-damp) + 1.0 * sqrt(((0,-1) * (0,-1)) - #macro(rh-motionK) - #macro(rh-motionS)) } 10 { (0,-1) > 0 }

where the macros are:

#BeginMacro (rh-motionK)

(25 / 001.0 *

(

(

(cellpos(1) - 25) * (cellpos(1) - 25) / 100

)

-

(

(cellpos(1) - 26) * (cellpos(1) - 26) / 100

)

)

)

#EndMacro

and

#BeginMacro (rh-motionS)

(2.0 / 001.0 * (0,-1) * 0.01)

#EndMacro

Test Execution and Experience

The mks simulation can be run by placing the mks folder as a subdirectory of the directory holding the simu.exe environment.

Mks.bat will run simu against mks.ma, and then run drawlog to make a .drw

The mks.drw file can then be loaded into cd++ guimodeler to visualize the results.

Note that this model does not incorporate calculated inertial delays yet,

A number of mass Mo, Ko, and So combinations were tried to demonstrate behaviour that spanned the nominal 50-cell grid .

Interesting results following the velocity of the mass in the .drw file was that it slowed down as the square on the displacement of the spring, which it should, into recoverable energy, whereas the dissipation was a linear function, so there was not a lot of dissipation at the outer boundaries.

Intentional Determinate fall thru (for now)

Initially, a convoluted algorithm was applied on the determinate to guess when the mass had stopped moving one direction and started moving the other. It turns out that within the nominal error of the system, allowing the determinate to blow up as the indication of a direction change worked satifactorily- the turnaround value consistently on a failure to compute the determinate was -0.381., reported below

State Variables

------------------

I desperately need temporary variables to reduce equation size, and multiple state variables per cell- I would gladly help put this into the product for you.!

Sample results here at time 1.9 seconds(fake time - 190 calcs )

Show the recoil from rh-motion to lh-motion.

--------------------

0|     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     1.769     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000|

0|     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000    ---0.398     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000     0.000|

Simu Bugs

--------------

I'm not complaining here,

An interesting time was had getting the lh-motion to work after recoil, regrettably under the influence of an interesting anomaly with Simu.exe. I truly did learn these the hard way, never the less, they are important learnings for improving the future of the tool. I do not mind helping fix them at all.

The key bug was the failure of the power() function to 'square' my velocities when they were negative (result was always -0.381). This caused improper results in the lh-motion direction only. As you can see I manually square the velocities in the .ma now.  It took a while for me to figure this one out.

% bugs 7 - power() function doesn't work on my negative numbers

these others are features maybe or maybe not for bugzilla posting: I don’t mind helping with these either.

% bugs 1 - parser needs space after every token even for curly braces

% bugs 2 - x,y convention is backwards

% bugs 3 - gui modeler won't load a 1 by n array

% bugs 4 - debug mode abend on option -r

% bugs 5 - can't nest macros or create temp variables / sequences

% bugs 6 - sqrt (-1) = -0.381

% bugs 7 - power() function doesn't work on negative numbers

Part II Testing Strategy

All model components will be individually tested, and then test as integrated subcomponents of coupled models. There are 3 regimes of testing to be applied. These are

1) nominal or normal inputs representing realistic run time situation

2) extreme inputs x - preset 0 transition times and simultaneous events

3) extreme inputs z - preset long transition times and simultaneous events

Atomic Model Testing

Cuthair

Cuthair is a timed server - it receives cutcontinue messages, taking the predetermined time to process a cut. When the cut is finished, it outputs progress.

Test inputs are applied for each of the 3 general test regimes described above, and results evaluated.

Checkhair

Checkhair is a counter and type server - it receives both cust arrived messages from reception, and progress messages from cuthair. Checkhair is required to be able to process both concurrently. On progress messages from cuthair, Checkhair will be tested that it correctly counts progress. On cust messages from reception, Checkhair will be tested that it correctly starts cutting hair. Test data for each of the 3 general testing regimes defined above will be applied and evaluated.

Reception

Reception is a FIFO queue. It's queue size is limited, so it has to keep track of total number of entries. It must respond to both newcust messages from the world, and 'finished' messages from checkhair. Reception is required to be able to process both concurrently. Reception will be tested with test data for each of the general testing regimes defined above,  and results evaluated.

Coupled Model Testing

Barber

Barber is the coupled checkhair/cuthair components. It receives customers, and informs when cutting is finished. It must be able to start when it receives a customer, finish the cut, and send output when finished. It will be tested with test data for each of the general testing regimes defined above,  and results evaluated.

Barbershop

Barbershop is the complete hierarchically coupled system, including Reception, checkhair and cuthair components. It receives customers enqueuing them if the barber is busy and passes queued customers to the barber when he sends a finished message. It must be able to start when it receives a customer, send a 'first customer in queue' to the barber, enqueuing others arriving, and manage open close times for the shop. It will be tested with test data for each of the general testing regimes defined above,  and results evaluated.

Part III Simulation Results

Components were developed , unit tested , and integration tested using the test strategy from part II.

Components include reception.cpp and .h, checkhair.cpp and .h, cuthair.cpp and .h .

Inputs are the standard .ma and .ev files for the atomic components, coupled components developed include barber.ma and .ev, and barbershop.ma and .ev

In testing the components, erratic out of sequence inputs were applied to see what would happen. The model code was adjusted to reject erratic inputs. Ie , if the barber has a customer, and receives another customer, he ignores him.

Note: through the course of testing, it became useful and necessary to mark with output comments all edges of the control flow path for external, internal transition functions of all code. These outputs were left active to facilitate interpretation of the test results. Note in the text outputed comments that the last transition time lastchange() was used, not msg.time() .

Further, the batch scripts to run the barbershop simulation have been enhanced to bring all outputs into one '… .outputs.txt ' listing file per simulation test regime and model combination, and a utility to run all included tests - runallbarbershop.bat.

Testing regimes were assigned a naming convention to help sort them out, and combine them.

Example of Conventions  Used

barber.bat runs the barber subcomponent using barber.ma and barber.ev .

barber.x.bat runs the barber subcomponent using barber.x.ma and barber.x.ev .

the .x. naming convention is for the extreme conditions no transition times and simultaneous events testing.

barber.z.bat runs the barber subcomponent using barber.z.ma and barber.x.ev . the .z. naming convention is for the extreme conditions long transition times and simultaneous events.

There is a suite of each corresponding to each of the 3 atomic and 2 coupled models for barbershop.

Atomic Testing

Cuthair, Checkhair, Reception

Normal operation.

Events were listed in each of the respective event input .ev files for Cuthair, checkhair and reception. The accompanying message from the input was the desired amount of hair to remove, ie in millimetres.

Under normal circumstances, all 3 performed adequately independently .  Complete runtime listings are in the appendices. For example, here is an excerpt from the reception runtime:

"*** reception.ma "

[top]

components : reception@Reception

in : newcust next

out : cust

Link : newcust newcust@reception

Link : next next@reception

Link : cust@reception cust

[reception]

numberofChairs : 8

preparationTime : 00:0:01:100

openingTime : 09:00:00:000

closingTime : 16:00:00:000

. . . screen>

Starting simulation. Stop at time: Infinity. > event inputs:

08:01:00:000 / newcust /     10.00000

09:10:00:000 / newcust /     20.00000

. . .

12:23:00:000 / next /      1.00000

. . . code marks

00:00:00:000rcptn  x we are closed - sorry            : 0 : 0

08:01:00:000rcptn  x queued a newcust request         : 1 : 20

08:01:00:000rcptn  x activating first cust is coming  : 1 : 20

09:10:00:000rcptn  o transition                       : 1 : 20

09:10:00:000rcptn  i transition                       : 0 : 0

09:10:01:100rcptn  x queued a newcust request         : 1 : 30

09:10:01:100rcptn  x activating first cust is coming  : 1 : 30

. . . outputs

"*** reception.out "

09:10:01:100 cust 20

09:10:11:100 cust 30

Coupled Testing

Barber and Barbershop

Normal operation.

Events were listed in each of the respective event input .ev files for Barber and Barbershop. Under normal circumstances, all 3 performed adequately independently .  Normal circumstances included testing for erratic incoming messages that would typically not occur. Complete runtime listings are in the appendices. For example, here is an excerpt from the Barber runtime outputs.

Note in the text outputed comments that the last transition time lastchange() was used, not msg.time() .

"*** barber.ma "

[top]

components : checkhair@Checkhair cuthair@cuthair

in : cust progress

out : finished cutcontinue

- Link : cust cust@checkhair

Link : progress@cuthair progress@checkhair

- Link : cutcontinue@checkhair cutcontinue@cuthair

Link : finished@checkhair finished

[checkhair]

preparationTime : 00:00:09:000

Starting simulation. Stop at time: Infinity.

08:03:00:000 / cust /     -3.00000

11:03:00:000 / cust /     -0.00000

11:03:01:000 / cust /     19.00000

11:03:02:000 / cust /     18.00000

00:00:00:000chkhr x normal receive new customer      : 0:0

08:03:00:000chkhr o  first cutcontinue ->cuthair     : 1:3

08:03:00:000chkhr i transition :                     : 1:3

00:00:00:000cuthr x ok i'm cutting now                 : 1

08:03:09:000cuthr o output reporting progress         :

08:03:09:000cuthr i internal transition               :

Simulation ended!

"*** barber.out "

08:04:09:000 finished 3

Extreme Conditions

All of the Atomic models, and the coupled models too performed MISERABLY under extreme conditions. This included testing with simultaneous events combined with both long and 0 transition times. The key failure was in the checkhair component, and the way it was architected as a counter.

Checkhair used a state variable 'cutcount' for counting how much hair had been cut.

Simultaneous Events

The difficulty with simultaneous events for all of the atomic models was that the external transition function could be triggered sequentially in real time before the output function could run. The external transition function was manipulating state variables such as cutcount, cutmax etc to count total hair cut, and queue size for the receptionist, and these would change 'volatile' before the output function would run. These nasty results were revealed in testing with both the .x. and the .z. extreme test suites. In order to puncture proof the MKS for extreme conditions,  a complete redesign of the receptionist and checkhair components would be required, one which did not use state variables.

A slight improvement to the Receptionist component under simultaneous event conditions was to move the queue unloading function to the output function from its default(as cloned from the QUEUE sample) location in the external transition function.

An example of simultaneous event mishandling from checkhair.x.outputs.txt:

Inputs:

00:00:00:000 / progress /      0.00000

00:00:00:000 / cust /      3.00000

00:00:00:000 / progress /      0.00000

Reported

00:00:00:000chkhr x no cust here for haircut         : 1:0

00:00:00:000chkhr x busy and interrupted no new cust : 1:0

00:00:00:000chkhr x no cust here for haircut         : 1:0

00:00:00:000chkhr x no cust here for haircut         : 1:0

00:00:00:000chkhr x busy and interrupted no new cust : 1:0

Abnormal Transition Times

0 transition times compounded the problems associated with simultaneous events.

The main problem with long transition times was the rejection of incoming messages at the attached components. If checkhair was cutting hair, ie active, it would reject a new customer it had requested. This is not acceptable. In this case, the significance and importance of setting appropriate transition times was identified. This helps a lot. The code however really should be able to handle this, I think.  Here is an example of abnormal transition times from reception.z.outputs.txt.

In this case, the first cust in is not dispatched on time before the queue is full, so a customer has to be turned away.

Inputs.

09:00:00:000 / newcust /     30.00000

09:00:00:000 / next /      1.00000

output text:

09:00:00:000rcptn  x NEW cust in queue                : 8 : 20

09:00:00:000rcptn  x we are full - sorry              : 8 : 20

09:00:00:000rcptn  x activating queued cust is coming : 8 : 20

09:00:00:000rcptn  x activating queued cust is coming : 8 : 20

summarizing . . .

this has been an amazing learning experience in real time programming.

There were some glitches in getting started, but once the development environment was stable, things went ok, though slowly.

The key learning has been not to use state variables across function calls.

The key suggestion would be to create a sendoutput that allowed 'tree'ing of an attached comment to the console for aid in debugging.

There would be no expectation of extending the log files contents, but knowing the transition components and functions does NOT reveal the all important edges and states at each edge. Perhaps there is a debugger out there that can animate into and out of these coupled models , as the method and message sequencing  is not in the control of the programmer, as with normal COBOL code.

Thank you for the help getting started.

Art

Appendix

Readme.txt

barbershop demo was developed under cygwin on a compaq laptop running windows xp home.

the cd++ was downloaded as cd++_exes_win.zip from the course website

win32 cygwin

1) unzip into directory with cd++ tool - you have to allow overwrite of register.cpp and makefile

2) open a cygwin prompt

3) make

coupled components:

4) barbershop.bat testruns the complete Barbershop coupled devs model using barbershop.ma barbershop.ev

5) barber.bat     testruns the Barber subcomponent, also a coupled devs model using barber.ma barber.ev

atomic components:

6) reception.bat  testruns the reception Atomic component  using reception.ma reception.ev

7) checkhair.bat  testruns the checkhair Atomic component  using checkhair.ma reception.ev

8) cuthair.bat    testruns the cuthair Atomic component    using cuthair.ma cuthair.ev

naming conventions - related components have identical name roots

ie there is a barber.bat barber.ma barber.ev

Atomic components: there is reception.cpp, reception.h, checkhair.cpp, checkhair.h, cuthair.cpp, cuthair.h .

OUTPUTS

the .bat files have been configured to organize the output info for easier analysis.

each .bat file runs the simulator with its respective .ev and .ma

the .OUT file, screen output, and diagnostic messages from the code are output to {componentname}.outputs.txt

ie when you run barbershop.bat, you get a barbershop.outputs.txt for convenient viewing.

the last instruction of each .bat file displays the .outputs.txt file to the console using 'less'.

you get around the file in 'less' with the up and down arrows , it's pretty intuitive.

to exist the 'less' viewer, type 'q' for quit.

have fun!

art

sysc5807

Typical .Outputs.txt file

(

"*** cuthair.ma "

[top]

components : cuthair@Cuthair

in : cutcontinue

out : progress

- Link : cutcontinue cutcontinue@cuthair

Link : progress@cuthair progress

[cuthair]

preparationTime : 0:0:11:000

"*** cuthair runtime msgs "

N-CD++: A Tool to Implement n-Dimensional Cell-DEVS models

----------------------------------------------------------

- Version 2.0-R.45 December-1999

Daniel Rodriguez, Gabriel Wainer, Amir Barylko, Jorge Beyoglonian

Departamento de Computacion. Facultad de Ciencias Exactas y Naturales.

Universidad de Buenos Aires. Argentina.

- Loading models from cuthair.ma

Loading events from cuthair.ev

Message log: cuthair.log

Output to: cuthair.out

Tolerance set to: 1e-08

Configuration to show real numbers: Width = 12 - Precision = 5

Quantum: Not used

Evaluate Debug Mode = OFF

Flat Cell Debug Mode = OFF

Debug Cell Rules Mode = OFF

Temporary File created by Preprocessor = /tmp/tb0.0

Printing parser information = OFF

Starting simulation. Stop at time: Infinity.

00:00:00:000 / cutcontinue /     -3.00000

00:01:20:000 / cutcontinue /      1.00000

00:02:45:000 / cutcontinue /      3.00000

00:03:20:000 / cutcontinue /      1.00000

00:04:40:000 / cutcontinue /      3.00000

00:05:20:000 / cutcontinue /      1.00000

00:06:40:000 / cutcontinue /      3.00000

00:07:20:000 / cutcontinue /      1.00000

00:08:40:000 / cutcontinue /      3.00000

00:09:20:000 / cutcontinue /      1.00000

00:10:40:000 / cutcontinue /      4.00000

00:11:20:000 / cutcontinue /      5.00000

00:00:00:000cuthr x ok i'm cutting now                 : -3

00:00:00:000cuthr o output reporting progress         :

00:00:00:000cuthr i internal transition               :

00:00:11:000cuthr x ok i'm cutting now                 : 1

00:01:20:000cuthr o output reporting progress         :

00:01:20:000cuthr i internal transition               :

00:01:31:000cuthr x ok i'm cutting now                 : 3

00:02:45:000cuthr o output reporting progress         :

00:02:45:000cuthr i internal transition               :

00:02:56:000cuthr x ok i'm cutting now                 : 1

00:03:20:000cuthr o output reporting progress         :

00:03:20:000cuthr i internal transition               :

00:03:31:000cuthr x ok i'm cutting now                 : 3

00:04:40:000cuthr o output reporting progress         :

00:04:40:000cuthr i internal transition               :

00:04:51:000cuthr x ok i'm cutting now                 : 1

00:05:20:000cuthr o output reporting progress         :

00:05:20:000cuthr i internal transition               :

00:05:31:000cuthr x ok i'm cutting now                 : 3

00:06:40:000cuthr o output reporting progress         :

00:06:40:000cuthr i internal transition               :

00:06:51:000cuthr x ok i'm cutting now                 : 1

00:07:20:000cuthr o output reporting progress         :

00:07:20:000cuthr i internal transition               :

00:07:31:000cuthr x ok i'm cutting now                 : 3

00:08:40:000cuthr o output reporting progress         :

00:08:40:000cuthr i internal transition               :

00:08:51:000cuthr x ok i'm cutting now                 : 1

00:09:20:000cuthr o output reporting progress         :

00:09:20:000cuthr i internal transition               :

00:09:31:000cuthr x ok i'm cutting now                 : 4

00:10:40:000cuthr o output reporting progress         :

00:10:40:000cuthr i internal transition               :

00:10:51:000cuthr x ok i'm cutting now                 : 5

00:11:20:000cuthr o output reporting progress         :

00:11:20:000cuthr i internal transition               :

Simulation ended!

"*** cuthair.out "

00:00:11:000 progress 1

00:01:31:000 progress 1

00:02:56:000 progress 1

00:03:31:000 progress 1

00:04:51:000 progress 1

00:05:31:000 progress 1

00:06:51:000 progress 1

00:07:31:000 progress 1

00:08:51:000 progress 1

00:09:31:000 progress 1

00:10:51:000 progress 1

00:11:31:000 progress 1