Modeling and Simulation of Traffic Switching and Routing in Computer Networks using DEVS and DEVS Graph

by

Aizaz Chaudhry

(100783511)

A Directed Studies Course Project

for

SYSC 5104: METHODOLOGIES FOR DISCRETE EVENT MODELLING AND SIMULATION

TABLE OF CONTENTS

LIST OF FIGURES	iv

Motivation	1

NS2	1

OPNET	1

DEVS Formalism (using CD++ Builder)	1

Part I:

Problem: Traffic Switching and Routing between Nodes in a Computer Network	2

Description of the problem	2

Brief sketch of the model structure	2

Brief description of the behavior of each component	3

LAN Coupled Model:	3

WAN Coupled Model:	4

LAN Switch Coupled Model:	5

WAN Router Coupled Model:	7

Workstation Coupled Model:	7

Queue Atomic Model:	8

LAN Switch Atomic Model:	9

WAN Switch Atomic Model:	10

Source Atomic Model:	10

Sink Atomic Model:	11

Part II:

Formal Specifications	12

Formal Specifications for Atomic Models	12

Sink:	12

Queue:	12

LAN Switch:	13

WAN Switch:	13

Source:	14

Formal Specifications for Coupled Models	15

LANWAN coupled:	15

WAN coupled:	16

LAN coupled:	16

WAN Router coupled:	17

LAN Switch coupled:	18

Workstation (coupled):	19

Part III:

Implementation of the Atomic Models using DEVS Graph	20

Source:	20

Sink:	21

LAN Switch:	22

WAN Switch:	24

Part IV:

Test Strategies	26

Source (atomic):	26

Sink (atomic):	27

LAN Switch (atomic):	28

WAN Switch (atomic):	29

Workstation (coupled):	30

LAN Switch (coupled):	31

WAN Router (coupled):	32

LAN (coupled):	33

WAN (coupled):	36

Switching-Routing Simulator (LANWAN (coupled)):	38

Part V:

Modification to the CD++ Builder Tool	42

CONCLUSION	45

References	46

# LIST OF FIGURES

Figure 1: Structure of the Switching-Routing Simulator	2

Figure 2: Structure of the LAN Coupled Model	3

Figure 3: Structure of the WAN Coupled Model	4

Figure 4: Structure of the LAN Switch Coupled Model	6

Figure 5: Structure of the WAN Router Coupled Model	7

Figure 6: Structure of the Workstation Coupled Model	8

Figure 7: Structure of the Queue Atomic Model	8

Figure 8: Structure of the LAN Switch Atomic Model	9

Figure 9: Structure of the WAN Switch Atomic Model	10

Figure 10: Structure of the Source Atomic Model	10

Figure 11: Structure of the Sink Atomic Model	11

Figure 12: Source atomic model using DEVS Graph	20

Figure 13: Sink atomic model using DEVS Graph	21

Figure 14: LAN Switch atomic model using DEVS Graph	22

Figure 15: WAN Switch atomic model using DEVS Graph	24

# Motivation

Simulation tools such as, NS-2 [1] and OPNET [2], are mostly used for the modeling and simulation of computer networks.

## NS2

This network simulator has been developed by the Virtual InterNetwork Testbed (VINT) project. It is popular with the networking research community as it is an open source software. It includes numerous models of common Internet protocols. Additionally, different levels of configuration are present in NS-2 due to its open source nature, including the capability of creating custom applications and protocols as well as modifying several parameters at different layers. This simulation tool does not have a Graphical User Interface (GUI) and provides a Linux-based command line interface to modify the parameters or to change the source code. In addition to a command line interface that is not user friendly, it requires an extensive learning curve to understand the source code before altering the built-in models or before adding new models to the source code.

## OPNET

This simulation tool is based on a finite state machine model in combination with an analytical model. It can model protocols, devices and behaviors and comes with a GUI. The GUI along with the considerable amount of documentation that comes with the license makes it an attractive tool. However, it is not an open source software and though model parameters can be altered, it also requires an extensive learning curve to understand the source code before making changes to it.

## DEVS Formalism (using CD++ Builder)

The DEVS formalism [3] [4] provides a strong simulation and modeling framework for defining discrete-event models in a modular way, where a system is implemented as a composition of behavioral (atomic) and structural (coupled) components. Unlike the event-based discrete-time simulation approaches such as NS-2 and OPNET, DEVS uses a continuous time base and allows for asynchronous model execution, improving the efficiency of the simulation without losing accuracy.

In this project, a simplified version of a computer network has successfully been modeled and simulated using DEVS formalism through the CD++ Builder tool [5] [6], which, therefore, provides an attractive alternative for the modeling and simulation of computer networks.

# Part I:

# Problem: Traffic Switching and Routing between Nodes in a Computer Network

## Description of the problem

The purpose of the model is to model and simulate traffic generation, switching, routing and traffic reception between nodes in a computer network. The model is based on a computer network, which consists of three Local Area Networks (LANs) connected through a Wide Area Network (WAN). A LAN consists of three workstations and a LAN switch. A workstation can be a traffic source as well as a sink. The LAN switch switches the traffic (packets) between nodes in the same LAN or forwards the traffic to the gateway, which is a WAN router, in case the traffic is destined for a different LAN. The WAN consists of three WAN routers. A WAN router receives the traffic forwarded to it by the LAN switch and routes it to the appropriate WAN router, based on the destination address of the incoming packet.

Brief sketch of the model structure

packets  generated1

packets  generated3

packets  dropped1

packets  dropped2

packets dropped3

LAN WAN Coupled

LAN 1

WAN

LAN 3

LAN 2

packets  received1

packets  received3

packets

generated2

packets

received2

Figure 1: Structure of the Switching-Routing Simulator

The outputs of the simulator consist of packets generated, packets received and packets dropped. The output “packets generated” outputs the packet generated by a source workstation as well as the address of the sink workstation. The output “packets received” outputs the number of packets received by a sink workstation. The output “packets dropped” outputs the packets received by the WAN which does not belong to any of the LANs connected to the WAN. The Switching-Routing simulator has no inputs and has been implemented using DEVS formalism through the CD++ Builder tool. The atomic models are implemented through “C ++” as well as by using “DEVS Graph” [7] [8].

## Brief description of the behavior of each component

### LAN Coupled Model:

Each LAN consists of the following nodes:

- Three Workstation nodes (coupled models)

- A LAN switch node (coupled model)

The workstations are connected to the switch in a star topology, as shown in Fig 2. Each workstation can be a traffic source as well as a traffic sink. The LAN switch receives the packet from a workstation, checks the sink address of the packet to find out the destination workstation and sends it to the corresponding workstation. In case the destination node is not one of the workstations directly connected to the LAN switch, it forwards the packet to the gateway node. The gateway node is a WAN router which is part of the WAN coupled model.

packets  generated1

packets  generated2

packets  received1

LAN Out

LAN Coupled Model

Workstation 1

LAN

Switch

Workstation 2

Workstation 3

Figure 2: Structure of the LAN Coupled Model

packets  generated3

packets  received2

packets  received3

LAN In

The outputs of the LAN coupled model consist of the following:

- packets generated – outputs the packet generated by a source workstation containing the address of the sink workstation

- packets received – outputs the number of packets received by a sink workstation

- LAN Out – outputs the packet as well as the address of the sink workstation in case the destination node lies in a different LAN and is not one of the workstations directly connected to the LAN switch

There is only one input of the LAN coupled model which is "LAN In". It is connected to a WAN output and receives those packets from the WAN router which are destined for any workstation in that LAN.

### WAN Coupled Model:

WAN Coupled Model

WAN

Router

WAN

Router

WAN

Router

The WAN coupled model consists of three router nodes, as shown in the Fig. 3. A WAN router receives the packet from either another WAN router or from a LAN and forwards the packet based on its sink address. Each WAN router is connected to the other two WAN routers, as shown in Fig. 3.

WAN Out1

WAN In1

packets dropped1

WAN Out2

WAN Out3

WAN In3

WAN In2

packets dropped2

packets dropped3

Figure 3: Structure of the WAN Coupled Model

The outputs of the WAN coupled model consist of the following:

- WAN Out1

- WAN Out2

- WAN Out3

- packets dropped1

- packets dropped2

- packets dropped3

If a WAN router receives a packet that does not belong to the LAN connected to that WAN router as well as does not belong to the two WAN routers connected to it, it drops the packet which appears at the "packets dropped" output of the WAN coupled model linked to that WAN router.

The inputs of the WAN coupled model consist of the following:

- WAN In1

- WAN In2

- WAN In3

A pair consisting of one input and one output is connected to a LAN such that:

- WAN Out1 is connected to LAN In1

- WAN In1 is connected to LAN Out1

### LAN Switch Coupled Model:

The LAN switch consists of a queue atomic model and a LAN switch atomic model, as shown in the Fig. 4. It switches the traffic in the following ways:

- from a workstation to another workstation in the same LAN

- from a workstation to the WAN router (gateway)

- from the WAN router to a workstation

The outputs of the LAN Switch coupled model consist of the following:

- switch Out1

- switch Out2

- switch Out3

- gateWay

The inputs of the LAN Switch coupled model consist of the following:

- switch In1

- switch In2

- switch In3

- switch In4

A pair consisting of one input and one output of the LAN Switch coupled model is connected to a workstation such that:

- switch Out1 is connected to packet In1

- switch In1 is connected to packet Out1

A pair consisting of one input and one output of the LAN Switch coupled model is connected to the WAN coupled model such that:

- gateWay is connected to LAN Out which is connected to WAN In

- switch In4 is connected to LAN In  which is connected to WAN Out

LAN Switch Coupled Model

Queue

LAN

Switch

done

Queue

Queue

Queue

LAN

Switch

LAN

Switch

LAN

Switch

done

done

done

switch In1

switch In2

switch In3

switch In4

switch Out1

switch Out2

switch Out3

gateWay

Figure 4: Structure of the LAN Switch Coupled Model

### WAN Router Coupled Model:

The WAN router coupled model is similar to the LAN Switch coupled model and is shown in the Fig. 5. If the WAN router receives a packet that does not belong to any of the LANs connected to the two WAN routers, it drops the packet which appears at the "packets dropped" output port.

WAN Router Coupled Model

Queue

WAN

Switch

done

Queue

Queue

WAN

Switch

WAN

Switch

done

done

router In1

router In2

router In3

router Out1

router Out2

router Out3

packets dropped

Figure 5: Structure of the WAN Router Coupled Model

### Workstation Coupled Model:

The Workstation coupled model, as shown in the Fig. 6, consists of a source atomic model and a sink atomic model. There is only one input of the workstation which is “packet In”. Outputs of the workstation are the following:

- packet Out

- traffic generated

- traffic received

The outputs “packets generated” and “packet Out” output the packet generated (by the source atomic model of the workstation) containing the address of the sink workstation. The output “packets received” outputs the number of packets received by the sink atomic model of the workstation.

Workstation Coupled Model

Sink

Source

packet In

packet Out

packets received

packets generated

Figure 6: Structure of the Workstation Coupled Model

### Queue Atomic Model:

Queue

done

in

out

The queue atomic model, which is a built-in atomic model in CD++ Builder, is shown in Fig. 7.

Figure 7: Structure of the Queue Atomic Model

Inputs of the queue atomic model are the following:

- done

- in

There is only one output of the queue atomic model which is “out”. The queue stores the incoming packet and passes it on to the switch atomic model after some delay. The queuing delay can be set by changing its “preparation” parameter.

### LAN Switch Atomic Model:

The LAN Switch atomic model is shown in the Fig. 8. There is only one input of the LAN switch atomic model which is “switch In”.

Outputs of the LAN switch atomic model are the following:

- switch Out1

- switch Out2

- switch Out3

- gateWay

The LAN switch atomic model receives a packet from the queue atomic model and switches it to the appropriate output port based on the sink address of the packet after some delay. The switching delay of the LAN switch atomic model can be set by changing its “preparation” parameter.

LAN

Switch

Atomic

Model

switch Out1

switch Out2

switch Out3

gateWay

switch In

Figure 8: Structure of the LAN Switch Atomic Model

If the LAN switch atomic model receives a packet that does not belong to the any of the workstations connected to the LAN switch coupled model, then it sends that packet to the “gateWay” output port.

### WAN Switch Atomic Model:

The WAN Switch atomic model is similar to the LAN switch atomic model and is shown in the Fig. 9. If the WAN switch atomic model receives a packet that does not belong any of the WAN routers connected to its WAN router coupled model, it drops the packet which appears at the "packets dropped" output.

WAN

Switch

Atomic

Model

switch Out1

switch Out2

switch Out3

packets dropped

switch In

Figure 9: Structure of the WAN Switch Atomic Model

### Source Atomic Model:

Source

packet Out

packets generated

The source atomic model is shown in the Fig. 10.

Figure 10: Structure of the Source Atomic Model

There is no input of the source atomic model. Its outputs are the following:

- packet Out

- packets generated

The outputs “packets generated” and “packet Out” output the packet generated (by the source atomic model) containing the address of the sink workstation. The source atomic model generates packets at a constant rate, e.g. one packet every 10 seconds. The packet generation rate can be changed by changing its “preparation” parameter.

### Sink Atomic Model:

Sink

packets received

packet In

The sink atomic model is shown in the Fig. 11. There is one input of this model which is “packet In”. After receiving a packet, it checks the incoming address to ensure that the received packet belongs to itself. The sink then outputs the received packets after a delay of 1 second which can be changed by changing its “preparation” parameter. The address of the sink can also be configured by setting the “address” parameter of the sink.

Figure 11: Structure of the Sink Atomic Model

There is one output of the sink atomic model which is “packets received” which outputs the number of packets received by the sink atomic model.

# Part II:

# Formal Specifications

## Formal Specifications for Atomic Models

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic models [7] are defined as follows:

### Sink:

S = {passive, active}

X = {packet In}

Y = {packetsReceived}

δint (active) = passive

δext (in, passive) = active

δext (in, active) = active

λ(active)

{

check the sink address of the incoming packet

if incoming address is equal to my address, send packets received to port “packetsReceived”

}

ta(passive) = INFINITY

ta(active) = processing delay

### Queue:

S = {passive, active}

X = {in, done}

Y = {out}

δint (active) = passive

δext (in, passive) = active

δext (in, active) = active

δext (done, active) = active

δext (done, active) = passive

λ(active)

{

when done, send next packet to the front of the queue

send packet from front of the queue to the port out

}

ta(passive) = INFINITY

ta(active) = queueing delay

### LAN Switch:

S = {passive, active}

X = {switchIn}

Y = {switchOut1, switchOut2, switchOut3, gateWay}

δint (active) = passive

δext (in, passive) = active

δext (in, active) = active

λ(active)

{	check address of the packet received at switchIn and send it to the appropriate output port

if the packet does not belong to any of the directly connected workstations, send it to the gateWay output port

}

ta(passive) = INFINITY

ta(active) = switching delay

### WAN Switch:

S = {passive, active}

X = {switchIn}

Y = {switchOut1, switchOut2, switchOut3, packetsDropped}

δint (active) = passive

δext (in, passive) = active

δext (in, active) = active

λ(active)

{	check address of the packet received at switchIn and send it to the appropriate output port

if the packet does not belong to any of the LANs, send it to the packetsDropped output port

}

ta(passive) = INFINITY

ta(active) = switching delay

### Source:

State Variables:

phase = active;

sinkAddr; 		// the address of the sink node which is also the message of the packet

state generate;  	//since the source is always generating packets, so it is always in this state

Formal specification:

X = {}

Y = {packetOut, packetsGenerated}

S = {{phase, sinkAddr, state}}

δext ()

{

}

δint (sinkAddr, state)

{	case generate:

active:

sinkAddr = rand() % 11 + 1;	//generates a random number between //1 and 11 and assigns it to the sink //address

passive: 	//Never happens

}

λ(active)

{	send sinkAddr  to the port packetsGenerated			//packet generated and its sink address

send sinkAddr to the port packetOut				//message of the packet

}

ta(passive) = INFINITY	//Never happens

ta(active) = packet generation rate

## Formal Specifications for Coupled Models

The formal specifications <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT > for the coupled models [7] are defined as follows:

### LANWAN coupled:

X = {};

Y = {packetsGenerated1, packetsGenerated2, packetsGenerated3, packetsGenerated4, packetsGenerated5, packetsGenerated6, packetsGenerated7, packetsGenerated8, packetsGenerated9, packetsReceived1, packetsReceived2, packetsReceived3, packetsReceived4, packetsReceived5, packetsReceived6, packetsReceived7, packetsReceived8, packetsReceived9, packetsDropped1, packetsDropped2, packetsDropped3};

D = {LAN1 coupled, LAN2 coupled, LAN3 coupled, WAN coupled};

I(LAN1 coupled) = {LAN2 coupled, LAN3 coupled, WAN coupled};

I(LAN2 coupled) = {LAN1 coupled, LAN3 coupled, WAN coupled};

I(LAN3 coupled) = {LAN1 coupled, LAN2 coupled, WAN coupled};

I(WAN coupled) = {LAN1 coupled, LAN2 coupled, LAN3 coupled};

Z(LAN1 coupled) = LAN2 coupled;

Z(LAN1 coupled) = LAN3 coupled;

Z(LAN1 coupled) = WAN coupled;

Z(LAN2 coupled) = LAN1 coupled;

Z(LAN2 coupled) = LAN3 coupled;

Z(LAN2 coupled) = WAN coupled;

Z(LAN3 coupled) = LAN1 coupled;

Z(LAN3 coupled) = LAN2 coupled;

Z(LAN3 coupled) = WAN coupled;

Z(WAN coupled) = LAN1 coupled;

Z(WAN coupled) = LAN2 coupled;

Z(WAN coupled) = LAN3 coupled;

SELECT:

({LAN1 coupled, LAN2 coupled, LAN3 coupled, WAN coupled}) = LAN1 coupled;

({LAN2 coupled, LAN3 coupled, WAN coupled}) = LAN2 coupled;

({LAN3 coupled, WAN coupled}) = LAN3 coupled;

### WAN coupled:

X = {WANIn1, WANIn2, WANIn3};

Y = {WANOut1, WANOut2, WANOut3, packetsDropped1, packetsDropped2, packetsDropped3};

D = {WANrouter1 coupled, WANrouter2 coupled, WANrouter3 coupled};

I(WANrouter1 coupled) = {WANrouter2 coupled, WANrouter3 coupled};

I(WANrouter2 coupled) = {WANrouter1 coupled, WANrouter3 coupled};

I(WANrouter3 coupled) = {WANrouter1 coupled, WANrouter2 coupled};

Z(WANrouter1 coupled) = WANrouter2 coupled;

Z(WANrouter1 coupled) = WANrouter3 coupled;

Z(WANrouter2 coupled) = WANrouter1 coupled;

Z(WANrouter2 coupled) = WANrouter3 coupled;

Z(WANrouter3 coupled) = WANrouter1 coupled;

Z(WANrouter3 coupled) = WANrouter2 coupled;

SELECT:

({WANrouter1 coupled, WANrouter2 coupled, WANrouter3 coupled}) = WANrouter1 coupled;

({WANrouter2 coupled, WANrouter3 coupled}) = WANrouter2 coupled;

### LAN coupled:

X = {LANIn};

Y = {LANOut, packetsGenerated1, packetsGenerated2, packetsGenerated3, packetsReceived1, packetsReceived2, packetsReceived3};

D = {workstation1 (coupled), workstation2 (coupled), workstation3 (coupled), LAN switch coupled};

I(workstation1) = {workstation2, workstation3, LAN switch coupled};

I(workstation2) = {workstation1, workstation3, LAN switch coupled};

I(workstation3) = {workstation1, workstation2, LAN switch coupled};

I(LAN switch coupled) = {workstation1, workstation2, workstation3};

Z(workstation1) = workstation2;

Z(workstation1) = workstation3;

Z(workstation1) = LAN switch coupled;

Z(workstation2) = workstation1;

Z(workstation2) = workstation3;

Z(workstation2) = LAN switch coupled;

Z(workstation3) = workstation1;

Z(workstation3) = workstation2;

Z(workstation3) = LAN switch coupled;

Z(LAN switch coupled) = workstation1;

Z(LAN switch coupled) = workstation2;

Z(LAN switch coupled) = workstation3;

SELECT:

({workstation1, workstation2, workstation3, LAN switch coupled}) = workstation1;

({workstation2, workstation3, LAN switch coupled}) = workstation2;

({workstation3, LAN switch coupled}) = workstation3;

### WAN Router coupled:

X = {routerIn1, routerIn2, routerIn3};

Y = {routerOut1, routerOut2, routerOut3, packetsDropped};

D = {queue1 (atomic), queue2 (atomic), queue3 (atomic), WAN switch1 (atomic), WAN switch2 (atomic), WAN switch3 (atomic)};

I(queue1) = {WAN switch1};

I(queue2) = {WAN switch2};

I(queue3) = {WAN switch3};

I(WAN switch1) = {queue1, self};

I(WAN switch2) = {queue2, self};

I(WAN switch3) = {queue3, self};

Z(queue1) = WAN switch1;

Z(queue2) = WAN switch2;

Z(queue3) = WAN switch3;

Z(WAN switch1) = queue1;

Z(WAN switch1) = self;

Z(WAN switch2) = queue2;

Z(WAN switch2) = self;

Z(WAN switch3) = queue3;

Z(WAN switch3) = self;

SELECT:

({queue1, queue2, queue3, WAN switch1, WAN switch2, WAN switch3}) = queue1;

({queue2, queue3, WAN switch1, WAN switch2, WAN switch3}) = WAN switch1;

({queue2, queue3, WAN switch2, WAN switch3}) = queue2;

({queue3, WAN switch2, WAN switch3}) =WAN switch2;

({queue3, WAN switch3}) = queue3;

### LAN Switch coupled:

X = {switchIn1, switchIn2, switchIn3, switchIn4};

Y = {switchOut1, switchOut2, switchOut3, gateWay};

D = {queue1 (atomic), queue2 (atomic), queue3 (atomic), queue4 (atomic), LAN switch1 (atomic), LAN switch2 (atomic), LAN switch3 (atomic), LAN switch4 (atomic)};

I(queue1) = {LAN switch1};

I(queue2) = {LAN switch2};

I(queue3) = {LAN switch3};

I(queue4) = {LAN switch4};

I(LAN switch1) = {queue1, self};

I(LAN switch2) = {queue2, self};

I(LAN switch3) = {queue3, self};

I(LAN switch4) = {queue4, self};

Z(queue1) = LAN switch1;

Z(queue2) = LAN switch2;

Z(queue3) = LAN switch3;

Z(queue4) = LAN switch4;

Z(LAN switch1) = queue1;

Z(LAN switch1) = self;

Z(LAN switch2) = queue2;

Z(LAN switch2) = self;

Z(LAN switch3) = queue3;

Z(LAN switch3) = self;

Z(LAN switch4) = queue4;

Z(LAN switch4) = self;

SELECT:

({queue1, queue2, queue3, queue4, LAN switch1, LAN switch2, LAN switch3, LAN switch4}) = queue1;

({queue2, queue3, queue4, LAN switch1, LAN switch2, LAN switch3, LAN switch4}) = LAN switch1;

({queue2, queue3, queue4, LAN switch2, LAN switch3, LAN switch4}) = queue2;

({queue3, queue4, LAN switch2, LAN switch3, LAN switch4}) = LAN switch2;

({queue3, queue4, LAN switch3, LAN switch4}) = queue3;

({queue4, LAN switch3, LAN switch4}) = LAN swicth3

({queue4, LAN switch4}) = queue4

### Workstation (coupled):

X = {packetIn};

Y = {packetOut, packetsGenerated, packetsReceived};

D = {source (atomic), sink (atomic)};

I(source) = {};

I(sink) = {};

Z(source) = {};

Z(sink) = {};

SELECT: 	({source, sink}) = source;

# Part III:

# Implementation of the Atomic Models using DEVS Graph

In addition to their C++ implementation, the atomic models have also been implemented using the DEVS Graph utility [8] of the CD++ Builder Tool. With DEVS Graph, it is possible to think about the problem at hand in a more abstract way and develop models as state machines based on formal DEVS specifications. In DEVS Graph, atomic models are implemented through symbolic objects to represent the states and state transitions. In order to be simulated in CD++ Builder, DEVS Graph are translated into GADscript (GrAphical DEVS script).

## Source:

The DEVS Graph specification of the source atomic model, as per the "source.ggaddesigner" file, is shown below in Fig. 12.

Figure 12: Source atomic model using DEVS Graph

The GADscript representation of the source atomic model, as per the "source.cdd" file, is as follows.

[Top]

out:packetOut packetsGenerated

var:sinkAddr

state:generate

initial:generate

int:generate generate packetOut!sinkAddr packetsGenerated!sinkAddr {sinkAddr=Round(Rand(1,11));}

generate:00:00:10:00

sinkAddr:1

Note the difference in the random number generation for the sink address between the DEVS Graph and the GADscript. The details regarding this issue and its solution are presented in part V of this report.

## Sink:

The DEVS Graph specification of the sink atomic model, as per the "sink.ggaddesigner" file, is shown below in Fig. 13.

Figure 13: Sink atomic model using DEVS Graph

The GADscript representation of the sink atomic model, as per the "sink.cdd" file, is as follows.

[Top]

in:packetIn

out:packetsReceived

var:packets myAddr

state:idle active

initial:idle

ext:idle active Equal(Value(packetIn),myAddr)?1{packets=Add(packets,1);}

int:active idle packetsReceived!packets

ext:idle idle NotEqual(Value(packetIn),myAddr)?1

idle:99:99:99:99

active:00:00:01:00

packets:0

myAddr:1

## LAN Switch:

The DEVS Graph specification of the LAN Switch atomic model, as per the "LANswitch.ggaddesigner" file, is shown below in Fig. 14.

Figure 14: LAN Switch atomic model using DEVS Graph

The GADscript representation of the LAN Switch atomic model, as per the "LANswitch.cdd" file, is as follows.

[Top]

in:switchIn

out:switchOut1 switchOut2 switchOut3 gateWay

var:OneAddr TwoAddr ThreeAddr destAddr

state:idle active1 active2 active3 active4

initial:idle

ext:idle active1 And(Less(Value(switchIn),99),Equal(Value(switchIn),OneAddr))?1{destAddr=Value(switchIn);}

int:active1 idle switchOut1!destAddr

ext:idle active2 And(Less(Value(switchIn),99),Equal(Value(switchIn),TwoAddr))?1{destAddr=Value(switchIn);}

int:active2 idle switchOut2!destAddr

ext:idle active3 And(Less(Value(switchIn),99),Equal(Value(switchIn),ThreeAddr))?1{destAddr=Value(switchIn);}

int:active3 idle switchOut3!destAddr

ext:idle active4 Or(Less(Value(switchIn),OneAddr),Greater(Value(switchIn),ThreeAddr))?1{destAddr=Value(switchIn);}

int:active4 idle gateWay!destAddr

ext:idle idle Greater(Value(switchIn),99)?1

idle:99:99:99:99

active1:00:00:01:00

active2:00:00:01:00

active3:00:00:01:00

active4:00:00:01:00

OneAddr:1

TwoAddr:2

ThreeAddr:3

destAddr:0

## WAN Switch:

The DEVS Graph specification of the WAN Switch atomic model, as per the "WANswitch.ggaddesigner" file, is shown below in Fig. 15.

Figure 15: WAN Switch atomic model using DEVS Graph

The GADscript representation of the WAN Switch atomic model, as per the "WANswitch.cdd" file, is as follows.

[Top]

in:switchIn

out:switchOut1 switchOut2 switchOut3 packetsDropped

var:A B C D E F G H I destAddr

state:idle active1 active7 active4 active10 active2 active3 active5 active6 active8 active9

initial:idle

ext:idle active1 And(Less(Value(switchIn),99),Equal(Value(switchIn),A))?1{destAddr=Value(switchIn);}

int:active1 idle switchOut1!destAddr

ext:idle idle Greater(Value(switchIn),99)?1

ext:idle active7 And(Less(Value(switchIn),99),Equal(Value(switchIn),G))?1{destAddr=Value(switchIn);}

int:active7 idle switchOut3!destAddr

ext:idle active4 And(Less(Value(switchIn),99),Equal(Value(switchIn),D))?1{destAddr=Value(switchIn);}

int:active4 idle switchOut2!destAddr

ext:idle active10 And(Greater(Value(switchIn),9),Less(Value(switchIn),99))?1{destAddr=Value(switchIn);}

int:active10 idle packetsDropped!destAddr

ext:idle active2 And(Less(Value(switchIn),99),Equal(Value(switchIn),B))?1{destAddr=Value(switchIn);}

int:active2 idle switchOut1!destAddr

ext:idle active3 And(Less(Value(switchIn),99),Equal(Value(switchIn),C))?1{destAddr=Value(switchIn);}

int:active3 idle switchOut1!destAddr

ext:idle active5 And(Less(Value(switchIn),99),Equal(Value(switchIn),E))?1{destAddr=Value(switchIn);}

int:active5 idle switchOut2!destAddr

ext:idle active6 And(Less(Value(switchIn),99),Equal(Value(switchIn),F))?1{destAddr=Value(switchIn);}

int:active6 idle switchOut2!destAddr

ext:idle active8 And(Less(Value(switchIn),99),Equal(Value(switchIn),H))?1{destAddr=Value(switchIn);}

int:active8 idle switchOut3!destAddr

ext:idle active9 And(Less(Value(switchIn),99),Equal(Value(switchIn),I))?1{destAddr=Value(switchIn);}

int:active9 idle switchOut3!destAddr

idle:99:99:99:99

active1:00:00:01:00

active7:00:00:01:00

active4:00:00:01:00

active10:00:00:01:00

active2:00:00:01:00

active3:00:00:01:00

active5:00:00:01:00

active6:00:00:01:00

active8:00:00:01:00

active9:00:00:01:00

A:1

B:2

C:3

D:4

E:5

F:6

G:7

H:8

I:9

destAddr:0

# Part IV:

# Test Strategies

The atomic models and coupled models have been tested using the “black box” testing method. Test cases are created by adding different combinations of inputs to the event file (.ev). The simulation is run using the batch file (.bat) and the outputs in the output file (.out) are checked to find out whether or not the outputs are as expected.

## Source (atomic):

The source atomic model has zero inputs and two outputs. From the start of the simulation to the end, it generates packets at a constant rate. The packet generation rate is set to 10 seconds per packet which means that the source is generating a packet every ten seconds as shown in the sourceOUT.OUT file below.

00:00:10:000 packetout 7

00:00:10:000 packetsgenerated 7

00:00:20:000 packetout 9

00:00:20:000 packetsgenerated 9

00:00:30:000 packetout 6

00:00:30:000 packetsgenerated 6

00:00:40:000 packetout 3

00:00:40:000 packetsgenerated 3

00:00:50:000 packetout 3

00:00:50:000 packetsgenerated 3

00:01:00:000 packetout 9

00:01:00:000 packetsgenerated 9

00:01:10:000 packetout 1

00:01:10:000 packetsgenerated 1

00:01:20:000 packetout 10

00:01:20:000 packetsgenerated 10

00:01:30:000 packetout 2

00:01:30:000 packetsgenerated 2

00:01:40:000 packetout 6

00:01:40:000 packetsgenerated 6

00:01:50:000 packetout 8

00:01:50:000 packetsgenerated 8

00:02:00:000 packetout 7

00:02:00:000 packetsgenerated 7

00:02:10:000 packetout 11

00:02:10:000 packetsgenerated 11

00:02:20:000 packetout 10

00:02:20:000 packetsgenerated 10

00:02:30:000 packetout 5

00:02:30:000 packetsgenerated 5

00:02:40:000 packetout 10

00:02:40:000 packetsgenerated 10

00:02:50:000 packetout 5

00:02:50:000 packetsgenerated 5

00:03:00:000 packetout 1

00:03:00:000 packetsgenerated 1

The generated packet is send to the output port “packetOut” and the message of the packet is the address of the sink. The address of the sink is selected randomly for each packet between 1 and 11 which is confirmed by the sourceOUT.OUT file shown below. When the source generates a packet, the packet is also sent to a second output port “packetsGenerated” along with the sink address and the same is shown by the sourceOUT.OUT file given below. Later on, the “packetsGenerated” output is used as an output of the Switching-Routing simulator.

## Sink (atomic):

The event file for the sink atomic model sink.EV, which is used as input for the sink, is shown below.

00:00:11:000 packetIn 0

00:00:21:000 packetIn 1

00:00:31:000 packetIn 2

00:00:41:000 packetIn 3

00:00:51:000 packetIn 0

00:01:01:000 packetIn 1

00:01:11:000 packetIn 0

00:01:21:000 packetIn 1

00:01:31:000 packetIn 0

00:01:41:000 packetIn 1

00:01:51:000 packetIn 0

00:02:01:000 packetIn 1

The sink.OUT file which is the output of the sink is shown below.

00:00:22:000 packetsreceived 1

00:01:02:000 packetsreceived 2

00:01:22:000 packetsreceived 3

00:01:42:000 packetsreceived 4

00:02:02:000 packetsreceived 5

The preparation time which is the processing delay of the sink node is set to 1 second. After checking the sink address of the incoming packet, the sink either drops the packet if the destination address of the incoming packet does match the sink address or sends the packet to the output port “packetsReceived” if the destination address of the incoming packet matches the sink address. Therefore, as expected, the sink drops all the packets highlighted in yellow in the event file sink.EV as they do not belong to the sink. However, it sends all the packets highlighted in green in the event file sink.EV to the output port “packetsReceived” as shown in the sink.OUT file. Note that the message in the packet at the input port “packetIn” is the destination address and the message of the packet sent to the output port “packetsReceived” is the packet number. Later on, the “packetsReceived” output is used as an output of the Switching-Routing simulator.

## LAN Switch (atomic):

The event file for the LAN switch atomic model, which is LANswitch.EV, is shown below.

00:00:10:000 switchIn 1

00:00:20:000 switchIn 2

00:00:30:000 switchIn 1

00:00:40:000 switchIn 3

00:00:50:000 switchIn 2

00:01:00:000 switchIn 4

00:01:10:000 switchIn 5

00:01:20:000 switchIn 3

00:01:30:000 switchIn 2

00:01:40:000 switchIn 7

00:01:50:000 switchIn 1

00:02:00:000 switchIn 3

It is assumed that:

- workstation1 with address 1 is connected to the output port “switchOut1”

- workstation2 with address 2 is connected to the output port “switchOut2”

- workstation3 with address 3 is connected to the output port “switchOut3”

The event file LANswitch.EV file serves as input to the LAN switch. The LANswitch_atomicOUT.OUT file which is the output of the switch is shown below. The preparation time which is the switching delay for the LAN switch is set to 1 second. As expected, after checking its destination address, the LAN switch sends the first packet received at 10 seconds (highlighted in green) to the output port “switchOut1” at 11 seconds. Similarly, the LAN switch forwards the next four packets to the appropriate output ports based on their destination address. Note that for the sixth received packet (highlighted in yellow), the destination address is 4. After checking its destination address, the LAN switch sends it to the “gateway” output port since the packet does not belong to any of the directly connected workstations. This is similar to a computer network in which the switch sends the packets not belonging to any nodes connected directly to the switch to the local gateway which is usually a router. Note that the message in the packet at the input and output ports of the LAN switch is the destination (sink) address.

00:00:11:000 switchout1 1

00:00:21:000 switchout2 2

00:00:31:000 switchout1 1

00:00:41:000 switchout3 3

00:00:51:000 switchout2 2

00:01:01:000 gateway 4

00:01:11:000 gateway 5

00:01:21:000 switchout3 3

00:01:31:000 switchout2 2

00:01:41:000 gateway 7

00:01:51:000 switchout1 1

00:02:01:000 switchout3 3

## WAN Switch (atomic):

The event file for the WAN switch atomic model, which is WANswitch.EV, is shown below.

00:00:10:000 switchIn 1

00:00:20:000 switchIn 2

00:00:30:000 switchIn 3

00:00:40:000 switchIn 7

00:00:50:000 switchIn 8

00:01:00:000 switchIn 9

00:01:10:000 switchIn 4

00:01:20:000 switchIn 5

00:01:30:000 switchIn 6

00:01:40:000 switchIn 10

00:01:50:000 switchIn 15

00:02:00:000 switchIn 18

It is assumed that:

- workstation1 with address 1, workstation2 with address 2 and workstation 3 with address 3, can be reached through the output port “switchOut1”

- workstation4 with address 4, workstation5 with address 5 and workstation 6 with address 6 can be reached through the output port “switchOut2”

- workstation7 with address 7, workstation8 with address 8 and workstation9 with address 9 can be reached through the output port “switchOut3”

The event file WANswitch.EV file serves as input to the WAN switch. The WANswitch_atomicOUT.OUT file which is the output of the switch is shown below.

00:00:11:000 switchout1 1

00:00:21:000 switchout1 2

00:00:31:000 switchout1 3

00:00:41:000 switchout3 7

00:00:51:000 switchout3 8

00:01:01:000 switchout3 9

00:01:11:000 switchout2 4

00:01:21:000 switchout2 5

00:01:31:000 switchout2 6

00:01:41:000 packetsdropped 10

00:01:51:000 packetsdropped 15

00:02:01:000 packetsdropped 18

The preparation time which is the switching delay for the WAN switch is set to 1 second. As expected, after checking its destination address, the WAN switch sends the first packet received at 10 seconds (highlighted in green) to the output port “switchOut1” at 11 seconds. Similarly, the WAN switch forwards the next eight packets to the appropriate output ports based on their destination address. Note that for the tenth received packet (highlighted in yellow), the destination address is 10. After checking its destination address, the switch sends it to the “packetsDropped” output port since the packet does not belong to any of workstations. This is similar to a computer network in which the router drops a packet if it does not know the route to the destination. Note that the message in the packet at the input and output ports of the WAN switch is the destination (sink) address. Later on, the “packetsDropped” output is used as an output of the Switching-Routing simulator.

## Workstation (coupled):

The event file for the workstation coupled model, which is workstation.EV, is shown below.

00:00:11:000 packetIn 0

00:00:21:000 packetIn 1

00:00:31:000 packetIn 2

00:00:41:000 packetIn 3

00:00:51:000 packetIn 0

00:01:01:000 packetIn 1

00:01:11:000 packetIn 0

00:01:21:000 packetIn 1

00:01:31:000 packetIn 0

00:01:41:000 packetIn 1

00:01:51:000 packetIn 0

00:02:01:000 packetIn 1

The event file workstation.EV file serves as input to the workstation coupled model. The workstationOUT.OUT file which is the output of the workstation is shown below.

00:00:10:000 packetout 2

00:00:10:000 packetsgenerated 2

00:00:20:000 packetout 4

00:00:20:000 packetsgenerated 4

00:00:22:000 packetsreceived 1

00:00:30:000 packetout 8

00:00:30:000 packetsgenerated 8

00:00:40:000 packetout 2

00:00:40:000 packetsgenerated 2

00:00:50:000 packetout 9

00:00:50:000 packetsgenerated 9

00:01:00:000 packetout 3

00:01:00:000 packetsgenerated 3

00:01:02:000 packetsreceived 2

00:01:10:000 packetout 11

00:01:10:000 packetsgenerated 11

00:01:20:000 packetout 2

00:01:20:000 packetsgenerated 2

00:01:22:000 packetsreceived 3

00:01:30:000 packetout 7

00:01:30:000 packetsgenerated 7

00:01:40:000 packetout 2

00:01:40:000 packetsgenerated 2

00:01:42:000 packetsreceived 4

00:01:50:000 packetout 7

00:01:50:000 packetsgenerated 7

00:02:00:000 packetout 3

00:02:00:000 packetsgenerated 3

00:02:02:000 packetsreceived 5

00:02:10:000 packetout 5

00:02:10:000 packetsgenerated 5

00:02:20:000 packetout 8

00:02:20:000 packetsgenerated 8

00:02:30:000 packetout 10

00:02:30:000 packetsgenerated 10

00:02:40:000 packetout 8

00:02:40:000 packetsgenerated 8

00:02:50:000 packetout 7

00:02:50:000 packetsgenerated 7

00:03:00:000 packetout 10

00:03:00:000 packetsgenerated 10

The workstation coupled model consists of the source atomic and the sink atomic models, as shown in Fig. 6. The packet generation rate for source atomic model inside the workstation is set to 10 seconds and hence, the workstation generates packets every 10 seconds, as highlighted in green in workstationOUT.OUT file. The address of the sink atomic model inside the workstation atomic is set to 1. The workstation checks the sink address of each incoming packet, discards those packets whose sink address is different from the workstation address (highlighted in blue the workstation.EV file) and sends those packets whose sink address matches its own address (highlighted in yellow the workstation.EV file).

## LAN Switch (coupled):

The event file for the LAN switch coupled model, which is LANswitch_coupled.EV, is shown below.

00:00:10:000 switchIn1 4

00:00:20:000 switchIn2 5

00:00:30:000 switchIn3 6

00:00:40:000 switchIn4 1

00:00:50:000 switchIn1 7

00:01:00:000 switchIn2 8

00:01:10:000 switchIn3 9

00:01:20:000 switchIn4 2

00:01:30:000 switchIn4 3

00:01:40:000 switchIn2 3

00:01:50:000 switchIn3 1

00:02:00:000 switchIn1 2

Similar to LAN switch atomic model, it is assumed that:

- workstation1 with address 1 is connected to the output port “switchOut1”

- workstation2 with address 2 is connected to the output port “switchOut2”

- workstation3 with address 3 is connected to the output port “switchOut3”

The event file LANswitch_coupled.EV file serves as input to the LAN switch coupled model. The LANswitch_coupledOUT.OUT file which is the output of the LAN switch coupled model is shown below.

00:00:14:000 gateway 4

00:00:24:000 gateway 5

00:00:34:000 gateway 6

00:00:44:000 switchout1 1

00:00:54:000 gateway 7

00:01:04:000 gateway 8

00:01:14:000 gateway 9

00:01:24:000 switchout2 2

00:01:34:000 switchout3 3

00:01:44:000 switchout3 3

00:01:54:000 switchout1 1

00:02:04:000 switchout2 2

The LAN switch coupled consists of the queue atomic model and LAN switch atomic model, as shown in Fig. 4.  The preparation time of the queue atomic model is set to 3 seconds and the preparation time of the LAN switch atomic model is set to 1 second. Hence, the total switching delay for the LAN switch coupled model is 4 seconds. As expected, after checking its destination address, the LAN switch coupled model sends the first packet received at 10 seconds (highlighted in green) to the output port “gateWay” at 14 seconds since the packet does not belong to any of the directly connected workstations. However, the LAN switch coupled model switches the fourth packet (highlighted in yellow), which is received at 40 seconds and which has a sink address of 1, to the output port “switchOut1” at 44 seconds after a switching delay of 4 seconds.

## WAN Router (coupled):

The event file for the WAN router coupled model, which is WANrouter_coupled.EV, is shown below.

00:00:10:000 routerIn1 4

00:00:20:000 routerIn2 5

00:00:30:000 routerIn3 6

00:00:40:000 routerIn3 7

00:00:50:000 routerIn2 8

00:01:00:000 routerIn1 9

00:01:10:000 routerIn1 7

00:01:20:000 routerIn1 5

00:01:30:000 routerIn2 6

00:01:40:000 routerIn2 10

00:01:50:000 routerIn3 15

00:02:00:000 routerIn3 18

The event file WANrouter_coupled.EV file serves as input to the WAN router coupled model. The WANrouter_coupledOUT.OUT file which is the output of the WAN router coupled model is shown below.

00:00:14:000 routerout2 4

00:00:24:000 routerout2 5

00:00:34:000 routerout2 6

00:00:44:000 routerout3 7

00:00:54:000 routerout3 8

00:01:04:000 routerout3 9

00:01:14:000 routerout3 7

00:01:24:000 routerout2 5

00:01:34:000 routerout2 6

00:01:44:000 packetsdropped 10

00:01:54:000 packetsdropped 15

00:02:04:000 packetsdropped 18

The WAN router coupled consists of the queue atomic model and WAN switch atomic model, as shown in Fig. 5.  Similar to WAN switch atomic model, it is assumed that:

- workstation1 with address 1, workstation2 with address 2 and workstation 3 with address 3, can be reached through the output port “switchOut1”

- workstation4 with address 4, workstation5 with address 5 and workstation 6 with address 6 can be reached through the output port “switchOut2”

- workstation7 with address 7, workstation8 with address 8 and workstation9 with address 9 can be reached through the output port “switchOut3”

The preparation time of the queue atomic model is set to 3 seconds and the preparation time of the WAN switch atomic model is set to 1 second. Hence, the total switching delay for the WAN router coupled model is 4 seconds. As expected, after checking its destination address, the WAN router coupled model routes the first packet received at 10 seconds (highlighted in green) to the output port “routerOut2” at 14 seconds since destination is reachable through that output port. In a similar fashion, the WAN router coupled model routes the next eight packets to the appropriate output ports. For the tenth received packet (highlighted in yellow), the destination address is 10. After checking its destination address, the WAN router sends it to the “packetsDropped” output port since the packet does not belong to any of workstations and hence the destination is unreachable as the route to the destination is not known.

## LAN (coupled):

The event file for the LAN coupled model, which is LANcoupled.EV, is shown below.

00:00:15:000 LANIn 1

00:00:25:000 LANIn 1

00:00:35:000 LANIn 2

00:00:45:000 LANIn 3

00:00:55:000 LANIn 2

00:01:05:000 LANIn 1

00:01:15:000 LANIn 3

00:01:25:000 LANIn 1

00:01:35:000 LANIn 2

00:01:45:000 LANIn 1

00:01:55:000 LANIn 2

00:02:05:000 LANIn 1

The event file LANcoupled.EV file serves as input to the LAN coupled model. The LANcoupledOUT.OUT file which is the output of the LAN coupled model is shown below.

00:00:10:000 packetsgenerated1 5

00:00:11:000 packetsgenerated2 7

00:00:12:000 packetsgenerated3 9

00:00:14:000 lanout 5

00:00:15:000 lanout 7

00:00:16:000 lanout 9

00:00:20:000 packetsgenerated1 3

00:00:20:000 packetsreceived1 1

00:00:22:000 packetsgenerated2 3

00:00:24:000 packetsgenerated3 2

00:00:25:000 packetsreceived3 1

00:00:27:000 packetsreceived3 2

00:00:29:000 packetsreceived2 1

00:00:30:000 packetsgenerated1 7

00:00:30:000 packetsreceived1 2

00:00:33:000 packetsgenerated2 4

00:00:34:000 lanout 7

00:00:36:000 packetsgenerated3 9

00:00:37:000 lanout 4

00:00:40:000 packetsgenerated1 9

00:00:40:000 packetsreceived2 2

00:00:40:000 lanout 9

00:00:44:000 packetsgenerated2 3

00:00:44:000 lanout 9

00:00:48:000 packetsgenerated3 11

00:00:49:000 packetsreceived3 3

00:00:50:000 packetsgenerated1 5

00:00:50:000 packetsreceived3 4

00:00:52:000 lanout 11

00:00:54:000 lanout 5

00:00:55:000 packetsgenerated2 9

00:00:59:000 lanout 9

00:01:00:000 packetsgenerated1 9

00:01:00:000 packetsreceived2 3

00:01:00:000 packetsgenerated3 1

00:01:04:000 lanout 9

00:01:05:000 packetsreceived1 3

00:01:06:000 packetsgenerated2 11

00:01:10:000 packetsgenerated1 5

00:01:10:000 packetsreceived1 4

00:01:10:000 lanout 11

00:01:12:000 packetsgenerated3 8

00:01:14:000 lanout 5

00:01:16:000 lanout 8

00:01:17:000 packetsgenerated2 4

00:01:20:000 packetsgenerated1 8

00:01:20:000 packetsreceived3 5

00:01:21:000 lanout 4

00:01:24:000 packetsgenerated3 2

00:01:24:000 lanout 8

00:01:28:000 packetsgenerated2 5

00:01:29:000 packetsreceived2 4

00:01:30:000 packetsgenerated1 4

00:01:30:000 packetsreceived1 5

00:01:32:000 lanout 5

00:01:34:000 lanout 4

00:01:36:000 packetsgenerated3 10

00:01:39:000 packetsgenerated2 6

00:01:40:000 packetsgenerated1 1

00:01:40:000 packetsreceived2 5

00:01:40:000 lanout 10

00:01:43:000 lanout 6

00:01:45:000 packetsreceived1 6

00:01:48:000 packetsgenerated3 1

00:01:50:000 packetsgenerated1 5

00:01:50:000 packetsreceived1 7

00:01:50:000 packetsgenerated2 3

00:01:53:000 packetsreceived1 8

00:01:54:000 lanout 5

00:01:55:000 packetsreceived3 6

00:02:00:000 packetsgenerated1 6

00:02:00:000 packetsreceived2 6

00:02:00:000 packetsgenerated3 7

00:02:01:000 packetsgenerated2 10

00:02:04:000 lanout 6

00:02:04:000 lanout 7

00:02:05:000 lanout 10

00:02:10:000 packetsgenerated1 6

00:02:10:000 packetsreceived1 9

00:02:12:000 packetsgenerated2 4

00:02:12:000 packetsgenerated3 8

00:02:14:000 lanout 6

00:02:16:000 lanout 4

00:02:16:000 lanout 8

00:02:20:000 packetsgenerated1 1

00:02:23:000 packetsgenerated2 1

00:02:24:000 packetsgenerated3 3

00:02:25:000 packetsreceived1 10

00:02:28:000 packetsreceived1 11

00:02:29:000 packetsreceived3 7

00:02:30:000 packetsgenerated1 10

00:02:34:000 packetsgenerated2 10

00:02:34:000 lanout 10

00:02:36:000 packetsgenerated3 6

00:02:38:000 lanout 10

00:02:40:000 packetsgenerated1 3

00:02:40:000 lanout 6

00:02:45:000 packetsgenerated2 7

00:02:45:000 packetsreceived3 8

00:02:48:000 packetsgenerated3 5

00:02:49:000 lanout 7

00:02:50:000 packetsgenerated1 10

00:02:52:000 lanout 5

00:02:54:000 lanout 10

00:02:56:000 packetsgenerated2 4

00:03:00:000 packetsgenerated1 11

00:03:00:000 packetsgenerated3 2

00:03:00:000 lanout 4

The LAN coupled model consists of the three workstation coupled models and a LAN switch coupled model, as shown in Fig. 2. The packet generation rates of the source atomic models of woksatiton1, workstaion2 and workstation3 are 10, 11 and 12 seconds respectively. Hence, they generate a packet after every 10, 11 and 12 seconds respectively, which appears at the “packetsGenerated1”, “packetsGenerated2” and “packetsGenerated3” output ports of the LAN coupled model. The processing (preparation) time for sink atomic models of all workstations is 1 second. The addresses of the sink atomic models of woksatiton1, workstaion2 and workstation3 are 1, 2 and 3 respectively. The queuing delay of all queue atomic models of the LAN switch coupled model is 3 seconds and switching delay of all LAN switch atomic models is 1 seconds.

Case – I: As expected, the first packet generated by workstation1 at 10 seconds with a sink address of 5 (highlighted in green in the .out file) appears at the “LANOut” output port of the LAN coupled model at 14 seconds (highlighted in green in the .out file) since the packet does not belong to any of the local workstations inside the LAN. A total delay of 4 seconds is experienced due to a queuing delay of 3 seconds and a switching delay of 1 second.

Case – II: As expected, the first packet received at 15 seconds with a sink address of 1 (highlighted in yellow in the .ev file) at the “LANIn” input port of the LAN coupled model is received at workstaion1 at 20 seconds (highlighted in yellow in the .out file) which is shown by the “packetsReceived1” output port of the LAN coupled model. A total delay of 5 seconds is experienced due to a queuing delay of 3 seconds, a switching delay of 1 second and a processing delay of 1 second at the sink.

Case – III: As expected, the packet generated by workstation1 at 20 seconds with a sink address of 3 (highlighted in blue in the .out file) is received at workstaion3 at 25 seconds (highlighted in blue in the .out file) which is shown by the “packetsReceived3” output port of the LAN coupled model. A total delay of 5 seconds is experienced due to a queuing delay of 3 seconds, a switching delay of 1 second and a processing delay of 1 second at the sink.

## WAN (coupled):

The event file for the LAN coupled model, which is WANcoupled.EV, is shown below.

00:00:10:000 WANIn1 4

00:00:20:000 WANIn2 1

00:00:30:000 WANIn3 5

00:00:40:000 WANIn3 6

00:00:50:000 WANIn2 8

00:01:00:000 WANIn1 9

00:01:10:000 WANIn1 7

00:01:20:000 WANIn1 5

00:01:30:000 WANIn2 2

00:01:40:000 WANIn2 10

00:01:50:000 WANIn1 15

00:02:00:000 WANIn3 18

The event file WANcoupled.EV file serves as input to the WAN coupled model. The WANcoupledOUT.OUT file which is the output of the LAN coupled model is shown below.

00:00:14:000 wanout2 4

00:00:24:000 wanout1 1

00:00:34:000 wanout2 5

00:00:44:000 wanout2 6

00:00:54:000 wanout3 8

00:01:04:000 wanout3 9

00:01:14:000 wanout3 7

00:01:24:000 wanout2 5

00:01:34:000 wanout1 2

00:01:42:000 packetsdropped2 10

00:01:52:000 packetsdropped1 15

00:02:02:000 packetsdropped3 18

The WAN coupled model consists of the three WAN router coupled models, as shown in Fig. 3. Similar to WAN switch coupled model, it is assumed that:

- workstation1 with address 1, workstation2 with address 2 and workstation 3 with address 3, can be reached through the output port “WANOut1”

- workstation4 with address 4, workstation5 with address 5 and workstation 6 with address 6 can be reached through the output port “WANOut2”

- workstation7 with address 7, workstation8 with address 8 and workstation9 with address 9 can be reached through the output port “WANOut3”.

The queuing delay of all queue atomic models of the WAN router coupled models and switching delay of all WAN switch atomic models is set to 1 seconds.

Case – I: As expected, the first packet received at 10 seconds with a sink address of 4 (highlighted in yellow in the .ev file) at the “WANIn1” input port of the WAN coupled model appears at the “WANOut2” output port of the WAN coupled model at 14 seconds (highlighted in yellow in the .out file). A total delay of 4 seconds is experienced due to a queuing delay of 1 second and a switching delay of 1 second inside WAN router1 and a queuing delay of 1 second and a switching delay of 1 second inside WAN router2. In a similar fashion, the WAN coupled model routes the next eight packets received to the appropriate output ports.

Case – II: As expected, the tenth packet received with a sink address of 10 (highlighted in green in the .ev file) at the “WANIn2” input port of the WAN coupled model appears at the “packetsDropped2” output port of the WAN coupled model (highlighted in green in the .out file). A total delay of 2 seconds is experienced due to a queuing delay of 1 second and a switching delay of 1 second inside WAN router2.

## Switching-Routing Simulator (LANWAN (coupled)):

There is no event file for the LANWAN coupled model since the Switching-Routing simulator has no inputs. The LANWANcoupledOUT.OUT file which is the output of the LANWAN coupled model is shown below.

00:00:10:000 packetsgenerated1 4

00:00:11:000 packetsgenerated2 7

00:00:12:000 packetsgenerated3 9

00:00:14:000 packetsgenerated4 7

00:00:15:000 packetsgenerated5 3

00:00:16:000 packetsgenerated6 6

00:00:17:000 packetsgenerated7 10

00:00:18:000 packetsgenerated8 10

00:00:19:000 packetsgenerated9 1

00:00:20:000 packetsgenerated1 7

00:00:21:000 packetsreceived6 1

00:00:22:000 packetsgenerated2 11

00:00:23:000 packetsdropped3 10

00:00:23:000 packetsreceived4 1

00:00:24:000 packetsgenerated3 1

00:00:25:000 packetsdropped3 10

00:00:25:000 packetsreceived7 1

00:00:28:000 packetsdropped1 11

00:00:28:000 packetsgenerated4 9

00:00:29:000 packetsreceived1 1

00:00:29:000 packetsreceived3 1

00:00:29:000 packetsreceived9 1

00:00:30:000 packetsgenerated1 10

00:00:30:000 packetsgenerated5 9

00:00:32:000 packetsgenerated6 8

00:00:33:000 packetsgenerated2 8

00:00:33:000 packetsreceived7 2

00:00:34:000 packetsreceived1 2

00:00:34:000 packetsgenerated7 9

00:00:36:000 packetsdropped1 10

00:00:36:000 packetsgenerated3 5

00:00:36:000 packetsgenerated8 6

00:00:37:000 packetsreceived7 3

00:00:38:000 packetsgenerated9 3

00:00:39:000 packetsreceived9 2

00:00:40:000 packetsgenerated1 11

00:00:41:000 packetsreceived9 3

00:00:42:000 packetsgenerated4 7

00:00:44:000 packetsgenerated2 6

00:00:45:000 packetsgenerated5 8

00:00:45:000 packetsreceived9 4

00:00:46:000 packetsdropped1 11

00:00:48:000 packetsgenerated3 3

00:00:48:000 packetsgenerated6 9

00:00:49:000 packetsreceived5 1

00:00:49:000 packetsreceived8 1

00:00:50:000 packetsgenerated1 6

00:00:51:000 packetsreceived3 2

00:00:51:000 packetsgenerated7 3

00:00:53:000 packetsreceived3 3

00:00:53:000 packetsreceived6 2

00:00:53:000 packetsreceived8 2

00:00:54:000 packetsgenerated8 3

00:00:55:000 packetsgenerated2 2

00:00:56:000 packetsgenerated4 6

00:00:57:000 packetsreceived6 3

00:00:57:000 packetsreceived7 4

00:00:57:000 packetsgenerated9 10

00:01:00:000 packetsgenerated1 9

00:01:00:000 packetsreceived2 1

00:01:00:000 packetsgenerated3 5

00:01:00:000 packetsgenerated5 3

The LANWAN coupled model consists of the three LANs connected through a WAN, as shown in Fig. 1. LAN1 consists of workstaion1, workstaion2, workstaion3 and a LAN switch. LAN2 consists of workstaion4, workstaion5, workstaion6 and a LAN switch. LAN3 consists of workstaion7, workstaion8, workstaion9 and a LAN switch. The WAN consists of three WAN routers. The addresses or workstation1, workstation2, workstation3, workstation4, workstation5, workstation6, workstation7, workstation8 and workstation9 are 1, 2, 3, 4, 5, 6, 7, 8 and 9 respectively.

The delays (preparation times) of the various atomic models inside the LANWAN coupled model are as follows:

queue atomic models of all WAN routers : 1 second

switch atomic models of all WAN routers : 1 second

queue atomic models of all LAN switches : 3 second

switch atomic models of all LAN switches : 1 second

sink atomic models of all workstations : 1 second

source atomic model of workstation1 : 10 seconds

source atomic model of workstation2 : 11 seconds

source atomic model of workstation3 : 12 seconds

source atomic model of workstation4 : 14 seconds

source atomic model of workstation5 : 15 seconds

source atomic model of workstation6 : 16 seconds

source atomic model of workstation7 : 17 seconds

source atomic model of workstation8 : 18 seconds

source atomic model of workstation9 : 19 seconds

Case – I: As expected, the first packet generated by workstation1 at 10 seconds with a sink address of 4 (highlighted in yellow in the .out file) is received at workstaion4 at 23 seconds (highlighted in green in the .out file) which is shown by the “packetsReceived3” output port of the LANWAN coupled model. Note that this packet generated by workstation1 in LAN1 is destined for workstation4 in LAN2. A total delay of 13 seconds is experienced due to the following:

- a delay of 4 seconds in the LAN switch of LAN1 including a queuing delay of 3 seconds and a switching delay of 1 second

- a delay of 4 seconds in the WAN including a delay of 2 seconds in WAN router1 and a delay of 2 seconds in WAN router2

- a delay of 4 seconds in the LAN switch of LAN2 including a queuing delay of 3 seconds and a switching delay of 1 second

- a delay of 1 second in the sink atomic model of the workstation4

Case – II: As expected, the packet generated by workstation7 at 17 seconds with a sink address of 10 (highlighted in yellow in the .out file) appears at the “packetsDropped3” output port of the LANWAN coupled model at 23 seconds (highlighted in yellow in the .out file). Note that this packet is generated by workstation7 in LAN3 but is dropped at the WAN router3 since the path to the destination is not known. A total delay of 6 seconds is experienced due to the following:

- a delay of 4 seconds in the LAN switch of LAN3 including a queuing delay of 3 seconds and a switching delay of 1 second

- a delay of 2 seconds in the WAN including a delay of 2 seconds in WAN router3

Case – III: As expected, the packet generated by workstation3 at 24 seconds with a sink address of 1 (highlighted in blue in the .out file) is received at workstaion1 at 29 seconds (highlighted in blue in the .out file) which is shown by the “packetsReceived1” output port of the LANWAN coupled model. Note that this packet generated by workstation3 in LAN1 is destined for workstation1 in the same LAN. A total delay of 5 seconds is experienced due to the following:

- a delay of 4 seconds in the LAN switch of LAN1 including a queuing delay of 3 seconds and a switching delay of 1 second

- a delay of 1 second in the sink atomic model of the workstation1

Case – VI: As expected, the packet generated by workstation8 at 18 seconds with a sink address of 10 (highlighted in gray in the .out file) appears at the “packetsDropped3” output port of the LANWAN coupled model at 25 seconds (highlighted in gray in the .out file). Note that this packet is generated by workstation8 in LAN3 but is dropped at the WAN router3 since the path to the destination is not known. A total delay of 7 seconds is experienced due to the following:

- a delay of 4 seconds in the LAN switch of LAN3 including a queuing delay of 3 seconds and a switching delay of 1 second

- a delay of 3 seconds in the WAN. This includes a delay of 2 seconds in the “queue7” of WAN router3 and a delay of 1 second in the “switch7”. Note that the packet experiences a delay of 2 seconds in the “queue7” of WAN router3 instead of the normal queuing delay of 1 second as there is already a packet (which was generated by workstation7 at 17seconds)  in the “queue7” ahead of this packet .

Case – V: As expected, the packet generated by workstation9 at 19 seconds with a sink address of 1 (highlighted in red in the .out file) is received at workstaion1 at 34 seconds (highlighted in red in the .out file) which is shown by the “packetsReceived1” output port of the LANWAN coupled model. Note that this packet generated by workstation9 in LAN3 is destined for workstation1 in LAN1. A total delay of 15 seconds is experienced due to the following:

- a delay of 4 seconds in the LAN switch of LAN1 including a queuing delay of 3 seconds and a switching delay of 1 second

- a delay of 6 seconds in the WAN. This includes a delay of 4 seconds in WAN router3 and a delay of 2 seconds in the WAN router1. In WAN router3, the packet experiences a delay of 3 seconds in the “queue7” of WAN router3 and a delay of 1 second in the “switch7”. Note that the packet experiences a delay of 3 seconds in the “queue7” of WAN router3 instead of the normal queuing delay of 1 second as there are already two packets (first generated by workstation7 at 17seconds and second generated by workstation8 at 18seconds)  in the “queue7” ahead of this packet .

- a delay of 4 seconds in the LAN switch of LAN1 including a queuing delay of 3 seconds and a switching delay of 1 second

- a delay of 1 second in the sink atomic model of the workstation1

# Part V:

# Modification to the CD++ Builder Tool

During the modeling of the source atomic model using the DEVS Graph utility of the CD++ Builder Tool, the following problem was encountered due to which the source code of the CD++ Builder Tool had to be modified.

While using the "Action Function" [7] Rand(1,11) to generate a random number between 1 and 11 to be used as the sink address (message) of the packet generated in the source atomic model, the random number generated was a real number such as "6.20501", and hence, was required to be rounded off to an integer. However, there was no action function for rounding in the CD++ Builder and the same is confirmed by Fig. 2 in [7].

So, a new action function "round" was added to the DEVS Graph facility of the CD++ Builder Tool using the following procedure.

- The source code for the CD++ Builder Tool was downloaded from the following link:

http://cell-devs.sce.carleton.ca/mediawiki/index.php/Image:Cd%2B%2B_src_StandAloneWithDEVSGraphs.zip

- The source code was unzipped to the following folder:

C:\cd++_srcToolkit

- A new file "ggadfuncround.h" containing the new action function "round" was added to the following folder of the source code:

C:\cd++_srcToolkit\models\ggad

The contents of the file "ggadfuncround.h" are as follows.

// $Header$

#ifndef __GGADFUNCROUND_H__

#define __GGADFUNCROUND_H__

#include "ggadfunc.h"

#include "math.h"

class GgadFuncRound : public GgadFunc {

public:

GgadFuncRound() {

setFormalParams(1);

}

virtual ~GgadFuncRound() {}

virtual std::string name() { return "round"; }

virtual GgadValue executeImpl () {

double vv;

GgadValue v = modf( getParameter(0), &vv );

if ( v >= 0.5 )

return ceil( getParameter(0) );

else

return floor( getParameter(0) );

}

virtual GgadFuncRound* create() { return new GgadFuncRound(); }

};

#endif // __GGADFUNCROUND_H__

- The new action function "round" was registered in the file "ggadfuncregister.cpp" of the source code.

The file "ggadfuncregister.cpp" was located in the following folder of the source code.

C:\cd++_srcToolkit\models\ggad

The modifications which were made to the file "ggadfuncregister.cpp" in order to register the new action function "round" are highlighted in green and the contents of the "ggadfuncregister.cpp" are as follows.

#include "ggadsymboltable.h"

#include "ggadfunc.h"

// functions to be registered for use in models

#include "ggadfuncvalue.h"

#include "ggadfuncadd.h"

#include "ggadfuncminus.h"

#include "ggadfuncmultiply.h"

#include "ggadfuncdivide.h"

#include "ggadfuncbetween.h"

#include "ggadfuncpow.h"

#include "ggadfunccompare.h"

#include "ggadfuncany.h"

#include "ggadfuncpi.h"

#include "ggadfuncequal.h"

#include "ggadfuncnotequal.h"

#include "ggadfuncand.h"

#include "ggadfuncor.h"

#include "ggadfuncnot.h"

#include "ggadfuncrand.h"

#include "ggadfuncless.h"

#include "ggadfuncgreater.h"

#include "ggadfuncgreaterequal.h"

#include "ggadfuncround.h"

/** Register builtin functions to be used.

@todo Use GgadFunc::name() to get the name to register. */

void GgadSymbolTable::RegisterFunctions() {

addFunction( "value", new GgadFuncValue() );

addFunction( "add", new GgadFuncAdd() );

addFunction( "minus", new GgadFuncMinus() );

addFunction( "multiply", new GgadFuncMultiply() );

addFunction( "divide", new GgadFuncDivide() );

addFunction( "pow", new GgadFuncPow() );

addFunction( "between", new GgadFuncBetween() );

addFunction( "compare", new GgadFuncCompare() );

addFunction( "any", new GgadFuncAny() );

addFunction( "pi", new GgadFuncPi() );

addFunction( "equal", new GgadFuncEqual() );

addFunction( "notequal", new GgadFuncNotEqual() );

addFunction( "and", new GgadFuncAnd() );

addFunction( "or", new GgadFuncOr() );

addFunction( "not", new GgadFuncNot() );

addFunction( "rand", new GgadFuncRand() );

addFunction( "less", new GgadFuncLess() );

addFunction( "greater", new GgadFuncGreater() );

addFunction( "greaterequal", new GgadFuncGreaterEqual() );

addFunction( "round", new GgadFuncRound() );

}

- After adding the new action function "round" to the CD++ Builder source code and registering it in the source code, the modified source code was compiled in the "cygwin" environment using the "make clean" and then "make" commands.

- The new "simu.exe" file containing the new simulator was created in the following folder:

C:\cd++_srcToolkit\

- The "simu.exe" file was copied to the internal folder of the Eclipse [9].

- A backup copy was made of the "simuOrig.exe" file. This file was already in the internal folder of the Eclipse.

- The "simu.exe" file copied earlier to the internal folder of the Eclipse was renamed as "simuOrig.exe".

- The text editor of the CD++ Builder Tool  in the Eclipse (or Notepad) was used to modify the "source.cdd" (file containing the text version of the source atomic model) to implement the newly added action function "round" as follows:

sinkAddr=Round(Rand(1,11));

- After the above modification to the "source.cdd", the DEVS Graph based source atomic model was tested by running the "source.bat" file and the model simulated properly on the command line as well as in the Eclipse.

- However, it should be noted that after adding the new action function to the DEVS Graph, it was no longer possible to modify the "source.ggaddesigner" (file containing the graphical version of the source atomic model). So, any further changes to the source atomic model were made by modifying the "source.cdd" file.

# CONCLUSION

The Switching-Routing Simulator models and simulates the traffic generation, switching, routing and traffic reception between nodes in a computer network using DEVS formalism through the CD++ Builder tool and generates expected results. The behaviors and features of the source (atomic), sink (atomic), LAN switch (atomic), WAN switch (atomic),  workstation (coupled), LAN switch (coupled),  WAN router (coupled), LAN (coupled), WAN (coupled) and LANWAN (coupled) are simulated by the respective models. The message of the packet is simplified as sink (destination) address in this simulator which is a random number between 1 and 11. The testing cases verify the specifications of models. The Switching-Routing Simulator works exactly as expected according to the specifications.

# References

- The VINT Project, “Network Simulator (NS) – version 2,” available at http://www.isi.edu/nsnam/ns.

- OPNET Modeler documentation, OPNET Technologies, Inc, available at http://www.opnet.com/

- Zeigler, B., “Theory of modeling and simulation,” Wiley, 1976.

- Zeigler, B.; Kim, T.; Praehofer, H., “Theory of Modeling and Simulation: Integrating Discrete Event and Continuous Complex Dynamic Systems,” Second Edition, Academic Press, 2000.

- Chidisiuc, C.; Wainer G. 2007, “CD++Builder: An Eclipse-Based IDE for DEVS Modeling,” Proceedings of SpringSim 2007, Norfolk, VA, USA.

- Matias Bonaventura, Gabriel A. Wainer, Rodrigo Castro, “Advanced IDE for Modeling and Simulation of Discrete Event Systems,” Proceedings of 2010 Spring Simulation Conference (SpringSim10), DEVS Symposium - April 2010.

- Gaston Christen, Alejandro Dobniewski, Gabriel A. Wainer, “Modeling State-Based DEVS Models in CD++,” Proceedings of MGA, Advanced Simulation Technologies Conference (ASTC04), Arlington, VA. U.S.A - 2004.

- Gabriel A. Wainer, Qi Liu, “Tools for Graphical Specification and Visualization of DEVS Models,” SIMULATION: Transactions of the Society for Modeling and Simulation International, Volume 85, Number 3, page 131-158 - Mar. 2009.

- Object Technology International Inc., “Eclipse Platform Technical Overview,” available at http://www.eclipse.org/whitepapers/eclipse-overview.pdf, 2001.