Asymmetric energy requirements in WSNs

Advanced CELL-DEVS Model Simulation Using enhanced CD++

Mohammad El-Shabani

EECS

University of Ottawa

Ottawa, ON, Canada

M.elshabani@uottawa.ca

Gabriel Wainer

Department of Systems and Computer Engineering

Carleton University

Ottawa, ON, Canada

gwainer@sce.carleton.ca

Abstract—This paper illustrates the importance of careful energy requirement planning in WSN to optimize and reduce cost. An experiment is carried using the enhanced CD++ tool and interesting results were generated that cover an adequate range of realistic scenarios

Keywords—WSN, Energy, CD++, Cell-DEVS, requirement planning

# Introduction

Wireless Sensor Networks is a very active research area today; this is due to the significance of applications envisioned to utilize this technology; in addition, the continuous advancements in technology embodied in the continuous miniaturization of integrated circuits and RF technologies that enable the development of WSNs. However, the large amount of intensive research dedicated to this area is generated by the unique problems created by the idea that require novel solutions; this is because WSNs is different than other types of networks that have been developing for the last three decades.

Energy requirements of nodes is one of topics receiving most attention, this is because it is the most limiting factor when it comes to implementing WSNs. In this paper, energy requirement variance of nodes in a WSN is the topic of interest.

However, one similarity is the use of modeling and simulation to test and understand the operation of networks; this is done by emulating certain behavior to the point where we could say that it matches real behavior within the experimental frame of interest. The use of simulations has become so popular due to the complexity of implementing and proving the correctness of distributed and real-time systems, which makes it very convenient to model and simulate this behavior since it reduces the time, effort and cost required.

CD++ is a DEVS (Discrete Event System Specification) based tool that can be used to model and simulate coupled models specified in a minimalistic rule set that is specified by the user, effectively reducing the effort required to model and simulate at least an order of magnitude. In addition, it provides the ability to specify and simulate cellular automata embodied in the Cell-DEVS formalism in addition to a tool to visualize the simulation as will be seen later.

In previous work,   the CD++ tool was used to simulate and visualize the energy requirement variance in WSN, but it was limited due to not being able to define state variables; however, the enhanced version of CD++ provides this functionality which enabled the improvement of the original model and running more realistic test cases, which will be discussed later.

Following in this paper, some background topics will be discussed in Section 2, providing essential context of the work presented; in Section 3, the enhanced model is defined and discussed in detail. Finally, in section 4, the simulation results for various test cases are presented in addition to a performance analysis comparing the old and new CD++ tool.

# Background

## What are WSNs?

A WSN (Wireless Sensor Network) consists of spatially distributed set of nodes that are able to communicate with an ad-hoc manner; In other words, they do not require any infrastructure to work. This is mainly because each node functions as a wireless router that is connected to a set of neighbors; then, each node forwards packets according to some routing algorithm it implements that is supposed to optimize the path according to a selected parameter, such as the number of hops or energy consumed.

One of the potential applications that drive research in WSN (Wireless Sensor Networks) is data collection. This application is envisioned as being able to deploy hundreds or thousands of enabled nodes in a random or specific manner. Those nodes will be able to auto-configure by setting up ad-hoc connection. An example is throwing the nodes out of a helicopter in human non-accessible areas such as volcanic centers or rainforests. Some other data collection applications include air quality monitoring in cities, water quality monitoring, structural health as in building and bridges.

Those nodes usually have constrained resources to reduce costs and energy consumption. Each node has a microcontroller chip; a 4MHz MC with 4 KB of RAM and 16 KB of ROM is typical. Leight weight operating systems are developed to be able to run on such constrained nodes, such as: TinyOS and ContikiOS, which made creating new applications on top of the OS much easier.   A node is usually equipped with certain sensors to collect data about their environment in a specified variable rate.

But how are networks interfaced with the outside world, or how is the collected data transferred to the user? A sink is used, all data collected is forwarded to the sink which act as a gateway to the WSN or alternatively forwards all data using some other technology such as GPRS or 802.11 to be stored in a database. A sink is able to do that because it is usually a more powered entity that is able to handle higher data transmission and energy consumption; such a node usually either has power connectivity or is equipped with energy collection method such as a solar panel.

## Energy in WSN

Energy efficiency is one of the most important factors in WSNs, since nodes must run for longest possible period of time while powered by a constrained battery; it is inconvenient or unviable to change the batteries of a big number of nodes; it would be a tedious process to change batteries for hundreds or thousands of nodes which could be difficult to access in the first place.

Energy in WSN nodes  is consumed by a number of activities or states of a node; for example, computation activity in the MCU, data transmission, data reception, idling and sleeping; the most energy consuming activity is data transmission, and the other factors fade in comparison; this is why it is considered more efficient to do more computation if it can reduce the size of data transmitted. To conclude, data transmission is considered the main energy consumption factor in WSN; thus, a nodes life time is measured by the amount of data it transmits.

The sink connects the WSN to the outside world; thus, data collected is forwarded to a sink either continuously or on demand. The sink will have to transmit the largest amount; but this is fine since it is equipped to do so; however, energy requirements of nodes around the sink are asymmetrical. Nodes that are closer to the sink will be on the path of more nodes towards the sink, thus, will forward more packets. In addition, obstacles in the field create additional variance in energy consumption. This will be illustrated in the model and simulation to be presented in this document.

The variance of energy requirements of nodes makes it harder to plan the power source capacity; it creates a compromise between the early death of nodes around the source or wasted battery nodes away from the sink. There are many possible solutions such as deploying a bigger number of nodes around the sink which are put to sleep according to the topology control algorithm; but it is still required to carefully plan the network. Simulation tools are commonly used to forecast the behavior of WSNs; in fact, they are used for all kinds of distributed and real time systems since it is very difficult to deterministically prove their behavior, and it would be too expensive and time consuming to test actual implementations.

## Cell-DEVS and the opportunity in Advanced Cell-DEVS

The Cell-DEVS is a cell based discrete event modeling and simulation method. It can simulate any cellular automata model; where a cellular automata consists of a grid of cells that could be in any number of dimensions; the most common being 2D automata. In addition, each cell has a number of states and a number of rules that are used to decide what its next state is based on the states of neighboring cells. Cell-DEVS simulations fit a wide range of problems.

CD++ is a powerful tool that enables the implementation and simulation of cellular automata; which was used in previous work to simulate a model of data collection WSN.

The model defined in the previous implementation involved five planes. Two of which were required to illustrate results, and others were used as state variables. This is because in this version of CD++ no state variables can be defined and all rules are only based on the states of the defined neighborhood. The variable of the first plane is the number of packets to be forwarded in the next step (packets in buffer). The other plane is for monitoring energy consumption.

Figure 1: Cell neighborhood of old Cell-DEVS model which was implemented in the original CD++ tool

The other two planes are used to generate new packets; the first plane is a rotation plane in which all nodes are initialized to a random number and rotate from 0 to 9; the other plane is set to one when the corresponding node in the rotation plane is equal to 1. Thus, all nodes generate packet with the same period, however they are not synchronized.

The sink is located in the lower left end in the suggested model. This because it is simpler to correctly follow the shortest path in such a scenario. To be able to locate a sink anywhere would require additional planes which will increase the computation time of running simulations as the number of cell entities increases.

In addition, nodes in the main plane read the states of energy and blocked planes associated with neighboring nodes; this is to make the decision on where to forward packets. Cells in the energy plane set themselves to a very large number if the associated cell in the blocked plane is set; thus, other nodes will not assume that packets are forwarded by the blocked cells.

The three extra planes simplify and reduce the rule set because they help account for many special cases in an ingenious way; for example, the new packet plane reduced the set of rules to half; In addition, before adding the blocked plane, a blocked cell was represented by setting a cell in the main plane to -1; however, the model required more than 20 rules to account for special cases. After introducing the plane, the cell in the main plane remains zero; this, in addition to setting corresponding cells in the energy plane to a very large number, eliminated all but one special case because they fit correctly.

In addition, the DRAWLOG tool enable users to visualize the output of simulations and make them useful; using the tool generates a text file that is formatted to illustrate the states of cells at each time step. This “.drw” file can then be used  by a visualization applet to generate visualization as shown in figure below.

Figure 2: Visualization of simulation results of the simple WSN energy requirement model. The variance from the sink node at the lower left corner to the top right corner can be seen; In addition, the variance created by blocks is illustrated as well.

In other words, it would be impossible to reduce the current implementation to a single plane using the older CD++ tool. This is because it is not possible to define state variables and extra planes are used instead. This also limits the functionality of the model by requiring only on sink to exist which is not necessarily the best solution; In addition, the sink needs to be located at the corner to simplify routing rules, which is not realistic.

The older version of CD++ was a good starting point and generated interesting results; but we are interested in enhancing the model to simulate more realistic cases. This is where the enhanced version of CD++ which implements advanced Cell-DEVS can help. In this version, it is possible to define as many state variables as required, in addition to ports which are used to be able to check publish the variable state to neighbors. In addition, the enhanced DRAWLOG tool can take an additional parameter indicating the port to be used; this is useful to visualize the values of any state variable in the model, removing the need of any extra planes for that purpose.

# Models Defined

## Conceptual Model

The model defined involves a single plane of cells and each cell has 8 state variables. A size of 20x20 was used since it models a realistic implementation. To simplify implementation, a modular design is used where forwarding in separated from routing; this was found to be much simpler that the older implementation.

An initialization phase starts by setting up the routing parameters or nodes; each node that does not have routing information looks if one of its neighbors does or if one of them is a sink, then, it defines the route by setting a route variable pointing at that node. There are actually three routing variables; the main one which is used for forwarding, and two others which are alternative options; the main routing variable is evaluated at each time step according the energy consumption of possible destinations; it chooses the one that has consumed less energy. The first alternative is the first route found by the node as described earlier, while the second alternative is any other possible path that is not the same as the first one and where the other node does not point back to the original one.

After the initialization phase is done for some cell, new packets start being generated one for every 10 time steps; at the beginning, a rotation is set up and for each 10 time steps, all cells will have send a single packet at a random point in that period. This is why a rotation variable is required.

At each time step, a node checks the routing of neighboring cells and collects packets that are pointing towards it by looking at the their packets variable in addition to packets generated by itself. Then, it sets that number to its own packets variable. I

The energy consumed by each node is evaluated to be the total number of packets a node forwarded; thus, the energy consumed variable counts that number and this variable is used to visualize the results of the model.

An additional variable is required, the entity variable. This one is initialized at the beginning to choose which cells are regular nodes, which are cells and which are blocking cells meaning that there is no node existing at this cell. This variable is used to create various test cases that will be shown later.

## Formal specifications

CD = < X, Y, S, N, d, int, ext, , , D >

X = Ø

Y = Ø

η=5

S  R (number of packets in buffer)

N { (0,-1,0), (-1,0,0), (1, 0,0), (0, 1,0), (0,0,0)}

d = 10 ms

τ: NS: the local transition function is indicated in the .ma file below.

## CD++ Implementation

The model described above was implemented in CD++ as following:

#include(shorts.inc) [top] components : wsn [wsn] type : cell dim : (20,20) delay : transport defaultDelayTime : 10 border : unwrapped neighbors : wsn(-1,-1) wsn(-1,0) wsn(-1,1) neighbors : wsn(0,-1) wsn(0,0) wsn(0,1) neighbors : wsn(1,-1) wsn(1,0) wsn(1,1) initialvalue : 0 localtransition : wsnrule statevariables : route route1 route2 entity energy packets rotation newPacket statevalues : 0 0 0 0 0 0 0 0 neighborports : route1 route2 route entity energy packets rotation newPacket initialvariablesvalue : wsn.stvalues [wsnrule] rule : {~route := $route1; ~route1 := $route1; ~route2:= $route2;#macro(routinePort)}{$route1 := -1; $route2 := -1;#macro(routineAssign) } 10 {$entity = 1} rule : {~route := $route1; ~route1 := $route1; ~route2:= $route2;#macro(routinePort)}{$route1 := 1; $route2 := 1;#macro(routineAssign) } 10 {$entity =2} rule : {~route := $route1; ~route1 := $route1;#macro(routinePort)}{$route1 := 2;#macro(routineAssign) } 10 {$route1 = 0 and (0,1)~route1 >0} rule : {~route := $route1; ~route1 := $route1;#macro(routinePort)}{$route1 := 3;#macro(routineAssign) } 10 {$route1 = 0 and (-1,0)~route1 >0} rule : {~route := $route1; ~route1 := $route1;#macro(routinePort)}{$route1 := 4;#macro(routineAssign) } 10 {$route1 = 0 and (0,-1)~route1 >0} rule : {~route := $route1; ~route1 := $route1;#macro(routinePort)}{$route1 := 5;#macro(routineAssign) } 10 {$route1 = 0 and (1,0)~route1 >0} rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 2;#macro(routineAssign) } 10 {$route2 =0 and $route1 !=0 and $route1 != 2 and (0,1)~route >0 and (0,1)~route1 !=4 and (0,1)~route2 !=4 } rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 3;#macro(routineAssign) } 10 {$route2 =0 and $route1 !=0 and $route1 != 3 and (-1,0)~route >0 and (-1,0)~route1 !=5 and (-1,0)~route2 !=5 } rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 4;#macro(routineAssign) } 10 {$route2 =0 and $route1 !=0 and $route1 != 4 and (0,-1)~route >0 and (0,-1)~route1 !=2 and (0,-1)~route2 !=2 } rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 5;#macro(routineAssign) } 10 {$route2 =0 and $route1 !=0 and $route1 != 5 and (1,0)~route >0 and (1,0)~route1 !=3 and (1,0)~route2 !=3 } rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 0;#macro(routineAssign) } 10 {$route2 =0 and $route1 !=0 and ($route1 = 2 or (0,1)~route =4) and ($route1 =4 or (0,-1)~route =2) and ( $route1 =3 or (-1,0)~route =5)and ($route1 =5 or (1,0)~route =3)} rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 0;#macro(routineAssign) } 10 {$route2 =2 and ( (0,1)~route1 =4 or (0,1)~route2 =4) and $energy <(0,1)~energy } rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 0;#macro(routineAssign) } 10 {$route2 =4 and ( (0,-1)~route1 =2 or (0,-1)~route2 =2) and $energy <(0,-1)~energy } rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 0;#macro(routineAssign) } 10 {$route2 =3 and ( (-1,0)~route1 =5 or (-1,0)~route2 =5) and $energy <(-1,0)~energy } rule : {~route := $route1; ~route2:= $route2;#macro(routinePort)}{$route2 := 0;#macro(routineAssign) } 10 {$route2 =5 and ( (1,0)~route1 =3 or (1,0)~route2 =3) and $energy <(1,0)~energy } rule : {~route := $route;#macro(routinePort)}{$route := 2;#macro(routineAssign) } 10 {(( $route2 =2 and $route1 =3) or ( $route2 =3 and $route1 =2)) and (0,1)~energy < (-1,0)~energy } rule : {~route := $route;#macro(routinePort)}{$route := 3;#macro(routineAssign) } 10 {(( $route2 =2 and $route1 = 3) or ( $route2 =3 and $route1 = 2)) } rule : {~route := $route;#macro(routinePort)}{$route := 2;#macro(routineAssign) } 10 {(( $route2 =2 and $route1 = 4) or ( $route2 =4 and $route1 = 2)) and (0,1)~energy < (0,-1)~energy } rule : {~route := $route;#macro(routinePort)}{$route := 4;#macro(routineAssign) } 10 {(( $route2 =2 and $route1 = 4) or ( $route2 =4 and $route1 = 2)) } rule : {~route := $route;#macro(routinePort)}{$route := 2;#macro(routineAssign) } 10 {(( $route2 =2 and $route1 = 5) or ( $route2 =5 and $route1 = 2)) and (0,1)~energy < (1,0)~energy } rule : {~route := $route;#macro(routinePort)}{$route := 5;#macro(routineAssign) } 10 {(( $route2 =2 and $route1 = 5) or ( $route2 =5 and $route1 = 2)) } rule : {~route := $route;#macro(routinePort)}{$route := 3;#macro(routineAssign) } 10 {(( $route2 =4 and $route1 = 3) or ( $route2 =3 and $route1 = 4)) and (-1,0)~energy < (0,-1)~energy } rule : {~route := $route;#macro(routinePort)}{$route := 4;#macro(routineAssign) } 10 {(( $route2 =4 and $route1 = 3) or ( $route2 =3 and $route1 = 4)) } rule : {~route := $route;#macro(routinePort)}{$route := 3;#macro(routineAssign) } 10 {(( $route2 =5 and $route1 = 3) or ( $route2 =3 and $route1 = 5)) and (-1,0)~energy < (1,0)~energy } rule : {~route := $route;#macro(routinePort)}{$route := 5;#macro(routineAssign) } 10 {(( $route2 =5 and $route1 = 3) or ( $route2 =3 and $route1 = 5)) } rule : {~route := $route;#macro(routinePort)}{$route := 4;#macro(routineAssign) } 10 {(( $route2 =4 and $route1 = 5) or ( $route2 =5 and $route1 = 4)) and (0,-1)~energy < (1,0)~energy } rule : {~route := $route;#macro(routinePort)}{$route := 5;#macro(routineAssign) } 10 {(( $route2 =4 and $route1 = 5) or ( $route2 =5 and $route1 = 4)) } rule : {~rotation := $rotation;#macro(routinePort)}{ #macro(routineAssign) } 10 { t}

Figure 3:The new improved model implemented in the enhanced CD++ using multiple state variables and a single plane.

Where shorts.inc that is imported in the previous snippet is:

#BeginMacro (routineAssign) $rotation := remainder ($rotation+1,10); $newPacket := ifu($rotation = 1 and $route != 0 ,1,0,0); $packets := if ( $entity != 1, $newPacket + ifu( (0,1)~route = 4 , (0,1)~packets ,0,0) + ifu( (-1,0)~route = 5 , (-1,0)~packets ,0,0) + ifu( (0,-1)~route = 2 , (0,-1)~packets ,0,0) + ifu( (1,0)~route = 3 , (1,0)~packets ,0,0),-1); $energy := if ( $entity != 1 , $energy + $packets , 10000 ); #EndMacro #BeginMacro (routinePort) ~rotation := $rotation; ~newPacket := $newPacket; ~packets := $packets; ~energy := $energy; #EndMacro

Figure 4: shorts.inc

The initialization file is 400 lines long, thus, 10 lines are shown below that are representative of its contents:

(0,0) = 0 0 0 0 0 0 2 0 (0,1) = 0 0 0 0 0 0 7 0 (0,2) = 0 0 0 0 0 0 8 0 (0,3) = 0 0 0 0 0 0 8 0 (0,4) = 0 0 0 0 0 0 5 0 (0,5) = 0 0 0 0 0 0 6 0 (0,6) = 0 0 0 0 0 0 9 0 (0,7) = 0 0 0 0 0 0 4 0 (0,8) = 0 0 0 0 0 0 2 0 (0,9) = 0 0 0 0 0 0 6 0

Figure 5: First ten lines of an example of the initialization file wsn.stvalues

To visualize the results, three .pal files were created. One to visualize the packets as they move towards sinks, one for visualizing energy consumption of nodes and the last one to visualize routing of packets. After each simulation, separate .drw files are generated to visualize each variable of interest.

Figure 6: palette used to visualize packets

Figure 7: pallet used to visualize energy consumption

Figure 8: pallet used to visualize routing of each node. Red means routing to the right, Yellow means routing upwards, green means routing to the left, pink means routing downwards, black means a blocked cell and blue indicates a sink node.

After simulation and drawlog execution using the provided .bat file, the .drw file can be visualized using the animation tool. Some test scenarios and visualization are provided in the following section.

# Simulation Results

Various testing scenarios were considered to cover best case and realistic situations; these cases were generated by creating initialization files that are generated using  MATLAB scripts; an example of a script is the following

clc for i=1:20 for j = 1:20 e = 0; if ( i-1== 0 && j-1== 0) e = 2; end if ( i-1== 0 && j-1== 19) e = 2; end if ( i-1== 19 && j-1== 10) e = 2; end if ( i-1== 10 && j-1 >=3 && j-1 <=8 ) e = 1; end if ( i-1== 10 && j-1 >=12 && j-1 <=17 ) e = 1; end if ( i-1 >=3 && i-1 <=10 && j-1== 8) e = 1; end if ( i-1 >=3 && i-1 <=10 && j-1== 12) e = 1; end if ( i-1== 15 && j-1 >=5 && j-1 <=15 ) e = 1; end fprintf('(%d,%d) = 0 0 0 %d 0 0 %d 0 \n',i-1,j-1,floor(e),floor(random('unif',0,10))) end end

Figure 9: MATLAB script used to generate initialization wsn.stvalues file

To start with, no blocked cells are considered and all nodes are allowed to forward packet naturally; In addition, Only one sink is set at the lower left corner similar to the old model. This should provide a uniform fade of the level of energy consumption from the lower left corner (the sink) to the upper right corner.

Another two scenarios introduce multiple sinks; the first uses two sinks at the upper left and right corners and another midway of lower row; while the other has a sink at each corner of the model. Another case adds some blocking to the three sink mentioned.

In addition, a case with one sink centered at the middle is simulated, it shows how placing the node in the middle changes the variance of energy consumption. Also, a bad blocking case with the sink placed in the middle and an almost closed circle of blocking around it.

Finally, two more realistic cases are simulated. One with four sinks placed strategically in the (5,5), (5,15), (15,5), (15,15); this shows how much such a WSN implementation can reduce variance. In addition, a realistic blocking case is simulated a field with almost 25% randomly placed blocked nodes. This case should show how packets are forwarded around the blocked nodes and thus further energy consumption variance can be seen in those areas.

The rest of this section shows the results produced by these scenarios; please note that of the .drw files produced for each scenario, the most interesting to visualize will be shown here: Total energy consumption, Packets and the routing map for the same time step. The chosen time step for the screen shots was 80; please refer to the animation or the videos for a better view of results.  Results without numerical values are shown for qualitative analysis; for further data please refer to the .drw files.

### Basic Case

Figure 10: Packets at time step 80 for the basic case

Figure 11: Energy consumption at time step 80 for the basic case

Figure 12: Routing at time step 80 for the basic case

### Multiple Sinks 1

Figure 13: Packets at time step 80 for the multiple sinks case

Figure 14: Energy consumption at time step 80 for the multiple sinks case

Figure 15: Routing at time step 80 for the multiple sinks case

### Multiple Sinks 2

Figure 16: Packets at time step 80 for the second multiple sinks case

Figure 17: Energy Consumption at time step 80 for the second multiple sinks case

Figure 18: Routing at time step 80 for the second multiple sinks case

### Multiple Sinks 1 with blocking

Figure 19: Packets at time step 80 for the multiple sinks with blocking case

Figure 20: Energy consumption at time step 80 for the multiple sinks with blocking case

Figure 21: Routing at time step 80 for the multiple sinks with blocking case

### Middle

Figure 22: Packets at time step 80 for the sink in the middle case

Figure 23: Energy consumption at time step 80 for the sink in the middle case

Figure 24: Energy consumption at time step 80 for the sink in the middle case

### Middle with bad blocking

Figure 25: Packets at time step 80 for the sink in the middle with blocking case

Figure 26: Energy consumption at time step 80 for the sink in the middle with blocking case

Figure 27: Routing at time step 80 for the sink in the middle with blocking case

### Realistic Placing

Figure 28: Packets at time step 80 for the sink in the realistic placing case

Figure 29: Energy consumption at time step 80 for the sink in the realistic placing case

Figure 30: Routing at time step 80 for the sink in the realistic placing case

### Realistic Placing with Random Blocking

Figure 31: Packets at time step 80 for the sink in the realistic placing case with random blocking case

Figure 32: Energy consumption at time step 80 for the sink in the realistic placing case with random blocking case

Figure 33: Routing at time step 80 for the sink in the realistic placing case with random blocking case

## Performance analysis

After implementing the enhanced version of the model using the enhanced CD++, some metrics were collected to evaluate the performance of the tool. Translating the old version as is to the new CD++ implementation was not possible due to being unable to apply zones; hence, another method for performance analysis of comparable models was chosen; which is creating the enhanced version in the older CD++. The following are the results obtained.

Figure 34:The number of messages generated and logged by the CD++ tool (comparison between original and enhanced tools)

Figure 35: The total time required for simulation by the CD++ tool (comparison between original and enhanced tools)

As illustrated in the figures above, the total number of messages actually increased using the enhanced version of CD++; however, the execution time required to simulate the model was reduced significantly to almost 30% of the original time required.

# Conclusion.

In the experiment provided, the variance of Energy requirement of WSN nodes was illustrated and various deployment cases were tested as planned. It was showed that careful planning of energy requirements is needed to reduce and optimize costs.

In addition, the possibilities provided by the enhanced version of CD++ are demonstrated; it was shown how scenarios can be built and tested rapidly due to the tool’s unique architecture. In addition, the enhanced version provided significant CPU time improvement that the older version.

# References

- G. Wainer: "CD++: a Toolkit to Define DiscreteEvent Models",  Software, Practice and Experience,Wiley, Vol. 32, No 3. pp. 1261-1306. November 2002.

- A.  López, G. Wainer. Improved Cell-DEVS model definition in CD++. P.M.A. Sloot, B. Chopard, and A.G. Hoekstra (Eds.): ACRI 2004, LNCS 3305.Springer-Verlag. 2004.

- E. Glinsky; G. Wainer: "Performance Analysis of Real-Time DEVS models", In Proceedings of 2002 Winter Simulation Conference, San Diego, U.S.A.