Effect of Door Delay on Aircraft Evacuation DEVS Implementation

Patrick Castonguay (100749930)

Carleton University

pcaston3@connect.carleton.ca

Assignment 2 Report

Presented to Dr. Wainer

July 21st, 2008

- Table of Content

1.	Initial Conceptual Model Description	1

2.	Formal Specification	2

2.a.	General Description	2

2.b.	Cell-DEVS Atomic Model Specification	3

2.c.	Testing and bugs	5

2.d.	Limitations	7

3.	Results	7

- Initial Conceptual Model Description

The proposed system was used to model the effect of delays at the emergency exit door has during the evacuation of a Very Large Transport Aircraft (VLTA).  Amos & Wood (2005) developed and applied a model of the emergency exit procedures from the Airbus A380 to study the effect of having delay opening the emergency doors or individual delays at the door when exiting.

In order for an aircraft to meet regulations, all passengers and crew have to be evacuated within 90 seconds, with only half of the emergency exits available.  They argue that due to the dangerous and expansive nature of the evacuation, real demonstrations are normally limited to a single instance.  As well, for commercial reasons, the results normally kept secret within the company.  The effect of the delay at the door was suspected to be of importance due to the higher than usual nature of the second deck of the A380 which was much higher than other aircraft at the time.

The model was initially implemented using NetLogo programming language and we are proposing to implement this agent based cellular automata using Cell-DEVS.  We will keep to the same conceptual model as proposed by Amos & Wood (2005).  A cell grid representing the seats, wall and doors of the aircraft will be implemented.  Each cell will have information relative to the nearest exit (id & distance).  Each agent (passenger) has the following attributes:

pi: grid position;

hi: heading;

mi: max speed

si: current spead; and

di: delay at door.

The system variables needed are defined as:

D: mean passenger door delay; and

O: exit opening time (fixed at 14 seconds).

di is defined as a Poisson-distribution of positive real numbers with mean D.  Each passenger has a minimum speed of 0 and a random (poisson-distributed) max speed between 0.3m/s and 1.05m/s.  Furthermore, each cell of the grid is assumed to have a dimension of 0.5m * 0.5m.

When the emergency procedure is activated, the passengers start heading to the nearest exit at max speed.  They adjust their speed to reflect the passenger directly in front of them if applicable (i.e. they do not pass, they follow in trail, but they run to catch up).  Figure 1 represents the floor layout of the A380 which was used as a construct for the original work.  In the original work the authors only modeled the second level as they identified that inter-deck travel during emergency evacuation is not permitted.  As well they only modeled passengers in their seats.  This is normal as passengers are expected to be in their assigned seats for emergency landing.  They did not model any passenger behavior like panic.

Figure 1: A380 floor plan

- Formal Specification

  - General Description

In order to provide more flexibility for the potential user of this simulation, it was decided to have only the floor plan as well as the occupancy layout to be generated manually by the user.  This configuration permits an easy representation of the plane passenger load as well as floor layout and the initial construction of the plane does not need knowledge of distance or travel information.  This was implemented using two different planes.  The first plane is a representation of the seating arrangement.  The states of the first plane are presented in table 1.  The second plane is used to calculate the distance to the closest emergency exit.  This is done at initialization only and states are presented in table 2.  Each plane needs to access each other in order to determine if movement will be allowed to the desired cell.  The neighbor list can be seen in the Atomic Model Specification.  The speed of travel of passenger is encapsulated in the cell delay for the calculation of the new states.  For the initial development of the system a speed of 1000m/s was used as this represent a one second evaluation interval on each cell of 0.5m section and therefore a speed of 0.5m/s which fall in the average selection provided by Amos and Wood (2005).

| Name | Value | Color (.pal file) | Description |
| --- | --- | --- | --- |
| Passenger | -2 | Blue | Cell occupied by a passenger |
| Wall | -1 | Black | Wall or Obstruction |
| Empty | 0 | White | Empty space |
| Exit | 9 | Green | Emergency exit |

Table 1: Plane 1 State Description

| Name | Value | Color (.pal file) | Description |
| --- | --- | --- | --- |
| Wall | -1 | Black | Wall or Obstruction |
| Exit | 0 | White | Empty space |
| Distance | 1++ | Shades of Green | Emergency exit |

Table 2: Plane 2 State Description

The system initially calculates distances from the exit on second plane for each cell. If the corresponding floor plane cell is a wall, assign distance as wall which will give information not to allow travel to this cell.  One the system is initialized; each occupied cell will want to travel toward the next unoccupied cell with smaller distance to the exit.  Priority is given to window seats and then to center isle.  This means that for the sector defined for [port seats] the priority will be given to cells traveling down and for the [starboard exit] the priority is given to the cells traveling up.  In order for a passenger to move it has to verify that other cells with lower priority will not be moving to the same wanted cell.  Currently there is no panic or erratic behavior modeled and when a passenger cannot move, it patiently waits for the cell to become available.  The model could be made more interesting by implementing behaviors where passengers would choose alternate routes if the one wanted is not available or have erratic movement like jumping over seats or pushing other passengers.

  - Cell-DEVS Atomic Model Specification

The following is the formal specification for the Cell-DEVS aircraft evacuation model:

AC_EVAC = < X, Y, I, S, θ, N, d, δint, δext, τ, λ, D >

X = Y = Ø

S = plane 1: { 0, -1, -2, 9 } // Empty, Wall or Obstruction, Occupied (passenger), Exit

plane 2:  { -1, 0..inf }   // Wall, Distance to exit 0 being exit itself

N = neighbors :

%Floor plane (Modified Moore or VonNeuman)

(-2,0,0)

(-1,-1,0) (-1,0,0) (-1,1,0) 	% NW, N, NE

(0,-1,0)  (0,0,0)  (0,1,0) 	% W, Orig,E

(1,-1,0)  (1,0,0)  (1,1,0)  	% SW, S, SE

(2,0,0)

%Dist plane to access floor plane (Moore)

(-1,-1,-1) (-1,0,-1) (-1,1,-1) 	% NW, N, NE

(0,-1,-1)  (0,0,-1)  (0,1,-1) 	% W, Orig,E

(1,-1,-1)  (1,0,-1)  (1,1,-1)  	% SW, S, SE

%Floor plane to access dist plane (Extended VonNeuman)

(-2,0,1)

(-1,-1,1) (-1,0,1) (-1,1,1) 	% NW, N, NE

(0,-2,1)  (0,-1,1)  (0,0,1)  (0,1,1) (0,2,1)	% W, Orig, E

(1,-1,1)  (1,0,1)  (1,1,1)  	% SW, S, SE

(2,0,1)

Zones had to be defined in order to have different calculation done by the different planes as well as giving different prioritization to the different section of the aircraft.  The zones are presented below and represent the distance plane as well as the port (left) side and starboard (right) side of the aircraft.

zone : prt_seats   { (1,0,0)..(5,20,0) }

zone : stb_seats   { (6,0,0)..(10,20,0) }

zone : dist_plane 	{ (0,0,1)..(11,20,1) }

d = 1000 ms // default delay used to represent the speed 1 second per cell or 0.5m (or 0.5m/s)

type = transport

τ: NS is defined by the different sets of rules introduced bellow:

The first set of rules calculates the distance to the nearest exit and stores this information in the second plane. This is only done at initialization and a zone describing the distance plane had to be defined.

rule : -1           0 { (0,0,-1) = -1 }

rule : 0            0 { (0,0,-1) = 9 }

rule : {(1,0,0)+1}  0 { (0,0,0) < 0 and (1,0,0)  >= 0 }

rule : {(-1,0,0)+1} 0 { (0,0,0) < 0 and (-1,0,0) >= 0 }

rule : {(0,1,0)+1}  0 { (0,0,0) < 0 and (0,1,0)  >= 0 }

rule : {(0,-1,0)+1} 0 { (0,0,0) < 0 and (0,-1,0) >= 0 }

The second set of rules describes the exiting at the emergency door condition.  When a cell is occupied and next to an exit the passenger will exit after a certain random delay.  To be true to the initial model from Amos and Wood (2005) we should also add a condition for the simulation time to be greater than a certain constant (14seconds).  The current model does not function properly if the time constraint is implemented as a bottle neck forms and passengers do not exit even after the time constraint.  If the time constraint is kept smaller than the bottle neck time, the system behave as expected

rule : 0 #Macro(DoorDelay) { (0,0,0)=-2 and (1,0,0)=9 } %exit down

rule : 0 #Macro(DoorDelay) { (0,0,0)=-2 and (-1,0,0)=9 } %exit up

rule : 0 #Macro(DoorDelay) { (0,0,0)=-2 and (0,1,0)=9 } %exit right

rule : 0 #Macro(DoorDelay) { (0,0,0)=-2 and (0,-1,0)=9 } %exit left

The DoorDelay Macro represents a random exit delay at the door and returns values (1000, 2000, 3000 or 4000).  It has been defined in the ac_eval.inc file as such:

#BeginMacro(DoorDelay)

{ 1000 + 1000*randInt(4) }

#EndMacro

The third set of rules defines when a cell will be receiving passenger. This example is for the starboard seats and therefore the priority has been given to the cells traveling down.  This will be true for the entire description of the rules.   As well, for all transition, we have to verify the desired direction of movement manually.  This is more complicated than having the direction of movement coded in the state but was felt to provide a better flexibility.

%Entering cell

rule : -2 #Macro(Speed) {(0,0,0)=0 and (-1,0,0)=-2 and (-1,0,1)>=(0,0,1)} %receive from up

rule : -2 #Macro(Speed) {(0,0,0)=0 and (1,0,0)=-2 and (1,0,1)>(0,0,1)} %receive from down

rule : -2 #Macro(Speed) {(0,0,0)=0 and (0,-1,0)=-2 and (0,-1,1)>=(0,0,1)} %receive from left

rule : -2 #Macro(Speed) {(0,0,0)=0 and (0,1,0)=-2 and (0,1,1)>(0,0,1) and (0,2,1) > (0,1,1)} %receive from right

The fourth set of rules describes a passenger leaving a cell.  The one traveling down have priority and therefore the others have to evaluate is a cell with higher priority will be moving to its position.

%Leaving down(has priority), up next

rule : 0 #Macro(Speed) {(0,0,0)=-2 and (1,0,0)=0 and (1,0,1)<=(0,0,1)} %move down

rule : 0 #Macro(Speed) {(0,0,0)=-2 and ((-1,0,0)=0 and (-1,0,1)<(0,0,1)) and ((-2,0,0)!=-2 or (-2,0,1)<(-1,0,1))} %move up

%Leaving right giving way to up/down trafic but not if heading away from ilse

rule : 0 #Macro(Speed) { #Macro(WantRight) and ((-1,1,0)!=-2) and (1,1,0)!=-2} %move right with give way to down and up

rule : 0 #Macro(Speed) { #Macro(WantRight) and (1,1,0)!=-2 and ((-1,1,0)=-2 and (-1,1,1)<(0,1,1))} %When next row right is exit row (moving up)

rule : 0 #Macro(Speed) { #Macro(WantRight) and (-1,1,0)!=-2 and ((1,1,0)=-2 and (1,1,1)<(0,1,1))} %When next row right is exit row (moving down)

%Leaving left giving way to up/down trafic but not if heading away from ilse

rule : 0 #Macro(Speed) { #Macro(WantLeft) and ((-1,-1,0)!=-2) and (1,-1,0)!=-2} %move left with give way to down and up

rule : 0 #Macro(Speed) { #Macro(WantLeft) and (1,-1,0)!=-2 and ((-1,-1,0)=-2 and (-1,-1,1)<(0,-1,1))} %When next row left is exit row (moving up)

rule : 0 #Macro(Speed) { #Macro(WantLeft) and (-1,-1,0)!=-2 and ((1,-1,0)=-2 and (1,-1,1)<(0,-1,1))} %When next row left is exit row (moving up)

  - Testing and bugs

The model was developed incrementally and tested at each step.  Originally the floor plan (which can be seen in Figure 5d) was a subset of the full A380 floor plan, this made the system somewhat faster and easier to debug.  The first test was to confirm that the distances were calculated properly.  The progressive green palette used for the second plane is show in figure 2 and results for one two and four exits are presented in figure 3.  The results show that the distances are calculated as expected in each condition.

Figure 2: Simulation Palette

Figure 3: Distance Test Results

The second step was to verify that a single passenger could exit the plane from any position.  This was done by having one passenger in each window isle as well as one in a center row.  Of note is that there are still some problems with the simulation in that respect.  If there is a window seat that is empty, with an isle seat being occupied at initialization; the simulation creates a new passenger in the window seat but runs properly after that.  If during the simulation the same condition is encountered, the problem does not arise.  The same is true for a passenger at an intersection of the isle and row.  This problem was not able to be fixed for this assignment but does not affect the full scenario execution of the system to be tested were the aircraft is full in all seating position.  A run of this problem can be found in the folder [one_pers] of the assignment package. Figure 4 give a graphical representation.

Figure 4: Passenger Creation Bug

Once the movement was verified to be correct, possible collisions were introduced gradually in order to confirm that passengers did not appear or disappear during the simulation.  This lead to some revision of movement rule to cover some situations that were originally overseen.  The last test was to use a full airplane and observe the state changes to confirm compliance with conceptual design.  Figure 5 shows a few snapshoot (time: 0sec; 2sec; 25sec; and 54sec) of the system as it executes, one can observe that movement is as expected and all passengers exit the airplane.

Figure 5: Full Simulation Execution

Some problems that are still to be investigated with the systems will be discussed.  There seems to be a problem when the simulation time step is not in sync. For example, if the speed is set at 1000 (1 second) and the door delay set to 1300 (1.3 seconds) there is a bottle neck that forms and the system behaves erratically sometimes going into an infinite loop.  As well it seems to be that the time steps need to be somewhat synchronized.  By this we mean that if the time steps are all multiples of 500, the system behaves as expected but if the time steps are left to be completely random, erratic behaviors arise again.  One last problem was found is the layout was set with a multiple of rows empty leading to the exit.  An example of this would be if a row is full of passengers with the next two rows being emergency exits.  This lead to duplication of passengers but could probably be fixed with some effort as it would just be a matter of changing some of the rules to be more stringent.

  - Limitations

Some limitations have already been discussed like the lack of panic modes and alternate routing search.  As well, in Amos & Wood (2005), each passenger is assigned a certain max speed.  This has not been replicated in our system.  One last limitation is that even with a random value for the delay of passengers of the doors, the simulation time (evacuation time) seems to be constant.  One can see by stepping the system that the time a passenger sends at the door is not always the same but it seems that the random function used for the generation of the delay at the door always generates the same sequence.  It is suspected that changing the seed of the random function in accordance to time may fix this problem but nothing of the sort was found in the manual or online for the CD++ language.  If this problem gets addressed, one could run the system with different average values of passenger delay at the door in order to give more stochastic results.  For the evacuation time of the system, one could calibrate the delay (speed) in order to match a representative value.  In this example we know that a test was carried out in Europe on the A380 which took 74 seconds to exit 853 passengers and 20 crew members.  A video of the evacuation can be found at the following URL: www.youtube.com/watch?v=weOcrP7u7Y8.  Of note is the interesting fact that no pregnant woman, elderly or obese passenger can be seen in the video.

- Results

The following table shows the simulated evacuation time for different door delay.  One can notice that the time for a delay of 1 second is respectfully close to the results of the original work (taking into account that the model of the plane was smaller) which expected 54 seconds and got results of 57.9 seconds with a mean deviation of 1.5 second.  One can expect that if the door delay could be set as per the original experiment similar results would be obtained.  The next step would be to reproduce the full size second deck of the A380 and run the same type of scenarios.

| Door Delay (milliseconds) | Evacuation time (seconds) |
| --- | --- |
| 0 (D = 1000, same as speed) | 38 |
| 1000 (D = 2000, speed + 1 second) | 56 |
| 2000 | 1:14 |
| 1000 + 1000randInt(4) (1000,2000,3000,4000,5000) | 1:04 |

Table 3: Simulation Results

The next step is to implement a full scale replica of the A380 seating plan and run simulations with half of the emergency exits disabled.  When a full floor representation was simulated with full passenger load, one of the row (#38) was misbehaving and deleting passengers.  This problem was not able to be traced and only happens to the one row.  When the problem was found a 4 passenger center row was used but in reality the aircraft would only have 2 passengers in that area as it is a first class seating.  The problem still exists when loaded as a first class configuration.  The total simulation time for a full plane layout with full load was found to be greater than expected (1min 14sec).  This was originally done with a delay of 1sec but was adjusted to 500msec in order to account for the propagation delay.  This adjustment brought the simulation time down to 59sec which can be considered acceptable.  Observing the behavior of the passenger once half of the exits have been disabled, it becomes obvious that in order to have a fully representative simulation, the choice of alternate route for the passengers under certain circumstances should be added to the system.