Aircraft Evacuation DEVS Implementation & Visualization

Patrick Castonguay

Carleton University

pcaston3@connect.carleton.ca

Gabriel Wainer

Carleton University

gwainer@sce.carleton.ca

Keywords: Discrete Event Simulation, DEVS, CD++, Blender, Visualization

Abstract

With the advent of new twin-deck Very Large Transport Aircraft (VLTA), concerns about the speed at which they may be evacuated emerged[1]. Many factors come into play when conducting an emergency evacuation of an aircraft but this paper focuses on the possible effect of hesitation passenger could have when exiting from an abnormally high elevation[2]. Amos and Wood developed and applied a model of the emergency exit procedures from the Airbus A380 to study the effect of individuals delay at the emergency door when exiting the plane[3]. This work was replicated using Cell-DEVS[4]. The CD++ tool suit developed at Carleton University[12] was used to develop the model and generate the initial results. These results where then visualized using the Open Source (OS) 3D tool Blender[5]. Previous work by Poliakov[6] and Tsui[7] was adapted to animate the results of our simulation, effectively proving the flexibility and portability of both CD++ and Blender. The technologies used in this project comprise of Blender, Python and CD++.

# INTRODUCTION

Results from DEVS simulation can be visualized in many different ways. The CD++ tool kit provides DEVSView as well as the CD++ Modeler add-on[12]. These tools provide a quick representation of the model and simulation behavior in the 2D space. 3D visualization of CD++ results have previously been done using Maya and Blender[6,7,11]. This project ported the previous application of Blender to the new aircraft evacuation DEVS implementation and extended the functionality of the script to be more inline with the general functionality of the CD++ toolkit. This was done in order to demonstrate the flexibility and portability of both the CD++ tool kit as well as Blender.

# background

## Experimental Frame

Emergency evacuation of a Very Large Transport Aircraft (VLTA) can be affected by many different factors[2]. Amos & Wood (2005) developed and applied a model of the emergency exit procedures from the Airbus A380 to study the effect of experiencing a delay when opening the emergency doors or individual delays at the door when a passenger exits the aircraft. In order for an aircraft to meet regulations, all passengers and crew have to be evacuated within 90 seconds, with only half of the emergency exits available. Amos & wood (2005) argue that due to the dangerous and expansive nature of the evacuation, real demonstrations are normally limited to a single instance. As well, for commercial reasons, the results are normally kept secret within the company. The effect of the delay at the door was suspected to be of importance due to the nature of the second deck of the A380 which was much higher than other aircraft at the time. The model was initially implemented using NetLogo programming language[3]. Figure 1 represents the floor layout of the A380 which was used as a construct for the original work.

Figure 1: A380 upper floor plan

## Conceptual Design

We implemented this agent based cellular automata using Cell-DEVS[4]. A cell grid representing the floor plan is defined using a . A single cell represents either a wall, a passenger, an exit or an empty space and the listing of state can be found in Table 1. A second plane, or dimension, was used to store information on distances to the nearest functioning emergency exit therefore keeping this information separate from the configuration of the aircraft. Table 2 illustrate the different states a cell in the distance plane can take and Figure 2 show the visual representation of that dimension after system initialization for a subset of the aircraft. As the variable under study is the hesitation at the door, no other factors like panic behavior or secondary routing seeking behavior where implemented. In order to provide more flexibility for the potential user of this simulation, it was decided to have only the floor plan as well as the occupancy layout be generated manually by the user. In CD++, this is done via a .VAL file which is read at system initialization. The speed of travel of passenger is encapsulated in the cell delay for the calculation of the new states. For the initial development of the system a speed of 1000m/s was used as this represent a one second evaluation interval on each cell of 0.5m section and therefore a speed of 0.5m/s which fall in the average selection provided by Amos and Wood (2005).

Table 1: Plane 1 State Description

| Name | Val | Color | Description |
| --- | --- | --- | --- |
| Passenger | -2 | Blue | Occupied (passenger) |
| Wall | -1 | Black | Wall or Obstruction |
| Empty | 0 | White | Empty space |
| Exit | 9 | Green | Emergency exit |

Table 2: Plane 2 State Description

| Name | Val | Color | Description |
| --- | --- | --- | --- |
| Wall | -1 | Black | Wall or Obstruction |
| Exit | 0 | White | Empty space |
| Distance | 1+ | Green shades | Emergency exit |

Figure 2: Distance Test Results

Figure 3: Full Simulation Execution

This can easily be changed to random numbers by replacing the static timing for a call to a random number generating macro. Once the system is initialized; each occupied cell will want to travel toward the next unoccupied cell with smaller distance to the exit. Priority was given to window seats and then to center isle. This means that for the sector defined for [port seats] the priority will be given to cells traveling down and for the [starboard exit] the priority is given to the cells traveling up. CD++ define behaviors via the description of mathematical rules which are evaluated sequentially until a valid statement is found. Therefore movement priority is implemented in the order in which the evaluation of cell state is described.

## Experiment Results

Table 3 shows the simulated evacuation time for different door delay. One can notice that the time for a delay of 1 second is respectfully close to the results of the original work (taking into account that the model of the plane was smaller) which expected 54 seconds and got results of 57.9 seconds with a mean deviation of 1.5 second. One can expect that if the door delay could be set as per the original experiment similar results would be obtained. The next step would be to reproduce the full size second deck of the A380 and run the same type of scenarios. The next step is to implement a full scale replica of the A380 seating plan and run simulations with half of the emergency exits disabled. The total simulation time for a full plane layout with full load was found to be greater than expected (1min 14sec). This was originally done with a delay of 1sec but was adjusted to 500msec in order to account for the propagation delay. This adjustment brought the simulation time down to 59sec which can be considered acceptable.

Table 3: Simulation Results (seconds)

| Door Delay | Evacuation time |
| --- | --- |
| 0 (no delay) | 38 |
| 1 | 56 |
| 2 | 1:14 |
| 1 + 1*randInt(4) | 1:04 |

Observing the behavior of the passenger once half of the exits have been disabled, it becomes obvious that in order to have a fully representative simulation, the choice of alternate route for the passengers under certain circumstances should be added to the system. Only one freely available demonstration of such evacuation was found. It was carried out in Europe on the A380 which took 74 seconds to exit 853 passengers and 20 crew members. A video of the evacuation can be found on YouTube[15] but no other documentation was found. Of note is the interesting fact that no children, pregnant woman, elderly or obese passenger can be seen in the video. This is of course to be expected as the airline company would not want to endanger the life of these people but strengthen the argument that simulation can give a complete representation of these situations.

## Limitations

To limit the scope of this project no passenger behavior other than seeking the closest emergency exit was implemented. In order to broaden the scope and applicability of this project one would need to implement behaviors such as the search of alternate valid routing and passenger panic leading to erratic movement. Also because of the original structure of the state of each cell, a maximum speed assign to each passenger was not implemented. This would be needed in order to simulate having a wide type of passenger such as elderly, obese, children and/or pregnant. A possible solution would be to have a third plane that keeps the max speed information or to have a state on the first plane that encodes this information. an other limitation that was found about the system and possibly the way random generation of variables in handled in CD++ is that even though a random generation function was called, the outcome was deterministic in that every passenger had a different delay at the exit but these waits were identical between all runs. In order to make this simulation more stochastic, a way to change the seed of the random generation function would be needed.

## Blender and Python

Blender is a free open source software (FOSS) 3D content creation suite released under GNU GPL[5]. It can create 2D or 3D graphics as well as movie quality animations. The main reasons why Blender was chosen was the fact that it natively support Python scripting, is free and provide powerful 3D animation capability. Some of the commercial counterparts to Blender are Maya and 3DStudioMax[13]. Python is also a FOSS product[14]. It provides a high-level dynamic object-oriented programming language. For this project we have not made any modification to Blender itself but have used it's native support for Python scripting in order to read and animate the results from CD++.

# VISUALIZATION

A script originally developed by Poliakov[6] has been used as the basic building block and has been extended and modified for this project. The CD++ toolkit uses a .MA file to define the behavior of an atomic model using the DEVS formalism. In turn this .MA file can use a .VAL file to set the initial values of each cell of the model in the simulation. these two file are used at run-time to define the behavior of the simulation and a .LOG file is generated containing the state of each cell for every time-advance for the simulation. In order for the script to be most flexible, it had to be extended to support the existence of a .VAL file and login of the script behavior itself. The cell space is represented in Blender by a direct coordinate transformation of the cell position to a coordinate inside Blender. Therefore the size of the passengers, walls and seats have to be no more than 1x1. Once loaded the script will read and parse the .MA file and look for a .VAL file if needed. The .VAL file will be processed before the .LOG file in order to adequately represent the simulation behavior. See figure 4 for a visual representation of the script architecture. When parsing the .LOG file, passengers will be displayed at the appropriate coordinate within Blender. Passengers will appear and disappear is sequence representing their movement.

CD++ Toolkit

.ma

.val

.log

Py Script

Blender

.blend

.py

log.txt

Figure 4: Visualization Architecture

## Execution

In order to visualize the simulation 3D objects are needed by Blender, these are normally included in a file of the .blend type. For our simulation, a simple human figure was used to represent passengers and a solid block for walls and seats (see figure 5). Nicer objects could easily be incorporated by a 3D artist if needed. These object are incorporated in the ac_evac.blend file. Install the application by following these steps:

- Install Blender v2.43 and Python v2.4.4. Different version of Blender may be used but the current script has only been test with this configuration;

- If a previous version of the CD++Python script is installed the appropriate .py file needs to be removed from the Blender\script folder. These can be found by searching for the "CD++ Simulation" string;

- Copy the files ac_evac.blend, ac_evac20.ma, ac_evac20.val and ac_evac20LOG.log to the default blender folder. This is usually the .blender folder under the Blender main directory (C:\Program Files\Blender Foundation\Blender);

- Place the ac_evac_gui.py in the Blender\script folder;

- Launch Blender by double-clicking the ac_evac.blend file;

- Inside the Blender application, navigate to the Script/Misc/CD++ Simulations and select it;

- Select the proper .MA and .LOG file via the CD++ Simulations interface, this will load and setup the visual representation of the simulation;

- Press Execute to visualize the simulation's results. the application can be stopped by pressing ESC or right-clicking on the application.

The simulation will execute and passengers will appear and disappear representing their movements until they have all exited the aircraft.

Figure 5: 3D Models

## Effort

In order to migrate the previously developed Python script to support the aircraft evacuation cell-DEVS simulation, a relatively small effort was required. Most of the man-hours were spent backtracking a problem that originated in the CD++ model itself. For an unknown reason, cell states were assigned more than once to the same thing for each time-step. This generated erratic behavior of the 3D animation. As the CD++ simulation was to be considered a completed project, effort was spent in making the script more 'rugged'. By this we mean that multiple assignments of the same state (i.e. cell occupied by passenger) needed to be handled inside the script. Once this problem was identified, the actual fix inside of the Python script was relatively small: 7 lines of code. Of note if the fact that this academic project spawned a period of one month with an effort of approximately 10 hours per week for a total of 40 man-hours. Also of note is that the author did not have previous knowledge of Python or Blender and had no previous expertise in 3D modeling or animation. The effort expanded, other than the learning of the tools, for this project is divided as follows:

- Reading .VAL file (new routine): 10 lines;

- Reading .MA file (modification): 2 lines;

- Processing .LOG file:

  - 3 lines for coordinate handling;

  - 10 lines for state handling (most for duplicate rest for mapping stateId to 3D model);

- Reading .LOG file, GUI, and Event handling required no change.

## Limitation

When the model was first developed, 3D animation was not something the author had in mind. This lead to some decision at the atomic model level that were later considered limiting when developing the 3D animation script. Having the state of each cell on the first plane contain limited information (empty, passenger, wall) lead to the all the motion information being calculated by the rules within CD++. This in turn would have increased the complexity of representing passengers movement in a smooth transition fashion. Having movement information like direction and speed available directly in the state of model would be considered important when doing 3D animation in Blender but is not need for 2D representation inside CD++. The fact that the basic CD++ toolkit keeps each cells independent from one an other and messages cannot be passed between them was at the basis of some of the limitations motioned previously. There exist an advanced CD++ toolkit that is more object-oriented and permits these type of advanced cellular behaviors. We believe that using this advanced framework would permit a more complete and complex representation of the behavior of passengers inside an aircraft during an emergency evacuation scenario.

# CONCLUSION

This project expanded on a previous implementation of a CD++ Python script generating 3D animations inside Blender. We effectively proved that with little or no expertise in the field of 3D animation, one can easily adapt a generic script to support visualization of results from a specific CD++ simulation. This was possible only due to the flexibility and portability of the CD++ toolkit, Python and Blender. Further interesting development in this area would be to extend the script to enable mapping of the stateID to the 3D model so that it can be used to visualize results from CD++ without having to modify the script itself every time. An other interesting area to research would be the automatic generation of a rendered animation instead of having to view the raw animation processing inside of Blender.

References

[1] Muir H.C., Bottomley DM & Marrison C. "Effects of motivation and cabin configuration on emergency aircraft evacuation behavior and rates of egress". International Journal of Aviation Psychology 6(1), 57-77. 1996.

[2] Jungermann H. & Gohlert C. "Emergency evacuation from double-deck aircraft". Proceedings of ESREL 2000, 989-982, Rotterdam.

[3] Amos M. & Woods A. "Effect of Door Delay on Aircraft Evacuation time". University of Exeter. 2005

[4] Castonguay P. "Effect of Door Delay on aircraft Evacuation DEVS Implementation". Carleton University. 2008.

[5] Blender Foundation, http://www.blender.org/

[6] Wainer G., Poliakov E., Hayes J. & Jemtrud M. "A Busy Day at the SAT Building". Proceedings of the International Modeling and Simulation Multiconference, Buenos Aires. 2007.

[7] Tsui K.W. "Auto Factory DEVS Simulation Animated in Blender". Carleton University. 2007.

[8] Galea E.R., Owen M. & Lawrence P.J. "Computer modeling of human behavior in aircraft fire accidents". Toxicology 115, 63-78. 1996.

[9] Galea E.R. & Perez Galparsoro J.M. "A Computer-based Simulation Model for the Prediction of Evacuation from Mass-Transport Vehicles". Fire Safety Journal 22(4), 341-366. 1994.

[10] Muir H. & Thomas L. "Passenger Safety and Very Large Transportation Aircraft". Aircraft Engineering and Aerospace Technology 76(5), 479-486. 2004.

[11] Wainer G. & Chen W. "A framework for remote execution and visualization of Cell-DEVS models". Simulation. 79, 626-647. November 2003.

[12] Wainer G. "CD++: a toolkit to define discrete-event models". Software, Practice and Experience. 32(3), 1261-1306. November 2002.

[13] Autodesk Maya & 3DSM, http://usa.autodesk.com

[14] Python Software Foundation, http://www.python.org/

[15] http://www.youtube.com/watch?v=XIaovi1JWyY

Biography

Patrick Castonguay is a student in the Department of Systems and Computer Engineering at Carleton University where he is pursuing an M.A.Sc. in Technology Innovation Management with specialization in Modeling and Simulation.  Patrick graduated in 1998 from the Royal Military College of Canada with a Bachelor in Software Engineering. He then worked for two years as a team lead for CAE, maintaining and developing the CF-18 fighter aircraft mission computer software. He is specialized in Navigation and Communication with the Canadian Air force and was employed with the CP-140 maritime patrol aircraft for 6 years.  During that time he helped develop, maintain and administer the squadron scheduling and proficiency-management database