94.587

#### Fall 2001

###### Final Project

##### Traffic Analysis using ATLAS

Lidan Li

#311135

# Abstract

This project intends to perform traffic analysis on a selected city section using ATLAS [1]. ATLAS defines a complex traffic model in a simple way. The TSC [3] compiler takes an ATLAS specified city section as input and translates it into executable models that can be used in CD++ [2] tool for further analysis. With this approach, a city section can be easily described and the model can be built and analyzed without writing a single line of source code.

# Introduction

The city section defined in this project is the Carleton University Campus of Ottawa, Canada. Step1, the ATLAS specification language is used to define the constructions in the campus. Step 2, these ATLAS definitions (.plan file) are translated into source code by the TSC [3] compiler. Finally, CD++ tool takes the generated source code as input and generate the output file as simulated result that contains the information of the traffic flow in the campus. Based on the original model of the campus, we slightly change the topology of the campus, such as adding the traffic lights in the crossing, adding the railway crossings in the campus, varying the traffic loads etc. in order to compare the traffic performances under different conditions within the campus. By applying this method, we can find an optimal topology of the Carleton University Campus and therefore avoid the traffic jam.

# ATLAS constructions

Before simulating the model in the CD++ tool, It is necessary to know how the ATLAS language defines the constructions. Here is a brief explanation.

The constructions defined in the ATLAS language are mapped into DEVS and Cell-DEVS models. Atomic DEVS includes TSCGenerator, TSCConsumer, TSCRailnet, TSCSincrorailnet, TSCTrafficLight, TSCSincrotrafficlight and TSCCounter. TSCGenerator and TSCConsumer provide inputs and outputs to the city section. Rest atomic models provide various functionality of the traffic. All these atomic models construct a high performance discrete-event simulation. Cell-DEVS also describes cell spaces as DEVS models with timing delays, which includes the segments and crossings.

Conceptual model definition for each atomic DEVS and Cell-DEVS in ATLAS [1]:

TSCGenerator: this model is to generate cars during the constant period. It provides the inputs to the city section.

TSCConsumer: this model is to count the number of cars leaving the city section. There are two outputs from this model, one is to calculate the total number of cars leaving, the other is to calculate the number of cars leaving in each time unit. It provides the outputs for the city section.

TSCCounter: this model is connected with each crossing, it counts the number of cars arriving and leaving the crossing.

TSCRailnet: this model is built as the station connected to the railtrack, it can send trains from its output port to the railtrack.

TSCSincrorailnet: this model is to work with the TSCRailnet model, which synchronize the train from the station to the segment.

TSCTrafficlight: this model is in charge of changing the traffic light in the crossing.

TSCSincrotrafficlight: this model is to work with the TSCTrafficlight, which synchronize the traffic light in the crossing.

Segment : this Cell-DEVS represents the cars move in the section between two corners. A set of rules is defined to describe the cars’ moving direction. When the parking or some other traffic constructions are built in the segment, a special rule is defined for these constructions.

Crossing: this Cell-DEVS represents the points where several segments intersect.

Formal specification for TSCGenerator:

No external input, the initial function start the internal function with time zero

int ( s, e )

{

holdIn (active, Time );

check the lanes in the segment;

}

 (s)

{

send number 1 to the output port;

}

Formal specification for TSCConsumer:

ext ( s, e, x)

{

quantity =  quantity + 1;

}

int ( s, e )

{

holdIn (active, frecuence);

}

 (s)

{

send quantity  to the port quantityAcum;

send quantity/time to the port quantity;

}

Formal specification for TSCCounter:

ext ( s, e, x)

{

case port

arrive:

arrivedCount++;

solved:

solvedCount++;

}

int ( s, e )

{

holdIn (active, frecuence );

}

 (s)

{

send arrivedCount / time to the port throughputA;

send solvedCount / time to the port throughputS;

}

Formal specification for TSCTrafficlight:

ext ( s, e, x)

{

case: port

x_si_luz:

color = msg.value();

holdIn (active, time: zero);

else

holdIn (active, time: zero);

}

int ( s, e )

{

passivate();

}

 (s)

{

send color to the port y_t_luz[i];

}

Formal specification for TSCSincrotrafficlight:

No external function, initial function start the internal function with time zero.

int ( s, e )

{

holdIn (active, time);

tlight = tlight +1;

if (tlight == tlights)

tlight = 0;

}

 (s)

{

case: tlight

if tlight is match with tlights;

send green light to the output port;

else

send red light to the output port;

}

Formal specification for TSCRailnet:

ext ( s, e, x)

{

train = msg.value();

holdIn( active, time: zero);

}

int ( s, e )

{

passivate();

}

 (s)

{

send train to the port y_t_tren_bt[i];

send train to the port y_t_tren_at[i];

}

Formal specification for TSCSincrorailnet:

No external function, initial function start the internal function with time zero.

int ( s, e )

{

holdIn (active, time);

railnet = railnet +1;

if (railnet == railnets)

railnet = 0;

}

 (s)

{

case: railnet

if railnet is match with railnets;

send train to the output port;

else

send no train to the output port;

}

Formal specification for cell segment:

TDCsegment = < X, Y, I, S, , N, d, int, ext, , , D >

X:  is the set of input external events from the tscgenerator or the crossing;

Y:  is the set of output external events to the tsccounter, tscconsumer or the crossing;

### I:  is model’s modular interface with crossing, tscgenerator, tscconsumer and tsccounter;

 = 3;

N = { (0, -1), (0, 0), (0, 1)};

B = {}; nowrapped border, the delay for this cell is transport delay.

a set of rules is defined for the local computing function.

Formal specification for cell crossing:

TDCcrossing = < X, Y, I, S, , N, d, int, ext, , , D >

X:  is the set of input external events from the segment;

Y:  is the set of output external events to the tsccounter and segment;

### I:  is model’s modular interface with segment and tsccounter;

 = 3;

N = { (0, -1), (0, 0), (0, 1)};

B = {}; nowrapped border, the delay for this cell is transport delay.

a set of rules is defined for the local computing function

TSC Definitions

According to the formal specification of the ATLAS constructions, a TSC (Traffic Simulator Compiler) compiler was built, which can generate code by using a set of templates. These codes can be used in CD++ tools directly as described above.

Here is a simple example to explain how the atomic DEVS and Cell-DEVS coupled model constructed. The following is the ATLAS specification (TSC definition) that describes two segments (roads) and one crossing (intersection) and the corresponding coupled model.

begin segments

t1 = (1,5),(1,1),1,straight,go,40,300,parkNone

t2 = (1,1),(5,1),1,straight,go,40,300,parkNone

end segments

begin crossings

c1 = (1,1),20, withoutTL, withoutHole, 300, 3

end crossings

The following figure is the coupled DEVS model [2] that was resulted from the ATLAS specification. This model can now be used in CD++ for simulation.

C1 counter

Big counter

T1 gen

T2 con

0

1

C1

T1

T2

3

2

1

0

3

2

1

0

City section description:

The city section to be described here is the Carleton University Campus. It includes the following main streets: Colonel By drive, Library road, University drive, Campus Avenue, Raven road, Anniversary road, Admin Drive, sunnyside drive and Bronson Avenue. These main streets cross with each other and vary in the number of lanes. Within each of the roads, there are different sections describing different traffic constructions, e.g., different number of lanes, traffic signs, speed limits, parking permissions and potholes. Further more, there is a railway go through the campus; however, it doesn’t cross with any one of the streets because bridges are built. There are four entries/exits allowing traffic enter and leave the campus. They are north/south entries of Bronson Avenue and north/south entries of colonel by drive. The traffic flow in the Bronson is very high as it is a 2 way 3 lane high way. Please refer to the following map for the geographic view of the campus and road positions.

The following is the detailed TSC definition of the Carleton University Campus. The TSC will translate them into DEVS and Cell-DEVS models and generate the MA file to be used under CD++ [2].

begin segments

Colonel_A = (10, 0), (10, 10), 1, straight, go, 40, 300, parkNone

Colonel_B = (10, 10), (17, 190), 1, straight, go, 40, 300, parkNone

Colonel_A1 = (10, 0), (10, 10), 1, straight, back, 40, 300, parkNone

Colonel_B1 = (10, 10), (17, 190), 1, straight, back, 40, 300, parkNone

/* colonel by drive. Colonel A, B is one lane from south to north,

colonel A1, B1 is one lane from north back south. Speed limited at 40 and there is no

parking allowed along the drive. An intersection exists at (10,10) that divide the

road into section A and B */

University_A = (10, 10), (20, 16), 2, straight, go, 40, 300, parkNone

University_B = (20, 16), (71, 61), 1, straight, go, 40, 300, parkNone

University_C = (71, 61), (86, 90), 1, curve, go, 40, 300, parkNone

University_D = (86, 90), (82, 104), 2, straight, go, 40, 300, parkNone

University_E = (82, 104), (74, 120), 2, straight, go, 40, 300, parkNone

University_F = (74, 120), (65, 165), 2, straight, go, 40, 300, parkNone

University_A1 = (10, 10), (20, 16), 2, straight, back, 40, 300, parkNone

University_B1 = (20, 16), (71, 61) , 1, straight, back, 40, 300, parkNone

University_C1 = (71, 61), (86, 90) , 1, curve, back, 40, 300, parkNone

University_D1 = (86, 90), (82, 104), 2, straight, back, 40, 300, parkNone

University_E1 = (82, 104), (74, 120), 2, straight, back, 40, 300, parkNone

## University_F1 = (74, 120), (65, 165), 2, straight, back, 40, 300, parkNone

/* University drive. Section A,D,E and F are 2 way and 2 lane. */

Library_A = (20, 16), (35, 107), 1, straight, go, 40, 300, parkNone

Library_B = (35, 107), (62, 113), 1, straight, go, 40, 300, parkNone

Library_C = (35, 107), (35, 130), 1, straight, go, 40, 300, parkNone

Library_A1 = (20, 16), (35, 107), 1, straight, back, 40, 300, parkNone

Library_B1 = (35, 107), (62, 113), 1, straight, back, 40, 300, parkNone

Library_C1 = (35, 107), (35, 130), 1, straight, back, 40, 300, parkNone

/* library road. 2 way 1 lane and no parking */

Campus_A = (71, 61), (62, 113), 1, straight, go, 40, 300, parkNone

Campus_B = (62, 113), (53, 160), 1, straight, go, 40, 300, parkNone

Campus_C = (53, 160), (25, 188), 1, straight, go, 40, 300, parkNone

Campus_A1 = (71, 61), (62, 113), 1, straight, back, 40, 300, parkNone

Campus_B1 = (62, 113), (53, 160), 1, straight, back, 40, 300, parkNone

Campus_C1 = (53, 160), (25, 188), 1, straight, back, 40, 300, parkNone

/* campus avenue. 2 way 1 lane and no parking */

Anniversary_A = (53, 160), (65, 165), 1, straight, go, 40, 300, parkNone

Anniversary_B = (65, 165), (90, 180), 2, straight, go, 40, 300, parkNone

Anniversary_A1 = (53, 160), (65, 165), 1, straight, back, 40, 300, parkNone

Anniversary_B1 = (65, 165), (90, 180), 2, straight, back, 40, 300, parkNone

/* anniversary road. Anniversary_A/A1 is 2 way 1 lane and Anniversary _B/B1 is 2 way 2

lane */

SunnySide_A = (90, 180), (102, 190), 1, straight, go, 40, 300, parkNone

SunnySide_A1 = (90, 180), (102, 190), 1, straight, back, 40, 300, parkNone

/* sunnyside drive. 2 way 1 lane */

Raven_A = (86, 90), (99, 95), 1, straight, go, 40, 300, parkNone

Raven_A1 = (86, 90), (99, 95), 1, straight, back, 40, 300, parkNone

Raven_B = (99, 95), (110, 128), 1, curve, go, 40, 300, parkNone

Raven_B1 = (99, 95), (110, 128), 1, curve, back, 40, 300, parkNone

/* raven road. 2 way 1 lane */

Drive_A = (82, 104), (92, 110), 1, straight, go, 40, 300, parkNone

Drive_A1 = (82, 104), (92, 110), 1, straight, back, 40, 300, parkNone

Drive_B = (92, 110), (99, 95), 1, straight, go, 40, 300, parkNone

Drive_B1 = (92, 110), (99, 95), 2, straight, back, 40, 300, parkNone

/* admin drive. Drive_A/A1 is 2 way 1 lane. Drive_B/B1 is 2 way and B1 is 2 lane */

Bronson_A = (124, 115), (90, 180), 3, straight, go, 60, 300, parkNone

Bronson_B = (90, 180), (80, 190), 3, straight, go, 60, 300, parkNone

Bronson_A1 = (124, 115), (90, 180), 3, straight, back, 60, 300, parkNone

Bronson_B1 = (90, 180), (80, 190), 3, straight, back, 60, 300, parkNone

/* bronson drive. 2 way 3 lane */

end segments

begin crossings

c1 = (10, 10), 20, withoutTL, withoutHole, 300, 3

/* intersection (10,10) of colonel by drive and university drive */

c2 = (20, 16), 20, withoutTL, withoutHole, 300, 3

c3 = (35, 107), 20, withoutTL, withoutHole, 300, 3

c4 = (53, 160), 20, withoutTL, withoutHole, 300, 3

c5 = (62, 113), 20, withoutTL, withoutHole, 300, 3

c6 = (71, 61), 20, withoutTL, withoutHole, 300, 3

c7 = (86, 90), 20, withoutTL, withoutHole, 300, 3

c8 = (65, 165), 20, withoutTL, withoutHole, 300, 3

c9 = (90, 180), 20, withoutTL, withoutHole, 300, 3

c10 = (82, 104), 20, withoutTL, withoutHole, 300, 3

c11 = (92, 110), 20, withoutTL, withoutHole, 300, 3

c12 = (99, 95), 20, withoutTL, withoutHole, 300, 3

c13 = (74, 120), 20, withoutTL, withoutHole, 300, 3

c14 = (35, 130), 20, withoutTL, withoutHole, 300, 3

c15 = (25, 188), 20, withoutTL, withoutHole, 300, 3

c16 = (110, 128), 20, withoutTL, withoutHole, 300, 3

end crossings

The TSC compiler will generate the Cell-DEVS specification according to the above TSC definition. The cell size of segment is determined by the boundaries of the segment in the map. The cell size of crossing is determined by number of input/output segments in a crossing. The following is the partial CD++ source code generated by TSC.

[TOP]

components : BigCounter@TSCCounter

components : Library_B

components : Library_B1

components : Raven_A

components : Raven_A1

components : Anniversary_A

components : Anniversary_A1

components : University_A

components : University_A1

components : Drive_A

components : Drive_A1

components : Anniversary_B

components : Anniversary_B1

components : SunnySide_ACons@TSCConsumer

components : SunnySide_A

components : SunnySide_A1Gen@TSCGenerator

components : SunnySide_A1

…

out : arrived_BigCounter

out : solved_BigCounter

out : qty_Cons_SunnySide_A

out : qty_Cons_Acum_SunnySide_A

…

link : y_c_puedesalir038@Campus_C x_t_puedeentrar1@c15

link : y_t_hayauto0@c15 solved@c15Counter

link : throughputA@c15Counter arrived_c15Counter

link : throughputS@c15Counter solved_c15Counter

[Library_B]

type : cell

width : 27

height : 1

delay : transport

defaultDelayTime : 200

border : nowrapped

neighbors : Library_B(0,-1) Library_B(0,0) Library_B(0,1)

initialvalue : 0

in : x_c_hayauto00

in : x_c_puedeentrar00

out : y_c_haylugar00

in : x_c_haylugar026

out : y_c_hayauto026

out : y_c_puedesalir026

link : x_c_hayauto00 x_c_hayauto@Library_B(0,0)

link : x_c_puedeentrar00 x_c_puedeentrar@Library_B(0,0)

link : y_c_haylugar@Library_B(0,0) y_c_haylugar00

link : x_c_haylugar026 x_c_haylugar@Library_B(0,26)

link : y_c_hayauto@Library_B(0,26) y_c_hayauto026

link : y_c_puedesalir@Library_B(0,26) y_c_puedesalir026

portInTransition : x_c_puedeentrar@Library_B(0,0) segment1-puedeentrar-startcross-rule

portInTransition : x_c_hayauto@Library_B(0,0) segment1-startcross-rule

portInTransition : x_c_haylugar@Library_B(0,26) segment1-endcross-rule

localtransition : segment1-lane-rule

...

[c15-cellOut-rule]

%%%rule : { 0 + send(y_t_puedesalir,0)} 200 { (0,0) = 0 and (0,-1)= 1  and random >= 1/3}

rule : { 3 + send(y_t_puedesalir,0)} 0 { (0,0) = 0 and (0,-1)= 1  and random >= 1/3}

rule : 0  200  { (0,0)= 3 }

rule : 1       200 { (0,0) = 0 and (0,-1) = 1 }

rule : 0       200 { (0,0) = 1 and (0,1) = 0 }

rule : {(0,0)} 200  { t }

# Simulation execution results and analysis

Based on the basic model of the Carleton University Campus, many changes have been added to obtain the results under different circumstances.

- Including the non synchronized traffic lights in the two crossings: intersection of colonel by/university Drive and sunnyside/bronson (c1 and c9).

c1 = (10, 10), 20, withTL, withoutHole, 300, 3

c9 = (90, 180), 20, withTL, withoutHole, 300, 3

The simulation is executed in ten minutes. From the figure 1, It is apparent that adding the traffic lights increases the total number of cars in the campus.

Figure 1

In the first minute, due to the non-synchronized traffic lights, the number of vehicles stay in the campus is increased by 10. In the following minutes, it reduced to the normal number but is still a little higher than without the traffic lights. This indicates that although traffic lights are useful in ordering traffic, the non-synchronized lights could actually slow down the vehicles traveling out of the area. Checking the cars go through the crossing C9 in Figure 2, the number of cars arriving and leaving this crossing is reduced comparing with no traffic light previously. From figure 2 we can see that traffic flow in crossing 9 is regulated when the traffic light is added.

Figure 2

From the other point of view, we check the cars leaving the campus from the segment Bronson_B in figure 3. The influence of the traffic light can be seen clearly, the number of cars leaving the segment Bronson_B is reduced when traffic light is added in crossing C9. In this case, the traffic light didn’t solve the campus traffic flow, on the contrary, it results in the high number of cars in the campus due to the fact that it is not synchronized. Because segment Bronson_B is a main traffic road, in order to avoid the traffic jams on this road, the traffic light limit the traffic flow leaving campus through this crossing.

Figure 3

- There is a railway go through the campus, but it didn’t affect the local traffic flow because there is bridges built to avoid crossing with roads in the campus. We suppose the railway is crossing with the road in order to test the influence of the railway.

begin railnets

rn = (University_C, 9), (University_C1, 4), (Anniversary_A, 7), (Anniversary_A1, 5), 200

end railnets

This railway has crossings with the segment University_C, University_C1, Anniversary_A, and Anniversary_A1. The first affected crossing is C7. As Figure4 indicates, in the first minute, the number of cars is reduced to 0 due to the closing of the barrier, and no cars arrive crossing C7. In the following minutes, the cars increase largely and reach the maximum number with all cars passing through the railway cross.

Figure 5 describes the number of cars going through crossing C7, C10, C11 and C12 in the first minute. The result shows that in the first minute, no cars went through crossing C10, C11, C12 and C7. This indicates that when a train come through the campus, all the cars are jammed in the current crossing and could not reach nearby crossings.

Since the traffic flow is not very high in the campus, adding the railway didn’t affect the total number of cars in the campus on average over period of time. However, the traffic load did increase dramatically in a short time in the crossings around the railway, which means the railway still has the potential to make the traffic jams.

In general, if a railway cross with the street, it will always induce the traffic jam in the current crossing.

Figure 4

Figure 5

- Adding traffic signs in some segments to check the traffic flow changes.

begin ctrElements

in University_B : stop, 0, 200

in University_C : stop, 0, 200

in University_D : stop, 0, 200

in University_E : stop, 0, 200

in Campus_A : stop, 36, 200

in Campus_B : stop, 0, 200

in Campus_C : stop, 0, 200

in Anniversary_A1 : stop, 0, 200

end ctrElements

Figure 8						Figure 9

Figure 6,7,8 and 9 compares these crossings (C7, C8, C10 and C13) with stop signs and without stop sign. The above figures explain the results obtained at each of the crossings, the number of cars go through these crossings is reduced. Therefore, the stop signs regulated the traffic flow in these crossings. With respect to the total number of cars in the campus in Figure 10, no big difference comparing with the model without stop signs. Only in the last two minutes, the cars stay in the campus increased as the stop signs slows down the traffic leaving the campus and cause the total number of cars increased at some point.

Figure 10

- Adding both traffic light and stop sign in these crossings to check the result.

In Figure 11, the total number of cars in the campus is increased by 12. The cars leaving the segment Bronson_B are also reduced due to the fact that less traffic left the campus. The traffic lights limit the number of cars leaving campus.

Figure 11

Figure 12

- Adding jobsite in the segment Campus

begin jobsites

in Campus_B1 : 1, 25, 1, 200

in Campus_A1 : 1, 12, 1, 200

end jobsites

Use the crossing C5 as a reference point, which is just before the jobsite in Campus_B. In Figure 13, the number of cars increased largely in the first minute due to the fact that people entering and exiting the job site. For such a small crossing, the arrived and solved cars accumulate to 30. In other word, job site could introduce potential traffic jam at its nearby crossings at certain instances. There is also another interesting fact, that is, the traffic flow went below the usual rate after certain point. This is because the job site absorbs/produce traffic flows at one point result less flow at other time since the total number of cars on average are similar. We can imagine this case being people go to work. During the rush hours, the traffic flow is high; however, for the rest of the time, the traffic flow is below average.

Figure 13

- Varying the experimental framework (TSCGenerator) input rates to increase the generated cars.

From Figure 14, during the first minute, the number of vehicles in the campus increased exponentially, after several minutes the number of cars is reduced. During the first few minutes, as the arrival increases all of a sudden, there are jams created in some of the crossings; however, after a while, as the cars started moving around/leaving the campus, the number of cars left in the area decreased. This indicates that the campus can absorb more traffic flow without causing delays than it was in the previous models.

Figure 14

Output for each of the sub-models (Cell-DEVS)

In the ATLAS construction, there are two basic Cell-DEVS sub-models: segment and crossing. Because the Carleton University Campus model is very big, it requires a long time to draw the output result. Here we use a simple example to obtain the output of these two cell sub-models since the cell behavior is the same. For detailed graphic output, please refer to the zip file attached.

begin segments

t1 = (1,5),(1,1),2,straight,go,40,300,parkNone

t2 = (1,1),(5,1),2,straight,go,40,300,parkNone

end segments

begin crossings

c1 = (1,1),20, withoutTL, withoutHole, 300, 3

end crossings

Output for t2:

Line : 240 - Time: 00:00:01:400

0  1  2  3

+------------+

0|  1  0  0  0|

1|  0  0  0  0|

+------------+

Line : 256 - Time: 00:00:01:600

0  1  2  3

+------------+

0|  0  1  0  0|

1|  0  0  0  0|

+------------+

Line : 274 - Time: 00:00:01:800

0  1  2  3

+------------+

0|  0  0  1  0|

1|  0  0  0  0|

+------------+

Line : 295 - Time: 00:00:02:000

0  1  2  3

+------------+

0|  0  0  0  1|

1|  0  0  0  0|

+------------+

…

Conclusion:

In this project, a complex traffic model (Carleton University Campus) is defined by ATLAS [1] specification language. The TSC [3] compiler is used as a tool to translate them into executable files that can be run under the CD++ [2]. The simulated traffic flow is analyzed under different topology of the campus. With this approach, several advantages are obtained in simulating such a complex traffic model. First of all, the high-level specification language describes the model. TSC tool automatically builds the coupled model structure and generates the rules for cell DEVS models. With this approach, the modeler can change the model specification efficiently without rewriting code and therefore saves a lot of time on problem solving. Secondly, by using the ATLAS, although traffic model is a complex simulation, each of the micro-simulation results can be analyzed individually which is difficult to be analyzed by the traditional analytical solutions. Finally, the TSC templates are adaptive. The templates can be modified accordingly to fit different simulation tools without having to change the high-level specification.

The analysis of the campus traffic flow under different circumstances concludes that the TSC approach is useful in analyzing such models. Different traffic signs, traffic lights can be modeled to reflect the real situations easily. This is especially helpful in city traffic planning in order to provide the optimal constructions. These analytical results also prove that the current constructions in the campus works well for traffic controlling and the campus can handle more traffic without introduce traffic jams.

References

[1] Mariana Lo Tartaro, Cesar Torres and Gabriel Wainer. Defining models of urban traffic using the TSC tool. Proceedings of the 2001 Winter Simulation Conference.

[2] Daniel A. Rodriguez and Gabriel A. Wainer. CD++ User’s Guide.

[3] Cesar Torres and Mariana Lo Tartaro. Structure and Functionality of TSC(traffic simulation compiler).