### 94.5807  Modelling Discrete-Event Systems Using DEVS (June 2006)

#### Assignment1: Bluetooth Simulator

# Shafagh Jafer 100305285

Carleton University

Conceptual Model Description

Bluetooth is a wireless connection that enables devices such as mobile phones, computers and PDAs to exchange information. This assignment uses CD++ tool to model a Bluetooth Simulator which demonstrates the behavior of sending data from a sender mobile to a receiver mobile. In real world, data such as sound clips, videos, images, and pure text messages are sent back and forth between two paired mobile devices. Figure 1 illustrates the components of the Bluetooth Simulator. As shown in the figure, the model consists of four components: SenderMobile, ReceiverMobile, DataTransmission, and CennectionManager. The ConnectionManager is further decomposed into PairedDeviceFinder, and PermissionVerifier.

Figure 1  Structure of Bluetooth Simulator

The sender is activated when it receives “enableBluetooth” command which means the user of the mobile device wants to send data to another mobile using Bluetooth. Once the sender mobile gets its Bluetooth enabled, the connection manager is responsible to search for other mobile devices that are in range and have their Bluetooth feature on. As soon as a paired device is found, its ID is sent to the sender mobile (it will be displayed on the mobile screen, for simplicity in this assignment it is assumed that there is only one paired device available in that range). Once the sender mobile is informed about the existence of the receiver mobile, it will issue a “connect” command to start the transmission of data. The connection manager is then activated again and will request permission from the receiver mobile to transfer the data. After that, the receiver mobile will grant the permission and the connection manager will notify the sender mobile to start the transmission. The sender mobile sends the data to the dataTransmission handler which takes care of ensuring enough capacity at the receiver as well as reliable transmission. Once the transmission is done successfully the sender is informed and it goes back to passive state by disabling its Bluetooth and waiting for another input command (i.e. “enableBT”).

All the actions mentioned above are based on constant time units that will be described in .cpp files as well as in Part II and Part III of the assignment.

# DEVS Formal Specification and Testing Strategies

As shown in Figure 1, the Bluetooth Simulator has 1 input and 1 output. The enableBT input indicates that the user of SenderMobile would like to start a Bluetooth transfer of data to a ReceiverMobile. Whenever this command is issued the SenderMobile is activated and its state changes from passive to active. The newMsgArrival output indicates that there is new message received. At the ReceiverMobile there is a counter which counts the number of new messages arrived. The coupled component ConnectionManager and the atomic component DataTransmission handle connection establishment and data transfer between SenderMobile and ReceiverMobile atomic components.

Formal Specifications for Atomic Models

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic models are defined as follows:

ReceiverMobile:

Assumption: the receiver will always accept the request of receiving data from other mobile device. It is assumed that the sender and receiver mobiles know each other. Also, the receiver’s Bluetooth is always enabled.

S = {phase, ID, totalMsgs, ID_requested, permission_given, gotMsg   }

X = {IDrequested, permit, data}

Y = {IDout, accepted, gotData, newMsgArrival}

δext  (S,e,X) {

Case port

IDrequest:

Case Phase

Passive:

Phase = busy;

ID_requested = 1;            //set it to True

permission_given = 0;     //set it to False

gotMsg = 0;                     //set it to False

busy:

// Ignore the request;

Permit:

Case Phase

Passive:

permission_given = 1;   //set it to True

ID_requested = 0;         //set it to False

gotMsg = 0;                 //set it to False

Phase = busy;

busy:

// Ignore the request;

Data:

Case Phase

Passive:

gotMsg = 1;                         //set it to True

ID_requested = 0;               //set it to False

permission_given = 0;        //set it to False

totalMsgs = totalMsgs +1; //increment # of msgs received

Phase = busy;

busy:

// Ignore the request;

}

λ (S) {

case phase

busy:

If   ID_requested

//if the ID is requested, send it to out

Send (IDout, ID);

If   permission_give

// the receiver has accepted to receive Data

Send (accepted);

Else   //gotMsg is True

// Data was received successfully

Send (gotData);

// notify the receiver that newMsg has arrived

Send (newMsgArrival);

Passive: // Should not happen.

}

δint (S) {

case phase

busy:

phase = passive;

passive: //never happens

}

Testing can be done by inputting one or more events at the input ports and observing the outputs:

Output:

Should get: ID 123

Should get: 1

Should get: 1

Should get: numOfMsgs 1

Input:

00:00:10:00 IDrequested 1

00:00:15:00 permit 1

00:00:30:00 data 1

DataTransmission:

Once connection is established, the SenderMobile will send the data through the DataTransmission to the ReciverMobile. At the receiver when the data is received, the DataTransmission will signal the sender that data was transmitted successfully. The Sender then goes back to passive state and waits for the next “enableBT” request.

S = {phase, data, dataReceived}

X = {dataOut, gotData}

Y = {dataSent, data}

δext  (S,e,X) {

Case port

dataOut:

Case Phase

Passive:

Phase = busy;

data_received = 0;  //set it to False

data = d;                 //data recived at the dataOut port

busy:

// Ignore the request;

gotData:

Case Phase

Passive:

dataReceived = 1;   //set it to True

Phase = busy;

busy:

// Ignore the request;

}

λ (S) {

case phase

busy:

If   dataReceived

//if the receiver got the data, notify the sender

Send (dataSent);

Else

// send the data to the receiver

Send (dataPort, data);

Passive: // Should not happen.

}

δint (S) {

case phase

busy:

phase = passive;

passive: //never happens

}

Testing can be done by inputting one or more events at the input ports and observing the outputs:

Input:

Output:

Should get: Data = 555

Should get: dataSent successfully

00:00:10:00 dataOut 555

00:00:15:00 gotData 1

PairedDeviceFinder:

This unit is responsible for searching paired devices (mobile sets that are Bluetooth enabled and are in range). SenderMobile requests to have the ID of the paired devices. Once a paired device is found its ID is sent to the SenderMobile. In this assignment it is assumed that there is only one receiver mobile available in that area.

S = {phase, receiverID, reqSearch}

X = {search, IDin}

Y = {getID, deviceID}

δext  (S,e,X) {

Case port

search:

Case Phase

Passive:

Phase = busy;

reqSearch = 1;  //set it to True

busy:

// Ignore the request;

IDin:

Case Phase

Passive:

receiverID = id; //get the id from the receiver

reqSearch = 0;  //set it to False

Phase = busy;

busy:

// Ignore the request;

}

λ (S) {

case phase

busy:

If   reqSearch

//ask the receiver to provide its ID

Send (getID);

Else

// send the ID to the sender

Send (DeviceID, receiverID);

Passive: // Should not happen.

}

δint (S) {

case phase

busy:

phase = passive;

passive: //never happens

}

Testing can be done by inputting one or more events at the input ports and observing the outputs:

Input:

Output:

Should get: getid

Should get: deviceid = 123

00:00:10:00 search 1

00:00:15:00 IDin 123

PermissionVerifier:

After getting the ID of the receiver, the sender will have to ask for permission in order to start the data transmission. PermissionVerifier unit will handle this by coordinating between the sender and receiver.

S = {phase, accessOK, reqConnection }

X = {connectDevice, granted}

Y = {connected, reqPermission}

δext  (S,e,X) {

Case port

connectDevice:

Case Phase

Passive:

Phase = busy;

reqConnection = 1;  //set it to True

accessOK = 0;         //set it to False

busy:

// Ignore the request;

granted:

Case Phase

Passive:

accessOK = 1;          //set it to True

reqConnection = 0;  //set it to False

Phase = busy;

busy:

// Ignore the request;

}

λ (S) {

case phase

busy:

If   reqConnection

//ask for permission form the receiver

Send (reqPermission);

Else    // accessOK is True

// inform sender to start transmission

Send (connected);

Passive: // Should not happen.

}

δint (S) {

case phase

busy:

phase = passive;

passive: //never happens

}

Testing can be done by inputting one or more events at the input ports and observing the outputs:

Output:

Should get: reqpermission

Should get: connected

Input:

00:00:10:00 connectDevice 1

00:00:15:00 granted 1

SenderMobile:

Assumption: the receiver will always accept the request of receiving data from other mobile device. It is assumed that the sender and receiver mobiles know each other. Also, the receiver’s Bluetooth is always enabled.

S = {phase, gotID, receiverID,enable, sending }

X = {enableBT, deviceFound, start, dataSent }

Y = {searchDevice, connect, dataOut }

δext  (S,e,X) {

Case port

enableBT:

Case Phase

Passive:

Phase = busy;

enable = 1;     //set it to True

sending = 0;   //set it to False

gotID = 0;      //set it to False

busy:

// Ignore the request;

deviceFound:

Case Phase

Passive:

gotID = 1;       //set it to True

sending = 0;   //set it to False

enable = 0;    //set it to False

receiverID = id; // got the ID of the receiver

Phase = busy;

busy:

// Ignore the request;

start:

Case Phase

Passive:

sending = 1;  //set it to True

gotID = 0;      //set it to False

enable = 0;    //set it to False

Phase = busy;

busy:

// Ignore the request;

dataSent:

Case Phase

Passive:

sending = 0;   //set it back to False

gotID = 0;      //set it to False

enable = 0;    //set it to False

Phase = busy;

busy:

// Ignore the request;

}

λ (S) {

case phase

busy:

If   enable

//search for available paired devices

Send (searchDevice);

If   gotID

// ask for permission to send data

Send (connect);

Else //   sending is True

// send data

Send (dataOut, theData);

Passive: // Should not happen.

}

δint (S) {

case phase

busy:

phase = passive;

passive: //never happens

}

Testing can be done by inputting one or more events at the input ports and observing the outputs:

Input:

Output:

Should get: deviceID= 123

Should get: connected

Should get: data = 555

00:00:10:00 enableBT 1

00:00:15:00 deviceFound 123

00:00:30:00 start 1

00:00:40:00 dataSent 1

Formal Specifications for Coupled Models

The formal specifications <X, Y, D, EIC, EOC, IC, SELECT > for the coupled model Network and ABP Simulator are defined as follows:

ConnectionManager:

X = {search, IDin, connectDevice, granted};

Y = {deviceID, getID, connected, reqPermission };

D = {PermissionVerifier, PairedDeviceFinder};

EIC = {( ConnectionManager.search, PariedDeviveFinder.search),                                                             (ConnectionManager.IDin, PariedDeviveFinder.IDin ),

(ConnectionManager. connectDevice, PermissionVerifier. connectDevice), (ConnectionManager.granted, PermissionVerifier. granted)}

EOC = {( ConnectionManager. deviceID, PariedDeviveFinder. deviceID),                                                             (ConnectionManager. getID, PariedDeviveFinder. getID),

(ConnectionManager. connected, PermissionVerifier. connected), (ConnectionManager. reqPermission, PermissionVerifier. reqPermission)}

IC = {f}

SELECT:  ({PermissionVerifier, PairedDeviceFinder }) = PairedDeviceFinder;

For the coupled model testing can be done by inputting one or more events at the input ports and observing the outputs. The atomic components work independently and therefore the test cases and their results are very similar to what was done for the atomic cases of these models.

Input:

Output:

Should get: getid 1

Should get: deviceid 123

Should get: reqpermission 1

Should get: connected 1

00:00:10:00 search 1

00:00:20:00 IDin 123

00:00:30:00 connectDevice 1

00:00:40:00 granted 1

BluetoothSimulator Simulator:

X = {enableBT };

Y = {newMsgArrival };

D = {SenderMobile, ReceiverMobile, ConnectionManager, DataTransmission };

EIC = {( BluetoothSimulator.enableBT, SenderMobile. enableBT),

EOC = {( BluetoothSimulator. newMsgArrival, ReceiverMobile. newMsgArrival),

IC = { (SenderMobile.searchDevice, ConnectionManager.search), (SenderMobile.connect, ConnectionManager.connectDevice),

( ConnectionManager.deviceID, SenderMobile.deviceFound),

( ConnectionManager.connected, SenderMobile.start) ,( SenderMobile.dataOut, DataTransmission.dataOut), ,( SenderMobile.dataSent, DataTransmission.dataSent) ,

( DataTransmission.data, ReceiverMobile.data) , ( DataTransmission.gotData, ReceiverMobile.gotData), (ReceiverMobile.IDrequested, ConnectionManager. IDrequested) , (ReceiverMobile.IDout, ConnectionManager. IDout)

(ReceiverMobile.permit, ConnectionManager. permit)

(ReceiverMobile.accepted, ConnectionManager. accepted) }

SELECT:  ({DataTransmission, ConnectionManager }) = ConnectionManager

({SenderMobile, ConnectionManager }) = SenderMobile

({SenderMobile, DataTransmission }) = SenderMobile

For the top coupled model testing can be done by inputting only one event which is “enableBT”. This is the command issued by the user of the SenderMobile requesting to enable its Bluetooth and transferring data to other paired devices. At the end of simulation the out put “newMsgArrival” is expected which shows how many massages were received. For example if 3 “enableBT” inputs are written in the “.ev” file the following results should be seen:

Input:

Output:

totalMsg =  1

totalMsg =  2

totalMsg =  3

00:00:00:00 enableBT 1

00:00:30:00 enableBT 1

00:00:60:00 enableBT 1

Results of the Simulation Using CD++

Assumption made for simulating the models:

- Data is hard coded as an integer = 555 and the receiverID = 123

- Other than data transfer process which takes 5 units of time, every other process such as searching for paired device, requesting permission, granting permission, establishing a connection, etc…take 2 units of time.

Execution Results for atomic model SenderMobile:

By compiling the project and choosing “senderMobile.ma” and “senderMobile.ev” the following results are observed:

Input:

Output:

00:00:12:000 searchdevice 1

00:00:17:000 connect 123

00:00:35:000 dataout 555

00:00:10:00 enableBT 1

00:00:15:00 deviceFound 123

00:00:30:00 start 1

00:00:40:00 dataSent 1

Execution Results for atomic model PairedDeviceFinder:

By compiling the project and choosing “PairedDeviceFinder.ma” and “PairedDeviceFinder.ev” the following results are observed:

Input:

Output:

00:00:12:000 getid 1

00:00:17:000 deviceid 123

00:00:10:00 search 1

00:00:15:00 IDin 123

Execution Results for atomic model PermissionVerifier:

By compiling the project and choosing “PermissionVerifier.ma” and “PermissionVerifier.ev” the following results are observed:

Output:

00:00:12:000 reqpermission 1

00:00:17:000 connected 1

Input:

00:00:10:00 connectDevice 1

00:00:15:00 granted 1

Execution Results for atomic model DataTransmission:

By compiling the project and choosing “DataTransmission.ma” and “DataTransmission.ev” the following results are observed:

Output:

00:00:12:000 dataport 555

00:00:17:000 datasent 555

Input:

00:00:10:00 dataOut 555

00:00:15:00 gotData 1

Execution Results for atomic model ReceiverMobile:

By compiling the project and choosing “ReceiverMobile.ma” and “ReceiverMobile.ev” the following results are observed: (repeating the requests twice to simulate a sender who sends two messages to the receiver)

Output:

00:00:12:000 idout 123

00:00:17:000 accepted 1

00:00:35:000 gotdata 1

00:00:35:000 newmsgarrival 1

00:00:42:000 idout 123

00:00:47:000 accepted 1

00:00:55:000 gotdata 1

00:00:55:000 newmsgarrival 2

Input:

00:00:10:00 IDrequested 1

00:00:15:00 permit 1

00:00:30:00 data 1

00:00:40:00 IDrequested 1

00:00:45:00 permit 1

00:00:50:00 data 1

Execution Results for coupled model ConnectionManager:

The simulation of this coupled model is very similar to what was explained already in the atomic simulation. The following show the results:

Output:

00:00:12:000 getid 1

00:00:22:000 deviceid 123

00:00:32:000 reqpermission 1

00:00:42:000 connected 1

Input:

00:00:10:00 search 1

00:00:20:00 IDin 123

00:00:30:00 connectDevice 1

00:00:40:00 granted 1

Execution Results for the top coupled model BluetoothSimulator:

The simulation of the top coupled model is performed by sending only the input “enableBT” since the top component has only one input and from the other hand we should expect only one out put “newMsgArrival”. The following show the results:

(Please note that the following simulation shows the case where there are 3 attempts of sending data to the receiver mobile.)

Output:

00:00:28:000 newmsgarrival 1

00:00:58:000 newmsgarrival 2

00:01:28:000 newmsgarrival 3

00:02:28:000 newmsgarrival 4

00:03:28:000 newmsgarrival 5

00:05:28:000 newmsgarrival 6

00:40:28:000 newmsgarrival 7

Input:

00:00:00:00 enableBT 1

00:00:30:00 enableBT 1

00:00:60:00 enableBT 1

00:00:120:00 enableBT 1

00:00:180:00 enableBT 1

00:00:2400:00 enableBT 1

00:00:300:00 enableBT 1

From the above simulation results we can see that at the receiver end 7 new messages were received. This result and also the corresponding timings verify the correctness of our model.