Elements of Brain Machine in DEVS and Cell-DEVS environments

Yuri Boiko and Gabriel Wainer

Carleton University

1125 Colonel By Drive,

Ottawa, ON, K1S 5B6 CANADA

Abstract: - Simulation of selected elements of Brain Machine is demonstrated in DEVS and Cell-DEVS environment employing CD++ toolkit.  Neural decoder based on spiking neurons is chosen for modeling in DEVS formal definitions, while modeling of quantum dot cellular devices is implemented within Cell-DEVS formal definitions.  Signal of the encoded in ternary alphabet test messages of spike sequences is employed to verify functionality of the spiking neural decoder.  Spike sequences are split between two channels – one for initiating spikes and another one for terminating ones.  Spiking neurons with rectangular response function are considered in the presented model.  Firing condition for the spiking neuron is reached when two rectangular responses, one for the initiating spike and another one for terminating spike, overlap in time domain, as a result producing “1” at the output (firing signal) or alternatively “0” (non-firing output signal).  For Cell-DEVS modeling the following quantum dot based devices have been selected: quantum XOR, quantum wire and majority vote gates.  Demonstrated is operation of models of these devices in Cell-DEVS environment.

Key-Words: - 	Brain Machine, spiking neuron, spiking decoding, neural spiking decoder, quantum automata, XOR, quantum wire, majority vote gates, DEVS, Cell-DEVS.

1   Introduction

Modeling of the various elements of the Brain Machine is currently in the focus of the simulation and design research communities [1 - 6] due to expected technological impact of the progress in the area as well as due to significant benefits, which modeling can offer in optimization of the design and achieving operational efficiency of the complex systems, which are expensive to implement and experiment with in hardware.   DEVS (Discrete Event System Specification) and Cell-DEVS methodologies recently gained recognition for its usefulness in modeling various systems of artificial and/or natural descent [7 - 10].  DEVS formalism divides systems under study into atomic models,   which contain discrete number of states and is equipped with the input and output ports for interaction with each other and external environment.  In this approach atomic models are used as building blocks of more complex coupled models, which constitute next level in the hierarchy of model complexity.  In turn, coupled models can be used as building blocks for the next hierarchical levels, thus opening way of creating models of any desired level of sophistication.  The discrete representation of the time scale of events, in which only meaningful events are accounted for in simulation, allows retaining speed of the simulation even of highly complex models.   Cell-DEVS is an extension of the DEVS formalism to the systems, which can be described by cellular automata approach, in which atomic models are represented by individual cells, connected with their neighbors and by means of the designated rules governing operation of the model.  Herewith the implementation of the quantum dot based devices models are demonstrated using Cell-DEVS formal specifications.  For both cases, DEVS and Cell-DEVS, CD++ toolkit is used for programming the models.  Simulation is implemented in Eclipse integrated environment with some functions implemented as C++ modules.

.

2   DEVS Problem Formulation

and Implementation

2.1	Model of Spiking Neural Decoder

Spiking neural networks are regarded as a next generation devices with significant advantage in speed over the traditional neural network devices [11 - 18]. In the present study DEVS formalism is employed in defining model of the neural spiking decoder, which function is to decode the incoming signal of spike sequences.  The incoming messages are encoded in terms of time intervals between the initiating and terminating spikes, which here for simplicity of recognition are split into two parallel but separate channels.   The general structure of the neural spiking decoder under consideration is shown in Fig.1.  The incoming message is the series of spikes, where symbols are encoded in terms of time intervals t between odd and even spikes, where odd spikes initiate the time intervals of the symbol and even spikes terminate it.  Ternary alphabet is used with symbols selected from the set {A; B; C}.  As it is depicted in Fig.1 , the encoding table of the alphabet symbols via t intervals is as follows: 1 msec< t < 4 msec for “A”, 5 msec< t < 8 msec for “B”, and 9 msec< t < 12 msec for “C”.  Spiking neurons A, B and C are responding to incoming symbol with output of 1 if the symbol is respectively “A”, “B” or “C”, while producing 0 as the output otherwise.  In other words, neuron A is responding with output of 1 if incoming spikes are separated by t as that for symbol “A” from the encoding table and  with output of 0 otherwise (i.e. for symbols  “B” and “C”).  Neurons B and C behave similarly, but for spikes with encoded symbols “B” and “C” respectively.

C

100 = A

010 = B

001 = C

{0;1}

{0;1}

{0;1}

3 msec

5 msec

9 msec

A: t= [1, 4] msec

B: t= [5, 8] msec

C: t= [9, 12] msec

CBA

A

B

Fig.1. Schematic of model of the neural spiking decoder. Alphabet is {A; B; C} with encoding rules listed via spike intervals.  At the input there are three test combinations encoded in time intervals, at the output – the decoded sequence in terms of {0;1} triples.  Neurons A, B and C are firing “1” in response to encoded by spikes intervals symbols “A”, “B” and “C”, or outputting “0” otherwise.

2.2	Model of Spiking Neuron Terminal

In the present model each spiking neuron is represented as a terminal consisting of two parallel routing lines connecting two nodes with rectangular response function, as it is shown in Fig.2.  One routing line (1) is for the initiating spikes (odd spikes) while the other one (2) is for terminating spikes (even spikes).  Nodes (3) and (4) are to generate rectangular response for the incoming spike as well as node (3) of the initiating spike is introducing time delay  to ensure that only for the

threshold

∑

Firing

conditions:

in

out

1

2

3

4

Fig.2. General schematic of the simplest Spiking Neuron Terminal with two separate routings for initiating and terminating spikes.

spikes with predetermined time interval between initiating and terminating spikes the rectangular pulses overlap.  The overlapping of the rectangular pulses (which are the node’s responses to the incoming spikes) ensures that the peak amplitude of the signal at the output node reaches threshold value or neuron firing condition, which makes the output node to produce “1”.  Alternatively, when pulses do not overlap in time, the threshold condition is not met and the output node of the neuron produces “0” (non-firing output).  Here the spiking neuron terminal as with functionality depicted in Fig.2 is designed to include the following atomic models: the timer to measure the time intervals and the controller to generate proper output value depending on the value of the time interval between initiating and terminating spikes, thus producing the effect of the overlapping or non-overlapping rectangular response pulses on incoming spikes.   Coupled model of the spiking neuron terminal is shown in Fig.3.  The functionality of the coupled model in Fig.3 is the same as the one shown in Fig.2 – i.e. the model is producing output “1” or “0” depending on whether time interval between initiating and terminating spikes meet the firing condition.

Pulses

Trans-former

Controller

Timer

Input #2

Input #1

Timer control output

Terminating signal

Firing

Output

Spiking Neuron

Spiking Neuron Terminal (SNT)

signal

reference

Input #1

Input #2

Fig.3. Coupled model of Spiking Neuron Terminal.  Its functionality is same as that in Fig.2.

There are two inputs in the Spiking Neuron – input #1 is the reference input, which enables time reference point to measure the delay between reference pulse and the next one signal pulse coming to the input #2, which is signal input.   Spiking Neuron will fire if and only if the signal impulse to the input #2 is following the reference impulse (to the input #1) within the required time frame (here chosen to be between 5 msec < t < 8 msec.  Signal pulses arriving without prior reference pulse or too early as well as too late after the reference pulse will be discarded and do not cause neuron to fire. The SNT Simulator consists of 2 major components: Spike Tramsformer (amplifies the incoming sequences of pulses to both inputs) and Spiking Neuron, which fires if and only if the condition is met of the chosen time delay between reference and the signal spikes.   Spiking Neuron in turn includes two subcomponents, namely Timer and Controller (Fig.3), which are detecting the required relations in timing of the incoming spikes.  The behavior of Spike Transformer is to adjust amplitude of the incoming spikes to the required level, satisfying sensitivity of the circuits.   As a result the two series of the spikes with the same amplitude (1 unit), but separated by various time intervals.  Information is encoded in the time intervals, which is called time encoding.  For testing purposes in present simulation it is convenient to produce the list of the spiking events, which represent various relations between time intervals of the reference and signal pulses.  Spiking Neuron has to identify those events when signal spike to the input  #2 is coming within the required time slot after reference spike arrival to the input #1, producing 1 at the output, which constitutes firing event for the neuron.   Spikes, which do not meet the requirement, including those arriving too early or too late after reference spike or without respective reference spike present are discarded, producing output 0 (or no firing for the neuron). Identification of qualified spike sequences is done by the neuron via interaction of the internal Timer with the output Controller.  Reference spike, entering the neuron via input #1, goes directly to the Timer, starting time count (in msec units, i.e. scale is 1 ms).   Timer produces output for the Controller, sending messages of the values of current time measure (variable “count”).  In turn, Controller ignores those messages until it receives the signal spike, entering neuron via input #2.   Receiving the signal spike initiates the Controller, which is taking the next coming value from the Timer ( “count”)and compares is with the internal firing condition (here it is 5 msec < count < 8 msec ).   If the condition is sat6isfied, the Neuron fires 1 at the output, is not, it produces 0 (or no firing output).  Controller simultaneously sends the stop signal to the Timer, indicating that the required value of variable count had been received and unless another reference spike comes in, there is no need to continue the counting process.  The Timer and Controller have two phases: passive and active. They are in passive phase initially. Whenever they receive an input (Timer via connection to input #2 of the Neuron and Controller – to input #1 of the Neuron) , they will be in active phase.  Timer will be in active state until the Controller sends the stop signal to it.    Count of the timer needs to be renewed each time the reference spike comes in.   There is controlling output in the Timer, which is helpful in testing procedures to verify the dynamic of the counting process with the sequence of the incoming spikes.

    - Properties of the atomic models

Validation of the properties of the atomic models involved is an essential part of designing the top model, as it ensures operation ability of the last one via verification of its elements. The atomic models and coupled models had been tested using the “black box” testing method.  Each Atomic model had been tested for consistency in single-component top model environment.  Test cases are created by adding different combinations of inputs to the event file (.ev), run the simulation (.scp) and check whether the outputs in the output file (.out) are what is expected.  In order to verify the atomic models and coupled models, the test cases had been created to test the models.  There are three different Atomic Models to test: {Amplifier; Timer; Controller}. There are three Coupled Models to test: {Pulses Transformer; Neuron; Spiking Neuron Terminal}.

    - Properties of the atomic model Amplifier

To test the properties of the amplifier, it is necessary to verify the rule that sequence of the input spikes on the input is transformed into the sequence of the spikes with larger amplitude (according to the amplification coefficient), but preserving time labels of the spikes (as there is no processing time for amplification).  The results of the Amplifier’s model validation are gathered in Table 1, which confirm the validity of the above rule.

[ … Table 1. Table_0_in_Report…]

    - Properties of the atomic model Timer

The external connections of the atomic model Timer are shown in Fig.4.  The Timer is activated by the input of the reference pulse (spike) via input1= m_inTurnOn and terminated by the stop signal (of 1) via input2= m_inTurnOff.

Timer

Input1

(reference)

Timer control  output

count

Terminating signal input

Fig.4. External connections of the atomic model Timer.

The following properties need to be verified in the test:

- that internal counting in the timer starts exactly on the entry of the reference signal (with no time delay on that);

- internal counting terminates at exactly same time as stop signal arrives;

- in the active state intervals between the state change of the  parameter clk ={(+1); (-1)} are exactly equal to the value of cycleTime parameter, i.e. 1 msec.;

- reference spikes arriving in the active state do not interrupt the counting of the clk cycles;

- stop signal arriving at the passive state does not activate the clk counter of the Timer.

The following sequence of the signals had been offered to the atomic model of Timer (in the *.ev file):

00:00:00:000 m_inTurnOn 1.0

00:00:00:010 m_inTurnOff 1.0

00:00:00:020 m_inTurnOff 1.0

00:00:00:025 m_inTurnOn 1.0

00:00:00:035 m_inTurnOn 1.0

00:00:00:040 m_inTurnOff 1.0

The resulting outputs are summarized in Table 2.   It is seen that the internal cycles counting in the Timer starts exactly on the entry of the reference signal (with no time delay on that), because the very key points of Timer activation produce values out_clk=1 and out_count=1 the first output after the occurrence of activation entry.  This is what is expected, because in passive state the respective values of the variables are clk=-1 and count=0.   Therefore above rule (i) is confirmed (see data gathered in Table 2, (i)).  The next rule, (ii) is also confirmed that internal counting terminates at exactly same time as stop signal arrives, as seen from the fact that termination signal “m_inTurnOff 1.0”  prevents output immediately, as there is no output at and after the timing of termination signal, as gathered in Table 2,(ii). The rule (iii) that in the active state intervals between the state change of the parameter clk ={(+1); (-1)} are exactly equal to the value of cycleTime parameter, i.e. 1 msec,  is seen in the data too, as illustrated in Table 2,(iii). The reference spike (00:00:00:035 m_inTurnOn 1.0 ) arriving in the active state is seen as not interrupting the counting of the clk cycles, thus confirming rule (iv), as Table 2, (iv) confirms. And finally, the stop signal (00:00:00:020 m_inTurnOff 1.0 ) arriving at the passive state of the Timer does not activate the clk counter of the Timer, as it is seen in the non-interrupted inactivity of the passive state until the next activating influence by arriving reference spike, as required by the rule (v) and gathered in the Table 2, (v).

Table 1.  Test data verifying operation of the Amplifier

|  | Initiating Events (the cause) | Output result, confirming the rules |
| --- | --- | --- |
|  | 00:00:00:000 in_1 0.1 00:00:00:006 in_1 0.2 00:00:00:007 in_1 0.2 00:00:00:008 in_1 0.2 00:00:00:009 in_1 0.2 00:00:00:010 in_1 0.2 00:00:00:011 in_1 0.2 00:00:00:012 in_1 0.2 00:00:00:014 in_1 0.1 | 00:00:00:000 out_1 1 00:00:00:006 out_1 2 00:00:00:007 out_1 2 00:00:00:008 out_1 2 00:00:00:009 out_1 2 00:00:00:010 out_1 2 00:00:00:011 out_1 2 00:00:00:012 out_1 2 00:00:00:014 out_1 1 |

Table 2.  Test data confirming the rules (i)-(v) for the atomic model of Timer

| rule | Initiating Events (the cause) | Output result, confirming the rule |
| --- | --- | --- |
| (i) | 00:00:00:000 m_inTurnOn 1.0 --------------------------- 00:00:00:025 m_inTurnOn 1.0 | 00:00:00:001 out_clk 1 00:00:00:001 out_count 1 ------------------------ 00:00:00:026 out_clk 1 00:00:00:026 out_count 1 |
| (ii) | 00:00:00:010 m_inTurnOff 1.0 00:00:00:040 m_inTurnOff 1.0 | 00:00:00:009 out_count 9 and no output the next cycle ------------------------- 00:00:00:039 out_count 14 and no output the next cycle |
| (iii) | 00:00:00:025 m_inTurnOn 1.0 | 00:00:00:026 out_clk 1 00:00:00:026 out_count 1 00:00:00:027 out_clk -1 00:00:00:027 out_count 2 00:00:00:028 out_clk 1 00:00:00:028 out_count 3 00:00:00:029 out_clk -1 00:00:00:029 out_count 4 |
| (iv) | 00:00:00:035 m_inTurnOn 1.0 | 00:00:00:034 out_clk 1 00:00:00:034 out_count 9 00:00:00:035 out_clk -1 00:00:00:035 out_count 10 00:00:00:036 out_clk 1 00:00:00:036 out_count 11 |
| (v) | 00:00:00:010 m_inTurnOff 1.0 00:00:00:020 m_inTurnOff 1.0 00:00:00:025 m_inTurnOn 1.0 | 00:00:00:009 out_clk 1 00:00:00:009 out_count 9 00:00:00:026 out_clk 1 00:00:00:026 out_count 1 |

    - Properties of the atomic model Controller

The external connections of the atomic model Controller are shown in Fig.5.

The Controller is activated by the input of the signal spike via input2= m_in and self-terminates after taking the first available count at the reference input (input1= m_inCount) and producing the output events {1;0} at the output1= m_outfire and {1} at the output2= m_outoff.   The following properties need to be verified in the test:

- that the Controller is producing outputs only when spike via input2= m_in is followed by the signal intake via input1= m_inCount (and not otherwise, as states the next criteria vii);

- signals to input1= m_inCount are ignored unless it was preceded by the signal at input2= m_in ;

- firing output of 1 is resulting only when the value the count from the input1= m_inCount satisfies the firing condition (i.e. 5 msec < count < 8 msec );

- when firing condition is not met, the output at output1= m_outfire is 0, even though other conditions of Controller activation are met.

- Firing condition  holds on the borders of the interval (i.e. upper and lower limits of the interval are still valid firing conditions).

Controller

Input2

(signal)

Firing

output

Terminating signal output

count

Fig.5. External connections of the atomic model Controller.

To verify the above rules (vi)-(x), the following input events had been offered to the Atomic model of Controller:

00:00:00:002 m_in 1

00:00:00:003 m_inCount 6

00:00:00:007 m_in 1

00:00:00:008 m_inCount 2

00:00:00:012 m_in 1

00:00:00:012 m_inCount 7

00:00:00:015 m_inCount 7

00:00:00:020 m_in 1

00:00:00:021 m_inCount 9

00:00:00:030 m_in 1

00:00:00:031 m_inCount 5

00:00:00:040 m_in 1

00:00:00:041 m_inCount 8

These input events resulted in the following outputs:

00:00:00:003 m_outoff 1

00:00:00:003 m_outfire 1

00:00:00:008 m_outoff 1

00:00:00:008 m_outfire 0

00:00:00:012 m_outoff 1

00:00:00:012 m_outfire 1

00:00:00:021 m_outoff 1

00:00:00:021 m_outfire 0

00:00:00:031 m_outoff 1

00:00:00:031 m_outfire 1

00:00:00:041 m_outoff 1

00:00:00:041 m_outfire 1

The confirmation that as required by the rule (vi) the Controller is producing outputs only when spike via input2= m_in is followed by the signal intake via input1= m_inCount is seen in the fact that all combinations

00:00:00:00t m_in 1

00:00:00:00(t+1) m_inCount a

do produce the response at the both output ports (even when non-firing signal of 0 occurs at the output1= m_outfire. This is summarized in the Table 3,(vi).  Combination

00:00:00:00t m_in 1

00:00:00:00t m_inCount a

does the same as the previous one, which means that time shift between the two input signals can be anything within interval [0 – cycleTime] (also included in the Table 3,(vi)).  Verification of the Controller’s feature (rule (vii)) that signals to input1= m_inCount are ignored unless it were preceded by the signal at input2= m_in  is seen in the  fact that the singular input of

00:00:00:015 m_inCount 7

unaccompanied by the preceding or simultaneous input to the port m_in is ignored, while others with accompanying pulse are not, (see Table 3,(vii)).  Spiking neuron firing output of 1 is resulting only when the value the count from the input1= m_inCount satisfies the firing condition (i.e. 5 msec < count < 8 msec ), as it is seen in the fact of  the rule (vii) requires and the Table 3,(vii) summarizes.  Outside of the firing timing interval (i.e. 5 msec < count < 8 msec ), for example as in the events below the lower limit, i.e. too early spike (00:00:00:008 m_inCount 2), as well as above it, as too late spike of  (00:00:00:021 m_inCount 9) – both produce non-firing output of 0, verifying the rule (ix) prescription (see Table 3,(ix)).  The borders of the interval are also verified by events (00:00:00:031 m_inCount 5) and (00:00:00:041 m_inCount 8) producing firing output 1, as the rule (x) requires (see Table 3,(x)).

    - Properties of the coupled model of Spiking Neuron

The external connections of the coupled model of Spiking Neuron are shown in Fig.6.

The coupled model of Spiking Neuron consists of two components or sub-models: the Timer and the Controller.  The Coupled model validation requires verification of the consistency in the interaction between the sub-models under various relations in the input combinations.   In the tests of the Atomic models such interaction was substituted by the external events listed in the *.ev files, separate for each sub-model.    Now, the external events will be supplied to the Coupled Model, and the consistency in the internal interactions will be estimated based on the resulting overall output.

Table 3.  	Test data confirming the rules (vi)-(x) for the atomic model of Controller

| rule | Initiating Events (the cause) | Output result, confirming the rule |
| --- | --- | --- |
| (vi) | 00:00:00:002 m_in 1 00:00:00:003 m_inCount 6 00:00:00:007 m_in 1 00:00:00:008 m_inCount 2 ---------------------------00:00:00:012 m_in 1 00:00:00:012 m_inCount 7 | 00:00:00:003 m_outoff 1 00:00:00:003 m_outfire 1 00:00:00:008 m_outoff 1 00:00:00:008 m_outfire 0 ------------------------ 00:00:00:012 m_outoff 1 00:00:00:012 m_outfire 1 |
| (vii) | 00:00:00:015 m_inCount 7 and 00:00:00:020 m_in 1 00:00:00:021 m_inCount 9 | Ignored, no outputs ------------------------- 00:00:00:021 m_outoff 1 00:00:00:021 m_outfire 0 |
| (viii) | 00:00:00:002 m_in 1 00:00:00:003 m_inCount 6 00:00:00:012 m_in 1 00:00:00:012 m_inCount 7 | 00:00:00:003 m_outoff 1 00:00:00:003 m_outfire 1 00:00:00:012 m_outoff 1 00:00:00:012 m_outfire 1 |
| (ix) | 00:00:00:007 m_in 1 00:00:00:008 m_inCount 2 00:00:00:020 m_in 1 00:00:00:021 m_inCount 9 | 00:00:00:008 m_outoff 1 00:00:00:008 m_outfire 0 00:00:00:021 m_outoff 1 00:00:00:021 m_outfire 0 |
| (x) | 00:00:00:030 m_in 1 00:00:00:031 m_inCount 5 00:00:00:040 m_in 1 00:00:00:041 m_inCount 8 | 00:00:00:031 m_outoff 1 00:00:00:031 m_outfire 1 00:00:00:041 m_outoff 1 00:00:00:041 m_outfire 1 |

Controller

Timer

Terminating signal

Firing Output

Spiking Neuron

controlling output

Input

#2

Input

#1

Fig.6. External connections of the coupled model of Spiking Neuron.

Helpful in this regard is the fact, that in addition to the firing output, which serves the functionality of the model, there is one controlling output going directly from the Timer, allowing to follow the state of both sub-models during the Coupled Model operation.  The neuronk.ev was created as shown in left column of Table 4.  which together with sequential neuron_on and neuron_off spikes, includes multiple sequential repetition of either the neuron_off spikes as well as neuron_on spikes to verify that such sequences do not disrupt neither fully passive state of the Spiking Neuron as a whole as well as activated Timer in combination with passive state of the Controller.  Additionally, various intervals are tested between neuron_on and neuron_off spikes to verify firing interval conditions within the interval itself, as well as on its borders and on outside of the interval.  The above external events of the input file had produced the outputs shown in the right column of the Table 4.

The rules to be verified for the coupled model of Spiking Neuron are:

- within the firing interval of the qualified spiking sequences the Spiking Neuron does produce firing output of 1;

- outside of the firing interval of the qualified spiking sequences the Spiking Neuron does produce non-firing output of 0;

- on the borders of the firing interval of the qualified spiking sequences the Spiking Neuron does produce firing output of 1;

- repetitions of signal spike sequences do not disturb passive state of the whole Neuron unless there if reference spike;

- repetitive reference spikes do not disturb combination of active Timer with passive Controller, which represent the waiting state of the Spiking Neuron for the signal spike to come; importantly, only first reference spike counts, and the rest are ignored until the input of the signal spike.

The rule (xi) is verified by sequences of the qualified spikes within the firing interval of the Spiking Neuron to produce firing output of 1, as illustrated in the Table 4,(xi).  Outside of the firing interval even for qualified spiking sequences the Spiking Neuron produces non-firing output of 0, thus confirming rule (xii), as illustrated in Table 4,(xii).  For the upper and lower limits of the firing interval the qualified spiking sequences result in the firing output of 1 of the Spiking Neuron, thus confirming validity of the borders of the firing interval of rule (xiii), as shown in Table 4,(xiii).  Repetitions of signal spikes in a sequences shown not disturb the passive state of the whole Neuron (i.e. those are ignored, producing no clk-outputs) unless the reference spike comes in, as shown in Table 4,(xiv) to verify rule (xiv).  Similarly repetitive reference spikes do not disturb operation of active Timer under passive Controller, in which case the waiting state of the Spiking Neuron is maintained until arrival of the signal spike.  As seen in Table 4,(xv), in this case only first reference spike produces effect (activates the Timer) while others are disregarded until the input of the signal spike, verifying the rule (xv).

    - Properties of the coupled model of Pulses Transformer

The external connections of the coupled model of Pulses Transformer are shown in Fig.7.

The major property to be verified in the test is to show that Pulses Transformer follows the rule:

- the two parallel series of spikes are amplified independently, i.e. with different amplification coefficient, routed to different outputs with preserved time differences.

These features of the coupled model under validation are demonstrated in Table 5.  It is seen that difference in the amplitude between two series is eliminated, connections between in_1 to out_1 and in_2 to out_2 are kept as well as preserved are time labels.   The spike (00:00:00:000 out_2 0) at the output 2 is 0, because there was no actually input for in_2 for the starting point (left column).

Pulses

Transformer

Input #1

signal

reference

Input #2

output #2

output #1

Amp-1

Amp-2

Fig.7. External connections of the coupled model of Pulses Transformer.

    - Properties of the coupled model of Spiking Neuron Terminal (SNT)

The external connections of the coupled model of spiking Neuron Terminal (SNT) are shown in Fig.2 above. At this stage the Coupled Model of SNT Simulator is very similar to that of the Coupled Model of the Spiking Neuron itself, as the Spike Source is represented by the list of the spiking events.   The major additional rule, which need to be verified in the Top-model compare to the Coupled Model of Spiking Neuron is that for:

(xvii)	 rules of the Coupled Model of Spiking Neuron are preserved for the SNT Model, but complimented with the rule (xvi) of the Coupled Model of Pulses Transformer.

This means that producing to the SNT Model similar event list as to the Neuron Model, but with reduced amplitude (to enable Pulses transformer to compensate for the amplitude implementing rule (xvii) ) has to result in the identical output of both models – SNT and Neuron’s.   This is illustrated in Table 6, where first row shows similarity in the offered events lists for the Neuron (on the left) and for the SNT with amplitude being the only difference.  Second row compares the outputs for both models, which are seen to be identical.  This verifies the validity of the coupled model under consideration – Spiking Neuron Terminal Simulator.  It is seen from the Table 6 that the initial differences in the event list of the input spikes produces the logically sound differences in the output of the two models until the time slot of 00:00:00:015 , starting from which the preserved similarity in the input events results in the identical end output for both models.

.

Table 4.  Test data confirming the rules (xi)-(xv) for the Coupled model of the Spiking Neuron, comprising Timer and Controller as components

| rule | Initiating Events (the cause) | Output result, confirming the rule |
| --- | --- | --- |
| (xi) | 00:00:00:000 neuron_on 1 00:00:00:006 neuron_off 1 | 00:00:00:001 clk_control 1 00:00:00:002 clk_control -1 00:00:00:003 clk_control 1 00:00:00:004 clk_control -1 00:00:00:005 clk_control 1 00:00:00:006 clk_control -1 00:00:00:006 neuron_out 1 |
| (xii) | 00:00:00:014 neuron_on 1 00:00:00:016 neuron_off 1 ------------------------- 00:00:00:019 neuron_on 1 00:00:00:029 neuron_off 1 | 00:00:00:015 clk_control 1 00:00:00:016 clk_control -1 00:00:00:016 neuron_out 0 ------------------------- 00:00:00:020 clk_control 1 00:00:00:021 clk_control -1 00:00:00:022 clk_control 1 00:00:00:023 clk_control -1 00:00:00:024 clk_control 1 00:00:00:025 clk_control -1 00:00:00:026 clk_control 1 00:00:00:027 clk_control -1 00:00:00:028 clk_control 1 00:00:00:029 clk_control -1 00:00:00:029 neuron_out 0 |
| (xiii) | 00:00:00:031 neuron_on 1 00:00:00:036 neuron_off 1 -------------------------- 00:00:00:038 neuron_on 1 00:00:00:046 neuron_off 1 | 00:00:00:032 clk_control 1 00:00:00:033 clk_control -1 00:00:00:034 clk_control 1 00:00:00:035 clk_control -1 00:00:00:036 clk_control 1 00:00:00:036 neuron_out 1 -------------------------- 00:00:00:039 clk_control 1 00:00:00:040 clk_control -1 00:00:00:041 clk_control 1 00:00:00:042 clk_control -1 00:00:00:043 clk_control 1 00:00:00:044 clk_control -1 00:00:00:045 clk_control 1 00:00:00:046 clk_control -1 00:00:00:046 neuron_out 1 |
| (xiv) | 00:00:00:006 neuron_off 1 00:00:00:007 neuron_off 1 00:00:00:008 neuron_off 1 00:00:00:009 neuron_off 1 00:00:00:010 neuron_off 1 00:00:00:011 neuron_off 1 00:00:00:012 neuron_off 1 00:00:00:014 neuron_on 1 | 00:00:00:006 clk_control -1 00:00:00:006 neuron_out 1 00:00:00:015 clk_control 1 |
| (xv) | 00:00:00:048 neuron_on 1 00:00:00:049 neuron_on 1 00:00:00:050 neuron_on 1 00:00:00:051 neuron_on 1 00:00:00:052 neuron_on 1 00:00:00:053 neuron_on 1 00:00:00:054 neuron_off 1 | 00:00:00:049 clk_control 1 00:00:00:050 clk_control -1 00:00:00:051 clk_control 1 00:00:00:052 clk_control -1 00:00:00:053 clk_control 1 00:00:00:054 clk_control -1 00:00:00:054 neuron_out 1 |

.

Table 5.  Test data confirming operation of Pulses Transformer (rule xvi)

| rule | Initiating Events (the cause) | Output result, confirming the rule |
| --- | --- | --- |
| (xvi) | 00:00:00:000 in_1 0.1 00:00:00:006 in_2 0.2 00:00:00:007 in_2 0.2 00:00:00:008 in_2 0.2 00:00:00:009 in_2 0.2 00:00:00:010 in_2 0.2 00:00:00:011 in_2 0.2 00:00:00:012 in_2 0.2 00:00:00:014 in_1 0.1 | 00:00:00:000 out_1 1 00:00:00:000 out_2 0 00:00:00:006 out_2 1 00:00:00:007 out_2 1 00:00:00:008 out_2 1 00:00:00:009 out_2 1 00:00:00:010 out_2 1 00:00:00:011 out_2 1 00:00:00:012 out_2 1 00:00:00:014 out_1 1 |

.

.

.

Table 6.  Comparative test of SNT Simulator and

Coupled model of the Spiking Neuron (rule xvii).

| rule (xvii) | Data for Coupled Model of Spiking Neuron | Data for the SNT Simulator |
| --- | --- | --- |
| inputs | 00:00:00:000 neuron_on 1 00:00:00:006 neuron_off 1 00:00:00:007 neuron_off 1 00:00:00:008 neuron_off 1 00:00:00:009 neuron_off 1 00:00:00:010 neuron_off 1 00:00:00:011 neuron_off 1 00:00:00:012 neuron_off 1 00:00:00:014 neuron_on 1 00:00:00:016 neuron_off 1 00:00:00:017 neuron_off 1 00:00:00:018 neuron_off 1 00:00:00:019 neuron_on 1 00:00:00:029 neuron_off 1 00:00:00:031 neuron_on 1 00:00:00:036 neuron_off 1 00:00:00:038 neuron_on 1 00:00:00:046 neuron_off 1 00:00:00:048 neuron_on 1 00:00:00:049 neuron_on 1 00:00:00:050 neuron_on 1 00:00:00:051 neuron_on 1 00:00:00:052 neuron_on 1 00:00:00:053 neuron_on 1 00:00:00:054 neuron_off 1 | 00:00:00:000 in_2 0.2 00:00:00:000 in_1 0.1 00:00:00:001 in_1 0.1 00:00:00:006 in_2 0.2 00:00:00:007 in_2 0.2 00:00:00:008 in_2 0.2 00:00:00:009 in_2 0.2 00:00:00:010 in_2 0.2 00:00:00:011 in_2 0.2 00:00:00:012 in_2 0.2 00:00:00:014 in_1 0.1 00:00:00:016 in_2 0.2 00:00:00:017 in_2 0.2 00:00:00:018 in_2 0.2 00:00:00:019 in_1 0.1 00:00:00:029 in_2 0.2 00:00:00:031 in_1 0.1 00:00:00:036 in_2 0.2 00:00:00:038 in_1 0.1 00:00:00:046 in_2 0.2 00:00:00:048 in_1 0.1 00:00:00:049 in_1 0.1 00:00:00:050 in_1 0.1 00:00:00:051 in_1 0.1 00:00:00:052 in_1 0.1 00:00:00:053 in_1 0.1 00:00:00:054 in_2 0.2 |
| outputs | 00:00:00:001 clk_control 1 00:00:00:002 clk_control -1 00:00:00:003 clk_control 1 00:00:00:004 clk_control -1 00:00:00:005 clk_control 1 00:00:00:006 clk_control -1 00:00:00:006 neuron_out 1 00:00:00:015 clk_control 1 00:00:00:016 clk_control -1 00:00:00:016 neuron_out 0 00:00:00:020 clk_control 1 00:00:00:021 clk_control -1 00:00:00:022 clk_control 1 00:00:00:023 clk_control -1 00:00:00:024 clk_control 1 00:00:00:025 clk_control -1 00:00:00:026 clk_control 1 00:00:00:027 clk_control -1 00:00:00:028 clk_control 1 00:00:00:029 clk_control -1 00:00:00:029 neuron_out 0 00:00:00:032 clk_control 1 00:00:00:033 clk_control -1 00:00:00:034 clk_control 1 00:00:00:035 clk_control -1 00:00:00:036 clk_control 1 00:00:00:036 neuron_out 1 00:00:00:039 clk_control 1 00:00:00:040 clk_control -1 00:00:00:041 clk_control 1 00:00:00:042 clk_control -1 00:00:00:043 clk_control 1 00:00:00:044 clk_control -1 00:00:00:045 clk_control 1 00:00:00:046 clk_control -1 00:00:00:046 neuron_out 1 00:00:00:049 clk_control 1 00:00:00:050 clk_control -1 00:00:00:051 clk_control 1 00:00:00:052 clk_control -1 00:00:00:053 clk_control 1 00:00:00:054 clk_control -1 00:00:00:054 neuron_out 1 | 00:00:00:001 control_output 1 00:00:00:001 terminal_output 0 00:00:00:013 terminal_output 0 00:00:00:015 control_output 1 00:00:00:016 control_output -1 00:00:00:016 terminal_output 0 00:00:00:020 control_output 1 00:00:00:021 control_output -1 00:00:00:022 control_output 1 00:00:00:023 control_output -1 00:00:00:024 control_output 1 00:00:00:025 control_output -1 00:00:00:026 control_output 1 00:00:00:027 control_output -1 00:00:00:028 control_output 1 00:00:00:029 control_output -1 00:00:00:029 terminal_output 0 00:00:00:032 control_output 1 00:00:00:033 control_output -1 00:00:00:034 control_output 1 00:00:00:035 control_output -1 00:00:00:036 control_output 1 00:00:00:036 terminal_output 1 00:00:00:039 control_output 1 00:00:00:040 control_output -1 00:00:00:041 control_output 1 00:00:00:042 control_output -1 00:00:00:043 control_output 1 00:00:00:044 control_output -1 00:00:00:045 control_output 1 00:00:00:046 control_output -1 00:00:00:046 terminal_output 1 00:00:00:049 control_output 1 00:00:00:050 control_output -1 00:00:00:051 control_output 1 00:00:00:052 control_output -1 00:00:00:053 control_output 1 00:00:00:054 control_output -1 00:00:00:054 terminal_output 1 |

.

.

.

2.3	Spiking Neural Decoder as top model

Relying on the atomic and coupled models of Section 2.2 the top model of Spiking Neural Decoder can be designed as shown in Fig.8.

.

.

Controller-1

Timer-1

Controller-2

Timer-2

Controller-3

Timer-3

Spiking Neuron A

Spiking Neuron B

Spiking Neuron C

Pulses

Trans-former

signal

reference

Input #1

Input #2

Spiking Neural Decoder

clk-1

A

B

C

clk-3

clk-2

Out

Fig.8. Schematic of the Simulator of Spiking Neural Decoder.

.

Table 7.  Comparative test of Spiking Neural Decoder and

Spiking Neuron Terminal Simulators (rules xviii-xx).

|  | Output data for the Spiking Neuron Terminal Simulator | Output data for the Spiking Neural Decoder Simulator |
| --- | --- | --- |
| * * * ** ** ** | 00:00:00:001 control_output 1 00:00:00:001 terminal_output 0 00:00:00:013 terminal_output 0 00:00:00:015 control_output 1 00:00:00:016 control_output -1 00:00:00:016 terminal_output 0 00:00:00:020 control_output 1 00:00:00:021 control_output -1 00:00:00:022 control_output 1 00:00:00:023 control_output -1 00:00:00:024 control_output 1 00:00:00:025 control_output -1 00:00:00:026 control_output 1 00:00:00:027 control_output -1 00:00:00:028 control_output 1 00:00:00:029 control_output -1 00:00:00:029 terminal_output 0 00:00:00:032 control_output 1 00:00:00:033 control_output -1 00:00:00:034 control_output 1 00:00:00:035 control_output -1 00:00:00:036 control_output 1 00:00:00:036 terminal_output 1 00:00:00:039 control_output 1 00:00:00:040 control_output -1 00:00:00:041 control_output 1 00:00:00:042 control_output -1 00:00:00:043 control_output 1 00:00:00:044 control_output -1 00:00:00:045 control_output 1 00:00:00:046 control_output -1 00:00:00:046 terminal_output 1 00:00:00:049 control_output 1 00:00:00:050 control_output -1 00:00:00:051 control_output 1 00:00:00:052 control_output -1 00:00:00:053 control_output 1 00:00:00:054 control_output -1 00:00:00:054 terminal_output 1 | 00:00:00:001 control_output 1 00:00:00:001 decoder_output 1 00:00:00:001 control1_output 1 00:00:00:001 decoder_output 0 00:00:00:001 control2_output 1 00:00:00:001 decoder_output 0 00:00:00:013 decoder_output 0 00:00:00:013 decoder_output 0 00:00:00:013 decoder_output 0 00:00:00:015 control_output 1 00:00:00:015 control1_output 1 00:00:00:015 control2_output 1 00:00:00:016 control_output -1 00:00:00:016 decoder_output 1 00:00:00:016 control1_output -1 00:00:00:016 decoder_output 0 00:00:00:016 control2_output -1 00:00:00:016 decoder_output 0 00:00:00:020 control_output 1 00:00:00:020 control1_output 1 00:00:00:020 control2_output 1 00:00:00:021 control_output -1 00:00:00:021 control1_output -1 00:00:00:021 control2_output -1 00:00:00:022 control_output 1 00:00:00:022 control1_output 1 00:00:00:022 control2_output 1 00:00:00:023 control_output -1 00:00:00:023 control1_output -1 00:00:00:023 control2_output -1 00:00:00:024 control_output 1 00:00:00:024 control1_output 1 00:00:00:024 control2_output 1 00:00:00:025 control_output -1 00:00:00:025 control1_output -1 00:00:00:025 control2_output -1 00:00:00:026 control_output 1 00:00:00:026 control1_output 1 00:00:00:026 control2_output 1 00:00:00:027 control_output -1 00:00:00:027 control1_output -1 00:00:00:027 control2_output -1 00:00:00:028 control_output 1 00:00:00:028 control1_output 1 00:00:00:028 control2_output 1 00:00:00:029 control_output -1 00:00:00:029 decoder_output 0 00:00:00:029 control1_output -1 00:00:00:029 decoder_output 0 00:00:00:029 control2_output -1 00:00:00:029 decoder_output 1 00:00:00:032 control_output 1 00:00:00:032 control1_output 1 00:00:00:032 control2_output 1 00:00:00:033 control_output -1 00:00:00:033 control1_output -1 00:00:00:033 control2_output -1 00:00:00:034 control_output 1 00:00:00:034 control1_output 1 00:00:00:034 control2_output 1 00:00:00:035 control_output -1 00:00:00:035 control1_output -1 00:00:00:035 control2_output -1 00:00:00:036 control_output 1 00:00:00:036 decoder_output 0 00:00:00:036 control1_output 1 00:00:00:036 decoder_output 1 00:00:00:036 control2_output 1 00:00:00:036 decoder_output 0 00:00:00:039 control_output 1 00:00:00:039 control1_output 1 00:00:00:039 control2_output 1 00:00:00:040 control_output -1 00:00:00:040 control1_output -1 00:00:00:040 control2_output -1 00:00:00:041 control_output 1 00:00:00:041 control1_output 1 00:00:00:041 control2_output 1 00:00:00:042 control_output -1 00:00:00:042 control1_output -1 00:00:00:042 control2_output -1 00:00:00:043 control_output 1 00:00:00:043 control1_output 1 00:00:00:043 control2_output 1 00:00:00:044 control_output -1 00:00:00:044 control1_output -1 00:00:00:044 control2_output -1 00:00:00:045 control_output 1 00:00:00:045 control1_output 1 00:00:00:045 control2_output 1 00:00:00:046 control_output -1 00:00:00:046 decoder_output 0 00:00:00:046 control1_output -1 00:00:00:046 decoder_output 1 00:00:00:046 control2_output -1 00:00:00:046 decoder_output 0 00:00:00:049 control_output 1 00:00:00:049 control1_output 1 00:00:00:049 control2_output 1 00:00:00:050 control_output -1 00:00:00:050 control1_output -1 00:00:00:050 control2_output -1 00:00:00:051 control_output 1 00:00:00:051 control1_output 1 00:00:00:051 control2_output 1 00:00:00:052 control_output -1 00:00:00:052 control1_output -1 00:00:00:052 control2_output -1 00:00:00:053 control_output 1 00:00:00:053 control1_output 1 00:00:00:053 control2_output 1 00:00:00:054 control_output -1 00:00:00:054 decoder_output 0 00:00:00:054 control1_output -1 00:00:00:054 decoder_output 1 00:00:00:054 control2_output -1 00:00:00:054 decoder_output 0 |

.

.

.

Properties of the model of Spiking Neural Decoder differ from that of Spiking Neural Terminal of Section 2.2.7 is that in the Decoder the whole range of the time intervals is covered with three Neurons A, B and C connected in parallel (see Fig.2 and Fig 8) and therefore given legitimate symbol at the input the model has to provide response "1" at the output of the corresponding neuron (i.e. neuron A if symbol is "A", neuron B if it is "B", and neuron C if it is "C") and output of "0" at the rest of neurons.  Therefore the properties of the decoder's model can be formulated as a following set of rules, confirmation for which can be seen in the Table 7:

(xviii) for legitimate symbol inputs ("A", "B" and "C") there is always "1" at one firing output  and "0" at the rest of firing outputs (where firing outputs are those comprising A, B and C outputs in Fig.8 and Fig.2);

(xix) for each control output in SNT Simulator (variable control_output  in Table 7) there are three consequtive control outputs of the same value ("1" or "-1") in the Decoder Simulator (outputs clk-1, clk-2 and clk-3 in Fig.8 and variables control_output, control1_output and control2_output in Table 7));

(xx) for each firing output in SNT Simulator (variable terminal_output in Table 7) there are three consequtive firing outputs in the Decoder Simulator (variable decoder_output in Table 7);

(xxi) for each "0" firing output in SNT Simulator (see marks "*" and "**" for terminal_output in Table 7) there are three sequential outputs of either 1, 0, 0 or 0, 0, 1 (see those marked by respectively "*" for 100 and by "**" for 001 in Table 7) at the Decoder Simulator firing output (output "Out" in Fig.8);

(xxii) for each "1" firing output in SNT Simulator (see marks "***" for terminal_output in Table 7) there are three sequential outputs of 0, 1, 0 at the Decoder Simulator firing output (output "Out" in Fig.8 and variable decoder_output in Table 7).

It is seen from the Table 7, that the rules (xviii)-(xxii) are followed precisely.  For instance, the rule (xviii) is always followed for all positions in Table 7 marked with either "*", "**" or "***".  The rule (xix) is followed too as there are 3 control outputs in the Decoders column for each control output in the SNT column.  Similarly, the rule (xx) is confirmed by the data of Table 7, as there are 3 firing outputs in the Decoders column for each firing output in the SNT column. The rule (xxi) is verified by entries marked by "*" and "**", while the rule (xxii) by entries with "***" marks in Table 7.

This concludes design, properties verification and validation of the top model of Spiking Neural Decoder Simulator, programmed in CD++ tool employing DEVS formalism approach.  The model is validated at the level of its atomic components, coupled model components and top integrated hierarchical level.

.

3   Cell-DEVS Problem Formulation

and Implementation

3.1	Quantum Automata for modeling of quantum dot devices

In the present study Cell-DEVS formalism is employed to model operation of basic quantum dot devices, which are described by quantum cellular automata.  For Cell-DEVS modeling the following quantum dot based devices have been selected: quantum XOR, quantum wire and majority vote gates.  Demonstrated herewith is operation of models of these devices in Cell-DEVS environment employing CD++ tool kit.   Therefore, the real system of choice here is Cellular Automata implementation of Quantum Logic Circuits, which are used in the current state-of-the-art Quantum Computing and Artificial Brain Machines.  Quantum Cellular Automata (QCA) refers to a model of quantum computation, which has been devised in analogy to conventional models of cellular automata introduced by von Neumann. Important for a model of quantum cellular automata is that it should be universal for quantum computation [2,19-21] (i.e. that it can efficiently simulate quantum Turing machines, or equivalently, uniform families of quantum circuits).  Additional restrictions are that quantum cellular automata should be reversible and/or local unitary, and have an easily determined global transition function from the rule for updating individual cells [2,21].   Examples of the quantum devices can be found in [2,19-37].

  - Overview of Structures for Cell-DEVS modeling

The subject of this project is implementation of the Cellular Automata for logic device based quantum dot cells.  Each quantum cell contains two electrons which interact with the neighbors Coulombically.  Each cell is designed having two perpendicular axes of preferential alignment of the pair of electrons in it.  Such states of preferential alignment are used to encode binary bits of 0 and 1, as shown in Fig.9.  Employing such cells, various quantum devices can be built for the digital circuits to operate at nanometer scale, which hold a promise of faster speed and reduced size.   Basic devices built employing quantum dot based cells include quantum wire, shown in Fig.10.

0

1

Fig.9. States of preferential alignment in quantum dot cells.

Fig.10. Quantum wire in transmitting bit 1 mode.

Input is equal to the output for the quantum wire, which introduces delay defined by the number of intermediate cells between input and output ports.

Quantum dot XOR gate shown in Fig.11, where two input signals produce output in the middle location, where M=(a+b)%2.  Unlike in quantum wire, where signal is transferred only in one direction, in XOR gate input signals from two inputs a and b are coming towards each other and meet at middle cell to complete the operation.

a

b

M=(a+b)%2

Fig.11. Quantum dot XOR gate: a, b are two inputs; output is at c=(a+b)%2.

Another basic device, namely – 3 inputs Majority Vote Gates, is shown schematically in Fig.12.

a

b

c

Output

ab+bc+ac

Fig.12. Quantum dot 3 inputs Majority Vote Gates with no delay: a, b, c are inputs; output function is (ab+bc+ac)%2.

It takes inputs to three of the available ports a, b and c simultaneously and outputs the function M=ab+bc+ac (modulo 2).

Combining both above mentioned devices of Fig.10 and Fig.12 in one the delayed version of the 3 input Majority Vote Gates can be obtained, as shown in Fig.13.  The additional feature of this device compare to the one in Fig.12 is the introduced delay which is defined by the number of the intermediate cells introduced between each input and the processing cell.   Output function in both cases is the same M=ab+bc+ac (modulo 2).  The mapping function M of the devices of Fig.12 and Fig.13 is shown in Table 1. Essentially, the output of this device is equal to the most frequent of the inputs.   For that reason the number of inputs has to be an odd number.

a

b

c

Delayed Output

ab+bc+ac

Delay line

Delay line

Delay line

Fig.13. Quantum dot 3 inputs Majority Vote Gates with delayed response: a, b, c are inputs; output function is (ab+bc+ac)%2.

Table 8.  Mapping function of Majority Vote Gates

| a | b | c | M=ab+bc+ac |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 0 |

  - Structures of Quantum Cellular Automata in Cell-DEVS

The cellular automata for XOR quantum gates (of Fig.11) is shown in Fig.14.

out@XOR(0,1)

In_2@XOR(0,2)

In_1@XOR(0,0)

(0,0)

(0,1)

(0,2)

Fig.14. Quantum Cellular Automata for XOR gates.

The cellular automata for the 3 inputs delayed Majority Vote Gates (of Fig.13) is shown schematically in Fig.15.   In the cellular field there are several zones of inactive cells, which are not involved in the operation of the device, but rather are affecting indexes of the operational cells.  Among operational cells there are input cells, transport cells, calculation cell and output cell.

0,0

0,1

0,2

0,3

0,4

1,0

1,1

1,2

1,3

1,4

2,0

2,1

2,2

2,3

2,4

3,0

3,1

3,2

3,3

3,4

4,0

4,1

4,2

4,3

4,4

5,0

5,1

5,2

5,3

5,4

6,0

6,1

6,2

6,3

6,4

a

b

c

ab+bc+ac

Input #1

Input #2

Input #3

Output

In_1@3inMVG(0,3)

In_2@

3inMVG(3,0)

In_3@3inMVG(6,3)

out@3inMVG(3,4)

Fig.15. Quantum Cellular Automata for 3 inputs Delayed Majority Vote Gate device.

  - Properties of Quantum Cellular Automata devices (Cell-DEVS)

The properties of the models of XOR and delayed Majority Vote Gates had been verified using the “black box” testing method.  Initially both models were tested by setting all inputs to the value of 1 by the following lines in the program:

- (for XOR gate)

initialvalue : 0

initialrowvalue :  0   101

- (and for 3 inputs Majority Vote Gates)

initialvalue : 0

initialrowvalue :  0   00010

initialrowvalue :  3   10000

initialrowvalue :  6   00010

Under such conditions the output of 1 is expected in both models.  Obtaining 1 at the output implies that that model is likely to be functional with other external inputs.  After that the external input had been introduced via external events file *.ev.  For XOR gates the rule was that output “-1” is if both inputs are equal (i.e. both to “1” or to “-1”) and output “1” otherwise and calculation as well as clearing time constants set both at 100 msec:

[calculus-rule]

rule : -1 100 { (0,-1)=-1 and (0,1)=-1 }

rule : 1 100 { (0,-1)=1 and (0,1)=-1 }

rule : 1 100 { (0,-1)=-1 and (0,1)=1 }

rule : -1 100 { (0,-1)=1 and (0,1)=1 }

rule : 0 100 { t }

Calculation rules for 3 inputs Majority Vote gates  were set according to the Table 8, but contained “-1” instead of “0” , which is needed to avoid confusion between neutral state of the cell and signal “0” (as also was the case for XOR gates):

[calculus-rule]

rule : 1 10 { (-1,0)=1 and (0,-1)=1 and (1,0)=1 }

rule : 1 10 { (-1,0)=1 and (0,-1)=1 and (1,0)=-1 }

rule : 1 10 { (-1,0)=1 and (0,-1)=-1 and (1,0)=1 }

rule : 1 10 { (-1,0)=-1 and (0,-1)=1 and (1,0)=1 }

rule : -1 10 { (-1,0)=1 and (0,-1)=-1 and (1,0)=-1 }

rule : -1 10 { (-1,0)=-1 and (0,-1)=1 and (1,0)=-1 }

rule : -1 10 { (-1,0)=-1 and (0,-1)=-1 and (1,0)=1 }

rule : -1 10 { (-1,0)=-1 and (0,-1)=-1 and (1,0)=-1 }

rule : 0 10 { t }

where calculation and clearing time constants are both set at 10 msec value.

Time: 00:00:00:000

0      1      2

+---------------------+

0|   -1.0          -1.0|

+---------------------+

Time: 00:00:00:100

0      1      2

+---------------------+

0|          -1.0       |

+---------------------+

Time: 00:00:00:200

0      1      2

+---------------------+

0|   -1.0           1.0|

+---------------------+

Time: 00:00:00:300

0      1      2

+---------------------+

0|           1.0       |

+---------------------+

Time: 00:00:00:400

0      1      2

+---------------------+

0|    1.0          -1.0|

+---------------------+

Time: 00:00:00:500

0      1      2

+---------------------+

0|           1.0       |

+---------------------+

Time: 00:00:00:600

0      1      2

+---------------------+

0|    1.0           1.0|

+---------------------+

Time: 00:00:00:700

0      1      2

+---------------------+

0|          -1.0       |

+---------------------+

Fig.16. Operation of quantum automata XOR gates in drawlog tool of CD++.

For both models the test cases were introduced  by adding different combinations of inputs to the event file (.ev), run the simulation (.scp) and check whether the outputs in the output file (.out) are what was expected (as indicated by Table 8 for Majority Vote Gates and XOR-rule for XOR gates).

The properties of the XOR gate model can be formulated as a following set of rules, which are confirmed by the data presented in Table 9:

(xxiii) signal inputs and signal outputs are connected with XOR relations;

(xxiv) delay for signal output is equal to the programmed processing time;

(xxv) each signal output is followed by clearing “0” output after additional delay of programmed clearing time.

Rule (xxiii) is confirmed in Table 9 by correspondence with the XOR relations.  The rule (xxiv) is also followed as the delay for all output signals are equal to the programmed value of 100 msec.  And the last rule (xxv) is confirmed by the fact, that each signal is followed by the clearing output of “0” after programmed value of clearing time (also 100 msec).

Drawlog tool in CD++ allows for graphic view of signal propagation, shown in Fig.16.  Similarly, animation tool of CD++ Modeler allow visualization of the automata XOR gates operation, as it is shown in Fig.17.

( 1 )

( 2 )

( 3 )

( 4 )

Fig.17. Visualization of the operation of quantum automata XOR gates in CD++ Modeler animation tool.  Lines (1)-(4) represent various input-output combinations.

The above consideration conclusively confirms functionality and validity of the quantum automata of XOR gates.

Table 9. Test results for comparing inputs and output mapping to the expected values of the XOR gates.

| # | Initiating Events (the cause) | Output result, confirming the rules |
| --- | --- | --- |
| 1 | 00:00:00:000 in1 -1 00:00:00:000 in2 -1 | 00:00:00:100 out -1 00:00:00:200 out 0 |
| 2 | 00:00:00:200 in1 -1 00:00:00:200 in2 1 | 00:00:00:300 out 1 00:00:00:400 out 0 |
| 3 | 00:00:00:400 in1 1 00:00:00:400 in2 -1 | 00:00:00:500 out 1 00:00:00:600 out 0 |
| 4 | 00:00:00:600 in1 1 00:00:00:600 in2 1 | 00:00:00:700 out -1 00:00:00:800 out 0 |

Similar consideration has been given to the properties of the quantum cellular automata of 3 inputs Majority Vote Gates shown in Fig.15.

The properties of the Delayed Majority Vote Gates model can be formulated as a following set of rules, confirmation of which is sought in Table 10:

(xxvi) signal inputs and signal outputs are connected with Majority Vote relations;

(xxvii) delay for signal output is equal to the number of steps multiplied by the programmed processing time at each step;

(xxviii) each signal output is followed by clearing “0” output after additional delay of programmed output clearing time.

Cases (1)-(4) in Table 10 provide majority of “1” at the input, so expected output value is “1” and the obtained value is also “1”, as it is seen in the Table 10 in accord with the rule (xxvi).  It is also seen that the output is delayed by 40 msec, which is due to total of four cells distance between the input and output cells, each cell having the delay time of 10 msec, thus confirming rule (xxvii).  There is also expected “0” signal 1 msec after output of “1” – this is due to reset of the output cell value to 0, which is programmed to occur with 1 msec delay as the rule (xxviii) requires.   Respectively, cases (5) - (8) provide majority of “-1” at the input, so expected output value is “-1” and the obtained value is also “-1”, as it is also confirmed in the Table 10 in accord with the rule (xxvi).  Again, the output is delayed by 40 msec, which is due to total of four cells distance between the input and output cells, each cell having the delay time of 10 msec (rule (xxvii)).  There is also expected “0” signal 1 msec after output of “-1”  as was the case above for the same reason of resetting the output cell value to 0 with 1 msec delay (rule (xxviii)).

Animation of the signal propagation in the Delayed Majority Vote Gates model is shown in Fig.18 produced by CD++ Modeler.  Similarly, Fig.19 shows comparison of visualization with drawlog tool (top line) and animation tool (bottom line) in CD++ toolkit.

Table 10. Test results for comparing inputs and output mapping to the expected values of the of the Delayed Majority Vote Gates.

| # | Initiating Events (the cause) | Output result, confirming the rules |
| --- | --- | --- |
| 1 | 00:00:00:000 in1 1 00:00:00:000 in2 1 00:00:00:000 in3 1 | 00:00:00:040 out 1 00:00:00:041 out 0 |
| 2 | 00:00:00:100 in1 1 00:00:00:100 in2 1 00:00:00:100 in3 -1 | 00:00:00:140 out 1 00:00:00:141 out 0 |
| 3 | 00:00:00:200 in1 1 00:00:00:200 in2 -1 00:00:00:200 in3 1 | 00:00:00:240 out 1 00:00:00:241 out 0 |
| 4 | 00:00:00:300 in1 -1 00:00:00:300 in2 1 00:00:00:300 in3 1 | 00:00:00:340 out 1 00:00:00:341 out 0 |
| 5 | 00:00:00:400 in1 1 00:00:00:400 in2 -1 00:00:00:400 in3 -1 | 00:00:00:440 out -1 00:00:00:441 out 0 |
| 6 | 00:00:00:500 in1 -1 00:00:00:500 in2 1 00:00:00:500 in3 -1 | 00:00:00:540 out -1 00:00:00:541 out 0 |
| 7 | 00:00:00:600 in1 -1 00:00:00:600 in2 -1 00:00:00:600 in3 1 | 00:00:00:640 out -1 00:00:00:641 out 0 |
| 8 | 00:00:00:700 in1 -1 00:00:00:700 in2 -1 00:00:00:700 in3 -1 | 00:00:00:740 out -1 00:00:00:741 out 0 |

1

2

3

4

6

5

7

8

Fig.18. Visualization of the operation of quantum automata Delayed Majority Vote Gates in CD++ Modeler animation tool.  Lines (1)-(8) represent the same input-output relations as those in Table 10.

.

Line : 1676 - Time: 00:00:00:500

0         1         2         3         4

+--------------------------------------------------+

0|                                      -1.0                       |

1|                                                                   |

2|                                                                   |

3|      1.0                                                        |

4|                                                                   |

5|                                                                   |

6|                                      -1.0                       |

+--------------------------------------------------+

Line : 1756 - Time: 00:00:00:510

0         1         2         3         4

+--------------------------------------------------+

0|                                                                    |

1|                                      -1.0                       |

2|                                                                    |

3|                 1.0                                              |

4|                                                                    |

5|                                      -1.0                       |

6|                                                                    |

+--------------------------------------------------+

Line : 1834 - Time: 00:00:00:520

0         1         2         3         4

+--------------------------------------------------+

0|                                                                    |

1|                                                                    |

2|                                      -1.0                       |

3|                             1.0                                  |

4|                                      -1.0                       |

5|                                                                    |

6|                                                                    |

+--------------------------------------------------+

Line : 1885 - Time: 00:00:00:530

0         1         2         3         4

+--------------------------------------------------+

0|                                                                    |

1|                                                                    |

2|                                                                    |

3|                                       -1.0                       |

4|                                                                    |

5|                                                                    |

6|                                                                    |

+--------------------------------------------------+

Line : 1910 - Time: 00:00:00:540

0         1         2         3         4

+--------------------------------------------------+

0|                                                                    |

1|                                                                    |

2|                                                                    |

3|                                                 -1.0             |

4|                                                                    |

5|                                                                    |

6|                                                                    |

+--------------------------------------------------+

6

Fig.19. Comparison of visualization of signal propagation with drawlog tool of CD++ (top line) and animation tool (bottom line) in CD++ Modeler toolkit. (Input condition of case 6 of Table 10).

.

The above verifications conclude validation of the model of the Delayed Majority Vote Gates in quantum cellular automata.

.

4   Conclusion

The following conclusions can be drawn form the above considerations.

- CD++ toolkit is demonstrated as a suitable environment for simulation of the elements of Brain Machine under DEVS and Cell-DEVS formalism.

- The model of the neural decoder based on spiking neurons with rectangular response function is successfully implemented and validated in DEVS formal definitions.

- The models of quantum dot cellular devices, such as XOR gates and the Delayed Majority Vote Gates, have been implemented in Cell-DEVS specifications of CD++ Modeler.  Models are successfully validated.

References:

[1] Zeigler, B.P., The brain-machine disanalogy revisited, BioSystems, Vol. 64, pp. 127-140. (2002).

[2] Buller A (2003a) CAM-Brain Machine and Pulsed Para-Neural Networks (PPNN): Toward a hardware forfuture robotic on-board brains, Proceedings of the Eighth International Symposium on Artificial Life and Robotics (AROB 8th '03), January 24-26, 2003, Beppu, Oita, Japan, 490-493.

[3] Eeckhaut H, Van Campenhout J., (2003) “Handcrafting Pulsed Neural Networks for the CAM-Brain Machine”,- Proceedings of the Eighth International Symposium on Artificial Life and Robotics (AROB 8th '02),January 24-26, 2002, Beppu, Oita, Japan, 494-501.

http://trappist.elis.ugent.be/~heeckhau/CBM/Publications/arob2003heeckhau.pdf

[4] M. Conrad, "The brain-machine disanalogy"- Biosystems, 1989 - vol.22, no.3, pp197-213, Elsevier, 1989. cat.inist.fr

[5]  Michael Korkin1, Norberto Eiji Nawa, Hugo de Garis, A "Spike Interval Information Coding" Representation for ATR's CAM-Brain Machine (CBM) Volume 1478 (1998).

[6]  Obeid, I.   Wolf, P.D, "Evaluation of spike-detection algorithms fora brain-machine interface application",-    Biomedical Engineering, IEEE Transactions on, Volume 51,  Issue 6, page(s) 905- 911, June 2004.

[7] Zeigler, B.P., DEVS Component-Based M&S Framework: An Introduction , Topics Seminar: Object Oriented Simulation-Discreet Event Modeling Spring 2007.

http://www.cs.gsu.edu/~cscxlh/CSC8910/ACIMS%20DEVSTut%20AIS2002%20Final.doc

[8] G.Wainer, N. Giambiasi. "Timed Cell-DEVS: modelling and simulation of cell spaces ". Invited paper for the book: Discrete Event Modeling &  Simulation: Enabling Future Technologies. Springer- Verlag, 2001.

[9] Q. Liu, G. Wainer. “Optimistic Simulation of DEVS and Cell-DEVS Models with PCD++”. Accepted for publication in Simulation, Transactions of the SCS. Accepted: May 2007.

[10] G. Wainer. “Applying Cell-DEVS Methodology for Modeling the Environment”. In Simulation, Transactions of the SCS. Vol. 82, No. 10, 635-660.  October 2006.

[11] R Mayrhofer, M Affenzeller, H Prahofer, G Hofer, A., “DEVS Simulation of Spiking Neural Networks”, - Proceedings of Cybernetics and Systems (EMCSR), 2002.

http://www.heuristiclab.com/publications/papers/mayrhofer02a.pdf

[12] Thomas Natschläger, Wolfgang Maass, “Fast Analog Computation in Networks of Spiking Neurons.”-  (1999)   http://www.cis.tu-graz.ac.at/igi/maass/src-esann99.ps.gz

[13] Wolfgang Maass, “Networks of Spiking Neurons: The Third Generation of Neural Network Models”, - (1996) http://www.cis.tu-graz.ac.at/igi/maass/85j.ps.gz

[14] Wolfgang Maass, “Networks of Spiking Neurons: The Third Generation of Neural .. (1996)

ftp://ftp.informatik.uni-trier.de/pub/Users-CTVD/eccc/reports/1996/TR96-031/Paper.ps

http://citeseer.ist.psu.edu/cachedpage/34831/1

[15] S.M. Bohte, J.N. Kok, H. La Poutre, "Error-backpropagation in temporally encoded networks of spiking neurons",- Neurocomputing, v.48, pp.17-37, 2002

http://homepages.cwi.nl/~sbohte/publication/backprop.pdf

[16] T. Hirose, T. Asai and Y. Amemiya, "Pulsed neural networks consisting of single-flux-quantum spiking neurons",- Physica C: Superconductivity, Vol. 463-465, pp. 1072-1075, 2007.

[17] T. Hirose, T. Asai and Y. Amemiya," Spiking neuron devices consisting of single-flux-quantum circuits",-Physica C: Superconductivity, Vol. 445-448, pp. 1020-1023, 2006.

[18] T. Hirose, K. Ueno, T. Asai and Y. Amemiya, "Single-flux-quantum circuits for spiking neuron devices",- International Congress Series, Volume 1291, pp. 221-224, 2006.

[19] Neto, O.P.V.; Pacheco, M.A.C.; Barbosa, C.R.H., "Neural Network Simulation and Evolutionary Synthesis of QCA Circuits",- Transactions on Computers, Volume 56,  Issue 2,  Feb. 2007 Page(s):191 - 201

[20] A. Buller, M Perkowski, Cellular Automata realization of Regular Logic- ee.pdx.edu

http://www.ee.pdx.edu/~mperkows/temp/April10/ULSI-B2.pdf

[21] Buller A (2003b) Reversible Cascades and 3D Cellular Logic Machine, Technical Report TR-0012, ATR Human Information Science Laboratories, Kyoto.

[22] W.J. Townsend, JA Abraham - Nanotechnology, 2004. 4th IEEE Conference on, Complex gate implementations for quantum dot cellular automata, 2004

www.cerc.utexas.edu/~whitney/pdfs/nano04.pdf

[23] J. Watrous, "On one-dimensional quantum cellular automata", Proc. 36th FOCS, 1995: pp. 528–537.

[24] B. Schumacher and R. Werner, "Reversible quantum cellular automata", quant-ph/0405174

[25] C. Pérez-Delgado and D. Cheung, "Local Unitary Quantum Cellular Automata", Phys. Rev. A 76, 032320, 2007. See also arXiv:0709.0006 (quant-ph) http://www.arxiv.org/abs/0709.0006v1

[26] Domash, L.H., Cronin-Golomb, M., "Cellular automaton computation with nonlinear optics"-Signals, Systems and Computers, 1989. Twenty-Third Asilomar Conference on, Volume 1, Page(s): 106 - 110, 1989.

[27] Aubrey Jaffer, "4-Neighbor 3-State Universal Cellular Automaton", Copyright 1974, 2002, 2004.

http://swiss.csail.mit.edu/~jaffer/Cell/CAN4S3

[28] C.S. Lent, “Bypassing the Transistor Paradigm,” Science, pp. 1597-1599, June 2000.

[29] Momenzadeh, M.; Jing Huang; Tahoori, M.B.; Lombardi, F.; “Characterization, test, and logic synthesis of and-or-inverter (AOI) gate design for QCA implementation”,- Computer-Aided Design of Integrated Circuits and Systems, IEEE Transactions on, Volume 24,  Issue 12,  Dec. 2005 Page(s):1881 – 1893.

[30] Neto, O.P.V.; Pacheco, M.A.C.; Barbosa, C.R.H.; “Neural Network Simulation and Evolutionary Synthesis of QCA Circuits”,- Transactions on Computers, Volume 56,  Issue 2,  Feb. 2007 Page(s):191 – 201.

[31] Rui Zhang; Gupta, P.; Jha, N.K.; “Synthesis of majority and minority networks and its applications to QCA, TPL and SET based nanotechnologies”,- VLSI Design, 2005. 18th International Conference on, 2005 Page(s):229 – 234.

[32] Vilela Neto, O.P.; Pereira Masiero, L.; C. Pacheco, M.A.; Hall Barbosa, C.R.; “Evolvable Hardware Applied to Nanotechnology”,- Adaptive Hardware and Systems, 2006. AHS 2006. First NASA/ESA Conference on, 15-18 June 2006, Page(s):88 - 96

[33] “Simulation-based design of modular QCA Circuits”,- Huang, J.; Momenzadeh, M.; Schiano, L.; Lombardi, F.; Nanotechnology, 2005. 5th IEEE Conference on, 11-15 July 2005 Page(s):533 - 536 vol. 2 .

[34] Bajec, I. L. Mraz, M., "Multi-Valued Logic Based on Quantum-Dot Cellular Automata",- International J., Unconventional Computing, vol.3; issue 4, pp. 311-322, 2007.

[35] Beard, Mary Jean, "Design and simulation of fault-tolerant Quantum-dot Cellular Automata (QCA) NOT gates",- Thesis (M.S.)--Wichita State University, College of Engineering, Dept. of Electrical and Computer Engineering "July 2006." http://hdl.handle.net/10057/561

[36] G Varga, "Investigation of possibility of high temperature quantum-dot cellular automata",- Journal of Physics: Conference Series, v.61, pp.1216-1220, 2007.

http://www.iop.org/EJ/article/1742-6596/61/1/240/jpconf7_61_240.pdf

[37] Dysart, Timothy J.   Kogge, Peter M., "Probabilistic Analysis of a Molecular Quantum-Dot Cellular Automata Adder",-Defect and Fault-Tolerance in VLSI Systems, 2007. DFT '07. 22nd IEEE International Symposium on, 26-28 Sept. 2007, pp. 478-486.