SYSC 5104 Fall 2008

Assignment 1

October 28, 2008

Keith Holman, 100749416

Jeremy Kuzub, 100754498

# Abstract

Using the Eclipsed-based DEVS++ discrete event toolkit and simulator, a robot controller with a unique locomotion system is designed, implemented and tested. Rather than wheels, the robot is designed to translate against a vertical surface using cables fixed at one end that can wind on motor-controlled spools attached to the robot. The attachment points to the surface (in this case instantiated as a Chalk Board) can be arbitrary, and the controller is modeled in such a way that target (x, y) Cartesian coordinates can be translated to desired cable take-up and incremental motor movements. The Robot model is implemented so that the path between the current Robot position and the desired position is calculated in steps of defined resolution. This allows linear Robot movement to the target position regardless of the geometry of the robot and cable attachment points. By taking advantage of the event-driven architecture of DEVS and using the inherently modular architecture of CD++, development and of unit testing of increasingly complex sub-models is demonstrated. The final robot controller meets all design goals.

NOTE: This document and all related source and documentation files can be downloaded from
http://jufaintermedia.com/5104/SYSC5104-Assignment1.zip

# Contents

Abstract	2

Contents	4

List of Figures	8

Part I: High-Level Design	9

1.	Introduction: Cable-Anchored Robots	9

1.1.	Design Challenges	9

1.2.	Chosen implementation	11

1.3.	Robot Controller Processing System	12

2.	Overall Model Structure	14

3.	Componentized Breakdown	15

3.1.	Robot	15

3.2.	Generator	15

3.3.	Path Generator	15

3.4.	Queue	15

3.5.	Planner	16

3.6.	Translator	16

3.7.	Chalk Engager	16

3.8.	Motor	17

Part II: Detailed Design & Testing Strategy	18

4.	Atomic Models	18

4.1.	Chalk Engager	18

4.2.	Motor	19

4.3.	Planner	21

4.4.	Translator	23

4.5.	Queue	25

5.	Coupled Models	26

5.1.	Generator	26

5.2.	Path Generator	26

5.3.	Robot	27

6.	Testing Strategy	28

6.1.	Test Plan: Tools	28

6.2.	Test Plan: Procedure	28

6.3.	Boundary Condition Test Description	28

6.4.	Full System Test with a Complex Path	29

Part III: Verification & Validation	31

7.	Verification of Atomic Model “Queue”	31

7.1.	Event file input	31

7.2.	ma file	31

7.3.	CD++ Modeller Trajectories	32

7.4.	Analysis	32

8.	Verification of Atomic Model “Planner”	33

8.1.	Event file input	33

8.2.	ma file	33

8.3.	CD++ Modeller Trajectories	34

8.4.	Analysis	34

9.	Verification of Atomic Model “Translator”	35

9.1.	Event file input	35

9.2.	ma file	35

9.3.	CD++ Modeller Trajectories	36

9.4.	Analysis	36

10.	Verification of  Atomic Model “Engager”	37

10.1.	Event file input	37

10.2.	ma file	37

CD++ Modeller Trajectories	38

10.3.	Analysis	38

11.	Verification of Atomic Model “Motor”	39

11.1.	Event file input	39

11.2.	ma file	39

11.3.	CD++ Modeller Trajectories	40

11.4.	Analysis	40

12.	Verification of Coupled Model “PathGenerator”	40

12.1.	Event file input	41

12.2.	ma file	41

12.3.	CD++ Modeller Trajectories	42

12.4.	Analysis	43

13.	Verification of Coupled Model “Generator”	43

13.1.	Event file input	43

13.2.	ma file	44

13.3.	CD++ Modeller Trajectories	45

13.4.	Analysis	47

14.	Verification of Coupled Model “Robot”	47

14.1.	Event file input	47

14.2.	.ma file	47

14.3.	CD++ Modeller Trajectories	49

14.4.	Analysis	51

15.	Real-World Robot Test	52

15.1.	Discussion and Conclusion	53

15.2.	Next Steps and Future Development	53

Bonus	54

APPENDIX I: Full Robot Test Output	55

APPENDIX II: DEVS++ install on Windows Vista 32 bit	57

1.	Cygwin folder permissions	57

2.	Other debug attempts	57

References	59

List of Figures

Figure 1: Robot path between start and target coordinates	10

Figure 2: Robot path between start and target coordinates using a Path Planner	10

Figure 3: Chosen system design	12

Figure 4: Process flow for the robot controller	13

Figure 5: Overall Model of Robot System	14

Figure 14: Queue Trajectories	32

Figure 15: Planner Trajectories	34

Figure 16: Translator Trajectories	36

Figure 17: Engager Trajectories	38

Figure 18: Motor Trajectories	40

Figure 19: PathGenerator Trajectories	42

Figure 20: Generator Trajectories	45

Figure 21: Additional Trajectories for Generator	46

Figure 22: Robot coupled model trajectories.	50

Figure 23: Hand-drawn desired path for robot next to reconstructed simulation output	52

Figure 24: Vista O/S setting screens in sequence for Cygwin application.	58

# Part I: High-Level Design

## Introduction: Cable-Anchored Robots

In applications where terrain is too difficult to traverse using legs or wheels, other forms of robot locomotion must be found. Examples include disaster areas, such as building collapse, or environmentally sensitive locations where no disturbance can be tolerated. One form of locomotion that could allow 2D and 3D locomotion involves a cable-anchored robot.

Rather than wheels, a cable-anchored robot is designed to hang from two or more cables which are fixed above and around the desired area of movement. The ends of the cables meet at the robot and are each wrapped around motor-driven spools which the robot can rotate to let out cable or take it in. This effectively allows motion through a space or across a plane.

### Design Challenges

The challenge with such a system developing a controller that can accurately translate target position coordinates into corresponding cable lengths. A Translator component is required to perform this geometric conversion.

A second challenge is controlling the path between from one target coordinate to the next. For example, if a linear translation from point A to point B is desired, the actual path with vary depending on cable lengths and differential spooling motor speeds. It becomes necessary to generate and move to intermediate “waypoints” between the start and finish coordinates of a robot movement. (See Figure 1 and Figure 2)

Figure 1: Robot path between start and target coordinates

With a cable-anchor system the path is never linear and varies with robot position, anchor position, cable lengths, and relative spool-motor speeds. This unpredictability in movement path is undesirable.

Figure 2: Robot path between start and target coordinates using a Path Planner

The path planner “pins” way-points during robot travel. This removes a large part of the variability that the geometry of the anchor points and cables introduces. Increasing the number waypoints increases the linearity of the path.  This acts as a smoothing operation on the final path of the robot.

It is also desirable to be able to pre-load a sequence of target coordinates into the robot controller, so that a new destination does not have to be loaded only when the current destination is reached.

### Chosen implementation

A subset of this problem using two cables and a planar surface for movement was used as a real-world design specification. The robot would translate against a vertical surface using fixed cables that can wind on motor-controlled spools attached to the top of the robot (Figure 3). The attachment points to the surface (in this case instantiated as a Chalk Board) can be arbitrary, and the controller is modeled in such a way that target (x, y) Cartesian coordinates can be translated to desired cable take-up and incremental motor movements. The Robot model is implemented so that the path between the current Robot position and the desired position is calculated in steps of defined resolution. This allows linear Robot movement to the target position regardless of the geometry of the robot and cable attachment points.

This controller is designed in such a way that it can ultimately be implemented using the Lego MindStorms™ robotics construction toolkit and DEVS-MindStorms programming interface.

Figure 3: Chosen system design

The robot can translate on a vertical surface, in this case represented by a chalk board. Two cables fixed to anchor point above the robot allow movement by spooling in or out excess cable from two motor on the robot chassis.

### Robot Controller Processing System

Developing architecture for the robot controller involved breaking down the behavior of the Robot into sequential, event-driven steps. These each had a unique function and could be more easily translated into Atomic models for CD++ implementation, as seen in Figure 4.

Queue Atomic Model

Event File with coordinates

Path Generator Atomic Model

Translator Atomic Model

Sequential list of target coordinates for Robot movement

Store  these waypoints for use by the Robot

Plan a linear path of intermediate waypoints between Robot and next target coordinate

Translate next intermediate waypoint to cable lengths

Current target coordinate reached, request next target.

Current intermediate waypoint reached. Request next intermediate waypoint

Done movement

(sent by each motor)

Move motors to spool in our appropriate out cable length

Motor Atomic Model

Figure 4: Process flow for the robot controller

The corresponding CD++ atomic models can then be matched to each step of the process.

## Overall Model Structure

out

Event File

Motor 2

out

Motor 1

out

Generator

in:

{X, Y, D}

done

out

Path Generator

in

done

out

Translator

in:

next(ΔX, ΔY)

done

motorOut1

out

Robot

nextPosition

Chalk Engager

in

engaged

in

motorOut2

in

Queue

Planner

out

in

ready

done

in

engaged

motorOut1

motorOut2

out

Figure 5: Overall Model of Robot System

## Componentized Breakdown

### Robot

The Robot is the ‘top’ level coupled model.  This represents a cable-stay robot.  Each robot is composed of a Generator and Chalk Engager and two motors.  This simulates how a real cable-stay robot would work.  A robot would contain controllers, in this case a chalk engager, and 2 motors that communicate with MindStorms.

A robot would also require a software portion, in our case the ‘Generator’, which translates input into a format understandable by each controller.   The robot can be in either passive or active state.  When in passive the robot is waiting for the next event from the event file when in active the robot is handling an event from the event file or interpreting a response from the subsystem.

### Generator

A Generator is a coupled model coupling a path generator and a translator.   The purpose of this model is to combine the translation of an x and y position to a format that the controllers (2 motors and a chalk engager) would be able to understand.    The purpose of the path generator is to handle multiple concurrent inputs and to perform a pseudo smoothing operation by splitting large requests by into smaller segments that can be sent into the translator.

The generator can be in either active or passive state.  Initially in passive mode the generator is waiting for an in event from the top model or a response from the internal system.  When in active mode the generator is processing input from the top model or interpreting a response from the subsystem.

### Path Generator

The path generator is a coupled model that combines a queuing feature of points and a planner which splits large user requests into manageable portions that will smooth the final output of the robot.

The path generator can be in either a passive or active state.  When passive the path generator is waiting for input from other component.  When in active the path generator is in the process of handling input.

### Queue

The queue is atomic model that buffers any input received.  It contains two inputs one that input may coming into that is buffered and a ‘ready’ port which when set active will output the next item at the beginning of the buffer.   The queue is either in an active or passive state.  In passive mode the queue is waiting for new data or for a ‘ready’ signal that will output the next item.  When in the active state the queue stores the next item if an input or if a ready signal it will output the next item in the queue.  If the queue is empty and it receives a ready signal the queue goes into a passive state to wait (which is internally controlled by a ‘waiting’ flag).  Upon start up the first item received is immediately pushed to the output.

### Planner

This model divides an X, Y destination coordinate into discrete movement steps.  This serves to perform a basic smoothing operation based resolution (or STEP_SIZE).   For an example of why smoothing is required see the overall design requirements in Figure 1 and Figure 2.

Each interim destination coordinates are sent via the nextPosition port.   The planner will wait until input is received on ‘done’ port before sending out the next position.  This ensures that the motor completes a translation before sending the next position.   If an error occurs the translation is aborted.  The final output indicating success or failure is sent via the ‘out’ port.

The planner contains two states passive and active.  It is in passive state while waiting for a new position to translate to, and while processing a plan for the next position it is in the active state.

### Translator

The translator is an atomic model which converts a point in Cartesian space to discrete cable lengths that each motor must expelled in order to draw at that point.  The translator contains the position of each motor which is required to calculate the new lengths required.   The translator waits until a response is received by both motors before reporting a success or failure condition.  Failure is considered the dominate condition, i.e. if any motor fails the entire translation is considered to have failed.

The translator may be either a passive or active state.  In a passive state the translator is waiting for input of either a new position to calculate and adjust the motors to or waiting for a response from each of the motors.    Once a response is received from both motors the translation reports the status to the ‘out’ port.

### Chalk Engager

The chalk engager is an atomic model that represents the motor that controls the engagement of the chalk for drawing.  The ‘in’ port accepts a Boolean that if set to true will move the chalk so that it will draw on the chalkboard.  The current state of the chalk is stored, so that a request to put the chalk into the state that it is currently in will be ignored.

The chalk engager can be either in a passive or active state.  In passive, the chalk engager is waiting for a command that will change the visibility flag of the motor.  In active the chalk engager is in the process of moving the motor controlling the chalk engagement.

### Motor

The motor is an atomic model that is provided with a new length for the amount of cable to expel.  This model contains the amount of cable that is on the spool and the amount that is requested to be expelled.  If the request is larger then the amount of cable on the spool then the motor moves to the maximum length but is considered to have failed and will report a failure condition to the ‘out’ port.   Otherwise a ‘success’ condition will be sent to the ‘out’ port when the moving is complete.

The motor in MindStorms can be provided with an angular position to rotate to. This functionality is wrapped within a model which also contains the motor cables length (total), amount of cable expelled and the motor spool’s radius (in order to relate angular rotation with cable length change).

The motor can be either in a passive or active state.  In the passive state the motor is waiting for a command that will change the amount expelled.  In the active state the motor is in the process of moving the MindStorms motor changing the amount of cable currently expelled.

# Part II: Detailed Design & Testing Strategy

## Atomic Models

### Chalk Engager

_engaged

in

out

For a description of the Chalk Engager see section 3.7.

Internally the chalk engager contains the ‘_engaged’ attribute which stores whether the chalk is currently engaged to prevent the motor from requesting a change to the current state.

Though not documented used in the final coupled model the chalk engager contains an output port indicating when the chalk motor is engaged that is used for testing purposes.

Figure 6: Model of Chalk Engager

#### Formal Specification

in

in

in

out

P

A

out

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in}

Y = {out}

δint (active) = passive

δext (in, *)

{

if ( msg.value != _engaged )

_engaged  msg.value

return active

else

return passive

end

}

λ(active) = _engaged

ta(passive) = INFINITY

ta(active) = preparation time specified in ma file

### Motor

For a description of the Motor see section 3.8.  Internally the motor contains the ‘_expelled’, ‘_target’, ‘_total’ and ‘_success’ attributes, representing the total amount of cable expelled, the target amount to be expelled and the total amount of cable on the spool and whether the motor movement completed with success.

T

_expelled

_target

_total

_success

out

in

he ‘_total’ and ‘_expelled’ attributes are initially set to values read in from the ma file.  The ‘_target’ is adjusted each time a request is made via the ‘in’ port.  The amount ‘_expelled’ is guaranteed to be less than or equal to the ‘_target’ amount.   If the motor is requested to move to an impossible location, the ‘_success’ flag is set to false and this status is outputted to the out port after moving the motor the most extreme position possible.  If the requested movement succeeds a ‘true’ value is outputted to the out port upon completion of the movement.    The amount of time required for the movement is a linear time depending on the change in distance that the motor must move.

Figure 7: Model of Motor

#### Formal Specification

in

in

in

out

P

A

out

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in}

Y = {out}

δint (active) = passive

δext (in, passive) =

{

_success  msg.value > 0 AND msg.value < _total

if ( _success )

_target  msg.value

else

if ( msg.value < 0 )

_target  0

else

_target  _total

end

end

Δdistance  abs( _target - _expelled )

return active

}

δext (in, active) = active

λ(active) = _success

ta(passive) = INFINITY

ta(active) = Minutes: Δdistance / 60; Seconds: Δdistance mod 60

### Planner

For a description of the Planner see section 3.5.  Internally the planner contains the ‘_destination’, ‘_position’, ‘_visible’ and ‘_success’ attributes, representing the requested destination, the current position, whether this line is to be drawn and whether the translation completed with success.

T

_destination

_current

_visible

_success

out

in

done

engaged

nextPosition

ready

he ‘_destination’ and ‘_visible’ attributes with every triple input received on the in port.   After each translation the ‘_current’ position is updated until the planner reaches are point where ‘_current’ is equal to ‘_destination’ as long as ‘_success’ is set to true.  On failure the planner quits outputting this failure case to the ‘_out’ port.   The engaged port is outputted at the beginning of a translation indicating whether the chalk should be set visible or not.

Figure 8: Model of Planner

#### Formal Specification

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in, done}

Y = {engaged, nextPosition, out, ready}

δint (active) = passive

δext (in, *) =

{

_input[index]  msg.value

if ( index == 3 )

_visible  input[0]

_destination  Point( input[1], input[2] )

_success  true

index  0

else

Index  index + 1

end

return active

}

δext (done, *) =

{

if ( _success )

_success  msg.value

end

Return active

}

λ(active) =

{

if ( _postion != _destination )

output( engaged, _visible )

Δposition  _destination - _position

if ( Δposition.x != 0 && Δposition.y != 0 )

if abs( Δposition.x ) > abs( Δposition.y )

Δposition.x  STEP_SIZE

Δposition.y  slope * STEP_SIZE

else

Δposition.x  (1/slope) * STEP_SIZE

Δposition.y  STEP_SIZE

end if

else if ( Δposition.x != 0 )

if ( Δposition.x < 0 )

Δposition.x  -STEP_SIZE

else

Δposition.x  STEP_SIZE

end

else if ( Δposition.y != 0 )

if ( Δposition.y < 0 )

Δposition.y  -STEP_SIZE

else

Δposition.y  STEP_SIZE

end

end

_position  _posittion + Δposition

output( nextPosition, _position.y )

output( nextPosition, _position.y )

end
}

ta(passive) = INFINITY

ta(active) = 0

### Translator

_complete

_success

_anchorPoints[2]

motorOut1

in

done

out

motorOut2

For a description of the Translator see section 3.6.  Internally the translator contains the ‘_complete’, ‘_success’ attributes, representing whether the movement has been completed, and whether it completed with success, respectfully.  The ‘_anchorPoints’ attribute is an array containing the anchor points of each motor that must be specified in the .ma file.

Figure 9: Model of Translator

#### Formal Specification

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in, done}

Y = {motorOut1, motorOut2, out}

δint (active) = passive

δext (in, *) =

{

_input[index]  msg.value

if ( index == 2 )

for each motor ( I: 0 to …)

x  _input[0] – anchorPoint[I].x

y  _input[1] – anchorPoint[I].y

angle  atan2( y, x )

if ( cos( angle ) == 0 )

_targetLength[I]  y

else

_targetLength[I]  x / cos( angle )

end

end

return active

else

index  index + 1

return passive

end

}

δext (done, *) =

{

if ( _success )

_success = msg.value

_complete  count >= NUMBER_OF_MOTORS

if ( _complete )

return passive

else

return active

}

λ(active) =

{

if ( _complete )

output( out, _success )

else

output( motorOut1, _targetLength[0] )

output( motorOut2, _targetLength[1] )

end
}

ta(passive) = INFINITY

ta(active) = 0

### Queue

_queue

_waiting

out

in

ready

For a description of the Queue see section 3.4.  Internally the queue contains a ‘_queue’ attribute for buffering input and a ‘_waiting’ attribute to indicate that it is waiting for a response before forwarding the next item in the queue.

Figure 10: Model of Queue

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in, ready}

Y = {out}

δint (active) = passive

in

in

in

out

P

A

out

δext (in, passive) = active

δext (in, active) = active

δext (ready, passive) = active

δext (ready, active) = active

λ(active)

{

if ( !_queue.empty )

output( out, _queue.pop )

}

ta(passive) = INFINITY

ta(active) = 0.0

## Coupled Models

### Generator

out

in

done

engaged

motorOut2

motorOut1

For a description of the Path Generator model be see section 3.2.

Figure 11: Model of Generator

The formal specifications <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT> for the coupled model is defined follows:

X = {in, done};

Y = {out, engaged, motorOut1, motorOut2};

D = {pathGenerator1, translator1};

M(pathGenerator1) = PathGenerator;

M(translator1) = Translator;

I(pathGenerator1) = {translator1};

I(translator1) = {pathGenerator1, self};

Z(pathGenerator1) = {translator1};

Z(translator1) = {pathGenerator1, self};

SELECT: 	({pathGenerator1, translator1}) = pathGenerator1;

### Path Generator

out

in

done

engaged

nextPosition

For a description of the Path Generator model be see section 3.3.

Figure 12: Model of Path Generator

The formal specifications <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT> for the coupled model is defined follows:

X = {in, done};

Y = {out, engaged, nextPosition};

D = {queue1, planner1};

M(queue1) = Queue;

M(planner1) = Planner;

I(queue1) = {planner1};

I(planner1) = {queue1, self};

Z(queue1) = {planner1};

Z(planner1) = {queue1, self};

SELECT: 	({queue1, planner2}) = queue1;

### Robot

Robot

out

Event File

Figure 13: Model of Robot

For a description of the Robot model be see section 3.1.

The formal specifications <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT> for the coupled model is defined follows:

X = {Event File};

Y = {out};

D = {generator1, engager1, motor1, motor2};

M(generator1) = Generator;

M(engager1) = Engager;

M(motor1) = Motor;

M(motor2) = Motor;

I(generator1) = {engager1, motor1, motor2};

I(engager1) = {self};

I(motor1) = {generator1, self};

I(motor2) = {generator1, self};

Z(generator1) = {motor1, motor2};

Z(engager1) = {engager1};

Z(motor1) = {generator1, motor1};

Z(motor2) = {generator1, motor2};

SELECT:

({generator1, engager1, motor1, motor2}) = generator1,

({generator1, motor1, motor2}) = generator1,

({generator1, engager1, motor1}) = generator1,

({generator1, engager1, motor2}) = generator1,

({engager1, motor1, motor2}) = engager1,

({generator1, engager1}) = generator1,

({generator1, motor1}) = generator1,

({generator1, motor2}) = generator1,

({engager1, motor1}) = engager1,

({engager1, motor2}) = engager1,

({motor1, motor2}) = motor1;

## Testing Strategy

### Test Plan: Tools

Integration and Unit testing was straightforward using the Eclipse-based CD++ development tool chain. Each atomic and coupled model was tested using a dedicated .ma architecture file with that model defined as [top] and one or more .ev event files. Resulting log files were viewed and interpreted using CD++ Modeller trajectory plots. Additional debugging was done using text outputs within the CPP model files that could be viewed in the simu.exe execution window.

### Test Plan: Procedure

The testing procedure was a full bottom-up unit test of all atomic and coupled models, finishing with the final coupled Robot model. This would allow early detection of bugs and algorithm errors resulting. The result was a smoother development process with error sources quickly narrowed to specific atomic models and applicable code. Where appropriate, up to four types of unit tests were conducted on models:

- Test model with valid, expected data. This tests soundness of algorithms and event-driven behaviour and can be checked against pencil and paper calculations.

- Test model valid, unexpected data. This could include redundant or repeat signal events. Robot should not enter an inappropriate state.

- Test model with invalid and malformed inputs. This could include negative numbers for cable lengths or incomplete coordinate sets. This represents malformed event files or other input sources to the robot.

- Test model on boundary conditions.  To see if the robot and path generator handle cases ensure that internal boundary conditions are correctly implemented.

- Upon successful completion of all the integration tests a full system test will be performed that will serve as an integration test of all parts and a regression test to ensure that none of the existing functionality is broken by other parts of the system

### Boundary Condition Test Description

#### Robot Model Boundary Test

This tested Robot model behaviour when the target coordinates were no longer between and below the two anchor points, but outside and to the right of the rightmost anchor point. Although this is physically impossible (unless the cables could have compression applied to them) it is algorithmically and geometrically valid. The Robot model should successfully plan a path to this coordinate and simulate movement and associated cable lengths. The actual testing of this boundary condition was carried out during integration testing of the Robot coupled model but is included here for completeness.

#### Path Generator Boundary Test

This was a boundary test in which the robot was vertically in-line with the anchors and to the outside-right of the rightmost anchor. Although physically impossible, it is algorithmically and geometrically valid with a boundary condition that cable angles are equal to zero. This test would verify that the simulation checks for divide-by-zero conditions during calculations, and return the appropriate cable length boundary values. A similar case occurs when the robot is directly below anchor 1 or 2. The actual testing of this boundary condition was carried out during integration testing of the Robot coupled model but is included here for completeness.

### Full System Test with a Complex Path

Test model with valid inputs which are physically impossible for the system. This could include attempts to move the robot outside the physically possible movement area between and below the anchor points or requesting that a motor spool out more cable that it has available. Test Plan

A “round trip” regression test was designed to verify that the final Robot coupled model would perform as expected with a more complex path to follow. The test plan consisted of 4 parts:

- Pen and paper design of a set of waypoints. In this case a 22 waypoint path was designed so that the Robot would write the word “Lego” on the chalkboard. This test would cover some key performance points of the Robot architecture, including:

  - Paths that would require intermediate waypoints to be generated by the Planner

  - Paths in all four possible slope quadrants (∆x, ∆y), (-∆x, ∆y), (∆x,- ∆y), (-∆x,- ∆y), as well as special cases where ∆x or ∆y were equal to zero.

  - Paths that would cover the same point multiple times – this retracing would verify that the planner is not “collapsing” the set of input waypoints and that each waypoint in the input sequence would be considered ordered and unique. It would also test for any accumulated error in the path Planner and the Translator, which would show up as non-overlapping output points that were identical in the input file.

- Input waypoint pairs into an event file – a pair of points and the chalk engage/disengage flag are passed to the Robot in triples with a specific sequence in time:

For each target waypoint:

<t+dt1> in [ 1 | 0 ]		// chalk engaged | disengaged
<t+dt2> in [ Value ]		// Target X coordinate
<t+dt3> in [ Value ]		// Target Y coordinate

- Configure the robot with .ma file anchor positions, initial robot position, corresponding cable lengths and maximum cable lengths. Configure Translator to output its cable lengths to a comma-separated variable (csv) file.

- Execute the simulation and input the resulting cable length sequence pairs (LengthMotor1, LengthMotor2) into Excel-based graphing application that was designed as part of the test to solve for Robot (x,y) coordinates based on input cable lengths using a different method than that used by the Robot’s Translator model. Matching the resulting plot visually against the plot from the input waypoint set showed any errors or deviations immediately.

# Part III: Verification & Validation

## Verification of Atomic Model “Queue”

### Event file input

The input file tested behaviour during loading the queue via the “in” port, emptying of the queue by sending any input to the “ready” port, reloading, and fully emptying:

00:00:01:00 in 1

00:00:02:00 in 2

00:00:03:00 in 3

00:00:04:00 ready 1

00:00:05:00 in 4

00:00:06:10 in 5

00:00:07:20 in 6

00:00:20:00 ready 2

00:00:21:00 ready 3

00:00:22:00 ready 4

00:00:23:00 ready 5

00:00:24:00 ready 6

00:00:25:00 ready 7

### ma file

[top]

components : queue1@Queue

out : out

in : in

in : ready

Link : in in@queue1

Link : ready ready@queue1

Link : out@queue1 out

### CD++ Modeller Trajectories

Figure 14: Queue Trajectories

Red is the queue loading input, blue is the output, and green shows the “ready” inputs which trigger output on “out”. An increasing sequence is used for clarity on the “ready” port to differentiate between signals.

### Analysis

The trajectory plots from CD++ Modeller show that the queue fills and empties as expected. When empty, it immediately passes the first value it receives to the Out port, then waits for an input at the “Ready” port to send the next value. Additional redundant “ready” inputs were sent to test output behaviour when the queue was empty – as expected, no output was sent in this case.

## Verification of Atomic Model “Planner”

### Event file input

The input file tested behaviour during loading the planner via the “in” port, simulation the movement of each point by sending any input to the “done” port:

00:00:01:00 in 1

00:00:02:00 in 3

00:00:03:00 in 6

00:00:06:00 done 1

00:00:08:00 done 2

00:00:10:00 done 3

00:00:12:00 done 4

00:00:14:00 done 5

00:00:16:00 done 6

### ma file

The planner .ma file includes initial conditions include the Robot’s initial coordinates {x,y}

[top]

components : planner1@Planner

out : nextPosition

out : ready

out : engaged

out : out

in : in

in : done

Link : in in@planner1

Link : done done@planner1

Link : nextPosition@planner1 nextPosition

Link : out@planner1 out

Link : ready ready@planner1

Link : engaged@planner1 engaged

[planner1]

intial.x : 0

intial.y : 0

### CD++ Modeller Trajectories

Figure 15: Planner Trajectories

Red shows the input sequence: Chalk Engaged, followed by desired X and desired Y coordinates for the robot. The nextPosition trajectory shows planner’s sub-goals X,Y coordinates, which are sent to the out port as sequential new targets when ”done” input signals are received. In the coupled model, these done inputs are sent from the motors via the translator. The pink trajectory shows the planner generating a “ready” output once an entire movement sequence goal is achieved.

### Analysis

The Planner is the single most complex atomic model in the system, as it is responsible for creating a series of waypoint from the robot’s current position to the desired target coordinates. The model behaves as expected, effectively linearizing the path using waypoints which are determined by a specified maximum step size (the maximum distance the robot can travel in any one direction before another waypoint must be calculated).

## Verification of Atomic Model “Translator”

### Event file input

A coordinate pair (x,y) of (3,4) was sent with initial conditions set in the .ma file of anchor point 1 at (0,0) and anchor point 2 at (5,0). The resulting cable length were checked against manual calculation.

00:00:01:00 in 3

00:00:02:00 in 4

00:00:10:00 done 1

00:00:15:00 done 2

### ma file

[top]

components : translator1@Translator

out : motorOut1

out : motorOut2

out : out

in : in

in : done

Link : out@translator1 out

Link : motorOut1@translator1 motorOut1

Link : motorOut2@translator1 motorOut2

Link : in in@translator1

Link : done done@translator1

[translator1]

anchor1.x : 0

anchor1.y : 0

anchor2.x : 5

anchor2.y : 0

### CD++ Modeller Trajectories

Figure 16: Translator Trajectories

The red plot shows a target X,Y coordinate pair being input for translation to corresponding cable lengths. Green and Yellow plots show the resulting cable 1 and cable 2 target cable lengths calculated by the translator and output to the motors. The blue plot is a simulated signal from each motor signalling that the specified length has been reached. Once the Translator has received “done” port signals from both motors, it triggers an output on its “out” port (bottom plot) indicating robot translation is complete.

### Analysis

The Translator atomic behaved as expected and successfully translated a simulated target coordinate pair into corresponding cable lengths. The Translator also successfully parsed the separate “done” messages from each motor and sent out the correct “out” message to signal motor motion complete. It is noted that this test plan did not cover the input of multiple coordinate pairs, which led to discovery of a input buffer clearing issue in the testing of the coupled model Generator (see section 13)

## Verification of  Atomic Model “Engager”

This atomic model manages the chalk or pen, which can be retracted or deployed. There is a ‘deploy time’ modelled in the Engager. Thje engager is designed such that requests to ‘engage’ or ‘diengage the chalk’ are sent as Boolean inputs to the “in” port. Completion of chalk engagement or retraction is sent via the “out” port.

### Event file input

The event file simulates requests to retract and deploy the chalk. Multiple requests are modelled to test verify that chalk stay in place.

00:00:01:00 in 1

00:00:10:00 in 1

00:00:20:00 in 0

00:00:30:00 in 0

00:00:40:00 in 1

### ma file

The default state of the Engager and its preparation time is set in the .ma file.

[top]

components : engager1@Engager

out : out

in : in

Link : out@engager1 out

Link : in in@engager1

[engager1]

engaged : 0

preparation : 00:00:02:00

### CD++ Modeller Trajectories

Figure 17: Engager Trajectories

The red plot shows input and the blue plot the confirmation output from the model. The preparation time delay is visible in as a time shift between input and output.

### Analysis

This model was relatively straightforward to implement, yet shows the advantages of the DEVS formalism in easily implementing unknowns of real hardware performance. The model behaved as expected.

## Verification of Atomic Model “Motor”

Initial value are set for “length”, the total length of cable available on the motor spool, and “expelled”, the total length of cable that has been unwound from the spool. Te actuation time of the motor is modelled as a calculated time advance function duration proportional on the length of cable to be would or unwound. New “length” port inputs are ignored until actuation is complete.

### Event file input

The event file first spools out cable, ten spools in cable, then tests an error condition in which the requested spool-out length is greater than the total length of cable available.

00:00:01:00 in 7.7

00:02:00:00 in 2.1

00:10:03:00 in 12

### ma file

[top]

components : motor1@Motor

out : out

in : in

Link : out@motor1 out

Link : in in@motor1

[motor1]

length : 10

expelled : 4

### CD++ Modeller Trajectories

Figure 18: Motor Trajectories

### Analysis

The Trajectories show that the model behaved as expected, sending success outputs when the desired length is reached, and sending an error code of zero when the cable expelled exceeds the total cable length, seen here after the third and last command from the even file at 10:11:000. Although it is not visible in the trajectories since it is an internal variable, the motor position was verified to be at its maximum of 10 length units spooled out at the time the “out of cable” error signal was to the “out” port. This behaviour is desirable, as it allows robot maximum range of movement even if the exact boundaries of the robots range of motion are not explicitly known when commands are sent. This is a “best effort” failure mode as opposed to a predictive failure mode.

## Verification of Coupled Model “PathGenerator”

This coupled model combines uses the Queue atomic model to buffer data into the Planner atomic model. The overall behaviour of the PathGenereator coupled model should be appearing the same as thee Planner model alone. Initial verification showed that the Planner was not passivating until the translator reported back a motor done condition via the pathGenerator “done” port, demonstrating the effectiveness of unit testing in detecting logic errors.

### Event file input

The event file tested three key performance aspects of this central part of the Robot:

- Loading the Queue with multiple sequential target coordinates before the actually movements could be completed, and verifying that motions are carried out sequentially and that no data is lost or out of sequence.

- Testing the algorithm for path generation in the planner along positive and negative diagonal axis (covering all four quadrants of movement)

- Sending redundant motor “done” commands in an empty Queue condition.

00:00:01:00 in 1		//Boolean: Chalk engage

00:00:02:00 in 3		//X-coordinate target

00:00:03:00 in 6		//Y-coordinate target, full ‘instruction’ complete

00:00:04:00 in 0		//start of next instruction: chalk disengage

00:00:05:00 in 0		//X-target

00:00:05:500 in 0		//Y-target

00:00:06:00 done 1	//emulated motor done commands,

00:00:08:00 done 2	//incremental for clarity on trajectory trace

00:00:10:00 done 3

00:00:12:00 done 4

00:00:14:00 done 5

00:00:23:00 done 6

00:00:25:00 done 7

00:00:27:00 done 8

00:00:29:00 done 9

00:00:31:00 done 10

00:00:33:00 done 11

00:00:35:00 done 12

### ma file

The .ma file for the PathGenereator coupled model adds a Queue atomic in ahead of a Planner atomic. Initial position for the Robot is set through initial.x and initial.y parameters.

[top]

components : queue1@Queue

components : planner1@Planner

out : out

out : engaged

out : nextPosition

in : in

in : done

Link : in in@queue1

Link : out@queue1 in@planner1

Link : ready@planner1 ready@queue1

Link : out@planner1 out

Link : engaged@planner1 engaged

Link : nextPosition@planner1 nextPosition

Link : done done@planner1

[planner1]

intial.x : 0

intial.y : 0

[queue1]

### CD++ Modeller Trajectories

Figure 19: PathGenerator Trajectories

The top trace show the coordinate pair (3,6) loaded into the queue model, followed by the coordinate pair (0,0). The blue trace on “Ready” shows the Planner requesting two different sets of coordinates from the Queue model as motions are completed. The Red trace shows the Planners incremental position output coordinate pairs first in 6 steps up to the target (3,6) then 6 further steps to the next target (0,0).

### Analysis

The test plan for the PathGenerator composite model focussed on the interaction of the Queue with the Planner in accepting and sequential sets of targets and outputting appropriate incremental coordinate targets. Testing showed that the second coordinate target, in which the lanner makes incremental steps in the negative X direction, showed problems with slopes calculation in the Planner, which were corrected. These were clearly visible in the red “nextPosition” race as a lack of stair-step output back to (0,0). The test plan also covered a Queue containing multiple and partial coordinate sets. The final Path Generator behaved as expected.

## Verification of Coupled Model “Generator”

### Event file input

For verification purposed, this event file sends the same pair of sequential target coordinates that were used to test the PathGenerator: (3,6) followed by (0,0). Compared to the PathGenerator test, twice as many inputs to the done port need to be simulated, as the translator waits for a “done” input from each of the two connected motors. At the end of the event file an out of bounds condition is tested in which a coordinate outside the capability of the robot is set (-1,-1).

00:00:01:00 in 1

00:00:02:00 in 3

00:00:03:00 in 6

00:00:04:00 in 0

00:00:05:00 in 0

00:00:05:500 in 0

00:00:06:00 done 1

00:00:07:00 done 2

00:00:09:00 done 3

00:00:10:00 done 4

00:00:12:00 done 5

00:00:13:00 done 6

00:00:15:00 done 7

00:00:16:00 done 8

00:00:18:00 done 9

00:00:19:00 done 10

00:00:26:00 done 11

00:00:27:00 done 12

00:00:29:00 done 13

00:00:31:00 done 14

00:00:33:00 done 15

00:00:34:00 done 16

00:00:36:00 done 17

00:00:37:00 done 18

00:00:39:00 done 19

00:00:40:00 done 20

00:00:41:00 done 19

00:00:42:00 done 20

00:00:50:00 in 1

00:00:51:00 in -1

00:00:52:00 in -1

### ma file

The Generator instance is composed of three atomics, two of which are contained in the coupled model pathGenerator1, which was tested previously (see section 12). The initial position of the Robot is specified for the Planner as initial.X and initial.Y. The positions of the two cable anchor points are specified for translator1 as (0,0) and (0,5).

[top]

components : pathGenerator1

components : translator1@Translator

in : in

in : done

out : out

out : engaged

out : motorOut1

out : motorOut2

Link : in in@pathGenerator1

Link : done done@translator1

Link : nextPosition@pathGenerator1 in@translator1

Link : out@translator1 done@pathGenerator1

Link : out@pathGenerator1 out

Link : engaged@pathGenerator1 engaged

Link : motorOut1@translator1 motorOut1

Link : motorOut2@translator1 motorOut2

[pathGenerator1]

components : queue1@Queue

components : planner1@Planner

in : in

in : done

out : nextPosition

out : engaged

out : out

Link : in in@queue1

Link : out@queue1 in@planner1

Link : ready@planner1 ready@queue1

Link : done done@planner1

Link : out@planner1 out

Link : nextPosition@planner1 nextPosition

Link : engaged@planner1 engaged

[planner1]

intial.x : 0

intial.y : 0

[queue1]

[translator1]

anchor1.x : 0

anchor1.y : 0

anchor2.x : 5

anchor2.y : 0

### CD++ Modeller Trajectories

Figure 20: Generator Trajectories

Trajectories for coupled model Generator. Note that CD++ Modeller plots the “in” and “done” port of the Generator over top of the similarly named ports of the PathGenerator and Translator. Target coordinates of (3,6) are loaded on the “in” port (start of top trace), followed by a second target of (0,0) immediately after, which was shown to be successfully queued in previous tests. These result in intermediate coordinate steps are seen again in the top trace – these are the inputs to the Translator. The translator converts each of these intermediate target coordinates to corresponding target cable lengths for motor1 and motor2, which are seen in traces motorOut1 and motorOut2. These were cross checked against a pen-and-paper simulation of the robot and anchor point geometry and found to be accurate. The pink trace shows stepped inputs to the “done” port of the Generator that emulate the motor models’ signals when they have wound or unwound the desired cable length. The traces show that the Generator sends the next set of motor commands only when both motors have reached the desired position, and that the generator sends the correct output to from its “out” port (blue trace) when a target coordinate is reached.

Figure 21: Additional Trajectories for Generator

Generator trajectories for simple boundary condition: The robot starts at (2,5) and attempts to move beyond anchor2.x position of (0.5) towards (10,0). The PathGenerator does not check for this boundary condition and generates an erroneous path plan, which then is accurately converted to motor cable lengths by the Translator. This boundary test demonstrates the desirability of boundary checking at the atomic model level if possible.

### Analysis

The Generator initially did not behave as expected, despite successful verification of each of its component atomics. This showed a flaw in the verification of the Translator atomic, in that only a single coordinate was translated in any one text. Had multiple coordinated been translated sequentially, an input buffer clearing bug would have been discovered. Because of the coupled structure of the overall model, previous testing and CD++ trajectory visual analysis, it was relatively straightforward to determine the root cause and correct it. The Generator behaves as expected when given in-bounds coordinates (those that are below and between the two anchor points), however the Translator was shown to not check for positive X out of bounds conditions where the target robot coordinates are not between te anchor points

## Verification of Coupled Model “Robot”

### Event file input

As testing moves to more integrated coupled models, the event file decreases in complexity, as only target coordinate pairs and chalk engagement Booleans need to be loaded into the model. In this case the robot is instructed to move from its current position of (0,0) to (3,6) with the chalk engaged, then to return to the original position with the chalk disengaged.

00:00:01:00 in 1

00:00:02:00 in 3

00:00:03:00 in 6

00:00:04:00 in 0

00:00:05:00 in 0

00:00:05:500 in 0

### .ma file

[top]

components : engager1@Engager

components : motor1@Motor

components : motor2@Motor

components : robotGenerator1

in : in

out : out

Link : in in@robotGenerator1

Link : out@robotGenerator1 out

Link : out@motor1 done@robotGenerator1

Link : out@motor2 done@robotGenerator1

Link : engaged@robotGenerator1 in@engager1

Link : motorOut1@robotGenerator1 in@motor1

Link : motorOut2@robotGenerator1 in@motor2

[robotGenerator1]

components : pathGenerator1

components : translator1@Translator

in : in

in : done

out : out

out : engaged

out : motorOut1

out : motorOut2

Link : in in@pathGenerator1

Link : done done@translator1

Link : nextPosition@pathGenerator1 in@translator1

Link : out@translator1 done@pathGenerator1

Link : out@pathGenerator1 out

Link : engaged@pathGenerator1 engaged

Link : motorOut1@translator1 motorOut1

Link : motorOut2@translator1 motorOut2

[pathGenerator1]

components : queue1@Queue

components : planner1@Planner

in : in

in : done

out : nextPosition

out : engaged

out : out

Link : in in@queue1

Link : out@queue1 in@planner1

Link : ready@planner1 ready@queue1

Link : done done@planner1

Link : out@planner1 out

Link : nextPosition@planner1 nextPosition

Link : engaged@planner1 engaged

[planner1]

intial.x : 0		// initial robot position is adjacent to anchor point 1

intial.y : 0

[translator1]

anchor1.x : 0

anchor1.y : 0

anchor2.x : 5		// anchor point two is 5 distance units to the

anchor2.y : 0		// right of Anchor point 1

[motor1]

length : 10		// initial conditions chould be physically consistent

expelled : 0		// since the Robot is adjacent to motor 1, 0 cable has been

// expelled from that motor’s spool

[motor2]

length : 10

expelled : 5		// physically consistent: robot is 5 units away from

// anchor point 2 and therefore must have expelled

// 5 units of cable

[engager1]

engaged : 0 				// initial condition: chalk is off of surface

preparation : 00:00:00:500		// time for chalk actuator to engage

### CD++ Modeller Trajectories

Figure 22: Robot coupled model trajectories.

The robotGenerator sub-model is shown here since the Robot model trajectories are simple the target coordinate inputs and an overall “done” signal when the target coordinate storage queue has been emptied. The behaviour shown here matches the earlier robotGenrator test results, the only change being that the motor models and [chalk] Engager sent the appropriate “done” signals, rather than the Event File. The notable traces are Motor1out and Motor2out which show that appropriate cable lengths were spooled out to move the robot between desired target coordinates using the waypoints generated by the PathGenerator. The “Engaged” trace shows the chalk movement matches desired behaviour, drawing a line for the first movement and retracting the chalk for the return movement. Cable lengths for a subset of intermediate waypoints were verified by pencil-and-paper calculation as accurate.

### Analysis

The Robot coupled model was expected to have similar behaviour to the earlier robotGenerator test, since the only change was the addition of the motor and [chalk] Engager atomics which simulated delay time for actuation. The model behaved as expected, planning and following a physically feasible set of paths. Additional testing was conducted in which the target robot position extended beyond the available lengths of one or both motors, resulting in Robot travel to the maximum of one cable, then stopping, as this motor would no longer send ‘done’ signals. Because the motors are likely to run out of cable at different times, it is likely in practice that the last motion of the robot will depart from that specified by the pathGenerator. This behaviour may not be desirable, and may point to code changes which verify that the next waypoint in the planned path is achievable before executing the motor commands. This would allow best effort behaviour without unexpected motion.

## Real-World Robot Test

Figure 23: Hand-drawn desired path for robot next to reconstructed simulation output

Figure 23 shows a scan of the hand-drawn desired path for robot, converted into a sequence of target coordinates. These were input as an Event file to the coupled Robot model and a simulation was run. The output shown in the inset graph is plotted using Microsoft Excel which calculates coordinates from target cable lengths sent to the motor models. (See Table 1 on page 52 and the include Excel spread sheet for full results). This demonstrates a regression (round-trip) test of the robot model successfully planning, converting, and following a path using the anchor cable geometry.

As seen in Figure 23, the resulting path plot matches within rounding error the path specified in the Event File. Additional waypoints added by the Planner than 1 grid unit are visible in the output plot as extra data points on the longer paths. This regression test shows that the Robot behaves as expected. The Planner generated 10 additional intermediate waypoints to produce linear longer paths between target waypoints, and the regression test of the translator showed no error in the cable length calculations.

This test initially was not completely successful, and indicated an error in the path planning algorithms was not initially detected in the Atomic model tests of the Planner and Translator. Specifically, the path planner failed to generate intermediate waypoints in the special case of vertical and horizontal lines. Initial model tests were done with only diagonal lines, so cases where the slope was infinite or zero were not part of that test plan. This was easily pinpointed and corrected. This shows the value of full-up tests, not just to verify successful integration, but also to allow more opportunities to test the System in a larger data space. The greater the level of integration when an error is found, the more difficult it is to pinpoint its source; the lesson is that exhaustive unit testing is necessary, but not sufficient, as some test cases will likely be missed.

This test also showed the value of testing algorithms in regression using two different derivation methods. Inputs and outputs should match if the system is working as expected; errors oversights are more easily spotted. The disadvantage may be in development time as a partial second set of Robot control algorithms must themselves be derived, tested and debugged. The result, however, provides an additional level of confidence on the path to hardware implementation.

### Discussion and Conclusion

Using a bottom-up development and testing process resulted in a complex robot controller system that met desired performance goals. The flexibility of the anchor-cable locomotion system is offset by its geometric complexity; however this was shown to be successfully addressed with a path planner that could linearize robot motions with controllable fidelity. The CD++ development environment requires some additional overhead, but results in an event-driven architecture that is modular and easily tested with Event Files and multiple sets of .ma architecture files.

Visualization using CD++ Modeller’s atomic animate function proved invaluable in quickly verifying performance and detecting bugs. When looking into specific internal state issues for models, it was desirable to track variables inside the CPP files by outputting text to the command window when simu.exe was run.

### Next Steps and Future Development

The robot system has been designed with hardware implementation in mind using Lego MindStorms™ as a platform, and atomic models such as the Motor were based on the capabilities of the equivalent MindStorms components.

Expanding the movement of the Robot from 2D to 3D is a second design goal that will require addition of a Z coordinate and an addition motor model and cable. The algorithm for the path planner will also have to be extended to include vectors in three dimensions.

# Bonus

An attempt was made at the bonus portions of this assignment (and files are included in the bonus/ home directory.)   Three of the atomic models were modelled with the CD++ Modeller (Queue, Planner, and Motor).   A coupled model was created that integrated the Queue and Planner representing the [path] Generator.  Another was built integrating the Queue and Motor to test the state transition.

Though a valid .ma file was successfully generated from the coupled model it was unable to run:  the modeller was unable to link queue1 instance with the Queue atomic model and the planner1 instance with the Planner atomic model stored within their respective files.  Several attempts were made to model the coupled model without success.

The exported file did not save the association between instances and atomic models.   When running these files in a batch file it reported the error “ggad” is an unregistered type.  Changing the ma file (replacing “@ggad” with the symbols “@Planner” and “@Queue”) allowed the simulation to run generating a log file.  One point to note is that a STACK exception was thrown at the end of the simulation.

The type after the ‘@’ symbol was consistently lost when loading the files into the coupled model animator.  The ‘@’ symbol would be ‘@null’ within the animation tool.   This occurred even after manually modifying the .ma file by replacing the ‘@’ with the correct name (as found in the .ma files made for the original tests.)

Other approaches were attempted, however the coupled model would not run in the animation tool.

# APPENDIX I: Full Robot Test Output

This table is the output from the full robot test described in the Real World Robot Test on page 48.  Given the inputs points described in the test, the simulation produced the lengths for each line in each L1, L2 column.  Using some basic trigonometry identities these are converted to the corresponding x & y values.   These were the x & y values that were charted with Microsoft Excel to produce the output graph seen in Figure 23.

| Anchor1 | x | 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | y | 0 |  |  |  |  |
| Anchor2 | x | 20 |  |  |  |  |
|  | y | 0 |  |  |  |  |
| L0 |  | 20 |  |  |  |  |
|  | L1 | L2 | 1 | 2 | x | y |
|  | 1.11803 | 19.0066 | 0.026311 | 0.463683 | 0.999979 | 0.500034 |
|  | 2.23607 | 18.0278 | 0.055503 | 0.463689 | 1.999961 | 1.000083 |
|  | 2.82843 | 18.1108 | 0.110659 | 0.785412 | 1.999973 | 2.000031 |
|  | 3.60555 | 18.2483 | 0.165149 | 0.982797 | 1.999988 | 3.000006 |
|  | 4.47214 | 18.4391 | 0.218669 | 1.107151 | 1.999991 | 4.000009 |
|  | 5.38516 | 18.6815 | 0.270946 | 1.190282 | 2.000038 | 4.99998 |
|  | 5.83095 | 17.72 | 0.286051 | 1.030369 | 3.000039 | 4.999974 |
|  | 6.40312 | 16.7631 | 0.302886 | 0.896063 | 3.999961 | 5.000026 |
|  | 7.07107 | 15.8114 | 0.321751 | 0.7854 | 4.999992 | 5.000012 |
|  | 7.81025 | 14.8661 | 0.343025 | 0.694743 | 5.999977 | 5.000028 |
|  | 8.60233 | 13.9284 | 0.367175 | 0.620251 | 6.999994 | 5.000017 |
|  | 9.43398 | 13 | 0.394791 | 0.558599 | 7.999999 | 4.999999 |
|  | 8.5 | 13.1244 | 0.309703 | 0.489957 | 7.500003 | 3.999994 |
|  | 7.61577 | 13.3417 | 0.226802 | 0.404899 | 6.999975 | 3.000051 |
|  | 6.7082 | 14.3178 | 0.211091 | 0.463642 | 6.000014 | 2.999964 |
|  | 6.80074 | 15.0416 | 0.269167 | 0.628795 | 5.500008 | 3.999997 |
|  | 7.07107 | 15.8114 | 0.321751 | 0.7854 | 4.999992 | 5.000012 |
|  | 8.13941 | 15.6924 | 0.392341 | 0.828855 | 5.499964 | 6.000032 |
|  | 9.21954 | 15.6525 | 0.463648 | 0.862173 | 5.999979 | 7.000012 |
|  | 9.89949 | 14.7648 | 0.493941 | 0.785396 | 7.000015 | 6.999978 |
|  | 10.6301 | 13.8924 | 0.52807 | 0.718824 | 8.000006 | 6.999923 |
|  | 11.4018 | 13.0384 | 0.566733 | 0.661044 | 9.000029 | 7.000037 |
|  | 12.2066 | 12.2066 | 0.610731 | 0.610731 | 10 | 7.000077 |
|  | 13.0384 | 11.4018 | 0.661044 | 0.566733 | 10.99997 | 7.000037 |
|  | 12.0934 | 11.2361 | 0.563317 | 0.519146 | 10.50001 | 6.00001 |
|  | 11.1803 | 11.1803 | 0.46364 | 0.46364 | 10 | 4.999911 |
|  | 10.8167 | 12.53 | 0.499352 | 0.588009 | 9.000002 | 6.00008 |
|  | 10.6301 | 13.8924 | 0.52807 | 0.718824 | 8.000006 | 6.999923 |
|  | 11.4018 | 13.0384 | 0.566733 | 0.661044 | 9.000029 | 7.000037 |
|  | 12.2066 | 12.2066 | 0.610731 | 0.610731 | 10 | 7.000077 |
|  | 13.0384 | 11.4018 | 0.661044 | 0.566733 | 10.99997 | 7.000037 |
|  | 13.4001 | 12.2296 | 0.713062 | 0.639772 | 10.74999 | 8.000026 |
|  | 13.8293 | 13.0863 | 0.758377 | 0.70863 | 10.49996 | 9.000024 |
|  | 14.32 | 13.9665 | 0.798055 | 0.773055 | 10.24998 | 10.00001 |
|  | 14.8661 | 14.8661 | 0.832983 | 0.832983 | 10 | 11.00004 |
|  | 14.2127 | 15.5563 | 0.7854 | 0.885063 | 9.000059 | 10.99999 |
|  | 13.4536 | 14.8661 | 0.737813 | 0.837984 | 8.999961 | 10 |
|  | 12.7279 | 14.2127 | 0.685728 | 0.7854 | 8.999965 | 9.000004 |
|  | 12.8062 | 12.8062 | 0.674736 | 0.674736 | 10 | 7.999922 |
|  | 13.0384 | 11.4018 | 0.661044 | 0.566733 | 10.99997 | 7.000037 |
|  | 13.4164 | 10 | 0.6435 | 0.463647 | 11.99999 | 5.999993 |
|  | 13.9284 | 8.60233 | 0.620251 | 0.367175 | 13.00001 | 5.000017 |
|  | 14.8661 | 7.81025 | 0.694743 | 0.343025 | 14.00002 | 5.000028 |
|  | 15.8114 | 7.07107 | 0.7854 | 0.321751 | 15.00001 | 5.000012 |
|  | 16.1555 | 7.81025 | 0.876059 | 0.380507 | 15 | 6.000004 |
|  | 16.5529 | 8.60233 | 0.950542 | 0.436627 | 14.99996 | 6.999978 |
|  | 15.6525 | 9.21954 | 0.862173 | 0.463648 | 14.00002 | 7.000012 |
|  | 14.7648 | 9.89949 | 0.785396 | 0.493941 | 12.99999 | 6.999978 |
|  | 14.3178 | 9.21954 | 0.708624 | 0.432407 | 12.99999 | 5.999978 |
|  | 13.9284 | 8.60233 | 0.620251 | 0.367175 | 13.00001 | 5.000017 |

Table 1: Output from the Full Robot Test

# APPENDIX II: DEVS++ install on Windows Vista 32 bit

Install on Vista 32 SP1 worked as per the instructions provided on the cell-devs wiki, the only requirement was to make sure full permissions were set for the c:/cygwin folder as below:

## Cygwin folder permissions

(may also apply to the c:/tmp folder and the C:/Eclipse folder. I did this process to all three to be sure)

Right click on c:/cygwin folder, select Properties. In the popup, select the Security tab and Highlight your appropriate user group. Click the Edit button under Group or user names. Make sure “Full Control” is checked. If a popup to apply this to all subfolders appears, click ‘OK’

Also, I had to follow the manual download and install procedure for adding C:/cygwin/bin/cat.exe. The link specified in the installation instructions required the use of a free OSS  tar.bz2 archive tool called 7-zip, which handled the extraction without issue.

I would consider this information “debug” if the regular process does not work.

CD++ modeller seems to work as well at least to the point I have tested it.

Sequential screen shots of the permissions process are below. (Images resized losslessly in Word). Other debug information on next page.

## Other debug attempts

If Cygwin setup.exe does not run properly, try setting it to run in Compatibility mode by right-clicking, selecting Properties, clicking the Compatibility tab, checking “Run this program in compatibility mode for…” and selecting Windows XP [(Service Pack 2)]”. It was also found that old installs of Java had to be removed and the latest Java 6 installed.

Figure 24: Vista O/S setting screens in sequence for Cygwin application.

# References

All robot controller concepts, algorithms and implementation were derived from first principles in with the goal of discovering and testing solutions within the DEVS framework.

The lego MindStorms motor’s servo and position tracking capability was key in designing the robot controller. References include:

“NXT® motor internals” http://www.philohome.com/nxtmotor/nxtmotor.htm, accessed October 2, 2008

“Lego mindStorms Official Site” http://mindstorms.lego.com/Overview/Interactive_Servo_Motors.aspx, accessed October 2, 2008

“CD++ Wiki” http://cell-devs.sce.carleton.ca/mediawiki/index.php/Main_Page, accessed September 18 through October 24, 2008