### METHODOLOGIES FOR DISCRETE EVENT MODELLING AND SIMULATION

### SYSC 5104

#### Assignment #1

GSM Cellular Network Authentication Simulator

Name:  Rachid Chreyh

Student Number:              350664

Date:       Oct. 19 2005

# Part I: The Conceptual Model

In a GSM Cellular Network there are a number of security services implemented between the cellular network and handsets in the network.  One of those security services is the authentication service used to authenticate the identity of a given handset.  The handset functionality we are concerned about here is performed in the SIM card (contained in the handset), therefore we will refer to SIM card and handset interchangeably.  Each SIM card is assigned a subscriber authentication key (Ki) which is known to the network.  This Key is only known to the SIM card and the network, and it is never transmitted over the airways so that “hackers” cannot steal it.  Each SIM card also has an “identity” (Id), which could simply be thought of as the phone number.  The network stores the information related to each SIM Card Id, including the Authentication Key (Ki).

The authentication method used is a “challenge response” method in which the handset first contacts the network claiming a certain identity (Id).  To verify the SIM card’s identity without giving away the authentication key (Ki), the Network generates a random 128 bit number which is broadcast to the handset.  Both the handset and the network use a pre-defined Algorithm (called A3) which takes as input the 128 bit random number and the authentication key (Ki) and generates a 32 bit number called the SRES (Signed Response).  The handset then transmits the SRES back to the Network which verifies that it is the expected SRES.  If it is, then the authentication is successfully completed, otherwise it is rejected.

The GSM Authentication Simulator consists of 2 top level components: the SIM Card (SIM), and the Mobile Network. The Mobile Network is decomposed further to 2 components: Access Control (AC), and the Mobile Switching Center (MSC). (Figure 1)

Id

SIM Card

RAND

SRES_2

Mobile Network

AC

MSC

GSM Authentication Simulator

SRES_1

OK

MaxValidId

On

Figure 1   Structure of GSM Authentication Simulator

As shown above, the GSM Authentication Simulator has two inputs and one output. The first input is the ‘MaxValidId’ and it is used to specify the value of the maximum Id that is recognized by the GSM network.  A mobile phone with an Id higher than this number will fail authentication.  This scheme of specifying which mobile phones will be authenticated is a simplification of the real world authentication key idea presented earlier.  The second input to the simulator is the ‘On’ input and it is an integer specifying the Id of the mobile phone that is requesting authentication. It is intended to simulate the event of a mobile phone being turned on somewhere in the network.  Finally, the output is the ‘Ok’ which takes a value of 1 for a successful authentication and a value of 0 otherwise.  For the purpose of our simulator the Mobile Network is assumed to be very fast compared to the SIM card, and therefore its operation is assumed to be instantaneous.  Meanwhile the SIM card takes 3 seconds to send the Id to the network after receiving an ‘On’ input, and takes another 3 seconds to calculate the value of SRES_2.  The following is a breif description of the behavior of each component of the simulator.

The SIM Card (SIM):

The SIM Card (SIM) has the following state variables:

Phase = {Passive, Active}

Authenticating = Boolean

IdNum, SRES2_Num, Rand_Num = Integer

The SIM card ignores any input if it is in the Active state. If it’s in the Passive state the following can takes place:

1- If an ‘On’ input is received and Authenticating is False (meaning we are not busy with a previous authentication) the following takes place:

- The integer number received at the ‘On’ port is stored in state variable IdNum.

- The phase is set to active and remains active for 3 seconds after which the value of IdNum is sent out the ‘Id’ output port. At this point the Phase is reset to Passive, but the state variable Authenticating is now set to True.

2- If a ‘RAND’ input is received and Authenticating is True (meaning we waiting for the RAND input to complete an authentication) the following takes place:

- The integer number received at the ‘RAND’ port is stored in state variable Rand_Num.

- The phase is set to active and remains active for 3 seconds after which the value of IdNum is added to the value of Rand_Num and the result of the addition is sent out the ‘SRES2’ port to the MSC. At this point the Phase is reset to Passive, and the state variable Authenticating is reset to False.

The Access Control (AC):

The Access Control (AC) has the following state variables:

IdNum, SRES1_Num, Rand_Num, and MaxValidIdNum = Integer

When the ‘MaxValidId’ input is received at the AC, its value is stored in the state variable MaxValidIdNum. Nothing else happens.

When the ‘Id’ input (an integer) is received at the AC, the following takes place:

- The value of the input is stored in state variable IdNum.

- A Random number is generated and stored in state variable Rand_Num.

- The Random number (Rand_Num) is sent out the ‘RAND’ port to the SIM.

- If the value of state variable IdNum is greater than or equal to MaxValidIdNum, the Id is considered as an authentic Id and the appropriate SRES value is calculated by adding Rand_Num to IdNum and storing the result in SRES1_Num. Otherwise, if the value of IdNum is less than MaxValidIdNum, the Id is considered as an invalid Id, and a value of zero is stored in SRES1_Num.

- SRES1_Num is then sent out the ‘SRES_1’ port to the MSC.

The Mobile Switching Center (MSC):

The Mobile Switching Center (MSC) has the following state variables:

Num_Inputs, SRES1_Num, SRES2_Num, OkValue = integer

When the SRES_1 input is received, the following takes place:

- The value of the received SRES_1 input is stored in SRES1_Num.

- If the state variable Num_Inputs is zero (first input), then it is set to 1. Otherwise, if the state variable Num_Inputs is greater than zero it means we have received two consecutive SRES_1 inputs; therefore state variable OkValue is set to 0 and its is output to the ‘Ok’ output port (indicating a failed authentication for the previous SRES_1).

When the SRES_2 input is received, the following takes place:

- The value of the received SRES_2 input is stored in SRES2_Num.

- If the sate variable Num_Inputs is equal to zero (meaning we don’t have an SRES_1 value to compare with), the SRES_2 input is ignored and nothing else happens.  If, however, the sate variable Num_Inputs is greater than zero (meaning we have an SRES_1 value to compare with), then SRES1_Num is compared with SRES2_Num. If they matched then state variable OkValue is set to 1, otherwise it is set to 0. In either case the value of state variable ‘OkValue’ is output at the ‘Ok’ output port.

# Part II: DEVS Formal Specification

# The conceptual model of the “Cellular GSM Network Authentication Simulator” described above shows that our model of the overall system is composed of two sub-models.  These sub-models are the SIM card and the Mobile Network. The SIM card is an atomic model, while the Mobile Network is a coupled model that can be de-composed further into two atomic sub-models. These are the Access Control (AC) and the Mobile Switching Centre (MSC).  The SIM sends its Id number to the AC. The AC than generates a random number that is sent back to the SIM.  The AC also (depending on the value of MaxIdNum) outputs a value for SRES_1 to the MSC.  When the AC gets the random number it generates SRES_2 and sends it to the MSC.  The MSC compares the inputs of SRES_1 and SRES_2 and outputs Ok = true if they are equal and false otherwise. As indicated in the conceptual model, the Mobile Network (and its sub-components) are assumed to be very fast that their processing is instantaneous, while the SIM card is slower and takes 3 seconds to produce an Id output and another 3 seconds to produce the SRES_2 output.

# The top model (i.e. our simulator) has 2 inputs and 1 output.  The “MaxIdNum” input specifies the maximum Id defined in the Mobile Network. The “On” input is a trigger that represents some handset being turned on (and thus requiring authentication). Finally the “Ok” output is a Boolean that indicates if the handset passed authentication by the network or not. The following are the DEVS formal specifications of each of the models involved.

The SIM Atomic Model:

# S = {Sigma, Phase, Authenticating, IdNum, SRES2_Num, RandNum}

X = {On, RAND}

Y = {Id, SRES_2}

δint(Sigma, Phase, Authenticating, IdNum, SRES2_Num, RandNum, e, x)

{

case Authenticating

False: {Authenticating = True // The Id is sent out and now

Sigma = INFINITY	  // waiting for authentication from Network

Phase = Passive

}

True: {Authenticating = False // The SRES_2 is sent out and now

Sigma = INFINITY    // setting back to initial state.

Phase = Passive

}

}

δext(Authenticating, Sigma, IdNum, SRES2_Num, RandNum, e, x)

{

case phase

Passive:

If (not Authenticating)

{

If x is from On

{

IdNum = x.value;

Sigma = Constant Preparation Time    //Wait for some time delay

// before outputting Id

Phase = Active

}

else if x is from RAND

{ // Should not be here. Input Ignored. }

}

else if (Authenticating)

{

If x is from On

{ // Should not be here. Input Ignored. }

else if x is from RAND

{

RandNum = x.value;

SRES2_Num  = RandNum + IdNum;

Sigma = Constant Preparation Time // Wait for some time delay

// before outputting SRES_2

Phase = Active

}

}

}

λ(Authenticating) = Send SRES2_Num to port SRES_2

λ(!Authenticating) = Send IdNum to port Id

The AC Atomic Model:

# S = {Phase, Sigma, IdNum, SRES1_Num, RandNum, MaxValidIdNum}

X = {MaxValidId, Id}

Y = {RAND, SRES_1}

δext(Phase, Sigma, IdNum, SRES1_Num, RandNum, MaxValidIdNum, e, x)

{

If x is from MaxValidId

{	MaxValidIdNum = x.value() }

else if x is from Id

{

IdNum = x.value()

RandNum  = A randomly generated integer

if (IdNum <= MaxValidIdNum)

SRES1_Num = RandNum + IdNum

else

SRES1_Num = 0

Sigma = 0 //Trigger Internal transition function immediately

// so that the outputs are produced at the output ports.

// It is assumed that the processing time of the AC is

// negligible, so outputs are produced immediately on

// receiving Id input.

Phase = active

}

}

δint(Phase, Sigma, IdNum, SRES1_Num, RandNum, MaxValidIdNum, e, x)

{

// The internal transition function does not do anything.

// The outputs are produced immediately

Sigma = INFINITY

Phase = passive

}

λ(s) = {

// These outputs occur immediately after Id inputs received

Send Rand_Num to port RAND

Send SRES1_Num to port SRES_1

}

The MSC Atomic Model:

# S = {Phase, Sigma, Num_Inputs, SRES1_Num, SRES2_Num, OkValue}

X = {SRES_1, SRES_2}

Y = {Ok}

δext(Phase, Sigma, Num_Inputs, SRES1_Num, SRES2_Num, OkValue, e, x)

{

If x is from SRES_1

{

SRES1_Num = x.value()

if (Num_Inputs == 0)

Num_Inputs++

else

{     OkValue = 0; 		// received another SRES_1 while one was

// pending, so output a result of Ok = false

// to indicate that the last SRES_1 value did not

// match an SRES_2 value.

Sigma = 0   //Trigger Internal transition function immediately

// so that the Ok output is produced.

// It is assumed that the processing time of the MSC is

// negligible, so outputs are produced immediately.

Phase = active

}

else if x is from SRES_2

{

SRES2_Num = x.value()

if (Num_Inputs != 0)

{

OkValue = (SRES2_Num == SRES1_Num);

Num_Inputs = 0;

Sigma = 0   //Trigger Internal transition function immediately

// so that the Ok output is produced.

// It is assumed that the processing time of the MSC is

// negligible, so outputs are produced immediately.

Phase = active

}

}

}

δint(Phase, Sigma, Num_Inputs, SRES1_Num, SRES2_Num, OkValue, e, x)

{

// The internal transition function does not do anything.

// The outputs are produced immediately

Sigma = INFINITY

Phase = Passive

}

λ(s) = {

Send OkValue to port Ok

}

The Mobile Network Coupled Model:

X = {MaxValidId, Id, SRES_2}

Y = {RAND, Ok}

D = {AC, MSC}

MAC = AC Atomic Model (see above)

MMSC = MSC Atomic Model (see above)

I (AC) = {MSC}

I (MSC) = {} // empty set

Z (AC) = MSC

SELECT: ({AC, MSC}) = MSC

The GSM Authentication Simulator Coupled Model:

X = {MaxValidId, On}

Y = {Ok}

D = {Mobile_Network, SIM}

MMobile_Network = Mobile Network Coupled Model (see above)

MSIM = SIM Atomic Model (see above)

I (SIM) = {Mobile_Network, self}

I (Mobile_Network) = { SIM, self }

Z (SIM) = Mobile_Network ; Z(SIM) = self

Z (Mobile_Network) = SIM ; Z(Mobile_Network) = self

SELECT: ({Mobile_Network, SIM}) = Mobile_Network

Testing Strategy:

The models will be tested starting at the level of the atomic models and moving up the hierarchy until all models have been tested.  The testing is done by creating input event files (*.ev) for each model. These event files will have combinations of inputs designed to test all the different possibilities and sequence of inputs. To verify correctness of the models, the expected results for each model’s input test file is determined a priori (see below). The simulation is then run and the result files (*.out) produced by the simulation are then compared to the expected results (recorded below).  The following sections describe the tests for each model.

The SIM atomic Model

The ‘On’ input of the SIM model is an integer representing the Id of the SIM card. When an input is received at this port, its value is sent out the ‘Id’ output port after a constant preparation time (currently defined in the .ma file to be 3 sec). Immediately after an input is received at the ‘On’ input port the SIM model is in a state of waiting for a ‘RAND’ input, so if during this time another ‘On’ input is received it will be discarded. However, when the ‘RAND’ input is received its value is added to the value of Id and then, after another constant preparation time, the result of this addition is sent out the ‘SRES_2’ output port. Any input received during this preparation time is ignored.

The SIM event input file used to test this Model is as follows (SIM.ev):

00:00:10:00 On 8

00:00:30:00 Rand 15

00:00:45:00 On 12

00:01:00:00 On 55

00:01:05:00 Rand 25

00:01:25:00 On 46

00:01:30:00 Rand 5

00:01:55:00 Rand 17

00:01:55:00 On 5

00:01:56:00 On 10

00:02:00:00 Rand 21

00:02:01:00 Rand 50

For each ‘On’ input , except the ones in bold, the output file should contain an ‘Id’ output with the same value but 3 seconds later. The ‘On’ inputs in bold are discarded.  Also, for each ‘Rand’ input, except the ones in bold, the output file should contain an ‘SRES_2’ output with the sum of the ‘Rand’ input and the previous (non-discarded) ‘On’ value. Again the ‘SRES_2’ outputs will appear 3 seconds later than the ‘Rand’ input.  The ‘Rand’ inputs in bold are discarded.  For the above test file the following is the expected output file.

00:00:13:00 id 8

00:00:33:00 sres_2 23

00:00:48:00 id 12

00:01:08:00 sres_2 37

00:01:28:00 id 46

00:01:33:00 sres_2 51

00:01:58:00 id 5

00:02:03:00 sres_2 26

The AC atomic Model

The ‘MaxValidId’ input of the AC model is an integer representing the maximum Id value for which the AC will produce the correct SRES_1. Receiving Ids with a greater value than that will result in an SRES_1 of zero.  When an input is received at the ‘MaxValidId’ input its value is stored, but no outputs are produced.

The ‘Id’ input of the AC model is an integer representing the Id of the SIM card requesting authentication. When an input is received at this port the following is performed immediately (i.e. there is no processing time since it’s assumed the time taken for the AC to do any processing is negligible):

- A random number is generated and sent out the ‘RAND’ output port.

- The value of SRES_1 is determined (as explained in earlier sections) and the SRES_1 value is output to the ‘SRES_1’ output port.

The AC event input file used to test this Model is as follows (AC.ev):

00:00:05:00 MaxValidId 12

00:00:10:00 Id 11

00:00:30:00 Id 25

00:00:45:00 MaxValidId 50

00:00:52:00 Id 10

00:01:25:00 Id 40

00:01:35:00 MaxValidId 19

00:01:55:00 Id 50

00:02:00:00 Id 19

For each ‘Id’ input, the output file should contain a ‘RAND’ and ‘SRES_1’ output with the same time as the ‘Id’ input. The ‘RAND’ outputs cannot be determined at this point since they are random numbers. The ‘SRES_1’ outputs however are going to be equal to the value of the corresponding ‘Id’ input plus the corresponding value of the ‘RAND’ output, except when the value of Id is greater than the value of the last ‘MaxValidId’ inputs, in which case the ‘SRES_1’ outputs will have a value of zero.  For the above test file the following is the expected output file.

00:00:10:000 rand x1

00:00:10:000 sres_1 (11 + x1)

00:00:30:000 rand x2

00:00:30:000 sres_1 0

00:00:52:000 rand x3

00:00:52:000 sres_1 (10 + x3)

00:01:25:000 rand x4

00:01:25:000 sres_1 (40 + x4)

00:01:55:000 rand x5

00:01:55:000 sres_1 0

00:02:00:000 rand x6

00:02:00:000 sres_1 (19 + x6)

The MSC atomic Model

The ‘SRES_1’ and ‘SRES_2’ inputs of the MSC model are integers received from each of the AC and the SIM modules respectively.  If these values matched then the authentication is successful, otherwise it has failed.  When an ‘SRES_1’ input is received it is stored until an ‘SRES_2’ input is received and a comparison can be made. A successful comparison will cause an output of one to be produced at the ‘Ok’ output port. If however another ‘SRES_1’ input is received before the ‘SRES_2’ input, then the authentication of the first one is considered to be failed, and an output of zero is produced at the ‘Ok’ output port. If an ‘SRES_2’ is received when no ‘SRES_1’ has been received then it will be discarded because it is not expected.

The MSC event input file used to test this Model is as follows (MSC.ev):

00:00:10:00 SRES_1 12

00:00:30:00 SRES_2 21

00:00:45:00 SRES_1 15

00:01:00:00 SRES_2 15

00:01:05:00 SRES_1 10

00:01:25:00 SRES_1 19

00:01:30:00 SRES_2 19

00:01:35:00 SRES_2 24

00:01:45:00 SRES_1 50

00:01:55:00 SRES_2 50

For each pair of consecutive ‘SRES_1’ and ‘SRES_2’ inputs, the output file should contain an ‘Ok’ output with the same time as the ‘SRES_2’ input. The value of the ‘Ok’ output is 1 if the values of ‘SRES_1’ and ‘SRES_2’ are the same and 0 otherwise. Also for each occurrence of two consecutive ‘SRES_1’ inputs, an output of ‘Ok’ = 0 is produced at the same time as the second ‘SRES_1’. The lines in bold in the event input file are inputs that are discarded by the MSC (for reasons described earlier). For the above test file the following is the expected output file.

00:00:30:000 ok 0

00:01:00:000 ok 1

00:01:25:000 ok 0

00:01:30:000 ok 1

00:01:55:000 ok 1

The Mobile Network Coupled Model

The Mobile Network coupled model is made up of the combination of the AC and the MSC atomic models. Therefore, the ‘MaxValidId’ input of the Mobile Network model has the same behavior described earlier for the AC model. The other inputs are the ‘Id’ input port and the ‘SRES_2’ input port. When an input is received at the ‘Id’ input port a Random number is generated and sent out the ‘RAND’ output port.  When an ‘SRES_2’ input is received it is either discarded (if received before Id) or it causes an output to be produced at the ‘Ok’ output port.

The Mobile Network event input file used to test this Model is as follows (Mobile_Network.ev):

00:00:10:00 MaxValidId 27

00:00:15:00 SRES_2 43

00:00:20:00 Id 14

00:00:23:00 SRES_2 112

00:00:30:00 Id 25

00:00:33:00 SRES_2 45

00:00:45:00 Id 15

00:00:50:00 Id 10

00:00:52:00 SRES_2 113

00:00:55:00 SRES_2 46

00:01:00:00 MaxValidId 15

00:01:20:00 Id 20

00:01:23:00 SRES_2 47

For each ‘Id’ input, the output file should contain a ‘RAND’ output with the same time as the ‘Id’ input. Also, for each ‘SRES_2’ input received after an ‘Id’, the output file should contain an ‘Ok’ output with the same time as the ‘SRES_2’ input. The ‘SRES_2’ inputs shown in bold are ignored because they are unexpected inputs. Also the ‘Id’ input in bold has a special behavior because it comes right after another Id input. It will basically overwrite the old Id and cause an ‘Ok’ output of zero to be produced (to indicate that the previous Id failed authentication). For the above test file it is not possible to determine the exact expected results because the RAND output is a random number. Nevertheless the following is the kind of output to be expected.

00:00:20:000 rand 98

00:00:23:000 ok 1

00:00:30:000 rand 20

00:00:33:000 ok 1

00:00:45:000 rand 4

00:00:50:000 rand 103

00:00:50:000 ok 0

00:00:52:000 ok 1

00:01:20:000 rand 18

00:01:23:000 ok 0

The GSM Authentication Simulator Coupled Model

The GSM Authentication Simulator coupled model is made up of the combination of the SIM atomic model and the Mobile Network Coupled model. The GSM Authentication Simulator has two inputs, ‘On’ and ‘MaxValidId’. The behavior of the inputs is as described in the earlier sections, namely: Receiving an ‘On’ input causes the SIM to start the authentication process by sending its Id (which is the value received at the ‘On’ input port) to the Mobile Network. Receiving a ‘MaxValidId’ input sets the maximum Id value that will be considered an authentic Id by the Mobile Network. The only output of the GSM Authentication Simulator is the ‘Ok’ output which indicates, for each received ‘On’ input, whether that particular mobile phone was successfully authenticated or not. The ‘Ok’ output should appear 6 seconds after the time the ‘On’ input was received due to “preparation time” delays in the SIM atomic model.

The GSM Authentication Simulator event input file used to test this Model is as follows (GSM_Auth_Sim_1.ev):

00:00:10:00 MaxValidId 27

00:00:15:00 On 25

00:00:25:00 On 45

00:00:35:00 On 15

00:00:40:00 MaxValidId 5

00:00:45:00 On 10

For each ‘On’ input with a value less than or equal to the last ‘MaxValidId’ input, the output file should contain an ‘Ok’ output of 1. Otherwise it should contain an ‘Ok’ output of 0. The outputs are 6 seconds after the inputs. For the above test file the following is the output to be expected.

00:00:21:000 ok 1

00:00:31:000 ok 0

00:00:41:000 ok 1

00:00:51:000 ok 0

# Part III: Implementation and Results

The model specifications (atomic and coupled) defined in part II above were implemented using CD++. The relevant source files are included in the package with this document for the interested reader.  Each of the Atomic and Coupled models were also tested using the test input event files developed in the previous section, and the resulting output files contained the expected results presented earlier.  The success of these tests proves (to a great extent) the correctness of the models and thus the correctness of the overall GSM Authentication Simulator.

In addition to the test input event file “GSM_Auth_Sim_1.ev” shown above, which was run to verify the correctness of the simulator, the following event files were also run and the outputs from them are presented and discussed below.

- Event file “GSM_Auth_Sim_2.ev”: This event file was designed such that it contains the same events as the first event file with an addition to the end of it that simulates a continuous input (every second for 7 seconds) at the ‘On’ port. The intent of this input file is to examine the behavior of the simulator when new inputs are received while it is busy with a previous input. The event file looks as follows:

00:00:10:00 MaxValidId 27

00:00:15:00 On 25

00:00:25:00 On 45

00:00:35:00 On 15

00:00:40:00 MaxValidId 5

00:00:45:00 On 10

00:00:46:00 On 1

00:00:47:00 On 1

00:00:48:00 On 1

00:00:49:00 On 1

00:00:50:00 On 1

00:00:51:00 On 1

00:00:52:00 On 1

The result of running the above event file is as follows:

00:00:21:000 ok 1

00:00:31:000 ok 0

00:00:41:000 ok 1

00:00:51:000 ok 0

00:00:58:000 ok 1

As expected, the last 7 consecutive inputs resulted in only one output corresponding to the last input.  All the other inputs from time 46 to 51 are ignored. The reason for this is because in our model the SIM card takes 3 seconds to prepare to send out the Id output to the network and 3 seconds to prepare to send out the SRES_2 output to the network (the network itself takes negligible time (no time) to do its processing). During these 6 seconds the SIM card will ignore any inputs it receives at the ‘On’ port. As a result, of the consecutive inputs, only the input received after 7 seconds is not discarded.

Using CD++ Modeler, the above described results can be seen graphically. The following figure was captured from the CD++ modeler screen and it shows the changes in each of the relevant state variables for the above input event file.

Figure 2   Graphical Display from CD++ Modeler

As can be seen above (figure 2) first thing that happens is that MaxValidId gets a value of 27 at time 10.  It keeps that value until at time 40 it becomes 5 (just like the input file specified).  Also following the input file, the ‘On’ state variable gets the appropriate values at the appropriate times. Notice the consecutive inputs at the end.  In response to the ‘On’ inputs, an output is seen at the ‘Id’ state variable 3 seconds after the ‘On’ input is received.  At the same time (3 seconds after input ‘On’) a random number is generated and put in state variable ‘rand’. Notice that for the consecutive inputs at the end no ‘Id’ or ‘rand’ values are generated since those inputs are ignored. Now, another 3 seconds after each of the ‘rand’ values, a ‘sres_2’ value is generated.  This value is sent to the MSC which immediately produces an output, which can be seen in the graph of ‘Ok’.

- Event file “GSM_Auth_Sim_3.ev”: This event file was designed to observe the behavior of the simulator when an ‘On’ input is received before a ‘MaxValidId’ input. In this case the Mobile network is assumed not to recognize any Ids as valid and thus authentication of all inputs should result in a failure. The event file looks as follows:

00:00:15:00 On 0

00:00:25:00 On 1

00:00:35:00 On 2

00:00:45:00 On 3

The result of running the above event file is as follows:

00:00:21:000 ok 0

00:00:31:000 ok 0

00:00:41:000 ok 0

00:00:51:000 ok 0

As expected, all of the outputs indicate a failed authentication.