Methodologies for Discrete-Event Systems

Modelling and Simulation

SYSC5104

Assignment 1

Simulator of a House Heating System

Blerim Qela

3067273

University of Ottawa

PART I

The House Heating System is a model that enables us to observe and study the effects of outdoor temperature changes and the corresponding dynamic response of a typical house heating system.

This is a simplified thermodynamic model of a House Heating System (refer to Fig.1 below) consisting of few essential components, such as the:

- Outdoor Temperature Generator

- Transducer

- House Heating unit and

- Thermostat unit

In this model, the Thermostat unit consists of two subsystems, which are the Indoor Temperature Sensor and Controller.

Troom

Outdoor Temperature Generator

Indoor Temperature Sensor

Controller

Thermostat

Transducer

Report

Tout

Heat Losses

Troom

House Heating

Unit

Troom

Heat Gain

HeaterCntrSignal

Figure 1 – Structure of the House Heating System

Outdoor Temperature Generator - Generates variation of temperatures with respect to the indoor temperature; It emulates outdoor temperatures and is used as an input to the house model.

Its phase state is active at time zero; Generates temperature variation based on the normal distribution with a mean of 22 ºC and deviation of 5 (i.e. different values and probability distribution can be chosen by the user).

Transducer - Based on the outdoor and indoor temperature inputs, and thermal isolation coefficients of a house model, Transducer computes the approximate heating losses of a house.

State variables used in this model are phase, IndoorTemp and  OutdoorTemp. Its phase state is active; it receives the updated Tout values from the Generator and Troom values from the Thermostat, computes heat losses and sends it out via output port HeatLosses.  Approximate values of the thermal resistance of a house and dimensions are considered as constants for a typical house model.

House Heating Unit - After receiving logic high control signal from the thermostat unit, the relay switch is on, which turns the heater on, providing heated air with constant air flow and temperature to the house; respectively the heater is off, after receiving the logic low control signal from the thermostat unit.  State variables are phase, IndoorTemp, Heat  and Switch.

When HeaterCntrSignal received from the Thermostat unit is 1, this means that the Switch = 1 (relay) is turned ON and Heat = 0.5; representing an increase in room temperature of 0.5 ºC.  When HeaterCntrSignal = 0, Switch = 0 (relay) is turned OFF and the generated heat is zero, hence value of Heat sent to the Thermostat is 0 ºC).

Thermostat - Thermostat unit is composed of two subsystems: Indoor temperature sensor and Controller.

Indoor Temperature Sensor - receives input from the Heater and Transducer and computes the room temperature accordingly (taking into account heat being generated and heat being lost).  It also provides a report on temperature changes throughout the simulation (generated each time indoor temperature changes with respect to the previous step).  State variables are phase, Heat Losses and Heat Gain. Initial phase state is active, since it needs to perform input processing starting at time zero, hence is the entire time active.

Controller - reacts when temperature value received from the Indoor Temperature sensor is different than the Set Point temperature (e.g. 25 ºC - 0.1 ºC) by sending the logic low (when Set Point temperature is higher or equal than Troom, high otherwise) or high signal to the heater - indicating the OFF/ON state for the heater.   State variables are phase, IndoorTemp, SetPoint and CntrSignal.  When Controller receives an input from the Indoor Temperature Sensor, its phase becomes active; it generates an output control signal and returns to a passive state (this is done under assumption that the relay switch is a latching type, i.e. it requires only an instant voltage level (low or high) in order to ‘latch’ to an open or closed state).

PART II

As described in the Part I, structure of the House Heating System consists of four components, which are the Outdoor Temperature Generator, Transducer, House Heating unit and Thermostat.  Three of the above components, are DEVS atomic models: Generator, Transducer and Heater while Thermostat is the coupled model (can be further decomposed in two different DEVS atomic models: Temperature Sensor and Controller).

Formal Specifications for the atomic and coupled models

1.  Outdoor Temperature Generator

Generator=<S, X, Y, δint, δext, λ, ta >

Outdoor Temperature Generator

Tout

X = {Ø}

Y = {Tout}

S = {phase, ta, Tout}

Tinit = 22 // initial temperature value set to 22 ºC

δint: δint (active) = active

{Generates the OutdoorTemp (outdoor temperature) with normal distribution, mean: 22 and deviation: 5 for the next time unit interval}

δext: unavailable

λ (active) = Tout (send OutdoorTemp via output port Tout)

ta (active) = preparationTime  // time delay between generated Tout, chosen as 1 sec //intervals between Tout generations

2.	Transducer

Tout

Troom

Transducer =<S, X, Y, δint, δext, λ, ta >

Transducer

X = {Tout, Troom}

Heat Losses

Y = {HeatLosses}

S = {phase, ta, IndoorTemp, OutdoorTemp}

K = 0.1 // initialize heat losses coefficient

// rate of losses multiplier K is assumed to be a constant value of 0.1 representing heat //losses of a house

δint (active) = preparationTime // time delay chosen as 1 sec

δext (active) = active // temperature changes dynamically every second

{

IndoorTemp = Troom;

OutdoorTemp = Tout;

HeatLosses  =  K*|IndoorTemp - OutdoorTemp|  }

ta (active)  = preparationTime //time delay interval, chosen as 1 sec

λ (active) = HeatLosses //sends output data to the Thermostat i.e. HeatLosses

3.  	House Heating Unit

HouseHeater =<S, X, Y, δint, δext, λ, ta >

X = {IndoorTemp, HeaterCntrSignal}

House Heating

Unit

Troom

HeatGain

Y = {HeatGain}

S = {phase, ta, Troom, HeaterCntrSignal, HeatGain}

// initialize variables, switch position to open and heat to zero

Switch = 0;   // zero representing open switch position

HeaterCntrSignal

Heat = 0.0;   // initial heat value is zero

δint (active) = active

δext (Switch, Heat, IndoorTemp, e, x)

{

case phase

active:

IndoorTemp = Troom  // necessary for future implementation of more accurate heat flow

Switch = HeaterCntrSignal

if (Switch==true)

{

Heat = 0.5;  // representing an increase in room temperature of 0.05 ºC

}

else

{

Heat = 0.0;  //  no heat being generated

}

ta = 0 ; //trigger an output immediately

passive:

//never happens;

}

λ (active) = Heat (outputs HeatGain at a rate of 0.5 ºC when switch is active)

ta (active) = preparationTime (time delay interval, chosen as 1 sec)

Indoor Temperature Sensor

Heat Losses

TempSensor =<S, X, Y, δint, δext, λ, ta >

X = {HeatLosses, HeatGain}

Indoor Temperature Sensor

HeatGain

Y = {Troom}

S = {phase, ta, RoomTemp, HeatLoss, HeatG}

Troom

Initialization of variables in init function:

RoomTemp = 0.0;	// room temperature set at 25 ºC initially

HeatLoss = 0.0;	// heat losses

HeatG = 0;  		// heat gain

δext (RoomTemp, HeatG, HeatLoss, e, x)

{

//based on the current temperature readings, heat gain and losses  updates the RoomTemp

RoomTemp = RoomTemp + HeatG - HeatLoss;

}

δint (active) = active

λ (active)  =  Troom 	// report/send out room temperature

ta (active) = preparationTime   // time delay interval, chosen as 1 sec

5.  	Controller

Controller =<S, X, Y, δint, δext, λ, ta >

Controller

Troom

X = {Troom}

Y = {HeaterCntrSignal}

S = {phase, ta, IndoorTemp, SetPoint, CntrSignal}

HeaterCntrSignal

Initialization of variables in init function:

SetPoint = 25.0 // initial set point temperature set to 25 ºC

δext (IndoorTemp, SetPoint, CntrSignal, e, x)

{

case phase

active:

if(IndoorTemp <= (SetPoint-0.1))

{				// if between Troom and SetPoint there is a 0.1 ºC

CntrSignal = 1;  	// difference, activate the heater, control signal = 1

}

else{

CntrSignal = 0;  	// otherwise, turn off the heater, control signal = 0

}

// time interval, coordinated with the TempSensor

ta (active) = preparationTime  	//  time delay interval, chosen as 1 sec

passive:

// never happens

}

δint (phase) = passive

λ (active)  =  HeaterCntrSignal   	//  send out control signal to turn ON/OFF heater

Coupled Model Formal Specification

The formal specification CM = < X, Y, D, {Mi}, {Ii}, {Zij}, select} for the coupled models Thermostat and HouseHeatSystem (House Heating System model) are:

Thermostat

X = { HeatLosses, HeatGain };

Y = { Troom, HeaterCntrSignal };

D = { TempSensor, Controller };

I (TempSensor) = {Controller, self};

I (Controller) =  self;

Z(TempSensor) = Controller;

Z(TempSensor) = self;

Z(Controller) = self;

Select: ({TempSensor, Controller}) = TempSensor;

HouseHeatSystem

X = {Ø};

Y = {Ø};

D = {Generator, Transducer, Thermostat, HouseHeater};

I(Generator) = {Transducer, self};

I(Transducer) = {Thermostat, self};

I(Thermosat) = { HouseHeater, Transducer, self};

I(HouseHeater) = { Thermostat, self};

Z(Generator) = Transducer;

Z(Transducer) = Thermosat;

Z(HouseHeater) = Thermosat;

Z(Thermostat) = HouseHeater;

Z(Thermostat) = Transducer;

Z(Thermosat) = self;

Select: ({Generator, Transducer, Thermostat, HouseHeater}) = Generator;

Test Strategies

Testing of each atomic and coupled model will be accomplished using a “black box” technique.  Each atomic model will be tested individually first; In addition, each coupled model (consisting of two or more atomic models) will be tested, and afterwards, the entire model will be integrated and tested.

Test cases will be performed by utilizing event files (.ev) with different input scenarios and analyzing the output files for each model being tested.

PART III

For each atomic and coupled model presented in this report, test cases are implemented in order to verify the functionality of each atomic/coupled model behavior with respect to their specifications.

Test cases and execution analysis

Generator (atomic model)

Generator has no input and only generates the output temperatures according to normal distribution with mean of 22 and deviation of 5;

Initial Tout (output temperature for the model was set to be 22 ºC).  The model was run for different time durations in order to observe the Tout displayed values.  The output temperature Tout generation results for the 15 seconds interval are shown below:

00:00:00:000 tout 22

00:00:01:000 tout 21.9634

00:00:02:000 tout 18.3883

00:00:03:000 tout 18.8734

00:00:04:000 tout 22.1063

00:00:05:000 tout 19.5477

00:00:06:000 tout 22.482

00:00:07:000 tout 24.5497

00:00:08:000 tout 23.9657

00:00:09:000 tout 23.3386

00:00:10:000 tout 21.4918

00:00:11:000 tout 20.2024

00:00:12:000 tout 24.0613

00:00:13:000 tout 16.1661

00:00:14:000 tout 24.0347

00:00:15:000 tout 24.0396

00:00:16:000 tout 27.2793

00:00:17:000 tout 22.2534

00:00:18:000 tout 20.6003

00:00:19:000 tout 18.1123

00:00:20:000 tout 20.2577

The results indicate that the ranges of output temperatures generated are normally distributed around 22 (mean) and 5 (deviation); hence provide good range of outdoor temperatures for testing the entire model.

Transducer (atomic model)

Based on the outdoor and indoor temperature inputs, and thermal isolation coefficients of a house model, Transducer computes the approximate heating losses of a house (rate of losses coefficient K is assumed to be a constant value of 0.1 representing heat losses of a house).

Transducer computes heat losses within the house if the indoor temperature is greater than outdoor temperature, multiplied by the house heat losses coefficient K = 0.1 (this is just an approximation of a real model). The transduc.ev was used to emulate different scenarios and test the atomic model; it has the events represented in the left column of the Table 1 (below) and right column are the output generated after the execution of the simulations.  File transduc.ev with the simulation stop time set at 5 seconds or the transduc.bat can be used to verify the results presented for the Transducer.

Table 1 – Transducer output results analyzed

transduc.ev				   transducMA.out

CASE 2

| First case: Represents the scenario when Troom > Tout and heatlosses are computed according to the formulation given in Transducer specification correctly (i.e. ¦25 – 20¦*0.1 = 5*0.1 = 0.5. 00:00:00:00 Tout 20 00:00:00:00 Troom 25 00:00:01:00 Tout 15 00:00:01:00 Troom 24 00:00:02:00 Tout 25 00:00:02:00 Troom 25 00:00:03:00 Tout 26 00:00:03:00 Troom 25 00:00:04:00 Tout 21 00:00:04:00 Troom 23 00:00:05:00 Tout 30 00:00:05:00 Troom 25 | 00:00:00:000 heatlosses 0.5 00:00:01:000 heatlosses 0.9 00:00:02:000 heatlosses 0 00:00:03:000 heatlosses 0 00:00:04:000 heatlosses 0.2 00:00:05:000 heatlosses 0 |
| --- | --- |

CASE 1

CASE 3

Second case:	Represent the scenario when Tout = Troom and heatlosses  = 0 (as per the specifications given in Part I and Part II)

Third case:	Represent the scenario when Troom < Tout and heatlosses = 0 (as per the specifications given in Part I and Part II).

HouseHeater (atomic model)

Based on the input signal received from the Thermostat, when the HeaterCntrSignal is 1, this means that the Switch = 1 (relay) is turned ON and Heat = 0.5; representing an increase in room temperature of 0.5 ºC.  When HeaterCntrSignal = 0, Switch = 0 (relay) is turned OFF and the generated heat is zero, hence value of Heat sent to the Thermostat is 0 ºC). While the controller takes care of the temperature check (if >= SetPoint it deactivates the control signal to the heater).

In order to test and emulate different scenarios the event file heater.ev was created and also the heater.bat.  The results shown in heaterMA.out indicate proper functionality of the atomic model as per specifications given in Part I and Part II.

Table 2 – HouseHeater results analyzed

heater.ev				heaterMA.out

| 00:00:00:00 Troom 25 00:00:00:00 HeaterCntrSignal 0 00:00:01:00 Troom 24 00:00:01:00 HeaterCntrSignal 1 00:00:02:00 Troom 20 00:00:02:00 HeaterCntrSignal 1 00:00:03:00 Troom 27 00:00:03:00 HeaterCntrSignal 0 00:00:04:00 Troom 15 00:00:04:00 HeaterCntrSignal 1 00:00:05:00 Troom 25 00:00:05:00 HeaterCntrSignal 0 | 00:00:00:000 heatgain 0 CASE 1 00:00:01:000 heatgain 0.5 CASE 2 00:00:02:000 heatgain 0.5 CASE 3 00:00:03:000 heatgain 0 00:00:04:000 heatgain 0.5 00:00:05:000 heatgain 0 |
| --- | --- |

First case:	Represents a scenario when Troom = 25 and HeaterCntrSignal = 0, and the heatgain = 0 (heater is off when the HeaterCntrSignal is 0, which is the desired action of the model, as per the specifications in the Part I and II)

Second case:	Represents a scenario when Troom = 24 and HeaterCntrSignal = 1 since the Troom is lower than the SetPoint (defined as 25), hence heatgain = 0.5 in this case, confirming the desired action of the model to be correct.

Third case:	Represents a scenario when Troom is higher than SetPoint, hence the

heatgain = 0 (desired action, as per the specifications in the Part I and II).

TempSensor (atomic model)

Tempsensor, as described in specifications of the model, receives input from the Heater and Transducer and computes the room temperature accordingly (taking into account heat being generated and heat being lost).

In order to test and emulate different scenarios, the event file tempsensor.ev and tempsensor.bat were created.  The results indicate proper functionality of the atomic model as per the specifications given in Part I and Part II.  Simulation results after the execution of TempSensor atomic model are presented below.

Table 3 –TempSensor results analyzed

tempsensor.ev				tempsensorMA.out

CASE 1

| 00:00:00:00 HeatLosses 1 00:00:00:00 HeatGain 1 00:00:01:00 HeatLosses 2 00:00:01:00 HeatGain 1 00:00:02:00 HeatLosses 3 00:00:02:00 HeatGain 5 00:00:03:00 HeatLosses 0 00:00:03:00 HeatGain 2 00:00:04:00 HeatLosses 2 00:00:04:00 HeatGain 3 00:00:05:00 HeatLosses 5 00:00:05:00 HeatGain 1 | 00:00:00:000 troom 25 00:00:01:000 troom 24 00:00:02:000 troom 26 00:00:03:000 troom 28 00:00:04:000 troom 29 00:00:05:000 troom 25 |
| --- | --- |

CASE 3

CASE 2

Based on the specification described in Part I and II, the influence of heat gain and heat losses on room temperature is defined as:

RoomTemp = RoomTemp+ HeatGain - HeatLosses;

Where, RoomTemp is represented as troom in the output file.

From the results shown on Table 3, we can observe and verify that RoomTemp computations are correctly represented in the tempsensorMA.out.

First case:  	RoomTemp = 25 + 1 – 1 = 25

Second case:	RoomTemp = 25 +1 – 2 = 24

Third case:	RoomTemp = 29 + 1 – 5 = 25

Controller (atomic model)

The Controller as described in the specifications of the model, reacts when the temperature value received from Indoor Temperature sensor is different than the Set Point temperature (e.g. 25 ºC - 0.1 ºC) by sending the logic low (when Set Point temperature is higher or equal than Troom, high otherwise) or high signal to the heater - indicating the OFF/ON state for the heater.

In order to test and verify the functionality of the Controller, the events file controller.ev and controller.bat were created.  Event file provides different input temperatures to the model and allows us to observe the heatercntrsignal output of the Controller.  The results are shown in the Table 4 below.

Table 4 –Controller results analyzed

CASE 1

controller.ev				controllerMA.out

| 00:00:00:00 Troom 21 00:00:01:00 Troom 25.5 00:00:02:00 Troom 25 00:00:03:00 Troom 24.95 00:00:04:00 Troom 24.9 00:00:05:00 Troom 26 | 00:00:00:000 heatercntrsignal 1 00:00:01:000 heatercntrsignal 0 00:00:02:000 heatercntrsignal 0 00:00:03:000 heatercntrsignal 0 CASE 2 00:00:04:000 heatercntrsignal 1 00:00:05:000 heatercntrsignal 0 CASE 3 |
| --- | --- |

First case: 	Room temperature is 21 ºC, which is below the (SetPoint – 0.1) = (25 – 0.1) = 24.9 ºC, therefore heater control signal of the Controller (heatercntrsignal) is 1, which is a logic high signal needed to turn ON the heater.

Second case:	Room temperature is 24.9 which is equal to the SetPoint -0.1 = 24.9.  Therefore the heater control signal is still 1.

Third case: 	Room temperature is above 24.9 ºC, therefore the heater control signal (heatercntrsignal) is 0, which is a logic low signal needed to turn OFF the heater.

Thermostat (coupled model)

Thermostat receives as input heat losses and heat gain from the Transducer and Heater unit respectively; it has two outputs which are the Troom (room temperature) and HeaterCntrSignal (heater control signal – to turn ON/OFF the heater).

Thermostat is a coupled model, which consists of two atomic models (TempSensor and Controller, which were proven to be functional individually).  However, it is important to verify that the same behavior of the model can be observed when the atomic models are integrated.  The test strategy is similar to the atomic model testing; Thermostat.ev (event file) with different input scenarios and Thermostat.bat were created in order to facilitate the verification and testing of the model.

Table 5 – Thermostat results analyzed

Thermostat.ev				ThermostatMA.out

| 00:00:00:00 HeatLosses 1 00:00:00:00 HeatGain 1 00:00:01:00 HeatLosses 2 00:00:01:00 HeatGain 1 00:00:02:00 HeatLosses 3 00:00:02:00 HeatGain 5 00:00:03:00 HeatLosses 0 00:00:03:00 HeatGain 2 00:00:04:00 HeatLosses 2 00:00:04:00 HeatGain 3 00:00:05:00 HeatLosses 5 00:00:05:00 HeatGain 1 | 00:00:00:000 troom 25 CASE 1 00:00:00:000 heatercntrsignal 0 00:00:01:000 troom 24 CASE 2 00:00:01:000 heatercntrsignal 1 00:00:02:000 troom 26 00:00:02:000 heatercntrsignal 0 00:00:03:000 troom 28 00:00:03:000 heatercntrsignal 0 00:00:04:000 troom 29 00:00:04:000 heatercntrsignal 0 00:00:05:000 troom 25 00:00:05:000 heatercntrsignal 0 |
| --- | --- |

First case:	Since heat losses and gain are equal, they cancel each other and room temperature remains 25 ºC, hence the heater control signal is 0 (i.e. turn OFF the heater).

Second case:	In this scenario, losses – gain = 1, hence room temperature falls below the SetPoint threshold, hence heater control signal is 1(i.e. turn ON the heater)

HouseHeat System (coupled model)

HouseHeatSystem is DEVS coupled model of a house heating system; it consists of three atomic models (Generator, Transducer and Heater) and one coupled model (which is further decomposable into two different atomic models: TempSensor and Controller).

After the integration of the atomic models and Thermostat coupled model into a HouseHeatSystem coupled model, overall functionality of the model was by creating a HouseHeatSysReportMA.ma file in addition to the original HouseHeatSysMa.ma.  House Heating System model is a self-contained model and has no outputs.  However, in order to see main outputs of the model (in addition to being able to observe them in the .log file) HouseHeatSysMA.ma file was modified into HouseHeatSysReportMA.ma and several outputs were added to the file (Tout, HeatLosses, Troom, HeaterCntrSignal and HeatGain) in order to be able to observe the functionality of the entire model in the HouseHeatSysReportMA.out file.

Reports of the outputs from the HouseHeatSysReportMA.out are:

00:00:00:000 tout 22

CASE 1

00:00:00:000 heatlosses 0

00:00:00:000 troom 25

00:00:00:000 heatercntrsignal 0

00:00:00:000 heatgain 0

00:00:01:000 tout 21.9634

CASE 2

00:00:01:000 heatlosses 0.303661

00:00:01:000 troom 24.6963

00:00:01:000 heatercntrsignal 1

00:00:01:000 heatgain 0.5

00:00:02:000 tout 18.3883

00:00:02:000 heatlosses 0.630806

00:00:02:000 troom 24.5655

00:00:02:000 heatercntrsignal 1

00:00:02:000 heatgain 0.5

00:00:03:000 tout 18.8734

00:00:03:000 heatlosses 0.569211

00:00:03:000 troom 24.4963

00:00:03:000 heatercntrsignal 1

00:00:03:000 heatgain 0.5

00:00:04:000 tout 22.1063

00:00:04:000 heatlosses 0.239004

00:00:04:000 troom 24.7573

00:00:04:000 heatercntrsignal 1

00:00:04:000 heatgain 0.5

00:00:05:000 tout 19.5477

00:00:05:000 heatlosses 0.520965

00:00:05:000 troom 24.7364

00:00:05:000 heatercntrsignal 1

00:00:05:000 heatgain 0.5

First case:	Results indicate that at time zero initial room temperature is 25 hence no change/action is required (refer to page 15 Case 1).

Second case:	Results indicate that outdoor temperature (tout) is lower than the room temperature (troom) hence the heater control signal is 1 and also the heatgain is 0.5 while heat losses are (25-21.9634)*0.1 = 0.30661, which is what we expected (refer to page 15, Case 2).

Conclusions:

Test results for the atomic and coupled models, including the final results, indicate that model is working as expected.  The corresponding outputs being generated (for different possible scenarios), show proper functionality of the entire simulation model as defined in the model specifications in Part I and II.