SYSC5104 Simulation and Modelling

Faezeh Rafsanjani Sadeghi 100612639

Chih-Wei Wang, 100335923

Part I

System to be designed: Adaptive hospital digital temperature/humidity controller

Every hospital needs a warming and cooling system. This system is used to heat the hospital when the weather is cold and cool the hospital when the weather is hot. We are trying to model and simulate a hospital which has an intelligent cooling and warming system. According to the season and time of day as well as number of patients coming and going out of hospital, it will automatically set the temperature/humidity. If the user is not comfortable with the automated temperature/humidity they can use their own setting. The user has the option of setting the humidity to turn on or off the humidifier as well.As seen in the diagram the hospital temperature controller consists of three coupled models, one is TC which is the temperature controller, SC which is a small controller for comparing temperature and determining if the conditioner/heater or humidifier/dehumidifier should be turned on or off.

CompareTemp

CompareHumidity

Memory

ProcessorTwo

SC

TC

tSensor1

hSensor1

tempIn1

humidIn1

humidOut

tempOut

In2

In3

In4

tempDiff

humidDiff

tempDiff

humidDiff

SPort1

TPort2

TPort1

SPort2

tempSensor

tempSetting

tempIn

humidIn

humiditySensor

humiditySetting

SeasonAndTimeGen

PatientInfo

ProcessorOne

TC

seasonPort

timePort

seasonIn

timeIn

infoOut

peopleIn

tempOut

humidOut

humidIn

tempIn

TPort1

TPort2

Hospital Temperature Controller

- Atomic model description:

Memory:

The memory takes in some temperature and humidity that is desired. The inputs come from the processing unit. This temperature and humidity is recorded onto memory for some later use. The output of this function is temperature, which is sent to the comparators.

CompareTemp:

CompareTemp model compares the temperature that was generated by processorOne and temperature from sensor and returns the difference.

CompareHumidity:

CompareHumidity model compares the humidity that was generated by processorOne and temperature from sensor and returns the difference.

ProcessorTwo:

The processor takes in the differences between the temperatures and turns on or off the heater, air conditioner or humidifier. This temperature and depend on humidity, if the humidity is low and temperature is low the heater and the humidifier are turned on, if the humidity is too high and the temperature is too high the air conditioner is turned on. And if humidity is low and temperature is high, the air conditioning is turned on.

ProcessorOne:

The processor takes in number of attributes from the environment. Those attributes (factors) include seasons, time of a day and the number of people in the hospital. The processor will calculate average desired value of temperature and humility level based on these information and inform the appropriate change of temperature and humility to memory setting.

SeasonAndTimeGenerator:

The generator is responsible for generating some random temperature and humidity which will depend on the season and time of day. There is no input to the system, it is either generating or not generating. The output is some temperature and humidity which is generated by the internal transition function. The result is sent to ProcessorOne.

PatientInfo:

This model generates a random number of people entering the hosp

Part II:

Formal Specifications

The formal specifications <S, X, Y, δint, δext, λ, ta>  for all the atomic models are as follows:

Formal Specifications

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic models are defined as follows:

Memory:

State variables:

Temp=0;

Humid=0;

Sigma=infinity, state = passive

Formal Specification:

S = {passive, active}

X = {tempIn humidIn}

Y = {tempOut humidOut}

δint (active) = passive

δext (tempIn, passive) = active

δext (humidIn, active) = active//please refer to external transition function from //Memory.cpp for this part

λ(active)

{	send tempIn via tempOut		//extract the alternating bit and send back

}

ta(passive) = INFINITY

ta(active) = preparation time

CompareTemp:

State variables:

TempDiff=0 ;

TempQue[2] ={0,0} //the temperature from sensor stored at TempQue[0] and from setting to //TempQue[1]

Formal Specification:

S = {passive, active}

X = {tempSensor tempSetting}

Y = {tempDiff }

δint (active) = passive

δext (tempSensor, passive) = active

δext (tempSetting, active) = active//please refer to external transition function from //CompareTemp.cpp for this part

λ(active)

{	send TempDiff via tempDiff		//extract the alternating bit and send back

}

ta(passive) = INFINITY

ta(active) = preparation time

CompareHumidity:

State variables:

HumidDiff=0 ;

HumidQue[2] ={0,0} //the temperature from sensor stored at HumidQue[0] and from setting to //HumidQue[1]

Formal Specification:

S = {passive, active}

X = {HumiditySensor HumiditySetting}

Y = {humidDiff }

δint (active) = passive

δext (HumiditySensor, passive) = active

δext (HumiditySetting, active) = active//please refer to external transition function from //CompareHumidity.cpp for this part

λ(active)

{	send HumidDiff via humidDiff		//extract the alternating bit and send back

}

ta(passive) = INFINITY

ta(active) = preparation time

ProcessorTwo:

State variables:

onOFF[2]={2,2} ;

//	   if onOFF[0] = 0;  //heater on/conditioner off

//	   if onOFF[0] = 1;  //conditioner on/heater off

//	   if onOFF[0] = 2;  //conditioner off/heater off

//	   if onOFF[1] = 0; //humidifier off

//             if onOFF[1] = 1;  //humidifier on

Formal Specification:

S = {passive, active}

X = {tempDiff1 humidDiff1}

Y = {port1 port2} //port 1 for heater/conditioner; port2 for humidifier

δint (active) = passive

δext (port1, passive) = active

δext (port2, passive) = active//please refer to external transition function from //ProcessorTwo.cpp for this part

λ(active)

{	send onOFF[0] via port1

send onOFF[1] via port2

}

ta(passive) = INFINITY

ta(active) = preparation time

SeasonAndTimeGen:

State variables:

season;        //0:spring, 1:summer, 2:fall, 3:winter

timeOfDay;		  //Only hours of a day is meaningful to this model

count;

Formal Specification:

S = {passive, active}

X = { }

Y = {seasonPort timePort}

δint (active) = passive

δext (seasonPort, passive) = active

δext (timePort, passive) = active//please refer to external transition function from //SeasonAndTime.cpp for this part

λ(active)

{	send season via seasonPort

send timeofDay via timePort

}

ta(passive) = INFINITY

ta(active) = preparation time

PatientInfo:

State variables:

Count=0;

Formal Specification:

S = {passive, active}

X = { }

Y = {infoOut}

δint (passive) = generate random number passive//please refer to internal transition function from //PatientInfo.cpp for this part

δext ();

λ(active)

{	send count via infoOut

}

ta(passive) = INFINITY

ta(active) = preparation time

ProcessorOne:

State variables:

numPatient = 10;  //assume 10 people in hospital initially

time = 9;	//assume 9am in the morning

season = 0;	//assume in Spring

calcTemp = 0;

calcHumid = 0;

Formal Specification:

S = {passive, active}

X = { timeIn seasonIn peopleIn} //inputs from SeasonAndTimeGen and PatientInfo

Y = {tempOut humidout}

δint (active) = passive

δext (timeIn, passive) = active //please refer to external transition function from //ProcessorOne.cpp for this part

δext (seasonIn passive) = active

δext (peopleIn passive) = active

λ(active)

{	send calcTemp via tempOut

send calHumid  via humidOut

}

ta(passive) = INFINITY

ta(active) = preparation time

The formal specifications <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT > for the coupled model SC and TC and hospitalTC are defined as follows:

SC:

X = {in1 in2 in3 in4};

Y = {SPort1 SPort2};

D = {CompareTemp, CompareHumidity, ProssorTwo};

I(ProcessorTwo) = {CompareTemp, CompareHumidity};

I(CompareTemp) = self;

I(CompareHumidity)=self;

Z(ProcessorTwo) = self;

Z(CompareTemp) = ProcessorTwo;

Z(CompareHumidity)=ProcessorTwo

SELECT:

TC:

X = {tSensor1 tempIn1 humidIn1 hSensor1};

Y = {TPort1 TPort2};

D = {Memory, SC};

I(Memory) = {SC};

I(Network) = {Memory};

Z(Memory) = self

Z(SC) = Memory

SELECT:

HospitalTC:

X = {};

Y = {TPort1 TPort2};

D = {SeasonAndTimeGen PatientInfo ProcessorOne TC};

I(SeasonAndTimeGen) = {};

I(PatientInfo) = { };

I(ProcessorOne) = SeasonAndTimeGen PatientInfo

Z(ProcessorOne) = TC

Z(PatientInfo)={ProcessorOne}

Z(SeasonAndTimeGen)={ProcessorOne}

Z(TC)=self

Test strategies:

Each of the atomic models were tested individually and then the coupled models were tested.

Part III

Memory

Memory should be a storing unit for temperature and humidity, to test this unit we generate some inputs and see the output result.

00:00:00:00 tempIn1 25

00:00:30:00 humidIn1 20

00:00:45:00 tempIn1 30

00:00:80:00 humidIn1 40

00:07:00:00 humidIn1 71

00:08:60:00 tempIn1 60

00:09:60:00 humidIn1 34

As can be seen there is a temperature input at time zero and the result should be shown on the output 5 seconds later. The humidity result is also generated but it is 0 because there was no input at that port. So following the output seems to be working correctly.

Output result

00:00:05:000 tempout1 25

00:00:05:000 humidout1 0

00:00:35:000 tempout1 25

00:00:35:000 humidout1 20

00:00:50:000 tempout1 30

00:00:50:000 humidout1 20

00:01:25:000 tempout1 30

00:01:25:000 humidout1 40

00:07:05:000 tempout1 30

00:07:05:000 humidout1 71

00:09:05:000 tempout1 60

00:09:05:000 humidout1 71

00:10:05:000 tempout1 60

00:10:05:000 humidout1 34

CompareTemp

This should return the difference between temperature showing on the sensor and temperature on the setting(here generated). We can see this in the output. We send two values through in1 and in2 ports to test our model:

00:00:10:00 in1 25

00:00:50:00 in2 20

00:01:00:00 in1 38

00:01:20:00 in2 40

00:01:50:00 in1 45

00:02:30:00 in2 45

The first result should be 25-20 which is 5 and so on, the result can be seen here:

00:00:55:000 tempdiff1 5

00:01:05:000 tempdiff1 18

00:01:25:000 tempdiff1 -2

00:01:55:000 tempdiff1 5

00:02:35:000 tempdiff1 0

CompareHumidity

This should return the difference between humidity showing on the sensor and humidity on the setting(here generated). We can see this in the output. We send two values through in1 and in2 ports to test our model:

00:00:10:00 in1 25

00:00:50:00 in2 20

00:01:00:00 in1 38

00:01:20:00 in2 40

00:01:50:00 in1 45

00:02:30:00 in2 45

The first result should be 25-20 which is 5 and so on, the result can be seen here:

00:00:55:000 humiddiff1 5

00:01:05:000 humiddiff1 18

00:01:25:000 humiddiff1 -2

00:01:55:000 humiddiff1 5

00:02:35:000 humiddiff1 0

ProcessorTwo

ProcessorTwo takes in the values from compareHumidity (which is the difference between sensor and setting) and compareTemperature, and uses these values to determine if the conditioner should be on or heater should be on or both should be off.

(Recall port1 is for conditioner or heater, port2 is for humidifier, if port 1 is 1 it means the conditioner is on if it is 0 it means the heater is on and if it is 2 it means non of them are on. If port2 is 1 it means humidifier is off and if it is 0 it means its off)

For testing the result we input 2 at port1 which is a positive number and it indicates that the conditioner should be turned on and the value should be 1, and since port 2 is showing a negative number it means the humidifier should be turned on

00:00:10:00 inp1  2

00:00:30:00 inp2 -3

00:00:50:00 inp1  -2

00:01:00:00 inp2 3

00:01:30:00 inp1  1

00:02:00:00 inp2 1

Results:

00:00:15:000 port1 1

00:00:15:000 port2 2

00:00:35:000 port1 1

00:00:35:000 port2 0

00:00:55:000 port1 0

00:00:55:000 port2 0

00:01:05:000 port1 0

00:01:05:000 port2 1

00:01:35:000 port1 1

00:01:35:000 port2 1

00:02:05:000 port1 1

00:02:05:000 port2 1

To simplify the following was generated on the console

conditioner on

humidifier off

heater on

humidifier on

conditioner on

humidifier on

if difference is possitve we should turn conditioner on and if negative we should turn heater on. If the result is negative the humidifier should be turned off and it is positive it should be turned on.

SeasonAndTimeGen

This is responsible for generating random season and time of day, the time of day is only in hours and goes from 0 to 23 and season is 0, 1, 2, or 3. The outputs were generated as follows:

00:00:00:000 seasonport1 0

00:00:00:000 timeport1 9

00:00:03:000 seasonport1 0

00:00:03:000 timeport1 10

00:00:06:000 seasonport1 0

00:00:06:000 timeport1 11

00:00:09:000 seasonport1 0

00:00:09:000 timeport1 12

00:00:12:000 seasonport1 0

00:00:12:000 timeport1 13

00:00:15:000 seasonport1 0

00:00:15:000 timeport1 14

00:00:18:000 seasonport1 0

00:00:18:000 timeport1 15

00:00:21:000 seasonport1 0

00:00:21:000 timeport1 16

00:00:24:000 seasonport1 0

00:00:24:000 timeport1 17

00:00:27:000 seasonport1 0

00:00:27:000 timeport1 18

00:00:30:000 seasonport1 1

00:00:30:000 timeport1 19

ProcessorOne

Processor one is responsible for taking in  the input from SeasonAndTimeGen and PatientInfo and output a temperature and humidity for TC.

Here we give some random inputs to ProcessorOne and it generates temperature and humidity according to rules.

Input:

00:00:10:00 seasonIn1  2

00:01:00:00 timeIn1	 8

00:02:00:00 peopleIn1	20

00:05:30:00 timeIn1	 17

00:07:60:00 peopleIn1	150

Output:

00:00:15:000 tempout1 25

00:00:15:000 humidout1 50

00:01:05:000 tempout1 20

00:01:05:000 humidout1 51

00:02:05:000 tempout1 19

00:02:05:000 humidout1 53

00:05:35:000 tempout1 21

00:05:35:000 humidout1 52

00:08:05:000 tempout1 19

00:08:05:000 humidout1 57

PatientInfo

This model generates a number of people who are in the hospital:

00:00:00:000 infoout1 200

00:00:05:000 infoout1 194

00:00:10:000 infoout1 194

00:00:15:000 infoout1 203

00:00:20:000 infoout1 201

00:00:25:000 infoout1 205

00:00:30:000 infoout1 201

00:00:35:000 infoout1 189

00:00:40:000 infoout1 188

00:00:45:000 infoout1 170

00:00:50:000 infoout1 170

....

00:49:50:000 infoout1 154

00:49:55:000 infoout1 137

00:50:00:000 infoout1 122

00:50:05:000 infoout1 107

00:50:10:000 infoout1 91

00:50:15:000 infoout1 73

00:50:20:000 infoout1 82

00:50:25:000 infoout1 85

00:50:30:000 infoout1 71

00:50:35:000 infoout1 73

00:50:40:000 infoout1 90

00:50:45:000 infoout1 107

00:50:50:000 infoout1 101

00:50:55:000 infoout1 115

00:51:00:000 infoout1 115

00:51:05:000 infoout1 102

00:51:10:000 infoout1 86

00:51:15:000 infoout1 81

00:51:20:000 infoout1 65

00:51:25:000 infoout1 52

00:51:30:000 infoout1 51

00:51:35:000 infoout1 69

00:51:40:000 infoout1 51

00:51:45:000 infoout1 51

00:51:50:000 infoout1 70

00:51:55:000 infoout1 85

00:52:00:000 infoout1 98

00:52:05:000 infoout1 106

00:52:10:000 infoout1 100

00:52:15:000 infoout1 104

00:52:20:000 infoout1 114

00:52:25:000 infoout1 103

00:52:30:000 infoout1 103

SC:

This model is a coupled model which has 4 inputs two from sensor and two from memory and it determines if the conditioner/heater should be on or off and if the humidifier should be on of off.

00:00:10:00 in1  23

00:00:50:00 in2  25

00:01:00:00 in3  55

00:02:50:00 in4  25

00:04:10:00 in1  59

00:07:50:00 in2  29

00:09:00:00 in3  13

00:12:50:00 in4  33

we can see that the results are satisfactory the difference between in1 and in2 is negative therefore the heater should be on and so on.

00:01:03:000 sport1 0

00:01:03:000 sport2 2

00:03:06:000 sport1 0

00:03:06:000 sport2 0

00:04:23:000 sport1 1

00:04:23:000 sport2 0

00:08:03:000 sport1 1

00:08:03:000 sport2 0

00:09:16:000 sport1 1

00:09:16:000 sport2 1

00:13:06:000 sport1 1

00:13:06:000 sport2

Console output:

heater on

humidifier off

conditioner on

conditioner on

humidifier on

humidifier on

TC:

This model consists of Memory, CompareTemp, CompareHumidity and ProcessorTwo. The inputs are sent to memory which is sent to SC and compared and generate outputs which shows if the conditioner/heater is on or off, humidifier is on or off. The event file is shown below:

00:00:10:00 tSensor  23

00:00:50:00 tempIn1  25

00:01:00:00 humidIn1 57

00:02:00:00 hSensor 55

The output is as follows:

(the heater should be turned on and humidifier should be turned off)

temperature is 25

heater on

humidity is 57

heater on

humidifier off

We see the correct result here.

PT:

The following is a test of the model ProcessorOne and TC:

00:00:10:00 season  3

00:00:50:00 time  17

00:01:10:00 people  101

00:02:50:00 tSensor2  30

00:04:10:00 hSensor2  59

The result are as follows:

00:02:56:000 condheat 1

00:02:56:000 humidd 2

00:04:16:000 condheat 1

00:04:16:000 humidd 1

The humidifier and heater/conditioner are turned on/off accordingly.