Building Electrical Circuits Using Bond Graphs

&

Designing an Electrical Circuit Bond Graph Modeller Java GUI

Ardeshir Golshani (100631620), Ismaeel Al Ridhawi (3385278)

Dept. of Systems and Computer, Engineering. Carleton University

1125 Colonel By Drive. Ottawa, ON, K1S 5B6, CANADA

INTRODUCTION

A bond graph is a graphical notation utilized to model physical systems in a clear and concise manner, independent of their domain.  Possible domains for modelling bond graphs are electrical, mechanical, and chemical.  This modelling of systems allows for a better understanding of their functionality.  Specifically, bond graphs are used to model the energy transformations between the subsystems, or elements, within a system.

Modelica is an object-oriented multi-domain modelling language, which is used for modelling complex systems such as mechanical, electrical, and power.

In this report, our purpose is to model systems represented in bond graph notation using CD++.  Specifically, we are dealing with systems of electrical circuits.  We begin with an electrical circuit, and then we will show its equivalent Modelica description language.  Later, we will translate the circuit into an equivalent bond graph, and attempt to simulate it in the CD++ Modeler tool.

Atomic models in CD++ Modeler are illustrated as blue bubbles.  What we have done in this project is to create icon images that represent specific atomic models instead of the generic blue bubble.  An example is to use the resistor symbol to represent a resistor .cpp file.  Furthermore, we have started the building blocks of a GUI, similar to that of CD++ Modeler, in which the setup is there, however, further development is needed.

BOND GRAPHS

A bond graph (or bondgraph) is an explicit graphical tool that is used to capture the energy structure and flow of systems, or in other words, to model a physical system.  It is an abstract depiction of a system in which a collection of components interact with each other via energy ports.  This representation expresses how power flows inside the system.  This is achieved by simply illustrating subsystems linked to one another by lines in order to show the connections of the dynamic system having a variety of energy types.  This helps to give more insight into a system’s behaviour and allows for giving a concise, clear, and broad description of complex systems.  Furthermore, the notation of bond graphs allow for formulation of system equations as well as intuition-based discussions of system behaviour [1].  Simulation studies show bond graphs to be superior to other techniques of simulation with respect to robustness.

HISTORY OF BOND GRAPHS

Bond graphs were introduced by the late Henry M. Paynter (1923-2002), a professor at UT Austin and MIT [2].  It took Paynter about a decade to put together the underlying concepts of a conceptual framework and corresponding notation.  In the 1960’s, his students further elaborated the notation; Ronald C. Rosenberg, a professor at Michigan State University, also designed the first computer tool, ENPORT, which supported simulation of bond graph models.  In the 1970’s, professors Jan J. Van Dixhoorn and Jean U. Thoma introduced bond graphs in Europe.  In recent years, bond graphs have become a topic of research or at least employed in research at many universities worldwide.  They are part of the engineering curricula of a steadily growing number of universities.  In the past decade, the use of bond graphs in industry has become much more important.

BENEFIT OF BOND GRAPHS

An advantage of using the bond graph technique is that not only is the representation brief and clear, but the technique can be used to model transformations of energy across many applications.  Some examples include electrical, mechanical, hydraulic, pneumatic, chemical, magnetic, and thermodynamic.  Refer to [3] for an application of bond graphs in hydraulic pipes.  Refer to [4] for an application in mechatronics, and [5] for an application in the human cardiovascular system.

CONCEPT OF BOND GRAPHS

- Bond, effort, and flow

The flow of energy between any two elements (and thus a bond) is characterized by two variables, the product of which is power.  According to the bond graph notation, these variables are called effort (e) and flow (f).

Figure 1: Effort and flow between elements

The two variables, effort and flow, make up a combination that is typical for a physical domain.  The product of effort and flow is always power.  Such pairs of variables are referred to as power conjugated variables.  For example, voltage and current are used for electrical networks and force and velocity are used for mechanical systems.  In the above figure, the connection between two elements, referring to the line in between the elements, is named the power bond or bond.  This bond indicates an ideal energy flow between the two elements.  The energy entering the bond from the element on one side immediately leaves the bond and enters the element at the other side.  This is known as power continuity.  The point at which the bond interacts with each element is called a power port or port for short.

To understand the connection established by a bond, the bond can be interpreted in one of the two following ways:

1. As the interaction of energy.  The connected elements/subsystems form a load to each other by their exchange of energy.  A power bond embodies a connection where a physical quantity is exchanged.

2. As a bilateral signal flow.  The connection is interpreted as two signals, an effort and flow, flowing in opposite directions, thus

determining the computational direction of the bond variables.  With regards to one element, the effort is input and the flow is output, while in the other element, this is backwards, having the effort as output and the flow as input.

These variables have different interpretations in different physical domains.  Yet, power can always be used as a generalized coordinate to model coupled systems residing in several energy domains.  The following figure displays effort and flow variables in a number of domains, including electrical and chemical.

| Systems | Effort (e) | Flow (f) |
| --- | --- | --- |
| Mechanical | Force (F) | Velocity (v) |
|  | Torque (t) | Angular velocity (w) |
| Electrical | Voltage (V) | Current (i) |
| Hydraulic | Pressure (P) | Volume flow rate (dQ/dt) |
| Thermal | Temperature (T) | Entropy change rate (ds/dt) |
|  | Pressure (P) | Volume change rate (dV/dt) |
| Chemical | Chemical potential (m) | Mole flow rate (dN/dt) |
|  | Enthalpy (h) | Mass flow rate (dm/dt) |
| Magnetic | Magneto-motive force (em) | Magnetic flux (f) |

Figure 2: Effort and flow in various domains

- Causality

Modelling is started by indicating the physical structure of the system. The bonds are first interpreted as interactions of energy, and then the bonds are endowed with the computational direction, interpreting the bonds as bilateral signal flows.  Causal stroke is a line that is added to one end of a bond, indicating the direction of the effort, and is called the causality of the bond.  This is depicted in the next figure.

Figure 3: Example showing causality

- Basic 1-port elements

A single-port element is interacted with through a single power port.  At that port, a single pair of effort and flow variables exists.  Ports are classified as passive ports and active ports.  Passive ports belong to elements that do not produce any power.  Examples of passive elements are the inductor, capacitor, and resistor.

The resistor is an element in which the effort and flow variables at the port are related by a static function.  Resistors dissipate energy; this must be true for simple electrical resistors.  The bond graph symbol for the resistor is shown below.

Figure 4: Resistor in bond graph

The half arrow pointing towards R means that the power (e*f) is positive and flowing into R.  The constitutive relationship between e, f, and R is given by the following equations:

e = R * f

Power = e * f = R * f2

A single-port element that has a static constitutive relation between an effort and a displacement stores and gives up energy without loss.  This is the definition of a capacitor in bond graph terminology.  A capacitor is an idealization of devices like springs, electrical capacitors, and gravity tanks.  The bond graph symbol for a capacitor is shown below.

Figure 5: Capacitor in bond graph

In a capacitor, the charge accumulated on the plates (Q) or voltage (e) is given by the equations below.  Here, the current is the cause and the total charge (and hence voltage) is the consequence.

Q = ò-t¥  i dt

e = C-1 ò-t¥  i dt

A second energy storing single-port element arises if momentum (P) is related by a static constitutive law to the flow (f).  This element is referred to as an inertial element in bond graph terms, and is used to model inductance electrical circuits and mass or inertia in mechanical and fluid systems.  The bond graph symbol for an inductor is depicted in the next figure.

Figure 6: Inductor in bond graph

The resulting equation of examining the mechanics of mass point by considering the impulse-momentum equations is given below.  Here, effort is the cause and velocity (and hence momentum) is the consequence.  The current in an inductor is also given below.

P = ò-t¥ edt

f = m-1ò-t¥ edt

i = L -1 ò-t¥ e dt

- Sources

In comparison with passive ports, active ports exist in elements that give reaction to the source; sources are called active ports.  Force is considered to be an effort source and the surface of a rigid body gives a velocity source.  They are represented as a half arrow pointing away from the source symbol.  The effort source and flow source are represented below by figures 7 and 8, respectively.

Figure 7: Source effort in bond graph

Figure 8: Source flow in blond graph

Modeling Electrical Circuits

Modelica [6] is an object-oriented, logic, multi-domain modeling language, used for modeling complex systems such as mechanical, electrical, hydraulic, thermal, electronic, control, chemical, electric power, thermodynamics, and much more. The main application area of medelica is the modeling of physical systems. It is used in a wide varaiety of applications such as, fluid systems, automotive applications, mechanical systems, and much more.

Modelica differs from object-oriented programming languages like Java or C++ by two aspects. First, it is a modeling language rather than a programming language, where its classes are translated into objects which then is simulated by the simulation engine. Second, the primary content of Modelica classes are sets of equations, where these equations describe equalities rather than assignments. The simulation engine will manipulate the equations symbolically to determine their order of execution; and which components in the equation are inputs and which are outputs.

One building block of the Modelica library is the electrical analog part. It contains important but simple models of electrical and electronic devices including very simple semiconductor device models. The models can be composed to any electrical circuit for which the model simplicity is sufficient enough. The electrical analog components are capable of interacting with the components of the other sub-libraries of Modelica. [7]

The electrical analog library in Modelica is a collection of electrical components that follow the components of the SPICE [8] simulator. The library is divided into five components:

- time dependent sources (ramp, sine, exponentials, trapezoid et. al. for voltages and currents)

• basic models (resistor, capacitor, inductor, transformer, linear controled sources, etc…)

• semiconductor devices (diode, bipolar transistors, metal-oxide semiconductor FETs)

• line models

• ideal elements (switch, diode, opamp, transformer, etc…)

We start with an electrical circuit, then we will show its Modelica description language, but later we will use these circuits to describe their bond graphs, and the simulation in the CD++ Modeler.

Figure 9 shows an electrical circuit made of a voltage source, two resistors, capacitor, and an inductor.

Figure 9:  Electrical Circuit

The following table describes the circuit components briefly:

| Component | Type | Parameters |  |
| --- | --- | --- | --- |
| V | Voltage Pulse Source | Amplitude of pulse | 10 V |
|  |  | Width of pulse | 50% |
|  |  | Time for one period | 2.5 sec |
|  |  | Voltage offset | 0 V |
|  |  | Time offset | 0 sec |
| R1 | Resistor | Resistance | 5 Ohms |
| C | Capacitor | Capacitance | 150 F |
| I | Inductor | Inductance | 30 H |
| R2 | Resistor | Resistance | 2.5 Ohms |

The following Modelica text shows the circuit description without graphical instructions. The instantiation of library components can be seen as well as the usage of parameters. Once instantiated the circuit elements are connected in the equation part.

model Circuit1

ConstantVoltage V(V=10);

Resistor R(R=5);

Resistor R2(R=2.5);

Capacitor C(C=150);

equation

connect(V.p, R1.p);

connect(R1.n, L.p);

connect(C.p, L.p);

connect(L.n, R2.p);

end  Circuit1;

Each component in the above Modelica description language refers to another specification. For example the Modelica Specification of the capacitor is:

model Capacitor "Ideal linear electrical capacitor"

extends Interfaces.OnePort;

parameter SI.Capacitance C=1 "Capacitance";

equation

i = C*der(v);

end Capacitor;

The Modelica Specification of the resistor is:

model NonlinearResistor " resistor"

extends Interfaces.OnePort;

parameter SI.Conductance Ga;

parameter SI.Conductance Gb;

parameter SI.Voltage Ve;

equation

i = if (v < -Ve) then Gb*(v + Ve) - Ga*Ve else if (v > Ve) then

Gb*(v - Ve) + Ga*Ve else Ga*v;

end NonlinearResistor;

Converting Electric Circuits to bond Graphs

Before using the CD++ modeler to model bond graphs, we have to know how to convert and electric circuit drawn as we have learned over the years into a bond graph.

There are four steps involved in the construction of a bond graph from an electric circuit. Each one will be discussed in details in what follows:

Step 1:  for each electrical element such as a resistor, conductor, inductor, etc… a 0-junction should be created. Where a parallel connection in an electrical circuit, and the sum of all the flows, considering the power direction, should equal zero. The zero junction is all considered a parallel junction.

Step 2:  is to insert 1-junction between the appropriate pair of 0-junctions. This will merge the nodes linked by the edges describing logical relations. This junction represents a serial junction, where all flows of the connecting bonds are equal. All effort sums should equal to zero.

Step 3:  assign power direction to all elements, where the direction of the power will specify power flow which is assumed to be positive.

Step 4:  if the circuit has explicit ground potential, delete the 0-junction and its connected elements. If there is no ground potential in the circuit, choose any 0-junction and delete it. The 0-junction that are selected should be only those that are associated with the negative pin of every source’s port.

By applying these rules the bond graph of the electrical circuit visualized in figure 1 would be the following, where a 1-junction is applied between the source voltage and resistor R1, a 0-junction is applied between resistor R1 and the capacitor, a 1-junction between the capacitor and the inductor, and finally a 0-junction between the inductor and resistor R2. The result (i.e. bond graph) is shown in fig. 10.

Figure 10:  bond graph of an electrical circuit

Modeling Bond Graphs using CD++ Modeler

Using the CD++ Modeler Graphical User Interface (GUI), atomic and coupled models can be created for the CD++ toolkit. The basic functions of the GUI include: creating atomic and coupled models, exporting the models to different formats, and animating the simulations.  The GUI also includes: - a simple text editor to directly modify Cell-DEVS models, - the RUN/SIMU commands to execute the simulation with the CD++ toolkit, and - the DRAWLOG command to generate the draw information from the simulation. This section describes in detail how to use the GUI for inputting DEVS model, and for visualizing the results of the Cell-DEVS, Atomic-DEVS, and Coupled-DEVS models. [9]

Therefore bond graphs are modeled according to conjunction bond graph shown in figure 10. MCD++ is an extension to the N-CD++ toolkit, allowing the modeling and simulation of simple electrical circuits under the discrete event paradigm.

The circuits are developed using the Modelica specification language. The simulation environment accomplishes model simulation based on the QSS theory, providing the extensions needed to the N-CD++ toolkit to simulate dynamical systems [10].

The MCD++ provides Atomic models based on the following DEVS formalism:

- QBGGyratorFlowIn

- QBGTransformer

- QBGSerialJunction

- QBGSourceEffort_Pulse

- QBGSourceFlow_Pulse

- QBGResistanceFlowIn

- QBGGyratorFlowIn

- QBGInductorEffortIn

Figure 11:  Bond modeled using CD++ Modeller

- QBGParallelJunction

- QBGSourceEffort_Step

- QBGSourceFlow_Step

- QBGSourceEffort_Constant

- QBGSourceFlow_Constant

- QBGResistanceEffortIn

- QBGSourceFlow_Sine

- QBGCapacitorFlowIn

- QBGSourceEffort_Sine

Therefore if we apply these atomic models, and use CD++ modeler, we will get the result as shown in figure 11.

We have modified the images for each of the above atomic models; therefore instead of having a blue circle for QBG Source Effort Sine, the following image will appear.

We have modified the image for QBG Parallel Junction; therefore instead of having a blue circle, the following image will appear.

We have modified the image for QBG Serial Junction; therefore instead of having a blue circle, the following image will appear.

We have modified the image for QBG Capacitor Effort In; therefore instead of having a blue circle, the following image will appear.

We have modified the image for QBG Capacitor Flow In; therefore instead of having a blue circle, the following image will appear.

We have modified the image for QBG Inductor Effort In; therefore instead of having a blue circle, the following image will appear.

We have modified the image for QBG Resistor Effort In; therefore instead of having a blue circle, the following image will appear.

Figure 12:  Bond modeled using CD++ Modeller with new node images

We have modified the image for QBG Resistor Flow In; therefore instead of having a blue circle, the following image will appear.

We have applied those changes to the Bond graph model shown in figure 11 to apply the changes of the images discussed earlier and the following model was designed as shown in figure 12.

We have imported the mcd++ atomic models using the .cpp import option, see figure 13 for details. The next step is to simulate the model by first exporting the model. The problem we ran into is that after following all these steps, no output is created when simulating. The following error is produced as shown in figure 14.

Figure 13: mcd++ atomic models

Figure 14: error screen shown after simulation

Java GUI:  Electrical Circuit Bond Graph Modeller

We have developed a graphical user interface using the Java object-oriented language. The concept behind this GUI is to develop an independent program that runs independently of the CD++ modeller. A snapshot of the GUI is provided in figure 15.

The user interface is composed of three parts:

- Top Toolbar

- Left Panel

- Modeling Panel

The top toolbar contains all the electrical circuit elements that we have discussed earlier, composed of resistor flow in, resistor effort in, capacitor effort it, capacitor flow in, inductor flow in, 0-junction, 1-junction, and source effort sine. This tool bar also contains blue rectangles and circles to cover any other electrical circuit element not covered by the shown images, so that the user is able to model his/her bond graph freely. A color icon is provided so that the node color can be freely chosen. A text toolbar is provided to name the nodes accord to the user’s preference. A link is provided in two formats, the first is a link with no arrow head indicating that it is an external transition function, while the link with the arrow head will indicate that it is an internal transition function. The user will be able to save bond graphs, open, and replace other ones, using the icons provided.

The Modeling Panel will be the place where the bond graphs are modeled using the available icons representing the electrical circuit elements in the top toolbar panel. Currently circuit element icons (i.e. resistor flow in, resistor effort in, capacitor effort it, capacitor flow in, inductor flow in, 0-junction, 1-junction, and source effort sine) cannot be placed in the modeling panel, while the rectangle and circle are available to be place in the modeling panel.

The user will be able to use both the internal and external links to connect the nodes.

The left panel was originally there to be similar to the lateral panel in the CD++ Modeller as seen in figure 3, but due to time constraint we were not able to finish this. The panel displays the types of objects that are available for defining the model. It is divided into ports (input and output), predefined atomic models, and predefined coupled models.  It also displays the objects in textual form that define the model.

Figure 15:  Electrical Circuit Bond Graph Modeller GUI

There are 5 classes involved in the creation of the graphical user interface, a small description of each class is provided next:

- GUI.java – creates all the icon buttons, the top toolbar panel, left panel, and modeling panel.

- GUImenu.java – creates a menu bar file, edit, and insert.

- LeftPanel.java – creates a left panel and all necessary things.

- Shapes.java – the class where objects such as circle, rectangle, arrow, line, etc… are created and dimensions are defined.

- WorkSpace.java – is that class that allows the user to draw freely in the Modeling panel.

- ContainsText.java – is the class displaying information as an output while working on the Electrical Circuit Bond Graph Modeller GUI.

To sum things up, this GUI is just a prototype where many functions are not performed. But as a starting point this will provide easiness for the user as it visualizes most of the steps involved in creating bond graphs.

Simple Example Created

After spending much time on the previous examples we decided to go back to a very simple electrical circuit example where it is composed of only two element, a voltage source and a resistor, shown in figure 16. The following table describes the circuit components briefly:

Figure 16: Simple Electrical Circuit

By applying the rules of converting electrical circuits into bond graphs, the bond graph of the electrical circuit visualized in figure 16 would be the following, where a 1-junction is applied between the source voltage and resistor R1, the result (i.e. bond graph) is shown in fig. 10.

| Component | Type | Parameters |  |
| --- | --- | --- | --- |
| V | Voltage Pulse Source | Amplitude of pulse | 10 V |
|  |  | Width of pulse | 50% |
|  |  | Time for one period | 2.5 sec |
|  |  | Voltage offset | 0 V |
|  |  | Time offset | 0 sec |
| R1 | Resistor | Resistance | 5 Ohms |

Figure 17:  Bond graph of a simple electrical circuit

After applying the atomic models available via the mcd++ folder, and using CD++ modeler, we will get the result as shown in figure 18.

Figure 18:  Bond graph modeled using CD++ Modeler

We have applied the changes to the Bond graph model shown in figure 18 to apply the changes of the images discussed earlier and the following model was designed as shown in figure 19.

Figure 19:  Bond graph modeled with image changes

The next step is to simulate the model by first exporting the model. The problem we ran into is that after following all these steps, no output is created when simulating. The following error is produced as shown in figure 20, which is the same error produce from the first example described in this paper.

Figure 20: error screen shown after simulation

The result of exporting the file, creates an MA file, where it shows the connections between the atomic models. This is shown below.

[top]

components : QBGResistanceFlowIn0@QBGResistanceFlowIn QBGSerialJunction1@QBGSerialJunction QBGSourceEffort_Sine2@QBGSourceEffort_Sine

Link : out@QBGSourceEffort_Sine2 e2p@QBGSerialJunction1

Link : f2p@QBGSerialJunction1 f1p@QBGResistanceFlowIn0

Link : e1p@QBGResistanceFlowIn0 e3p@QBGSerialJunction1

[QBGResistanceFlowIn0]

10

[QBGSourceEffort_Sine2]

10

Conclusion

We have seen that modelling of electrical systems allows for a better understanding of their functionality.  Specifically, bond graphs are used to model the energy transformations between the subsystems, or elements, within a system.

Modelica is an object-oriented multi-domain modelling language, which is used for modelling complex systems such as mechanical, electrical, and power.

In this project, our purpose is to model systems represented in bond graph notation using CD++.  Specifically, we are dealing with systems of electrical circuits.  We begin with an electrical circuit, and then we will show its equivalent Modelica description language.  Later, we will translate the circuit into an equivalent bond graph, and attempt to simulate it in the CD++ Modeler tool.

We have started the building blocks of a GUI, similar to that of CD++ Modeler, in which the setup is there, however, further development is needed.

References

[1]	Bond Graph: (http://www.bondgraph.info/about.html)

[2] 	P. Breedveld, “Bond Graphs”, Encyclopedia of Life Support Systems, Theme 6.43, Topic 7, July 2004.

[3] 	Higo, H.; Yamamoto, K.; Tanaka, K.; Sakurai, Y.; Nakada, T., "Bondgraph analysis on pressure fluctuation in hydraulic pipes," Industrial Electronics Society, 2000. IECON 2000. 26th Annual Confjerence of the IEEE , vol.3, no., pp.1556-1561 vol.3, 2000

[4]	Jiachuan Wang; Zhun Fan; J.P. Terpenny; E.D. Goodman, "Knowledge interaction with genetic programming in mechatronic systems design using bond graphs," Systems, Man, and Cybernetics, Part C: Applications and Reviews, IEEE Transactions on , vol.35, no.2, pp.172-182, May 2005

[5]	Diaz-Insua, M.; Delgado, M., "Modeling and simulation of the human cardiovascular system with bond graph: A basic development," Computers in Cardiology 1996 , vol., no., pp.393-396, 8-11 Sep 1996

[6]	Modelica:  (www.modelica.org)

[7]	C. Claub, A.Schneider, T.Letner, P.Schwarz, “Modeling of electrical circuits with Modelica”, Modelica Workshop Proceedings, pp. 3-12, Oct. 2000.

[8]	Johnson, B.; Quarles, T.; Newton, A.R.; Pederson, D.O.; Sangiovanni-Vincentelli, A.:SPICE3

Version 3e, User’s Manual., Univ. of California, Berkeley, Ca., 94720, 1991

[9]	G.Wainer, “CDModeller Main Manual”, Carleton University.

[10]	Mariana C. D’Abreu , “MCD++ - A extension to N-CD++ for continuous systems simulation using Modelica”, University of Carleton.