SYSC 5104: Methodologies for Discrete-Event Modeling and Simulation

Fall 2007

Assignment 1: Industrial Imaging System Simulator

Brian Webb 100727967

Carleton University

Description of Problem

An industrial imaging system is a system consisting of cameras, frame grabbers and a computer.  The cameras produce image data, which is collected by the frame grabbers and then software running on the computer processes the image data and makes decisions based on the image contents.

Systems such as this are used for a variety of applications, one of the most common being inspection.  A stream of objects: pills, fish fillets, bolts, lumber, lottery tickets, ICs or printer ink cartridges, for example, is moved past a camera which takes pictures of the objects.

The camera is controlled by a frame grabber board, which deals with external signals such as frame and line triggers, encoders and strobe controls, and which collects the image data created by the camera.  The frame grabber either passes this data on to a program running on the computer it is installed in, or performs its own processing on the image before passing it on.  A frame grabber's processing capability varies depending on the sophistication of the board.  This can range from no processing at all, simple buffering of the image data, specific mathematical operations such as convolution, to fully programmable CPUs which allow the frame grabber to act as an independent sub-system in the computer it is installed in.

For the purposes of this model I will assume a digital area scan camera with a 32 bit CameraLink connection and a pixel clock of 40 MHz.  The CameraLink 1.X standard offers either 32 or 64 bit connections and a pixel clock of between 20 and 80 MHz.  A 32 bit connection with 40MHz clock is common with many Dalsa and Jai/Pulnix brand cameras.  A simple frame grabber with a CameraLink transceiver, timers, triggers and no on board processing will be modeled.  This is similar to low end boards such as the Matrox Solios.

Model diagram

Figure 1.

Model components

Camera:  The camera produces image data either continuously or in response to an external signal.  For the style of digital camera I have chosen to simulate the state variables are:

- Size: the size of the image in KB

- exposure_state: Whether the camera is currently exposing, starting to expose or waiting to expose and image.

- continuous_mode:  Is the camera constantly generating images or is it waiting for an external trigger.

- ExposureTime:  How long is the camera's CCD exposed to light before the charge is read

- TransmitTime: How long does it take the camera to transmit an image.  A function of the image size (X*Y*bit_depth*# of colours), the camera's bus width and the camera's pixel clock.

- ImageList[] : A list of image data, DEVS events can't send images directly so we pass indices for this array as camera image data.

- CurrentIndex: The index of imageList[] that will be sent out when the camera takes its next image.

Phases are:

- passive: the camera is waiting for an external trigger

- exposing: the camera is exposing an image

- transmitting: the camera is sending an image out

Because DEVS events do not allow anything other than integers or reals to be passed, and images are far to large to fit inside a single integer another method must be used if image data is to be passed around the simulation.  The camera uses the file images.txt to build an array, one entry per line.  The indices of this array are used as the image data.  Any other component of the simulation can build an identical array from images.txt and access the data that way.

Images.txt has the form:

number of images

data

data

...

For example:

3

first.bmp

second.bmp

third.bmp

Images.txt is currently a list of image files, however, it does not have to be.  As long as the parts of the simulation that access the image data all agree on what must be in images.txt.  It could be actual images to allow actual image processing to be done, or it could simply be the results of what the image processing should be.  Ie. Each line of images.txt could be the number of vertical lines found in the image, or a string of text that Optical Character Recognition (OCR) software was supposed to find.  It depends on the needs of the simulation.

The cameras initial starting values, image size (X, Y, bit depth, colour bands) pixel clock and other relevant values are provided in a file identified by the parameter configfile.  These are conventionally named with .cfg but this is not strictly necessary.  The term bands refers to the number of colour bands in the image, 1 for greyscale, 3 for RGB, YUV or HSL colour are the standards, although some IR capable cameras may have 4 (RGB+IR).

A config file has the form:

sizeX(int)

sizeY(int)

bit_depth(int)

bands(int)

CameraLink bit width (int, 32 or 64)

pclk (int in MHz)

frames per second (int)

continuous (int 1 = continuous)

Image data is transmitted as pairs of size and imageList index, with a positive value indicating the start of the image and a negative value indicating the end of the image transmission.  Because of the use of negative numbers the indices start at 1.  To get the C/C++ style index for an image, subtract 1 from the DEVS event data value.

Frame Grabber:  The frame grabber is a compound system.  It has an image acquisition module,

a storage buffer, a timer module and a host system interface module.

Image acquisition:  This can be a digital transceiver, custom or an industry standard, A/D converter or TV-decoder.  It syncs with the camera's output and sends it to the Frame Grabber's control logic.  State variables are:

- Size:  The size of the image being grabbed.  This should match what the camera is transmitting, as there can be problems if they do not.  Buffer overflow and lost data if the camera is sending more than expected, or extra errors and timeout problems if the camera is sending less.

- Mode:  Is the module ignoring image data, waiting for the next image or grabbing every image that comes along.

- Image: The image that was just grabbed.

- Response:  Error reports and other information sent to the Frame Grabber's control logic.

Phases:

- passive:  waiting for input, either a command or an image

- command:  processing a command that has been given

- grabbing: acquiring an image from the camera

Timer module:  The timer module consists of a programmable timer which can be used to generate clock signals, filter and reshape trigger signals and generate any kind of timed output needed by the system. State variables:

- activeTime:  How long the timer pulse is active

- delayTime:  How long to wait between an incoming trigger and the start of the active pulse

- inactiveTime: How long after the active pulse to wait before we can acknowledge a trigger again.

- Mode:  Is the timer off, in triggered mode, or periodic mode.

- pulse_location:  Is the timer pulse off, in delay, in active or in inactive.

- pulse_abort: Was the timer turned off before an active pulse was finished.

- Response: Error messages and other information to be sent to the Frame Grabber's control logic.

Phases:

- Passive:  waiting for a command or a trigger.

- Delay: waiting for delayTime to expire.

- Active: waiting for activeTime to expire.

- Inactive: waiting for inactiveTime to expire.

Delays are used when there is a period between when the external trigger is generated and when the timer needs to send it's pulse.  The inactive period serves a similar purpose, when there is a time after a pulse has been sent that a trigger must be ignored.

The timer pulse is sent out as a pair of DEVS events.  One event at the beginning of the Active period, with a value of +1.0 and one event at the end of  the Active period, with a value of -1.0.  This is to simulate the rising and falling edges of a logic transition.

Storage buffer:  Holds image data as it is collected by the Image Acquisition section. Storage buffers may be ignored to allow the Frame Grabber to transmit directly to the host system, or they may be used to store image data before it is sent on.  This is usually done to ensure that the bus that connects the frame grabber and host system is being used as efficiently as possible.  With large images, or analog cameras it is possible for a direct image transfer to tie up the system bus long enough to cause problems with other devices using the bus.

State variables:

- totalSpace:  How much room there is in the buffer.  In Kilobytes.

- FreeSpace: How much room there is available for new buffers.

- ImageBufferList:  A list of all the storage buffers allocated in the storage buffer.

- ImageBufferSizes: A list of the sizes of each storage buffer.

- CurrentIndex:  The index to the buffer in ImageBufferList that is currently being used.

- Response:  Error messages and other information to be sent to the Frame Grabber's control

logic.

- internal_bus_speed: the speed at which data can be transferred inside the Frame Grabber.  In KB/s.

Phases:

- Passive:  waiting for a command.

- BufferList manipulation:  changing the internal storage according to a command

- Storing:  Placing image data in ImageBufferList.

- Retrieving:  Getting image data out of ImageBufferList.

The internal bus speed values used in the simulation are arbitrary, and highly dependent on the particular hardware being modeled.  A value that reflects the specific hardware being simulated should be placed in the storage buffer's .ma file.

Host System Interface:  Handles communication with the host system, accepting commands from the image processing software running on the main CPU and transferring image data to the main memory of the system. The Frame Grabber's control logic, unless there is a more sophisticated processor (DSP or microprocessor) in the Frame Grabber.

State variables:

- command:  What the frame grabber was asked to do by the host system

- grab type: Is the grab being buffered on board and then automatically sent on to the host, being buffered on board and not sent on to the host, or not being buffered on board at all.

- Image: The image data currently being dealt with.

- Response:  A response to be sent to the host, either an error message from the his or another module that must be sent to the host or other information.

- external_bus_speed: The maximum data transfer rate of the bus connected to the host system, in KB/s.  This is the fastest that the images can be sent to the host system, as this model knows nothing of the state of the bus it is connected to.

Phases:

- passive: waiting for a command or image data.

- commanding:  passing a command or commands on to one of the other modules

- grabbing: sending image data from the image acquisition module to either the storage buffer or the host system.

- Responding: Processing a response from another module and possibly passing it on to the host system.

The external bus speed used in the simulation is arbitrary and dependent on the hardware being modeled.  A value that corresponds to the system being modeled should be put the HSI's .ma file.

DEVS Formal Specification

The industrial imaging system (IIS) simulator, as described in Figure 1,  has 2 inputs and 2 outputs.  The controlIn input represents commands sent by the larger system that the imaging system is a part of.   This larger system will be called the Host System.  The trigIn input represents a signal sent by an outside transducer, indicating that an image should be taken now.

The two outputs are responseOut, which reports errors or problems from within the frame grabber to the host system, and imageOut, which provides that host system with the image acquired by the imaging system.

The IIS simulator has two components, a camera and a frame grabber.  The camera is an atomic model.  The frame grabber a coupled model made up of four atomic models:  the image acquisition model, the timer model, the on board buffer model and the host system interface model.

The camera generates image data, the frame grabber collects image data and passes it on to the host system

In order to simplify the system and avoid sending multiple events to issue a single command, events with multiple pieces of information are sent as combined values.  The least significant 4 bits of the value are the command or image index.  The rest of the value is the command information or the image size.

Atomic Model Specifications

The formal specifications of the models  <S, X, Y, δint, δext, λ, ta>  for the atomic models are defined as follows:

Camera:

State variables:

image index = 0 // start of image list

image list           //  loaded from a file

continuous_mode = false  // true send images constantly, false, wait for a trigger

grabbing = 0 // what part of the grab the camera is in: off, starting to expose, starting to

// transmit, finished transmitting;

exposureTime = 1/fps // how long it takes the camera to generate an image

transmitTime = f(pclk,size,bus) // how long it takes the camera to transmit the image

Formal specification

S = {passive, active}

X = {trigIn}

Y = {imageDataOut}

δint( active)

{

if grabbing == start

grabbing = on

state = active

else if grabbing == on

grabbing = done

state = active

else if grabbing == done

inc imageIndex

if continuous_mode

grabbing = start

state = active

else

grabbing = off

state = passive

}

δext ( trigIn )

{

if !continuous_mode & trigIn > 0 // trigIn < 0 is  the falling edge of the trigger

grabbing = start

state = active

}

λ( active )

{

case grabbing == start

nothing

case grabbing == on

send currentIndex.size to port imageDataOut

case grabbing == done

send -1.0 x currentIndex.size to port imageDataOut

}

ta( passive ) = INFINITY

ta( active )

{

case grabbing = start

ta = 0

case grabbing = on

ta = exposureTime

case grabbing = done

ta = transmitTime

}

Image Acquisition:

State variables:

mode =off	 // are we grabbing, continuously grabbing, or idling.

response = NONE

size = 0 // size of the image being grabbed.

Image = none // the last image that came in.

Formal specification:

S = {passive, active}

X = {commandIn, imageIn}

Y = {responseOut, imageOut}

δint( active  )

{

if response != NONE

response = NONE

else // image came in

image = NONE

if mode = grab

mode = off

passive

}

δext( commandIn )

{

local command = commandIn  bitmask ( EVENT_MASK)

local data  = commandIn bit_shift_right(DATA_SHIFT)

switch command

case SET_SIZE

size = data

state = passive

case GRAB_SINGLE

mode = grab

state = passive

case GRAB_CONT

mode = continuous

state = passive

case GRAB_ABORT

if mode != off

mode = off

response = GRAB_ABORTED

else

response = ERROR_NO_GRAB_IN_PROGRESS

state = active

case unknown command

response = ERROR_BAD_COMMAND

state = active

}

δext( imageIn )

{

if mode != off

if imageIn < 0 & imageIn.size != size

response = ERROR_IMAGE_SIZE_DIFFERENT_THAN_EXPECTED

else

response = NONE

image = imageIn

state = active

}

λ( active )

{

if response != NONE

send response to port reponseOut

if image != NONE

send image to port imageOut

}

ta( passive ) = INFINITY

ta( active )  = 0 // all delays are imposed by the camera

Timer:

State variables:

command = NONE // last command received

mode = off	// 3 modes:	off – timer does not output or react to trigIn

//		trigged – timer sends timerOut on trigIn

//		periodic – timer sends timerOut periodically

delayTime = 50.0 // msec after trigIn timer waits before sending timerOut

activeTime = 50.0 // msec the timer is active (high or low)

inactiveTime = 0.0 // msec after timer is active when a new trigIn will be ignored.

pulse_location = delay // where in the pulse, delay, active or inactive

Formal specification:

S = {passive, active}

X = {trigIn, commandIn}

Y = {timerOut, errorOut}

δint( active )

{

if command == error

if mode == off  // should always be true

state = passive

else

mode = off

state = passive

else

if pulse_location == delay

pulse_location = active

state = active

else if pulse_location == active

pulse_location == inactive

state = active

else // pulse_location == inactive

pulse_location = delay

if mode == periodic

state = active

else

state = passive

}

δext( commandIn)

{

command =commandIn bitmask( EVENT_MASK )

local data = commandIn bit_shift_right( DATA_SHIFT )

switch ( command )

case SET_MODE :

mode = data

if mode == periodic & activeTime != 0 & (delayTime or

inactiveTime != 0)

state = active

else if mode == triggered & activeTime != 0 & (delayTime or

inactiveTime != 0)

state = passive

else if mode = off

state = passive

else

mode = off

command = error // timer isn't set or bad mode

state = active

case SET_ACTIVE:

activeTime = data

if activeTime == 0 & mode != off

mode = off

command = error // no active time

case SET_DELAY:

delayTime = data

if delayTime & inactiveTime == 0 & mode != off

mode = off

command = error // time always active

case SET_INACTIVE:

inactiveTime = data

if delayTime & inactiveTime == 0 & mode != off

mode = off

command = error // time always active

default:

command = error

}

δext( trigIn )

{

if state == passive && mode == triggered

pulse_location = delay

state = active

else

// we're not waiting for a trigger

}

λ( active )

{

if command == error

send error to port errorOut

else if pulse_location == active

send 1 to port timerOut

else if pulse_location == inactive

send -1 to port timerOut

}

ta( passive ) = INFINITY

ta( active )

{

if command != NO_COMMAND

ta = 0

else // active timer

if pulse_location == delay

ta = delayTime

else if pulse_location == active

ta = activeTime

else  // pulse_location == inactive

ta = inactiveTime

}

Storage Buffer:

State variables:

response = NONE

bufferList[LIST_SIZE]

bufferSizes[LIST_SIZE]

image = NONE

internal_bus_speed = 1024 // KB/s, this is a random number, the internal speed of the

// FG is very hardware and implementation dependent.

// it can generally be assumed to be better than the camera

// data rate, except for extreme cases.

totalSpace

freeSpace

currentIndex

lastIndex

Formal specification:

S = { passive, active}

X = {commandIn }

Y = { imageOut, responseOut }

δint(active )

{

if response != NONE

response = NONE

state = passive

else if image > 0

image = -1 * image

state = active

else if image < 0

image = NONE

state = passive

}

δext( commandIn )

{

local command = commandIn bitmask( EVENT_MASK )

local data = commandIn bit_shift_right( DATA_SHIFT )

switch command

case CREATE_BUFFER

if data < freeSpace & lastIndex != LIST_SIZE

bufferSize[lastIndex] = data

lastIndex++

state = passive

else

response = ERROR_NO_FREE_SPACE

state = active

case SET_INDEX

if data >= 0 & < lastIndex

currentIndex = data

state = passive

else

response = ERROR_INDEX_OUT_OF_RANGE

state = active

case STORE_IMAGE

size = data bit_shift_right( DATA_SHIFT )

if (size > bufferSize[currentIndex]

response = ERROR_IMAGE_BUFFER_SIZE_MISTACH

state = active

else

bufferList[currentIndex] = data

state = passive

case RETRIEVE_IMAGE

image = bufferList[currentIndex]

state = active

case unknown

response = ERROR_BAD_COMMAND

state = active

}

λ( active )

{

if response != NONE

send response to port responseOut

if image != NONE

send image to port ImageOut

}

ta( passive ) = INFINITY

ta( active )

{

if image > 0

ta = image.size / internal_bus_speed

else

ta = 0

}

Host System Interface:

State variables:

grab = buffered

send_image = NONE

response = NONE

buf_image = NONE

off_board_buffered_grab_image_from_IA = true;

external_bus_speed  // max speed for external bus type.

// we know nothing of the state of the bus the FG is connected to,

// so send data as fast as the board is capable of IE. A PCI board

// will send data at PCI rates. While a PCI-X board will send

// data at PCI-X rates.

IA = NONE		// im acq commands

BUF = NONE		// buf commands

TIM = NONE		// tim commands

Formal specification:

S = { passive, active}

X = {imageIn, responseIn, commandIn}

Y = {imageOut, bufOut, iaOut, timOut, errorOut}

δint( active  )

{

if response != NONE

response = NONE

state = passive

if IA != NONE

IA = NONE

state = passive

if BUF != NONE

BUF = NONE

state = passive

if TIM != NONE

TIM = NONE

state = passive

if send_image > NONE

send_image = -1 * send_image

state = active

else if send_image < NONE

send_image = NONE;

state = passive

if buf_image != NONE

buf_image = NONE

state = passive

if  !off_ board_buffered_grab_image_from_IA & grab = off_board_buffered

BUF = BUF_RETRIEVE_IMAGE

state = active

}

δext( imageIn )

{

if grab == not_buffered

send_image = imageIn

state = active

else if grab == buffered

if imageIn > 0

buf_image = imageIn

state = active

else

state = passive

else // off_board_buffered

if off_ board_buffered_grab_image_from_IA

if imageIn < 0

off_ board_buffered_grab_image_from_IA = false

else

buf_image = BUF_STORE_IMAGE +

(imageIn bit_shift_left( DATA_SHIFT) )

state = active

else // from buffer

send_image = imageIn

state = active;

}

δext( responseIn )

{

response = responseIn

state = active

}

δext( commandIn )

{

command = commandIn bitmask( EVENT_MASK)

data = (commandIn) bit_shift_right( DATA_SHIFT )

switch command

case HSI command

sub_command = data bitmask( EVENT_MASK )

data =  data bitshift

case SET_GRAB_TYPE

case data = ON_BOARD_BUFFERED

grab = buffered

state = passive

case data = OFF_BOARD_BUFFERED

grab = off_board_buffered

state = passive

case data = OFF_BOARD_UNBUFFERED

grab = unbuffered

state = passive

case

response = ERROR_BAD_COMMAND

state = active

case unknown

response = ERROR_BAD_COMMAND

state = active

case IA command

IA = data

state = active

case TIM command

TIM = data

state = active

case BUF command

BUF = data

state = active

case unknown

response = ERROR_BAD_COMMAND

state = active

}

λ( active )

{

if response != NONE

send response to port responseOut

if BUF

send BUF to port bufOut

if IA

send IA to port imOut

if TIM

send TIM to port timOut

if send_image

send send_image to port imageOut

if buf_image

send buf_image to port bufOut

}

ta( passive ) = INFINITY

ta( active )

{

if send_image < NONE

ta = send_image.size / external_bus_speed

else

ta = 0

}

Coupled Model Specifications

The formal specifications of the models <X, Y, D, {Mi}, EIC, EOC, IC, SELECT > for the frame grabber coupled model and IIS are defined as follows:

Frame Grabber:

X = {trigIn, imageIn, commandIn};

Y = {timerOut, imageOut, responseOut};

D = {image acquisition, timer, storage buffer, host system interface};

Md = {Mimage acquisition, Mtimer, Monboard buffer, Mhost system interface };

EIC = { ((Self, trigIn),(timer, trigIn));

((Self, imageIn),(image acquisition, imageIn ));

((Self, commandIn),(host system interface, commnadIn)) };

EOC = { ((timer, timerOut),( Self, timerOut));

((host system interface, imageOut),(Self, imageOut));

((host system interface, responseOut),( Self, responseOut)) };

IC = { ((host system interface, bufOut),( storage buffer, commandIn));

((host system interface, iaOut),(image acquisition,

commandIn));

((host system interface, timOut),(timer, commandIn));

((image acquisition, imageOut),(host system interface, imageIn));

((image acquisition, responseOut),(host system interface, responseIn));

((timer, responseOut),(host system interface, responseIn));

((storage buffer, imageOut),(host system interface, imageIn));

((storage buffer, responseOut),(host system interface, responseIn)) };

SELECT = {timer, image acquisition, host system interface, storage buffer}

Industrial Imaging Simulator:

X = {trigIn, commandIn};

Y = {imageOut, responseOut};

D = {camera, Frame Grabber};

Md = {Mcamera, Mframe grabber };

EIC = { ((Self, trigIn), (Frame Grabber, trigIn));

((Self, commandIn),(Frame Grabber, commandIn)) };

EOC = { ((Frame Grabber, imageOut),(Self, imageOut));

((Frame Grabber, responseOut),(Self, responseOut)) };

IC = { ((camera, imageDataOut),(Frame Grabber, imageIn));

((Frame Grabber, timerOut),(camera, trigIn)) };

SELECT = {Frame Grabber, camera}

Test strategies

The testing for the IIS can be broken into several parts:  Testing each atomic model individually using various .ev files,  testing the frame grabber coupled model and finally testing the entire industrial imaging simulator.

Atomic model tests

Tests for the individual components of the simulator.

camera:

The camera configuration file is set up for 640x480x8 bit greyscale (300 KB) at 30 frames per second with a pixel clock of 40 MHz and a CameraLink bandwidth of 32 bits.  The camera also ignores the falling edge of a trigger pulse.

Images.txt has two entries, which means we should have 2 different indices.

When set in continuous mode the camera should generate image data output pairs at 30 per second

and ignore any triggers in the event file camera.ev.  The simulation is set to stop after 1 minutes (cont_camera.bat).

00:00:01:000 trigIn 1.0

00:00:03:000 trigIn -1.0

00:00:05:000 trigIn 1.0

00:00:05:003 trigIn 1.0

00:00:07:000 trigIn 1.0

00:00:07:040 trigIn -1.0

An excerpt of the results file cont_cameraMa.out covering this range in times shows that we are achieving ~ 30fps ( 60 lines every second = 30 images) and that the output is independent of any triggers.  We also see that the images are the correct size, 300KB.

00:00:00:033 imagedataout 4801

00:00:00:033 imagedataout -4801

00:00:00:066 imagedataout 4802

00:00:00:066 imagedataout -4802

00:00:00:099 imagedataout 4801

00:00:00:099 imagedataout -4801

00:00:00:132 imagedataout 4802

00:00:00:132 imagedataout -4802

00:00:00:165 imagedataout 4801

00:00:00:165 imagedataout -4801

00:00:00:198 imagedataout 4802

00:00:00:198 imagedataout -4802

00:00:00:231 imagedataout 4801

00:00:00:231 imagedataout -4801

00:00:00:264 imagedataout 4802

00:00:00:264 imagedataout -4802

In triggered mode the camera is set to use the same event file, camera.ev.  However, it should only produce images in response to the triggers at 00:00:01:00,  00:00:05:00 and 00:00:07:00.  These images should show up in the output 1/30 second after the input trigger, simulating the exposure time.

The triggers at 00:00:03:00 and 00:00:07:40 are falling edges which the camera should ignore and the trigger at 00:00:05:03 should come in as the camera is exposing an image and also be ignored. (camera.bat)

Camera.ev					CameraMa.out

00:00:01:000 trigIn 1.0

00:00:01:066 imagedataout 4801

00:00:01:066 imagedataout -4801

00:00:03:000 trigIn -1.0

00:00:05:000 trigIn 1.0

00:00:05:0003 trigIn 1.0

00:00:05:066 imagedataout 4802

00:00:05:066 imagedataout -4802

00:00:07:000 trigIn 1.0

00:00:07:066 imagedataout 4801

00:00:07:066 imagedataout -4801

00:00:07:040 trigIn -1.0

As desired the camera responds only to rising edge triggers, and ignores the fast trigger at 5:0003.  Also we can see that the camera is correctly cycling through the two entries in images.txt.

Note: with the image size being used (640x480x8bits greyscale, standard B/W video) the transmition time for 32 bit CameraLink at 40MHz is smaller than CD++ can measure (~ 0.00024 seconds).  This means that the image start event and image end event are simultainious as far as CD++ is concerned.  This causes problems with the image acquisition module as found in the coupled model testing.  With the current time resolution provided by the CD++ tools, it is not possible to acurately model small images from a CameraLink camera.

The slowest that a CameraLink camera can transmit is 20MHz, which imposes a minimum size on the image of 625 KB in order for the image start and image end events to be discrete.  Anaolg cameras do not have this concern as they typically have much slower effective pixel clocks, and their transmition times are closer to their exposure times.

For the purposes of further testing a camera pixel clock of 4MHz was used, even though this violates the CameraLink specifications.

timer:

Following timer.ev the timer starts off and does not generate any pulses.  The active, delay and inactive times are 0 by default.  Before the timer is set or turned on it should ignore any triggers it recieves.  It should not allow it self to be turned on before the active time and either the delay or inactive time are set.  Once those are set and it is set in triggered mode it should only output when a trigger is received and then only for the set times.  Once it is set to periodic mode it should start to output constantly until turned off. (timer.bat)

timer.ev					timerMa.out

00:00:05:000 trigIn 1

00:00:10:000 commandIn 4		00:00:10:000 responseout -8

00:00:11:000 commandIn 801

00:00:12:000 commandIn 4		00:00:12:000 responseout -9

00:00:15:000 commandIn 802

00:00:20:000 commandIn 4

00:00:21:000 trigIn 1

00:00:21:050 timerout 1

00:00:21:100 timerout -1

00:00:23:000 trigIn 1

00:00:23:050 timerout 1

00:00:23:100 timerout -1

00:00:25:000 commandIn 6

00:00:21:050 timerout 1

00:00:21:100 timerout -1

00:00:23:050 timerout 1

00:00:23:100 timerout -1

00:00:25:050 timerout 1

00:00:25:100 timerout -1

00:00:25:150 timerout 1

00:00:25:200 timerout -1

00:00:25:250 timerout 1

00:00:25:300 timerout -1

00:00:25:350 commandIn 5

00:00:26:000 commandIn 2

00:00:27:000 commandIn 803

00:00:28:000 commandIn 6		00:00:28:000 timerout 1

00:00:28:050 timerout -1

00:00:28:100 timerout 1

00:00:28:150 timerout -1

00:00:28:200 timerout 1

00:00:28:250 timerout -1

00:00:28:300 timerout 1

00:00:28:350 timerout -1

00:00:28:400 commandIn 5

As expected the first trigger at 05:000 is ignored, and the first two attempts to start the timer before it has been set result in errors.  However, once the timer is set the next two triggers, at 21:00 and 23:00 produce pulses that are delayed by the desired 50 microseconds, and which are 50 microseconds wide.

Setting the timer into periodic mode causes it to output a square wave until the stop command is sent.  Changing the delay to 0 and then the inactive to 50 changes the behavior so that the rising edge occurs at the start of the cycle instead of middle.  This can be seen most effectively when the timer is started 00:00:25:000 where the first rising edge occurs at 00:00:25:050 (after the delay) compared to 00:00:28:000 where the first rising edge occurs at the same time the timer is started.

image acquisition:

There are several tests to be done, first the expected size is set, then an image is sent when none is expected.  Next a single grab is requested and an image is sent.  Then the mode is set to continuous grabbing and several images are set.  Finally the mode is returned to single grab and a grab of the wrong size is sent. (im_acq.bat)

Input						im_acq.ev

Set size to 300KB				00:00:00:005 commandIn 4801

Input image 4801 (300 KB, ind 1)	00:00:00:020 imageIn 4801

00:00:00:021 imageIn -4801

Grab next image				00:00:30:000 commandIn   2

Input image 4806 (300KB, ind 6)	00:00:31:000 imageIn 4806

00:00:31:001 imageIn -4806

Grab continuously				00:00:33:000 commandIn 3

Image inputs				00:00:33:030 imageIn 4807

00:00:33:031 imageIn -4807

00:00:34:000 imageIn 4808

00:00:34:001 imageIn -4808

00:00:35:030 imageIn 4809

00:00:35:031 imageIn -4809

Abort the grab				00:00:36:000 commandIn 4

Grab next image				00:00:37:000 commandIn 2

input image 5901 (368KB, ind 13)	00:00:38:000 imageIn 5901

00:00:38:001 imageIn -5901

im_acq.ev:					im_acqMa.out

00:00:00:005 commandIn 4801

00:00:00:020 imageIn 4801

00:00:00:021 imageIn -4801

00:00:30:000 commandIn   2

00:00:31:000 imageIn 4806		00:00:31:000 imageout 4806

00:00:31:001 imageIn -4806		00:00:31:000 imageout 4806

00:00:33:000 commandIn 3

00:00:33:030 imageIn 4807		00:00:33:030 imageout 4807

00:00:33:031 imageIn -4807		00:00:33:031 imageout -4807

00:00:34:000 imageIn 4808		00:00:34:000 imageout 4808

00:00:34:001 imageIn -4808		00:00:34:001 imageout -4808

00:00:35:030 imageIn 4809		00:00:35:030 imageout 4809

00:00:35:031 imageIn -4809		00:00:35:031 imageout -4809

00:00:36:000 commandIn 4		00:00:36:000 responseout 1

00:00:37:000 commandIn 2

00:00:38:000 imageIn 5901		00:00:38:000 imageout 5901

00:00:38:001 responseout -6

00:00:38:001 imageIn -5901		00:00:38:001 imageout -5901

The image sent at 00:020 is ignored as the module is not waiting for one. The two bold lines are responses.  The response to the grab abort command (4) is grab aborted (1) as defined in fg_codes.h.  At 38:001 the response is an error, negative number, and -6 corresponds to an image size mismatch.

storage buffer:

The commands in buffer.ev set up an image buffer, store data in it and retrieve that data.  An attempt is made to create a buffer larger than the available space and the index is set to an invalid value. Then a new, valid buffer is created, the index set to this buffer and a new image stored and retrieved. (buffer.bat)

Input						buffer.ev

Create a 300KB buffer			00:00:00:020 commandIn 4801

Store image 4803 (300KB, index 3)	00:00:01:000 commandIn 76851

Retrieve the image			00:00:05:000 commandIn 4

Create a 600KB buffer			00:00:06:000 commandIn 9601

Set index to 3				00:00:07:000 commandIn 50

Create a 300KB buffer			00:00:08:000 commandIn 4801

Set the index to 1			00:00:09:000 commandIn 18

Store image 4802 (300KB, index 2)	00:00:10:000 commandIn 76835

Retrieve the image			00:00:11:000 commandIn 4

Buffer.ev					bufferMa.out

00:00:00:020 commandIn 4801

00:00:01:000 commandIn 76851

00:00:05:000 commandIn 4		00:00:05:000 imageout 4803

00:00:05:600 imageout -4803

00:00:06:000 commandIn 9601		00:00:06:000 responseout -3

00:00:07:000 commandIn 50		00:00:07:000 responseout -4

00:00:08:000 commandIn 4801

00:00:09:000 commandIn 18

00:00:10:000 commandIn 76835

00:00:11:000 commandIn 4		00:00:11:000 imageout 4802

00:00:11:600 imageout -4802

At 00:020 a 300 KB buffer is created out of the 700 KB total space (as defined in bufferMa.ma).  At 06:000 there is an attempt to create a 600 KB buffer in the remaining 400 KB of free space, and fail (-3).  Then at 07:000 the index is set to 3, which is outside of the current range (0 to 0) and an out of bounds error is returned (-4).

host system interface:

The Host System Interface (HSI) has three jobs.  It must direct commands to the correct frame grabber module, it must accept errors back from those modules and pass them to the host system, it must accept image data and according to its mode do one of three things:  Pass it directly to the host, store it in the storage buffer or temporarily store it in the storage buffer and then pass it on to the host.

hsi.ev sends several commands for each of the other modules,  two for the image acquisition module, two for the storage buffer module and two for the timer module.  It then receives several responses and must forward them on.  Finally it accepts an image in pass through mode, and image in buffered mode and an image in buffer then forward mode. (hsi.bat)

Inputs						hsiMa.ev

IA single grab (2)			00:00:00:004 commandIn 34

IA grab abort (4)				00:00:00:020 commandIn 66

Timer, start timer (4)			00:00:01:000 commandIn 68

Timer, set active to 50ms (801)	00:00:01:300 commandIn 12820

Buf set index to 0 (2)			00:00:01:600 commandIn 35

Buf retrieve image (4)			00:00:02:000 commandIn 67

Error timer not set			00:00:02:400 responseIn -8

HSI command grab to on board		00:00:03:000 commandIn 273

Image in					00:00:03:200 imageIn 4801

00:00:03:860 imageIn -4801

HSI command grab off board		00:00:05:000 commandIn 785

Image in					00:00:05:120 imageIn 4802

00:00:05:780 imageIn -4802

HSI command grab off board buffered	00:00:07:100 commandIn 529

Image in from IA				00:00:07:200 imageIn 4803

00:00:07:860 imageIn -4803

Image in from buffer			00:00:08:000 imageIn 4803

00:00:08:050 imageIn -4803

hsi.ev					hsiMA.out			meaning

00:00:00:004 commandIn 34	00:00:00:004 ia 2		command 2 sent to IA

00:00:00:020 commandIn 66	00:00:00:020 ia 4		command 4 sent to IA

00:00:01:000 commandIn 68	00:00:01:000 tim 4	command 4 sent to Timer

00:00:01:300 commandIn 12820	00:00:01:300 tim 801	command 801 to Timer

00:00:01:600 commandIn 35	00:00:01:600 buf 2	command 2 sent to buffer

00:00:02:000 commandIn 67	00:00:02:000 buf 4	command 4 sent to buffer

00:00:02:400 responseIn -8	00:00:02:400 responseout -8	Passed the error on

00:00:03:000 commandIn 273						Set HSI grab to on board

00:00:03:200 imageIn 4801	00:00:03:200 buf 76819		Send image to buffer

00:00:03:860 imageIn -4801

00:00:05:000 commandIn 785						set HSI grab to offboard

00:00:05:120 imageIn 4802	00:00:05:120 imageout 4802	send image to host

00:00:05:610 imageIn -4802

00:00:05:720 imageout -4802

00:00:07:100 commandIn 529						set to buffered offboard

00:00:07:200 imageIn 4803	00:00:07:200 buf 76851		Send image to the buffer

00:00:07:860 imageIn -4803

00:00:07:860 buf 4		req image from buffer

00:00:08:000 imageIn 4803	00:00:08:000 imageout 4803	send image to host

00:00:08:050 imageIn -4803

00:00:08:600 imageout -4803

The HSI passes commands through to the correct modules, it passes the correct commands to those modules, and when dealing with incoming images, routes them correctly.

Coupled model tests

Tests for the coupled models of the simulator.  There are two such models, the frame grabber and the  industrial imaging system.  The frame grabber is made up of the host system interface, timer, storage buffer and image acquisition modules.  The industrial imaging system is the frame grabber and the camera.

frame grabber:

Commands for the frame grabber are largely the same as those for the HSI except we now have all four modules working together and do not have to fake their responses and inputs to the HSI.  All the HSI's modes, the timer's modes and the image acquisition must be exercised. (fg.bat)

Test:

- Set up the image acquisition module to recieve images of 300 KB

- Set the timer to send a trigger with an active cylce of 50 microseconds and an inactive of 50 microseconds.

- Create a 300 KB image buffer in the storage buffer

- Set the HSI to do an off board buffered grab

- request a grab from the image acquisition module

- send in a trigger to the timer

- include triggers before the setup of the timer (should be ignored)

- send an image to the input before a grab is requested (should be ignored)

- send an extra image after the grab is completed (should be ignored)

- switch HSI grab modes

- request more images from the image acquisition module

fg.ev

00:00:01:000	commandIn	76818		Set im_acq to expect 300KB image

00:00:02:000	commandIn	65		Non-existant HSI command

00:00:02:040	trigIn	1		Should be ignored

00:00:02:500	commandIn	12820		Set active time

00:00:02:750	imageIn	4801		Ignored image, no grab pending

00:00:02:950	imageIn	-4801

00:00:03:000	commandIn	12852		Set inactive time

00:00:03:300	trigIn	1		Ignored

00:00:04:000	commandIn	68		Start timer

00:00:04:600	commandIn	76819		Create an image buffer in storage

00:00:05:000	trigIn	1		Should cause timer output

00:00:05:350	commandIn	529		Set HSI to off board buffered grab

00:00:05:640	trigIn	1		Should cause timer output

00:00:06:000	commandIn	34		Grab next image

00:00:06:200	imageIn	4802		Next image

00:00:06:220	imageIn	-4802

00:00:07:000	commandIn	785		Set HSI to off board grab

00:00:07:200	commandIn	803		Bad buffer index command

00:00:07:400	commandIn	34		Grab next image

00:00:07:700	imageIn	4803		Next image

00:00:07:800	imageIn	-4803

00:00:08:400	commandIn	67		Get image stored onboard

The results: FrameGrabber.out

00:00:02:000 responseout -1		Error non existant command (correct)

00:00:05:000 timerout 1			Trig causes timer output, rising edge

00:00:05:050 timerout -1		Falling edge of timer output

00:00:05:640 timerout 1			2nd trig causes timer pulse, rising edge

00:00:05:690 timerout -1		Falling edge of 2nd timer pulse

00:00:06:220 imageout 4802		06:000 grab command succeeds

00:00:06:820 imageout -4802

00:00:07:200 responseout -4		Error bad index, correct buffer error

00:00:07:700 imageout 4803		07:400 grab command succeeds

00:00:08:300 imageout -4803

00:00:08:400 imageout 4802		retrieve the previously buffered image

00:00:09:000 imageout -4802		  the value is correct.

The Frame Grabber model is capable of grabbing images and sending them to the correct places.  The timer can be controlled, and responds properly to triggers.  Incorrect commands result in the correct errors.

industrial imaging system:

The tests for the IIS are built on the frame grabber tests, except the camera inputs are from the actual model.  There are two groups of tests with identical inputs, the only difference being if the camera is in continuous (iis_cont_cam.bat) or triggered mode (iis_trig_cam.bat).

The event file has been modified to use the continuous grabbing mode of the image acquisition module, and to provide triggers for some of the grabs.  These triggers should be ignored when the camera is in continuous mode, and synced with the images when the camera is in triggered mode.

As noted under the camera tests, attempting to use a CameraLink standard compliant pixel clock with a small image caused problems with the image acquisition module as both the image start and image end events occured at the same time. The following test was run with iis_fast_cam.bat.

The error reports in iis_fast_cam.out are correct, iis1.ev does more than just showcase this problem.

iis1.ev					iis_fast_cam.out

00:00:01:000 cmndIn 76818

00:00:02:000 cmndIn 65		00:00:02:000 respout -1

00:00:02:040 triggerIn 1

00:00:02:500 cmndIn 12820

00:00:03:000 cmndIn 12852

00:00:03:300 triggerIn 1

00:00:04:000 cmndIn 68

00:00:04:600 cmndIn 76819

00:00:05:000 triggerIn 1

00:00:05:640 triggerIn 1

00:00:07:000 cmndIn 785

00:00:07:200 cmndIn 803		00:00:07:200 respout -4

00:00:07:400 cmndIn 34

There should be a grab in iis_fast_cam,out as a result of the last line of iis1.ev.  It is not there.  Looking at the relevant section of iis_fast_cam.log we can see that the camera generates both the start and end events at 00:00:07:425 and that im_acq, the image acquisition module only ever passes on the end of image event.

Mensaje Y / 00:00:07:425 / camera(02) / imagedataout /   4801.00000 para top(01)

Mensaje D / 00:00:07:425 / camera(02) / 00:00:00:000 para top(01)

Mensaje X / 00:00:07:425 / top(01) / imagein /   4801.00000 para framegrabber(03)

Mensaje X / 00:00:07:425 / framegrabber(03) / imagein /   4801.00000 para im_acq(05)

Mensaje D / 00:00:07:425 / im_acq(05) / 00:00:00:000 para framegrabber(03)

Mensaje D / 00:00:07:425 / framegrabber(03) / 00:00:00:000 para top(01)

Mensaje D / 00:00:07:425 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:07:425 / Root(00) para top(01)

Mensaje * / 00:00:07:425 / top(01) para camera(02)

Mensaje Y / 00:00:07:425 / camera(02) / imagedataout /  -4801.00000 para top(01)

Mensaje D / 00:00:07:425 / camera(02) / 00:00:00:000 para top(01)

Mensaje X / 00:00:07:425 / top(01) / imagein /  -4801.00000 para framegrabber(03)

Mensaje X / 00:00:07:425 / framegrabber(03) / imagein /  -4801.00000 para im_acq(05)

Mensaje D / 00:00:07:425 / im_acq(05) / 00:00:00:000 para framegrabber(03)

Mensaje D / 00:00:07:425 / framegrabber(03) / 00:00:00:000 para top(01)

Mensaje D / 00:00:07:425 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:07:425 / Root(00) para top(01)

Mensaje * / 00:00:07:425 / top(01) para camera(02)

Mensaje D / 00:00:07:425 / camera(02) / 00:00:00:033 para top(01)

Mensaje D / 00:00:07:425 / top(01) / 00:00:00:000 para Root(00)

Mensaje * / 00:00:07:425 / Root(00) para top(01)

Mensaje * / 00:00:07:425 / top(01) para framegrabber(03)

Mensaje * / 00:00:07:425 / framegrabber(03) para im_acq(05)

Mensaje Y / 00:00:07:425 / im_acq(05) / imageout /  -4801.00000 para framegrabber(03)

Mensaje D / 00:00:07:425 / im_acq(05) / ... para framegrabber(03)

Mensaje X / 00:00:07:425 / framegrabber(03) / imagein /  -4801.00000 para interface(04)

Mensaje D / 00:00:07:425 / interface(04) / ... para framegrabber(03)

Mensaje D / 00:00:07:425 / framegrabber(03) / ... para top(01)

Mensaje D / 00:00:07:425 / top(01) / 00:00:00:033 para Root(00)

Using a slow camera (4MHz pclk) and iis_slow_cam.bat with iis1.ev iis_slow_cam.out looks like:

00:00:02:000 respout -1

00:00:07:200 respout -4

00:00:07:418 imagebufout 4802

00:00:08:018 imagebufout -4802

And the relevant sections from iis_slow_cam.log show im_acq passing on both the start and end of the image:

Mensaje Y / 00:00:07:418 / im_acq(05) / imageout /   4802.00000 para framegrabber(03)

Mensaje Y / 00:00:07:420 / im_acq(05) / imageout /  -4802.00000 para framegrabber(03)

slow.cfg  and slowtrig.cfg will be used for all further tests of the IIS simulator.

To test the system with the camera in triggered mode iis_trig_cam.bat and iis2.ev are used.  iis2.ev  Sets up the frame grabber to generate triggers and continuously grab images.  Then it sends several triggers, each of which should get a corresponding image.

iis2.ev

00:00:00:020	cmndIn		76818		set im_acq for a 300KB buffer

00:00:00:100	cmndIn		785		send next grab(s) directly to host

00:00:00:300	cmndIn		12820		set timer active (50msec)

00:00:00:500	cmndIn		15396		set timer delay (60msec)

00:00:00:700	cmndIn 		68		start timer in triggered mode

00:00:01:000	cmndIn		50		set im_acq to continuous grab

00:00:01:050	triggerIn	1			generate an image 60msec after this

00:00:02:000	triggerIn	1			generate an image 60msec after this

00:00:02:300	triggerIn	1			generate an image 60msec after this

00:00:02:305	triggerIn	1			too close to other trig, ignore

00:00:03:000	cmndIn		66		Stop grabbing

00:00:03:200	cmndIn		273		send next grab to a storage buffer

00:00:03:700	triggerIn	1			generate an image but don't grab

00:00:04:000	cmndIn		76819		create a storage buffer, 300KB

00:00:04:250	cmndIn		34		grab the next image

00:00:04:440	triggerIn	1			generate the next image in 60msec

00:00:06:000	cmndIn		67		transfer the image to host

iis_trig_cam.out

00:00:01:175 imagebufout 4801		First grab, starts at trig+60msec+exposureTime

00:00:01:204 imagebufout -4801

00:00:02:125 imagebufout 4802		Second grab

00:00:02:154 imagebufout -4802

00:00:02:425 imagebufout 4801		Third grab

00:00:02:454 imagebufout -4801

00:00:03:000 respout 1			continuous grab aborted

00:00:06:000 imagebufout 4801		image retrieved from buffer storage

00:00:06:029 imagebufout -4801

This matches what was expected.  3 grabs, one trigger ignored because it was too close to another one, and aborted grab and a retrieved buffer.  All the times match what was expected given the exposure time of (1/30) seconds, transmission time of 2 msec, and internal and external bus speeds of 10,000 KB/s

To test the system with the camera in continuous mode iis_cont_cam.bat and iis3.ev are used.  iis3.ev is similar to iis2.ev, but spends a shorter time with the frame grabber in continuous grabbing because of the large number of images the camera generates.

iis3.ev

00:00:00:020	cmndIn		76818		set im_acq for a 300KB buffer

00:00:00:100	cmndIn		785		send next grab(s) directly to host

00:00:00:300	cmndIn		12820		set timer active (50msec)

00:00:00:500	cmndIn		15396		set timer delay (60msec)

00:00:00:700	cmndIn 		68		start timer in triggered mode

00:00:01:000	cmndIn		50		set im_acq to continuous grab

00:00:01:050	triggerIn	1

00:00:02:025	cmndIn		34		stop im_acq and grab one last image

00:00:02:500	cmndIn		76819		create a buffer in storage

00:00:02:700	cmndIn		273		send the next grab to storage

00:00:03:200	triggerIn	1

00:00:03:350	cmndIn		34		grab the next image

00:00:03:540	triggerIn	1

00:00:04:124	cmndIn		67		retrieve the stored image

iis_cont_cam.out

00:00:01:013 imagebufout 4801		Start grabbing first image after grab command

00:00:01:042 imagebufout -4801

00:00:01:048 imagebufout 4802

00:00:01:077 imagebufout -4802

00:00:01:083 imagebufout 4801

00:00:01:112 imagebufout -4801

00:00:01:118 imagebufout 4802

00:00:01:147 imagebufout -4802

00:00:01:153 imagebufout 4801

00:00:01:182 imagebufout -4801

00:00:01:188 imagebufout 4802

00:00:01:217 imagebufout -4802

00:00:01:223 imagebufout 4801

00:00:01:252 imagebufout -4801

00:00:01:258 imagebufout 4802

00:00:01:287 imagebufout -4802

... cut imagebufout for space ..

00:00:01:923 imagebufout 4801

00:00:01:952 imagebufout -4801

00:00:01:958 imagebufout 4802

00:00:01:987 imagebufout -4802

00:00:01:993 imagebufout 4801

00:00:02:022 imagebufout -4801	Last grab before single

00:00:02:028 imagebufout 4802		Single grab

00:00:02:057 imagebufout -4802

00:00:04:124 imagebufout 4802		Image retrieved from buffer

00:00:04:153 imagebufout -4802

Conclusion

The IIS model simulates the most basic behavior of an area scan digital camera and frame grabber system.  It generates image data according to the camera settings, it has a programmable timer that can be used to reshape trigger inputs, it can store image data on board, buffer it there temporarily during a grab or send it directly to the host system.

However, there is an issue with regards to the transmission time of an image from the camera.  If the transmission time is too short (small image, high pixel clock) the image acquisition module will miss the end of image event.  Too small is defined as smaller than CD++'s time resolution (~1 msec).