# Conceptual Model Description

Modern aircraft avionics suites are composed of dozens of systems. This goal of this model is to model the interaction between two navigation systems, the Global Positioning System (GPS) and the Inertial Reference System (IRS). By sensing signals constantly transmitted by GPS satellites, a GPS receiver can determine its position in terms of latitude, longitude and elevation. An IRS measures its own acceleration very precisely in three axes, and that data is then used by the IRS to determine velocity and position estimates.

IRS systems measure forces and acceleration at a rate higher than a GPS calculates position. Some IRS systems have a hybrid feature that take GPS position as input and calculate and output position data at a rate high than that of the GPS. The IRS does this by using its acceleration measurements in addition to GPS position updates to calculate its position.

This model will have an IRS that contains three gyros, one for each of the x, y, and z axes, and an IRS computer that calculates positions. The IRS will also receive inputs from the GPS.

The IRS computer will have 4 states: OFF, ALIGN, NAV, HYBRID. Each of these states will also have a position (x,y,z), velocity (x,y,z), and acceleration (x,y,z) associated with it. When the IRS is turned on, it must first wait for its gyroscopes to spin up and for its built in tests to finish, this is the ALIGN state. After the ALIGN state the IRS enters the NAV state, where it uses its gyros to calculate displacement in 3D. The IRS enters HYBRID state from NAV state when it has had valid GPS inputs for at least 2 minutes. When in the NAV and HYBRID states, the IRS will output positions at 50Hz.

The gyros will have 3 states: OFF, SPINUP, ON. The gyros will transition from OFF to SPINUP when powered on, and will transition to ON at a time that is a normal random variable with mean 0.5 minutes and variance 0.25 minutes. When in the ON state, the gyros will output accelerations at 100 Hz. The gyro’s state will also include an acceleration value which indicates the currently sensed acceleration.

The GPS will have 3 states: OFF, ACQ, NAV. There will be an (x,y,z) position included in the state. The GPS system will transition from OFF to ACQ when powered on. When in the ACQ state, the GPS is acquiring GPS signals and it will stay in this state for a time that is a normal random variable with mean 4 and variance 2. When transitioning out of the ACQ state, the GPS will enter the NAV state, where it will output its current latitude, longitude, and elevation at 10 Hz.

There is also a Power model that will either be OFF or ON and will turn on after a random amount of time and send the POWER signal to the GPS and IRS.

# Formal Specifications & Test Strategy

## Coupled Model Specifications

### Navigation Avionics

X = {POWER}

Y = {IRS_POS}

D = {GPS, IRS}

EIC = { (NavigationAvionics.power, GPS.power), (NavigationAvionics.power, IRS.power)}

EOC = { (IRS.IRS_position , NavigationAvionics.IRS_position) }

IC = { (GPS.GPS_position, IRS.GPS_position) }

SELECT : 1. IRS, 2. GPS

### IRS

X: {POWER,GPS_POS}

Y: {IRS_POS}

D : {IRS_COMPUTER, GYRO_X, GYRO_Y, GYRO_Z}

I(GYRO_X) = {IRS_COMPUTER}

I(GYRO_Y) = {IRS_COMPUTER}

I(GYRO_Z) = {IRS_COMPUTER}

I(IRS_COMPUTER) = ø

SELECT: 1. IRS, 2. GYRO_X, 3. GYRO_Y, 4. GYRO_Z

## Atomic Model Specifications

### GPS

S = {OFF, ACQ, NAV}

X = {POWER}

Y = {GPS_POSITION)

dint(NAV) = NAV, also update ‘sensed’ GPS position

dint(ACQ) = NAV

dext(POWER=1,S=OFF) = ACQ

dext(POWER=0) = OFF

λ(NAV) : output internal representation of GPS coordinates

ta(ACQ): STDEV(4,2) sec

ta(NAV): 0.1 sec

### Gyro

S = {OFF, SPINUP, ON}

X = {POWER}

Y = {ACCEL)

dint(SPINUP) = ON

dint(ON) = ON, also update ‘sensed’ acceleration of gyro

dext(POWER=1,S=OFF) = SPINUP

dext(POWER=0) = OFF

λ(ON) : output sensed acceleration value

ta(ACQ): STDEV(0.5,0.25) sec

ta(NAV): 0.01 sec (100Hz)

### IRS Computer

S = {OFF, ALIGN, NAV, PRE_HYBRID, HYBRID, ON}

X = {POWER, GPS_POSITION, X_ACCEL, Y_ACCEL, Z_ACCEL}

Y = {IRS_POSITION)

dint(ALIGN) = NAV

dint(NAV) = NAV

dint(PRE_HYBRID& have continuous GPS updates for 2 minutes) = HYBRID

dint(PRE_HYBRID & dropped a GPS update) = NAV

dint(HYBRID & still have continuous GPS updates) = HYBRID

dint(HYBRID & dropped a GPS update) = NAV

dext(POWER=1,S=OFF) = ALIGN

dext(POWER=0) = OFF

dext(GPS_POSN, S=NAV) = PRE_HYBRID, update IRS_POSITION using GYRO data

dext(GPS_POSN, S=PRE_HYBRID for 2 minutes) = HYBRID, resync IRS position to GPS position

dext(GPS_POSN, S=HYBRID) = HYBRID, , resync IRS position to GPS position

dext(POWER=1,S=OFF) = ALIGN

dext(X_ACCEL) = update IRS coordinates by adding function of acceleration in x direction

dext(Y_ACCEL) = update IRS coordinates by adding function of acceleration in y direction

dext(Z_ACCEL) = update IRS coordinates by adding function of acceleration in z direction

λ(NAV, HYBRID, & PRE_HYBRID): output IRS_POSITION

ta(ALIGN): STDEV(30,10) sec

ta(PRE_HYBRID): 2 min

## Testing Strategy

Each of the atomic models and coupled models should produce outputs once power has been turned on and they have waited the required amount of time to transition between states.

The testing strategy will be to turn the power on and off at times that are both before and after the required spin up and alignment time to verify that the models do in fact wait for the correct state to being outputting their data.

Once each model is tested independently, they will be integrated and the testing will continue in the same fashion. When all the models are integrated and producing outputs a regression test will be performed by decoupling the power inputs between the models and having power from one or more of the units removed, and the resulting system behaviour analysed.

# Model Implementation & Simulation Results

## Differences Between Model & Spec

The major difference between the final implementation of the model and the spec is that the PRE_HYBRID state in the IRS_COMPUTER is not used and that a state called IRS_STATE_ALIGN_WAITING_FOR_GYROS is used.

It did not makes sense to have a state for PRE_HYRBID. PRE_HYBRID would have been defined as:

- In NAV state

- Had consecutive GPS inputs for less than 2 minutes

Also, any time a GPS update was missed, the IRS would have to drop out of PRE_HYBRID into NAV, but it is not possible to have a state transition based on a lack of an event.

Instead, the transition from NAV to HYBRID was taken care of by counting the amount of time GPS inputs had been received in the external transition function for GPS inputs, and then checking for that time to be greater than 2 minutes in the NAV internal transition function. The same check was used in the HYBRID internal transition function to drop back to NAV is a GPS input was missed. The HYBRID and NAV time advances are small so having the checks here for missing a GPS input made sense.

An intermediate state was needed to wait for all the gyros to spin up, this time it was possible to use a state for this because we were no longer looking for a non-event, we were waiting for each of the gyros to start producing outputs. This was not simply an oversight in how things were to be implemented, but a mistake in the formal specification. The formal specification states:

dint(ALIGN) = NAV

whereas the textual specification states:

When the IRS is turned on, it must first wait for its gyroscopes to spin up and for its built in tests to finish, this is the ALIGN state.

So to move from the ALIGN to NAV state it’s not just a case of time passing, but of time passing and then seeing all 3 gyros producing data. So instead, the internal transitions look like this:

dint(ALIGN) = ALIGN_WAITING_FOR_GYROS

dext(GYRO_*) = add new check for state is ALIGN_WAITING_FOR_GYROS and all gyros now producing then S = NAV, else S = unchanged.

The last discrepancy between the model implementation and the specification is that wherever a sample distribution was called for, for example in the spinup time of the gyro, a static value was instead loaded from the .ma file. This was done mainly for debugging purposes, the timing of the state changes was used to validate the model, and also because of time constraints.

Also note that the times states in the specification for the various spinup and acquire state times were not used in the tests, this was an omission and had time allowed the tests would have been rerun with the values stated in the specification.

## Test & Simulation Results

### Gyro

The gyro test consisted of sending power on port_in_power 1 and power off  port_in_power 0 inputs to the Gyro atomic model and examining the resulting model outputs.

The gyro.ev event file is reproduced in Figure 1.

00:00:00:000 port_in_power 1

00:00:05:000 port_in_power 0

00:00:10:000 port_in_power 1

00:00:32:000 port_in_power 0

01:09:52:000 port_in_power 1

01:10:30:000 port_in_power 0

01:22:00:000 port_in_power 1

01:22:01:000 port_in_power 1

01:22:02:000 port_in_power 1

01:22:21:000 port_in_power 1

01:22:23:000 port_in_power 0

01:22:25:000 port_in_power 0

Figure 1 - Gyro.ev Input File

The gyro is a simple model, starting to produce outputs every 10 milliseconds 20 seconds after power is applied. The gyro model’s output rules are simple, start at 0 and increment by 1 each time step.

We expect no outputs until 30 seconds into the simulation, and that is what the first line of our result file shows in Figure 2.  No outputs are expected for the first power on/off cycle because the 20 second spinup time is not reached before the power is turned off.

00:00:30:010 port_out_accel 0

00:00:30:020 port_out_accel 1

00:00:30:030 port_out_accel 2

00:00:30:040 port_out_accel 3

00:00:30:050 port_out_accel 4

...

00:00:31:950 port_out_accel 194

00:00:31:960 port_out_accel 195

00:00:31:970 port_out_accel 196

00:00:31:980 port_out_accel 197

00:00:31:990 port_out_accel 198

01:10:12:010 port_out_accel 199

01:10:12:020 port_out_accel 200

01:10:12:030 port_out_accel 201

01:10:12:040 port_out_accel 202

01:10:12:050 port_out_accel 203

...

01:10:29:950 port_out_accel 1993

01:10:29:960 port_out_accel 1994

01:10:29:970 port_out_accel 1995

01:10:29:980 port_out_accel 1996

01:10:29:990 port_out_accel 1997

01:22:20:010 port_out_accel 1998

01:22:20:020 port_out_accel 1999

01:22:20:030 port_out_accel 2000

01:22:20:040 port_out_accel 2001

01:22:20:050 port_out_accel 2002

...

01:22:22:950 port_out_accel 2292

01:22:22:960 port_out_accel 2293

01:22:22:970 port_out_accel 2294

01:22:22:980 port_out_accel 2295

01:22:22:990 port_out_accel 2296

Figure 2 - Gyro Output File

The second set of outputs start at 01:10:12:010 , which is 20 seconds (and one output increment) after the power is applied at 01:09:52:000, again showing correct functionality.

The simulation results also show correct functionality when the power is turned off and the gyro was producing outputs, it stops. The times power was shut off are 00:00:32:000 , 01:10:30:000, and 01:22:23:000 as seen in Figure 1 and the outputs can be seen to have been stopped at those times in Figure 2.

The latter half of Figure 1 shows multiple power on then off signals, and the Gyro result file shows no ill effects as a result of these extraneous inputs.

### GPS

The GPS atomic model is very similar to the gyro model, with the difference being the GPS model has 3 output ports for the X,Y,Z coordinates it outputs. The GPS outputs also follow a simple rule starting at a position of (100,200,300) and having a vector of (1,-1,2) added at each time step. The simulation inputs can be seen in Figure 3 and the resulting outputs in Figure 4. Again, as was the case in the gyro simulation results, the model waits properly for it’s acquisition time (20 seconds) and produced outputs after that time at the proper rate. It can be seen in Figure 4 that at each output time the outputs are produced on each of the three output ports.

00:00:10:000 port_in_power 1

00:00:32:000 port_in_power 0

01:09:52:000 port_in_power 1

01:11:00:000 port_in_power 0

01:22:00:000 port_in_power 1

01:22:01:000 port_in_power 1

01:22:02:000 port_in_power 1

01:22:21:000 port_in_power 1

01:22:23:000 port_in_power 0

01:22:25:000 port_in_power 0

Figure 3 - GPS.ev Input File

00:00:30:100 port_out_gps_x 100

00:00:30:100 port_out_gps_y 200

00:00:30:100 port_out_gps_z 300

00:00:30:200 port_out_gps_x 101

00:00:30:200 port_out_gps_y 199

00:00:30:200 port_out_gps_z 302

00:00:30:300 port_out_gps_x 102

00:00:30:300 port_out_gps_y 198

00:00:30:300 port_out_gps_z 304

00:00:30:400 port_out_gps_x 103

00:00:30:400 port_out_gps_y 197

00:00:30:400 port_out_gps_z 306

...

00:00:31:800 port_out_gps_x 117

00:00:31:800 port_out_gps_y 183

00:00:31:800 port_out_gps_z 334

00:00:31:900 port_out_gps_x 118

00:00:31:900 port_out_gps_y 182

00:00:31:900 port_out_gps_z 336

01:10:12:100 port_out_gps_x 119

01:10:12:100 port_out_gps_y 181

01:10:12:100 port_out_gps_z 338

01:10:12:200 port_out_gps_x 120

01:10:12:200 port_out_gps_y 180

01:10:12:200 port_out_gps_z 340

01:10:12:300 port_out_gps_x 121

01:10:12:300 port_out_gps_y 179

01:10:12:300 port_out_gps_z 342

...

01:10:59:700 port_out_gps_x 595

01:10:59:700 port_out_gps_y -295

01:10:59:700 port_out_gps_z 1290

01:10:59:800 port_out_gps_x 596

01:10:59:800 port_out_gps_y -296

01:10:59:800 port_out_gps_z 1292

01:10:59:900 port_out_gps_x 597

01:10:59:900 port_out_gps_y -297

01:10:59:900 port_out_gps_z 1294

01:22:20:100 port_out_gps_x 598

01:22:20:100 port_out_gps_y -298

01:22:20:100 port_out_gps_z 1296

01:22:20:200 port_out_gps_x 599

01:22:20:200 port_out_gps_y -299

01:22:20:200 port_out_gps_z 1298

01:22:20:300 port_out_gps_x 600

01:22:20:300 port_out_gps_y -300

01:22:20:300 port_out_gps_z 1300

...

01:22:22:800 port_out_gps_x 625

01:22:22:800 port_out_gps_y -325

01:22:22:800 port_out_gps_z 1350

01:22:22:900 port_out_gps_x 626

01:22:22:900 port_out_gps_y -326

01:22:22:900 port_out_gps_z 1352

Figure 4 - GPS Output File

### IRS Computer

The IRS computer produces outputs if it has power for more than 2 minutes and has inputs coming from the gyros. Each gyro input is added to the IRS position to produce a new IRS position. If the IRS has also been receiving GPS inputs for 2 minutes (changed to 5 seconds to reduce the size of the output files), the IRS will resync it’s position to the last position provided by the GPS each time it receives an input from the GPS as well as use the gyro acceleration values to update the position in between GPS updates.

To test the IRS computer independently of the other atomic models as significant number of GPS inputs were needed for the IRS_computer.ev file. These were produced using a simple perl script, gps_output.pl, which is included in the zip file for this project. This file simply increments time and sends static GPS positions to the IRS’s GPS input ports.

Figure 5 shows the test input set for the IRS computer model.

00:00:10:000 port_in_power 1

00:00:32:000 port_in_power 0

01:09:52:000 port_in_power 1

01:13:00:000 port_in_power 0

01:15:39:000 port_in_power 1

01:19:00:000 port_in_gyro_x 1

01:19:00:000 port_in_gyro_y 1

01:19:00:000 port_in_gyro_z 1

01:19:00:000 port_in_gps_x 5

01:19:00:000 port_in_gps_y 6

01:19:00:000 port_in_gps_z 7

01:19:00:100 port_in_gps_x 5

01:19:00:100 port_in_gps_y 6

01:19:00:100 port_in_gps_z 7

01:19:00:200 port_in_gps_x 5

01:19:00:200 port_in_gps_y 6

01:19:00:200 port_in_gps_z 7

01:19:00:300 port_in_gps_x 5

01:19:00:300 port_in_gps_y 6

01:19:00:300 port_in_gyro_x 1

01:19:00:300 port_in_gyro_y 1

01:19:00:300 port_in_gyro_z 1

01:19:00:300 port_in_gps_z 7

01:19:00:400 port_in_gps_x 5

01:19:00:400 port_in_gps_y 6

01:19:00:400 port_in_gps_z 7

01:19:00:500 port_in_gps_x 5

01:19:00:500 port_in_gps_y 6

01:19:00:500 port_in_gps_z 7

...

01:19:03:200 port_in_gps_x 5

01:19:03:200 port_in_gps_y 6

01:19:03:200 port_in_gps_z 7

01:19:03:300 port_in_gps_x 5

01:19:03:300 port_in_gps_y 6

01:19:03:300 port_in_gps_z 7

01:19:03:300 port_in_gyro_x 1

01:19:03:300 port_in_gyro_y 1

01:19:03:300 port_in_gyro_z 1

01:19:03:400 port_in_gps_x 5

01:19:03:400 port_in_gps_y 6

01:19:03:400 port_in_gps_z 7

01:19:03:500 port_in_gps_x 5

01:19:03:500 port_in_gps_y 6

01:19:03:500 port_in_gps_z 7

01:19:06:600 port_in_gps_x 5

01:19:06:600 port_in_gps_y 6

01:19:06:600 port_in_gps_z 7

01:19:06:700 port_in_gps_x 5

01:19:06:700 port_in_gps_y 6

01:19:06:700 port_in_gps_z 7

01:19:06:800 port_in_gps_x 5

01:19:06:800 port_in_gps_y 6

01:20:00:000 port_in_power 0

01:21:00:000 port_in_power 1

01:25:00:000 port_in_gyro_x 1

01:25:00:000 port_in_gyro_y 1

01:25:00:000 port_in_gyro_z 1

Figure 5 - IRS_computer.ev Input File

We expect output after immediately following the 3 gyro inputs at  01:19:00:000 because at that time the power has been on for two minutes,  since 01:15:39:000. This can be seen to be the case in the first line of the output file in Figure 6.

The GPS inputs at the beginning of the input file should have no effect because the IRS computer only takes them into account after they have been around for 5 seconds.

The next event of interest is at 01:19:00:300 where each gyro sends another value of 1. Figure 6 correctly shows an increase in the position output of the IRS by a value of 1 in each of its coordinates.

At 01:19:05:100, the GPS inputs start to be used to sync the IRS position, 5 seconds after they were first sensed by the IRS.

At 01:20:00:000 the power is turned off. This is reflected in the output file by seeing the last IRS output around that time being made at 01:19:59:980.

The power is again turned on at 01:21:00:000, but outputs do not appear until 01:25:00:020. This is as per design because the IRS must see inputs from the gyros at some point following power on (in addition to waiting 2 minutes) before it starts sending it’s position on the output ports. The 3 gyros sent inputs to the IRS at 01:25:00:000, so seeing the first IRS output after the internal NAV state transition time (00:00:00:020) is correct

01:19:00:020 port_out_irs_x 1

01:19:00:020 port_out_irs_y 2

01:19:00:020 port_out_irs_z 3

01:19:00:040 port_out_irs_x 1

01:19:00:040 port_out_irs_y 2

01:19:00:040 port_out_irs_z 3

01:19:00:060 port_out_irs_x 1

01:19:00:060 port_out_irs_y 2

01:19:00:060 port_out_irs_z 3

...

01:19:00:280 port_out_irs_x 1

01:19:00:280 port_out_irs_y 2

01:19:00:280 port_out_irs_z 3

01:19:00:300 port_out_irs_x 2

01:19:00:300 port_out_irs_y 3

01:19:00:300 port_out_irs_z 4

01:19:00:320 port_out_irs_x 2

01:19:00:320 port_out_irs_y 3

01:19:00:320 port_out_irs_z 4

01:19:00:340 port_out_irs_x 2

01:19:00:340 port_out_irs_y 3

01:19:00:340 port_out_irs_z 4

...

01:19:05:060 port_out_irs_x 3

01:19:05:060 port_out_irs_y 4

01:19:05:060 port_out_irs_z 5

01:19:05:080 port_out_irs_x 3

01:19:05:080 port_out_irs_y 4

01:19:05:080 port_out_irs_z 5

01:19:05:100 port_out_irs_x 5

01:19:05:100 port_out_irs_y 6

01:19:05:100 port_out_irs_z 7

01:19:05:120 port_out_irs_x 5

01:19:05:120 port_out_irs_y 6

01:19:05:120 port_out_irs_z 7

01:19:05:140 port_out_irs_x 5

01:19:05:140 port_out_irs_y 6

01:19:05:140 port_out_irs_z 7

...

01:19:59:940 port_out_irs_x 5

01:19:59:940 port_out_irs_y 6

01:19:59:940 port_out_irs_z 7

01:19:59:960 port_out_irs_x 5

01:19:59:960 port_out_irs_y 6

01:19:59:960 port_out_irs_z 7

01:19:59:980 port_out_irs_x 5

01:19:59:980 port_out_irs_y 6

01:19:59:980 port_out_irs_z 7

01:25:00:020 port_out_irs_x 5

01:25:00:020 port_out_irs_y 6

01:25:00:020 port_out_irs_z 7

01:25:00:040 port_out_irs_x 5

01:25:00:040 port_out_irs_y 6

01:25:00:040 port_out_irs_z 7

01:25:00:060 port_out_irs_x 5

01:25:00:060 port_out_irs_y 6

01:25:00:060 port_out_irs_z 7

Figure 6 - IRS Computer Outputs

### IRS

The IRS coupled model includes the IRS computer and the 3 gyros. The outputs are the same as the IRS computer, and inputs are the same as the IRS computer except the gyros now provide their own outputs.

The input file used to test the IRS coupled model is show in Figure 7.

00:00:10:000 port_in_power 1

00:00:32:000 port_in_power 0

01:09:52:000 port_in_power 1

01:13:00:000 port_in_power 0

01:15:39:000 port_in_power 1

01:19:00:000 port_in_gps_x 5

01:19:00:000 port_in_gps_y 6

01:19:00:000 port_in_gps_z 7

...

01:19:06:500 port_in_gps_x 5

01:19:06:500 port_in_gps_y 6

01:19:06:500 port_in_gps_z 7

01:19:06:600 port_in_gps_x 5

01:19:06:600 port_in_gps_y 6

01:19:06:600 port_in_gps_z 7

01:19:06:700 port_in_gps_x 5

01:19:06:700 port_in_gps_y 6

01:19:06:700 port_in_gps_z 7

01:19:06:800 port_in_gps_x 5

01:19:06:800 port_in_gps_y 6

01:20:00:000 port_in_power 0

01:21:00:000 port_in_power 1

01:30:00:000 port_in_power 0

Figure 7 - IRS.ev Input File

Note: the way in which the gyro’s output values was changed when the test results were generated. Simply incrementing the output value of the gyro by 1 at every step made the IRS position values explode. Instead, each time the new gyro value was generated, a coin was flipped and the value was either increased by one or decreased by one.

The simulation results are shown in 01:11:52:020 irs_x_pos 1.

01:11:52:020 irs_x_pos 1

01:11:52:020 irs_y_pos 0

01:11:52:020 irs_z_pos 1

01:11:52:040 irs_x_pos -1

01:11:52:040 irs_y_pos -2

01:11:52:040 irs_z_pos 1

...

01:12:59:960 irs_x_pos 41

01:12:59:960 irs_y_pos 88

01:12:59:960 irs_z_pos -83

01:12:59:980 irs_x_pos 41

01:12:59:980 irs_y_pos 88

01:12:59:980 irs_z_pos -85

01:17:39:020 irs_x_pos 44

01:17:39:020 irs_y_pos 89

01:17:39:020 irs_z_pos -84

01:17:39:040 irs_x_pos 44

01:17:39:040 irs_y_pos 91

01:17:39:040 irs_z_pos -84

01:17:39:060 irs_x_pos 44

01:17:39:060 irs_y_pos 93

01:17:39:060 irs_z_pos -86

...

01:17:39:080 irs_x_pos 44

01:17:39:080 irs_y_pos 95

01:17:39:080 irs_z_pos -86

01:17:39:100 irs_x_pos 46

01:17:39:100 irs_y_pos 95

01:17:39:100 irs_z_pos -84

01:17:39:120 irs_x_pos 48

01:17:39:120 irs_y_pos 97

01:17:39:120 irs_z_pos -84

...

01:19:05:060 irs_x_pos 10

01:19:05:060 irs_y_pos 79

01:19:05:060 irs_z_pos -60

01:19:05:080 irs_x_pos 10

01:19:05:080 irs_y_pos 77

01:19:05:080 irs_z_pos -60

01:19:05:100 irs_x_pos 6

01:19:05:100 irs_y_pos 7

01:19:05:100 irs_z_pos 8

01:19:05:120 irs_x_pos 6

01:19:05:120 irs_y_pos 5

01:19:05:120 irs_z_pos 8

01:19:05:140 irs_x_pos 6

01:19:05:140 irs_y_pos 7

01:19:05:140 irs_z_pos 8

01:19:05:160 irs_x_pos 8

01:19:05:160 irs_y_pos 5

01:19:05:160 irs_z_pos 10

01:19:05:180 irs_x_pos 10

01:19:05:180 irs_y_pos 7

01:19:05:180 irs_z_pos 10

01:19:05:200 irs_x_pos 6

01:19:05:200 irs_y_pos 7

01:19:05:200 irs_z_pos 8

01:19:05:220 irs_x_pos 6

01:19:05:220 irs_y_pos 5

01:19:05:220 irs_z_pos 10

01:19:05:240 irs_x_pos 6

01:19:05:240 irs_y_pos 5

01:19:05:240 irs_z_pos 12

01:19:05:260 irs_x_pos 6

01:19:05:260 irs_y_pos 5

01:19:05:260 irs_z_pos 10

01:19:05:280 irs_x_pos 6

01:19:05:280 irs_y_pos 5

01:19:05:280 irs_z_pos 10

01:19:05:300 irs_x_pos 6

01:19:05:300 irs_y_pos 7

01:19:05:300 irs_z_pos 8

01:19:05:320 irs_x_pos 4

01:19:05:320 irs_y_pos 9

01:19:05:320 irs_z_pos 6

01:19:05:340 irs_x_pos 2

01:19:05:340 irs_y_pos 11

01:19:05:340 irs_z_pos 6

01:19:05:360 irs_x_pos 0

01:19:05:360 irs_y_pos 9

01:19:05:360 irs_z_pos 6

...

01:19:59:960 irs_x_pos 22

01:19:59:960 irs_y_pos 27

01:19:59:960 irs_z_pos 28

01:19:59:980 irs_x_pos 24

01:19:59:980 irs_y_pos 25

01:19:59:980 irs_z_pos 28

01:23:00:020 irs_x_pos 23

01:23:00:020 irs_y_pos 24

01:23:00:020 irs_z_pos 29

01:23:00:040 irs_x_pos 21

01:23:00:040 irs_y_pos 22

01:23:00:040 irs_z_pos 29

01:23:00:060 irs_x_pos 21

01:23:00:060 irs_y_pos 24

01:23:00:060 irs_z_pos 29

...

01:29:59:960 irs_x_pos -129

01:29:59:960 irs_y_pos 374

01:29:59:960 irs_z_pos 303

01:29:59:980 irs_x_pos -127

01:29:59:980 irs_y_pos 374

01:29:59:980 irs_z_pos 305

Figure 8 - IRS Simulation Results

01:11:52:020 irs_x_pos 1 shows that the IRS starts to produce outputs 2 minutes and one Navigation State internal delay after it receives power (for the second time at 01:09:52:000), as per design. The power being turned on and off before this time has no effect on the IRS output because the IRS did not have enough time to align. The power is turned off at 01:13:00:000 so there are no position outputs at that time, the last is at 01:12:59:980, again, by design.

The IRS is turned on one again at 01:15:39:000 and the outputs begin 2 minutes and one Navigation State internal transition later. At 01:19:00:000 the GPS begins sending inputs to the IRS, although the IRS ignores them until 01:19:05:100, which greater than 5 seconds after the GPS started sending inputs, which is the rule by which the IRS starts using the GPS inputs to sync it’s position. Note that the GPS is always sending (5,6,7) as it’s position, and in between these re-syncs which occur every tenth of a second, the position is being updated by the randomly generated gyro acceleration, so this is working as designed.

The power is turned off at 01:20:00:000 and as per design the last output happens one step before that, at 01:19:59:980.

The power is turned on at  01:21:00:000, positions being to be output by the IRS at 01:23:00:000 and continue until power is turned off at 01:30:00:000. The outputs during this time period are only being changed by the random fluctuations in gyro output, as no GPS messages are being sent.

### Navigation Avionics

Once all the models are coupled in the top level Navigation Avionics file, the only inputs needed are to power the units, as seen in Figure 9.

00:00:10:000 port_in_power 1

00:00:32:000 port_in_power 0

01:09:52:000 port_in_power 1

01:13:00:000 port_in_power 0

01:15:39:000 port_in_power 1

01:20:00:000 port_in_power 0

Figure 9 - Navigation Avionics Input File

As in the previous cases, the IRS produces output only 2 minutes after power is received, and only uses the GPS inputs to sync its position once it has been receiving GPS inputs for 2 minutes – with the internal transition times also added. This can be seen in .

01:11:52:020 irs_x_pos 1

01:11:52:020 irs_y_pos 2

01:11:52:020 irs_z_pos 5

01:11:52:040 irs_x_pos 1

01:11:52:040 irs_y_pos 2

01:11:52:040 irs_z_pos 7

01:11:52:060 irs_x_pos 1

01:11:52:060 irs_y_pos 2

01:11:52:060 irs_z_pos 5

01:11:52:080 irs_x_pos -1

01:11:52:080 irs_y_pos 0

01:11:52:080 irs_z_pos 3

01:11:52:100 irs_x_pos -3

01:11:52:100 irs_y_pos -2

01:11:52:100 irs_z_pos 3

01:11:52:120 irs_x_pos 1119

01:11:52:120 irs_y_pos -819

01:11:52:120 irs_z_pos 2338

01:11:52:140 irs_x_pos 1119

01:11:52:140 irs_y_pos -821

01:11:52:140 irs_z_pos 2338

...

01:12:59:960 irs_x_pos 1795

01:12:59:960 irs_y_pos -1497

01:12:59:960 irs_z_pos 3690

01:12:59:980 irs_x_pos 1793

01:12:59:980 irs_y_pos -1499

01:12:59:980 irs_z_pos 3692

01:17:39:020 irs_x_pos 1796

01:17:39:020 irs_y_pos -1500

01:17:39:020 irs_z_pos 3693

01:17:39:040 irs_x_pos 1794

01:17:39:040 irs_y_pos -1500

01:17:39:040 irs_z_pos 3695

...

01:17:39:060 irs_x_pos 1796

01:17:39:060 irs_y_pos -1500

01:17:39:060 irs_z_pos 3695

01:17:39:080 irs_x_pos 1794

01:17:39:080 irs_y_pos -1502

01:17:39:080 irs_z_pos 3697

...

01:17:39:100 irs_x_pos 1794

01:17:39:100 irs_y_pos -1504

01:17:39:100 irs_z_pos 3695

01:17:39:120 irs_x_pos 2796

01:17:39:120 irs_y_pos -2496

01:17:39:120 irs_z_pos 5694

...

01:19:59:960 irs_x_pos 4202

01:19:59:960 irs_y_pos -3908

01:19:59:960 irs_z_pos 8514

01:19:59:980 irs_x_pos 4202

01:19:59:980 irs_y_pos -3908

01:19:59:980 irs_z_pos 8516

Figure 10 - Navigation Avionics Output File