Carleton University

Systems and Computer Engineering Department

SYSC-5807

Assignment 2:Pattern

To: Professor Wainer

Student: Ping Wang

Student No. 100633744

Part1

# Description

By searching about Cellular Automata, I am going to simulate the growing process of some complex patterns as in Fig. 1 by using two-dimension model.  A pattern grows up from a cell or some cells. We use outer-totalistic rules to generate these patterns which means a cell’s next value is determined by its current’s value and its neighbour’s value. I am going to generate Fig.1 and Fig. 2 with a five-neighbour and nine-neighbour Cell-DEVS respectively

Fig. 1                            Fig. 2

Part 2

# Formal specification for the coupled Cell-DEVS model

## Generate Fig. 1 pattern

To generate Fig.1, we will define five neighbours for a cell. Each cell has value 0 or 1. The definition is as in Fig. 3. The red cell stands for self, the green cells stand for neighbours

The rule is:

- If the cell’s current value is 0 and only one neighbour’s value is 1, the cell’s next value is 1

- If the cell’s current value is 0 and three neighbour’s value is 1, the cell’s next value is 1

- If the cell’s current value is 0 and two neighbour’s value is 1 or if the cell’s current value is 1 and two neighbour’s value is 1, the cell’s next value is 1

- otherwise, the cell’s next value is 0

Fig. 3

The following specification is for five neighbours

M=<Xlist,Ylist,I,X,Y,N,η,{f,c},C,B,Z,select>

Xlist={ Φ }

Ylist={ Φ }

X=Y={ Φ }

η =5

I=<Px,Py> with Px={ Φ }, Py={ Φ }

N={(0,0),(0,-1),(0,1),(-1,0),(1,0)}

f=20

c=20;

C={Ci,j /  i ε [1,20], j ε [1,20]

B={ Φ } the cell space is wrapped

Z:

The inverse neighbourhood list: {(0,0),(0,1),(0,-1),(1,0),(-1,0)}

Pi,j Y1->Pi,j X1                                            Pi,jX1<-Pi,j Y1

Pi,j Y2->Pi,j-1 X2                                         Pi,j X2<-Pi,j+1 Y2

Pi,j Y3->Pi,j+1 X3                                         Pi,j X3<-Pi,j-1 Y3

Pi,j Y4->Pi-1,j X4                                         Pi,j X4<-Pi+1,j Y4

Pi,j Y5->Pi+1,j X5                                        Pi,j X5<-Pi-1,j Y5

Select={(0,0),(0,-1),(0,1),(-1,0),(1,0)}

## Generate Fig. 2 pattern

To generate Fig.2, we will define nine neighbours for a cell. Each cell has value 0 or 1. The definition is as in Fig. 4. The red cell stands for self, the green cells stand for neighbours

The rule is:

- If the cell’s current value is 0 and only one neighbour’s value is 1, the cell’s next value is 1

- If the cell’s current value is 1 and three neighbour’s value is 1, the cell’s next value is 1

- If the cell’s current value is 0 and four neighbour’s value is 1, the cell’s next value is 1

- Otherwise, the cell’s next value is 0

Fig. 4

The following specification is for five neighbours

M=<Xlist,Ylist,I,X,Y,N,η,{f,c},C,B,Z,select>

Xlist={ Φ }

Ylist={ Φ }

X=Y={ Φ }

η =9

I=<Px,Py> with Px={ Φ }, Py={ Φ }

N={(-1,-1),(0,-1),(1,-1),(-1,0),(0,0),(1,0),(-1,1),(0,1),(1,1)}

f=20

c=20;

C={Ci,j /  I ε [1,20], j ε [1,20]

B={ Φ } the cell space is wrapped

Z:

The inverse neighbourhood list:{(1,1),(0,1),(-1,1),(1,0),(0,0),(-1,0),(1,-1),(0,-1),(-1,-1)}

Pi,j Y1->Pi-11,j-1 X1                                     Pi,jX1<-Pi+1,j+1 Y1

Pi,j Y2->Pi,j-1 X2                                         Pi,j X2<-Pi,j+1 Y2

Pi,j Y3->Pi+1,j-1 X3                                      Pi,j X3<-Pi-1,j+1 Y3

Pi,j Y4->Pi-1,j X4                                         Pi,j X4<-Pi+1,j Y4

Pi,j Y5->Pi,j X5                                           Pi,j X5<-Pi,j Y5

Pi,j Y6->Pi+1,j X6                                        Pi,j X6<-Pi-1,j Y6

Pi,j Y7->Pi-1,j+1 X7                                     Pi,j X7<-Pi+1,j-1 Y7

Pi,j Y8->Pi,j+1 X8                                       Pi,j X8<-Pi,j-1 Y8

Pi,j Y9->Pi+1,j+1 X9                                    Pi,j X9<-Pi-1,j-1 Y9

Select={(-1,-1),(0,-1),(1,-1),(-1,0),(0,0),(1,0),(-1,1),(0,1),(1,1)}

# Test cases

To generate Fig. 1, I create pattern1.ma and pattern1.bat and pattern1.pal. In the pattern1.ma file, I use transport delay and the border is wrapped. The rules in section 2.1 are implemented in this file. The graphic output can be seen in Modeler. Simulation shows the Fig.1 is created

To generate Fig. 2, I create pattern2.ma and pattern2.bat and pattern2.pal. In the pattern2.ma file, I use transport delay and the border is wrapped. The rules in section 2.2 are implemented in this file. The graphic output can be seen in Modeler. Simulation shows the Fig.2 is created.

# Bonus part

I tried to visualize my pattern1 and pattern2 results in VRML GUI with pattern1.daw, pattern1.pal and pattern2.daw, pattern2.pal respectively, but I found a bug in VRML GUI expressed as bellow

- After I created my .pal and .drw files under Linux, I switched to Windows to run VRML GUI. When I tried to load pattern1.drw file, VRML GUI prompted a error message “the color file is not correct”.

- I created a pattern1.pal from VRML GUI, then load pattern1.drw. This time, the pattern1.drw was successfully loaded in. If I make some change in pattern1.pal in cdModeler under Linux, then switch back Windows and try to load pattern1.drw, the same error message “color file is not correct” pops up again.

- So the bug is: .pal file created in cdModeler doesn’t work in VRML GUI.

I visualized pattern1.drw and pattern2.drw with pattern1.pal and pattern2.pal  which are all created in VRML GUI. But in VRML GUI, I only saw a square and can not see the 3D picture.