### METHODOLOGIES FOR DISCRETE EVENT MODELLING AND SIMULATION

### SYSC 5104

#### Assignment #2

Simulation of Pesticide Percolation in Soil Using Cell-DEVS

Name:  Rachid Chreyh

Student Number:              350664

Date:       Nov. 8 2005

# Part I: The Conceptual Model

In this assignment we will use Cell-DEVS to simulate Pesticide Percolation in the Soil. This assignment is based on [1] in which a parallel model based on Cellular Automata was used to Simulate Pesticide Percolation in the Soil.  The model details mention in [1] are simplified to fit the time constraints for this assignment.

The general idea we are trying to simulate is how water (e.g. from rain) gets contaminated with pesticides that exist in the soil and how the rain washes away the pesticides from the soil. When pesticides are used on farmland the soil absorbs some of them. Then, when water flows through the soil the pesticides are released into the water which flows down the soil until it reaches the groundwater.

Conceptually, each cell in our model has two states, the first defining its identity (I), which could have a value of water, soil or empty. The second state is an integer number representing the amount of pesticide particles (P) contained in it. Only a cell with an identity of water or soil can have a non zero value for the pesticide particles. The Von Neumann neighborhood was used in [1], which means every cell has four neighbors as shown below. (The neighborhood used in our Cell-DEVS model is more complex, see below)

U

R

D

L

C

Note that the original proposal of how the model will be implemented in Cell-DEVS has been modified. The reason for the modification is because after more detailed study of the original proposal, it was determined that the rules required to calculate the amount of pesticide in each cell as the water percolates through the model (keeping in mind that the total quantity of pesticide in the system should not change) were going to be too complex and require more time and effort than deemed reasonable for this assignment.  As a result the model has been modified to just track the contamination of the water and soil without keeping track of the exact quantities of the contaminants.  This is explained more clearly below.

The paper [1] suggested that the amount of pesticide in the water will be calculated based on averaging the amount of pesticide in the neighboring cells. As mentioned earlier, we will not do this. In our model the water particles will have only two states either contaminated or clean. Soil particles on the other hand have four states, one of them is the clean state, and the other three are varying degrees of contamination. Our model will not track the exact amounts of contaminants in the water; it will just model how the water carries away the contaminants from the soil. As the water washes away the contaminants in the soil, the soil particles change through the different degrees of contamination until they reach the “clean” state. The rules for our model are shown below:

- Reaction Rule:  For every cell with I = water that is not contaminated, the water cell becomes contaminated if any of its neighbors is a contaminated soil particle (Note that the clean water in such a cell will actually become contaminated water only after the water has moved to a neighboring empty cell). Contaminated water particles are assumed to be saturated and cannot get any more contaminated. They also do not cause contamination to clean soil particles.  Soil particles that are contaminated and that have at least one neighboring clean water cell will loose some of their contamination (one level) until they reach the “clean” state.

- Diffusion Rule: In this step the water is moved down the soil.  Therefore, for each water cell, if there is an empty cell to its bottom the water will be moved there (by making the identity of the empty cell equal to “water” and the original water cell equal to empty).  However if the bottom cell is occupied, but both the Left and Right cells are empty then the water will spread to both the left and right (note that our model does not represent quantities of water or contamination, therefore spreading the water in both directions is reasonable) . Otherwise if only one of the right or left cells is empty then the water will move there. If none of the Left, Right or Down neighbors are empty, then the water will not move. In the case where water is moving (for example) from the left and the right into the same cell, the water will be combined so that it joins in that cell.  Clean water joining with Dirty water will result in a clean water cell (since we assume that after joining the water is not saturated any more and will carry more contaminants).

The simulation will be started with a percolation bed composed of bunches of soil cells that are fully contaminated, and with empty cells in between the soil bunches. To start the simulation all empty cells at the top of the percolation bed are filled with water and are continually filled with water (simulating a flood condition). The percolation of the water and its contamination is then examined. Water is also allowed to leave the percolation bed by emptying the bottom row of the percolation bed.

# Part II: Formal Specifications, Test Strategy and Results

The Neighborhood

The neighborhood used in our Cell-DEVS model is shown below. We determined that this is the required neighborhood because basically we needed information about the Von Neumann neighborhood of each of the cells to our top, bottom, left and right. This information was used to determine when the water becomes contaminated (e.g. clean water coming into our cell that originally had dirty soil around it will become dirty water when it enters our cell).  It was also needed to determine the movement of water in the left and right directions. The neighborhood is as follows:

(-1, 0)

(0, 1)

(0, -1)

(0, 0)

(1, 0)

(-2, 0)

(-1, -1)

(0, -2)

(1, -1)

(1, 1)

(-1, 1)

(0, 2)

Formal Specification

The formal specification <X, Y, I, S, , N, d, , δint, δext, λ, ta> for the atomic Cell-DEVS model is defined as follows:

X = { x | x  [-1,5] }

Y = { y | y  [-1,5) }

I =  <12, 0, {Px1, Px2, .. , Px12}, {}>

S = { s | s  [-1,5] }  	// where:

-1 = Clean Water,

0 = Empty Space,

1 = Contaminated Water,

2 = Clean Soil,

3, 4, and 5 = Contaminated Soil.

 = { (s, phase, queue, )}

where	s [-1,5], phase{passive, active},

queue = {(vi, i) / i  N, vi [-1,5], i  R0+}

and   R0+		//user transport delay

N  S12

d = 100ms 		//fixed for the cell in the model

δint:	internal transition function which is defined by CD++ automatically

δext:	external transition function which is defined by CD++ automatically

λ: 		output function which is defined by CD++ automatically

ta(passive) = INFINITY

ta(active) = d

 = The local computing function described below: (See .MA file for details)

Empty Cells will get:

- A value of -1 (clean water) if any of the Left, Right, or Top neighbors is a “source of clean water”. “Source of clean water” means that the cell has a value of -1 and does not have a direct neighbor with a value >2. (Note that for the Left and Right cells to be considered “sources of water” they must Not have an empty cell below them, because if they did the water would flow to the bottom only).

- A value of 1 if :

  - The Right and Top neighbors are both not sources of clean water AND either:

    - The left cell has a value of -1, does not have an empty cell below it, and has at least one neighbor with a value >2.

OR

    - The left cell has a value of -1 and does not have an empty cell below it.

OR

  - The Left and Top neighbors are both not sources of clean water AND either:

    - The right cell has a value of -1, does not have an empty cell below it, and has at least one neighbor with a value >2.

OR

    - The right cell has a value of -1 and does not have an empty cell below it.

OR

  - The Left and Right neighbors are both not sources of clean water AND either:

    - The Top cell has a value of -1 and has at least one neighbor with a value >2.

OR

    - The Top cell has a value of -1.

**Note: As mentioned earlier, clean water and dirty water that move into the same cell will result in a cell with clean water.

Water Cells will get:

- A value of 0 (empty space) if any of the Left, Right, or Bottom neighbors is empty.

Soil Cells will get:

- A value of 4 if it had a value of 5 and any of the Top, Left, Right, or Bottom neighbors is a clean water cell (-1).

- A value of 3 if it had a value of 4 and any of the Top, Left, Right, or Bottom neighbors is a clean water cell (-1).

- A value of 2 if it had a value of 3 and any of the Top, Left, Right, or Bottom neighbors is a clean water cell (-1).

Special Rules:

Because the borders are not wrapped, there are also special rules for the following:

- Top Row: Always filled with clean water.

- Bottom Row: If not a soil cell then always get the value of the above cell if it is also not a soil cell. If soil cell then same as regular rules above.

- The Two Left Columns: Have special rules based on the above rules but without checking the cells that do not exist in each case.

- The Two Right Columns: Have special rules based on the above rules but without checking the cells that do not exist in each case.

The formal specification <Xlist, Ylist, I, X, Y, η, N {f, c}, C, B, Z, select> for the coupled Cell-DEVS model is defined as follows:

Xlist = {Ф}

Ylist = {Ф}

I = {Ф}

X = {Ф}

Y = {Ф}		//no external inputs and outputs

η = 12

N = {			     (-2, 0),

(-1,-1), (-1, 0), (-1, 1),

(0,-2), (0,-1),   (0, 0),  (0, 1), (0, 2)

(1,-1),   (1, 0),  (1, 1) }		//neighborhood

{f,c} = {10,10}	//10x10 cell space

C = { Cij | i [2,9], j[3,8]}

Where Cij is an atomic component defined in the previous part.

B = {C1j, C10j, Ci1, Ci2, Ci9, Ci10} //the border is Not wrapped

Where:  C1j is as defined for the Top Row in the previous part.

C10j is as defined for the Bottom Row in the previous part.

Ci1 is as defined for the Very Left Column in the previous part.

Ci2 is as defined for the Next Left Column in the previous part.

Ci10 is as defined for the Very Right Column in the previous part.

Ci9 is as defined for the Next Right Column in the previous part.

Z(Pijy1) = Pijx1	Z(Pijy2) = Pi-1jx2	Z(Pijy3) = Pij+1x3	Z(Pijy4) = Pij-1x4

Z(Pijy5) = Pi+1jx5	Z(Pijy6) = Pi-1j-1x6	Z(Pijy7) = Pi-2jx7 Z(Pijy8) = Pi-1jx8

Z(Pijy9) = Pij-2x9	Z(Pijy10) = Pij+2x10    Z(Pijy11) = Pi+1j-1x11 Z(Pijy12) = Pi+1j+1x12

Select = {(0, 0), (-1, 0), (0, 1), (0,-1), (1, 0), (-1,-1)

(-2, 0), (-1, 1), (0,-2), (0, 2), (1,-1), (1, 1)}

Discussion of the Test Cases

To test our model we determined that the two major behaviors that needed to be tested are, first the movement of the water from one cell to the next; and second the transfer of the contaminants from soil particles to water particles.  To test our model, we created two “.MA” files containing the same rules (explained in the previous section) but each with a different simulation environment (i.e. different locations of soil particles in our system). These test files are described below:

Test File #1: “PesticideMA1.ma”

This first test file represents the very simple environment in which there are no soil particles at all.  It shows how the water will flow straight down in the absence of any soil particles. The screen shots below show the environment before and during execution. These screen shots are taken from the CD++ Modeler tool which was used to analyze the behavior of our model. As expected, in this simple environment, our model behaves correctly with the water particles moving from one cell to the next, leaving the previous cell empty as they move on.  Note that we are simulating a “flood condition” in which the top cells are always filled with water.

Initial Environment                     After 0.5 seconds                     During Execution

Test File #2: “PesticideMA2.ma”

This second test file represents a more complicated environment. Different parts of this environment were intended to test different aspects of the model. In general this test file shows how water will be blocked by soil particles and will flow around them if possible. It shows how the contaminants get transferred from the soil particles to the clean water particles. It shows how the water will spread to the left and the right cells if there is no open path below it. And it shows that the same behavior still holds at the left and right boundaries where the rules are a bit different.  The screen shots below (again obtained from the CD++ Modeler tool used to analyze the model) show these behaviors as they are being tested. Again, our model behaves as expected and according to the rules set out in the beginning.  Note again that we are simulating a “flood condition” in which the top cells are always filled with water.  The screen shots below are accompanied by commentary to elaborate on the aspects being tested in each one.

For the following diagrams this is the meaning of the color codes:

White               : Empty Space.

Light blue        : Clean Water.

Dark blue        : Contaminated Water.

Black               : Highly Contaminated Soil.

Dark grey         : Medium Contaminated Soil.

Grey                 : Low Contaminated Soil.

Light grey        : Clean Soil.

| The Screen Shot to the right shows the initial environment. As you can see the black cells represent highly contaminated soil particles. The rest of the cells are empty. The water will be applied from the top and it will flow through the rest of the environment as shown in the next screen shots. | Initial Environment |
| --- | --- |

| The Screen Shot to the right shows the environment as water starts flowing. As can be seen the top row of soil is now all clean because of its continuous contact with clean water from the top row. Water is only able to flow through one hole in the top soil row. Also, as expected, when the water flows though the hole and reaches the second row of soil it “spreads out” moving to the right and to the left. The Dirty soil cell right beneath the “hole” has started to get cleaner (now it is at medium contamination). Also we can see here how water that has picked up contaminants became dark blue and can no longer “clean” dirty soil cells. | After 0.5 seconds |
| --- | --- |

| The Screen Shot to the right shows the environment as clean water continues to flow through it. Note how the second row of soil particles is now fully clean, and only now does the clean water reach the lower soil particles and start to clean them. Also note the placement of some soil and water particle at the right and left borders to prove that the behavior still holds in these “special” regions. Also notice how when clean water is moving into a cell it will be contaminated based on its original Von Neumann neighbors (not those of the cell it is moving into). This represents the behavior of the water “cleaning” the dirty cells as it passes by them. | After 3 seconds |
| --- | --- |

| The Screen Shot to the right shows the environment as clean water has completely washed away all contaminants it will ever get in contact with. Note how the soil particles at the bottom right corner are not cleaned because clean water never reaches them. | After 6 seconds |
| --- | --- |

Discussion of the Simulation Example

The above test cases have proved that the cells behave as we expected them to behave. The final step is to construct an environment that resembles soil through which water particles are to flow, and to run the simulation using this environment.  To do this we created a third “.MA” file called “PesticideMA3.ma”. This file contains the same rules that we had defined earlier but with a placement of soil particles more resembling a piece of soil. The screen shots below (again obtained from the CD++ Modeler tool used to analyze the model) show the behavior of our model in this new environment.  The execution of the simulation performs as expected.  Water flows in the correct manner carrying away contaminants as it peculates to the bottom of the soil.  Eventually all the soil particles coming in contact with the water flow have become clean.  It is worth noting while running this simulation that (as intended) when water from two neighbors is moving into our cell, our cell will become filled with clean water if at least one of the water sources is clean, otherwise it will become filled with dirty water.

Initial Environment                                                 After 1 second

After 3 seconds                                                 After 6 seconds

Reference:

[1] Stefania Bandini, Giancarlo Mauri, Giulio Pavesi, and Carla Simone: A Parallel Model Based on Cellular Automata for the Simulation of Pesticide Percolation in the Soil, PaCT-1999: 383–394