CARLETON UNIVERSITY

SYSC 5104

METHODOLOGIES FOR DISCRETE EVENT MODELING AND SIMULATION

Fall 2008

Instructor: Prof. Gabriel A. Wainer

Assignment # 1

Prepared by: Ahmed Sayed Ahmed

Student Number: 784 206

Email: asahmed@sce.carleton.ca

Tank Fire System Specifications:

A tank fire system will be modeled and simulated. The real system works as follows; first the loader should load the barrel of the tank with the required ammunition then the barrel lock is closed (to prevent the ammunition from sliding out of the barrel). After that the loader press the safety loader button to inform that he is in a safe position. Then the gunner fire ammunition by one of the three options stated below:-

- A Push button in the control handle is used when the electrical control fire system works.

- A Push button in the mechanical or manual system when there is failure in the electrical control fire system.

- Emergency button can be used when there are problems in the electrical control fire system and the manual system.

Once the ammunition fired from the barrel of the tank, loader can reload another one. It is impossible to input two ammunitions at the same time.

The structure of the model is introduced in Figure 1. As we can see the top level is the Tank is a coupled model composed of two basic components.

- The LOADER coupled model consists of 2 atomic components: the Barrel receive input of ammunition, input from the safety that it is on, and input of the Gunner that it trigger fire (and informs that it is ready through the out output port), and the Safety_loader receive the input that the loader press the button of safety (and informs that it is on through the out output port).

- The GUNNER coupled model consists of 4 atomic components: the Electric_fire, Manual_fire, and Emergency_fire receive input of fire from the Gunner that trigger electrical fire, manual fire or emergency fire respectively (and informs that it is on through the out output port of each one respectively), the Fire receive that the fire button is triggered (and informs that it is ready through the out output port).

The Coupled Model definitions show the structure of the whole model, but then we need to define the behavior for each atomic model, which should use the following description:

- BARREL: when it receives the input Ammunition, the input Safety is pressed, and the fire is triggered then it transmits 1 to the output port Ammu_out to trigger the ammunition.

- SAFETY_LOADER which holds pressing of safety loader through the Sin then it transmits 1 to the output port to the Barrel.

- ELECTRIC_FIRE which holds the electrical input fire and transmit 1 to the output port out to Fire.

- MANUAL_FIRE which holds the manual fire and transmit 1 to the output port out to Fire.

- EMERGENCY_FIRE which indicates the emergency fire and transmit 1 to the output port out to Fire.

- FIRE when it receives the input fire it enables the output port out to transmit 1 to the Barrel to Fire.

TANK

LOADER

Ammu_in

GUNNER

Emergency Fire

Barrel

Safety Loader

Manual Fire

Electric Fire

Amm_out

Emerg_in

Safety_in

Elec_in

Manu_in

out

Sfin

Sin

Ain

Ain

Ammu_out

Ammu_out

Sin

Fin

Fin

out

Ecin

in

Ecin

Fire

out

out

Min

in

Min

Emin

out

Emin

in

Figure 1 Tank Fire System

We can define the Coupled Model for TANK system as:

MTank = <X, Y, D, {Md | d Є D}, EIC, EOC, IC, select>

X	= {(Ammu_in, B); (Safety_in, B); (Elec_in, B); (Manu_in, B); (Emerg_in, B)};

Y	= {(Ammu_out, B)};

D	= {(Loader, Gunner)};

Md	= {MLoader, MGunner} (where MLoader and MGunner are coupled models);

EIC	= {(self, Ammu_in), (Loader, Ain); (self, Safety_in), (Loader, Sin); (self, Elec_in),

(Gunner, Ecin); (self, Manu_in), (Gunner, Min); (self, Emerg_in), (Gunner, Emin)};

EOC	= {((Loader, Ammu_out), (self, Ammu_out))};

IC	= ((Gunner, out), (Loader, Fin))};

Select	= {Loader, Gunner}.

The LOADER coupled model can be defined as:

MLOADER = <X, Y, D, {Md | d Є D}, EIC, EOC, IC, select>

X	= {(Ain, B); (Sin, B); (Fin, B)};

Y	= {(Ammu_out, B)};

D	= {(Barrel, Safety_Loader)};

Md	= {MBarrel, MSafet_Loader} (where MBarrel and MSafet_Loader are atomic models);

EIC	= {(self, Ain), (Barrel, Ain); (self, Fin), (Barrel, Fin); (self, Sin),

(Safety_Loader, Sin)};

IC	= {((Safety_Loader, out), (Barrel, Sfin))};

EOC	= {((Barrel, Ammu_out), (self, Ammu_out))};

Select	= {Barrel, Safety_Loader}.

The GUUNER coupled model can be defined as:

MGUUNER  = <X, Y, D, {Md | d Є D}, EIC, EOC, IC, select>

X	= {(Ecin, B); (Min, B); (Emin, B)};

Y	= {(out, B)};

D	= {(Electric_Fire, Manual_Fire, Emergency_Fire, Fire)};

Md	= {MElectric_Fire, MManual_Fire, MEmergency_Fire, MFire} (where MElectric_Fire,

MManual_Fire, MEmergency_Fire, and MFire are atomic models);

EIC	= {(self, Ecin), (Electric_Fire, in); (self, Min), (Manual_Fire, in); (self, Emin),

(Emergency_Fire, Fin)};

IC	= {((Electric_Fire, out), (Fire, Ein); (Manual_Fire, out), (Fire, Min);

(Emergency_Fire, out), (Fire, Emin))};

EOC	= {((Fire, out), (self, out))};

Select	= {Electric_Fire, Manual_Fire, Emergency_Fire, Fire.

The functionality of each atomic model must be described using the formal specification for DEVS. For instance the Barrel model can be formally described as:

Barrel = < X, S, Y, δint, δext, λ, ta >

X = {(Ain, B); (Sfin, B); (Fin, B)};

S = {state є {active, passive}, Sfin_val, Ain_val, Fin_val, Ammu_out_val є B, preparation Time є R0+};

Y = {(Ammu_out, B)};

δext (s, e, x)

{

if (x.port == Sfin)   	    // Indicates the safety loader is on

{

Sfin_val = 1;					// Assign Sfin_val to 1

}

else

{

Sfin_val = 1;					// Assign Sfin_val to 0

}

if (x.port == Ain)   	    // Indicates there is ammunition

{

Ain_val = 1;					// Assign Ain_val to 1

}

else

{

Ain_val = 0;					// Assign Ain_val to 0

}

if (x.port == Fin)   	    // Indicates the gunner trigger fire

{

Fin_val = 1;					// Assign Fin_val to 1

}

if (Sfin_val && Ain_val && Fin_val)	// Condition to send output

{

Ammu_out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

}

λ (s)

{

sendOutput(time, out, Ammu_out_val );

}

δint (s)

{

Ammu_out_val = 0;

Ain_val = 0;

Fin_val = 0;

passivate();

}

The Safety_Loader model can be formally described as:

Safety_Loader = < X, S, Y, δint, δext, λ, ta >

X = {(Sin, B)};

S = {state є {active, passive}, out_val є B, preparation Time є R0+};

Y = {(out, B)};

δext (s, e, x)

{

if (x.port == Sin)   	    // Indicates the safety loader is pressed

{

out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

else

{

out_val = 0;

State = passive; ta(state) = preparationTime;	//it would be executed

}

}

λ (s)

{

sendOutput(time, out, out_val );

}

δint (s)

{

State = state; ta(state) = ∞;

}

The Electric_Fire model can be formally described as:

Electric_Fire = < X, S, Y, δint, δext, λ, ta >

X = {(in, B)};

S = {state є {active, passive}, out_val є B, preparation Time є R0+};

Y = {(out, B)};

δext (s, e, x)

{

if (x.port == in)   	    // Indicates the Electric  Fire is pressed

{

out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

}

λ (s)

{

sendOutput(time, out, out_val );

}

δint (s)

{

out_val = 0;

passivate();

}

The Manual _Fire model can be formally described as:

Manual _Fire = < X, S, Y, δint, δext, λ, ta >

X = {(in, B)};

S = {state є {active, passive}, out_val є B, preparation Time є R0+};

Y = {(out, B)};

δext (s, e, x)

{

if (x.port == in)   	    // Indicates the Manual  Fire is pressed

{

out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

}

λ (s)

{

sendOutput(time, out, out_val );

}

δint (s)

{

out_val = 0;

passivate();

}

The Emergency _Fire model can be formally described as:

Emergency _Fire = < X, S, Y, δint, δext, λ, ta >

X = {(in, B)};

S = {state є {active, passive}, out_val є B, preparation Time є R0+};

Y = {(out, B)};

δext (s, e, x)

{

if (x.port == in)   	    // Indicates the Emergency Fire is pressed

{

out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

}

λ (s)

{

sendOutput(time, out, out_val );

}

δint (s)

{

out_val = 0;

passivate();

}

The Fire model can be formally described as:

Fire = < X, S, Y, δint, δext, λ, ta >

X = {(in, B)};

S = {state є {active, passive}, out_val є B, preparation Time є R0+};

Y = {(out, B)};

δext (s, e, x)

{

if (x.port == Ecin)   	    // Indicates the Electric Fire was pressed

{

out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

if (x.port == Min)   	    // Indicates the Manual Fire was pressed

{

out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

if (x.port == Emin)   	    // Indicates the Emergency Fire was pressed

{

out_val = 1;

State = active; ta(state) = preparationTime;	//it would be executed

}

}

λ (s)

{

sendOutput(time, out, out_val );

}

δint (s)

{

out_val = 0;

passivate();

}

Testing:

In order to check if the model behaves like the real Tank Fire System, the following test cases are made:

1- BARREL: In order to check if the model behaves like the real BARREL System, the following test cases are made:

Input file Barrel.ma

[top]

components : Barrel@Barrel

in : Sfin Ain Fin

out : Ammu_out

Link : Sfin Sfin@Barrel

Link : Ain Ain@Barrel

Link : Fin Fin@Barrel

Link : Ammu_out@Barrel Ammu_out

Input file Barrel.ev

00:00:20:000 Sfin 1

00:00:60:000 Ain 1

00:02:30:000 Fin 1

00:03:25:000 Ain 1

00:03:40:000 Sfin 0

00:04:10:000 Fin 1

00:05:25:000 Ain 1

00:05:40:000 Sfin 1

00:06:15:000 Fin 1

Output file Barrel.out

00:02:30:002 ammu_out 1

00:05:40:002 ammu_out 1

TEST RESULTS: Test passed. The model works as it is expected to behave.

2- Safety_Loader: In order to check if the model behaves like the real Safety_Loader System, the following test cases are made:

Input file Safety_Loader.ma

[top]

components : Safety_Loader@Safety_Loader

in : Sin

out : out

Link : Sin Sin@Safety_Loader

Link : out@Safety_Loader out

Input file Safety_Loader.ev

00:00:20:000 Sin 1

00:00:50:000 Sin 0

00:02:30:000 Sin 1

00:03:20:000 Sin 0

Output file Safety_LoaderOUT.out

00:00:20:000 out 1

00:00:50:000 out 0

00:02:30:000 out 1

00:03:20:000 out 0

TEST RESULTS: Test passed. The model works as it is expected to behave.

3- Electric_Fire : In order to check if the model behaves like the real Electric_Fire System, the following test cases are made:

Input file Elec_Fire.ma

[top]

components : Electric_Fire@Electric_Fire

in : in

out : out

Link : in in@Electric_Fire

Link : out@Electric_Fire out

Input file Elecev

00:00:20:000 in 1

00:00:60:000 in 0

00:02:30:000 in 1

00:03:20:000 in 0

Output file Elec_FireOUT.out

00:00:20:004 out 1

00:02:30:004 out 1

TEST RESULTS: Test passed. The model works as it is expected to behave.

4- Manual_Fire : In order to check if the model behaves like the real Manual_Fire System, the following test cases are made:

Input file Manu_Fire.ma

top]

components : Manual_Fire@Manual_Fire

in : in

out : out

Link : in in@Manual_Fire

Link : out@Manual_Fire out

Input file Manual.ev

00:00:20:000 in 1

00:00:60:000 in 0

00:02:30:000 in 1

00:03:20:000 in 0

Output file Manu_FireOUT.out

00:00:20:004 out 1

00:02:30:004 out 1

TEST RESULTS: Test passed. The model works as it is expected to behave.

5- Emergency_Fire : In order to check if the model behaves like the real Emeregncy_Fire System, the following test cases are made:

Input file Emeregncy.ma

[top]

components : Emergency_Fire@Emergency_Fire

in : in

out : out

Link : in in@Emergency_Fire

Link : out@Emergency_Fire out

Input file Emeregncy.ev

00:00:20:000 in 1

00:00:60:000 in 0

00:02:30:000 in 1

00:03:20:000 in 0

Output file Emeregncy.out

00:00:20:004 out 1

00:02:30:004 out 1

TEST RESULTS: Test passed. The model works as it is expected to behave.

6- Fire : In order to check if the model behaves like the real Fire System, the following test cases are made:

Input file Fire.ma

[top]

components : Fire@Fire

in : Ecin Min Emin

out : out

Link : Ecin Ecin@Fire

Link : Min Min@Fire

Link : Emin Emin@Fire

Link : out@Fire out

Input file Fire.ev

00:00:20:000 Ecin 1

00:00:60:000 Min 1

00:02:30:000 Emin 1

Output file FireOUT.out

00:00:20:004 out 1

00:01:00:004 out 1

00:02:30:004 out 1

TEST RESULTS: Test passed. The model works as it is expected to behave.

7- COUPLED GUNNER: In order to check if the model behaves like the real Coupled GUNNER System, the following test cases are made:

Input file GUNNER.ma

[top]

components : Electric_Fire@Electric_Fire  Manual_Fire@Manual_Fire Emergency_Fire@Emergency_Fire Fire@Fire

in : Ecin Min Emin

out : out

Link : Ecin in@Electric_Fire

Link : Min in@Manual_Fire

Link : Emin in@Emergency_Fire

Link : out@Electric_Fire Ecin@Fire

Link : out@Manual_Fire Min@Fire

Link : out@Emergency_Fire Emin@Fire

Link : out@Fire out

Input file Gunner.ev

00:00:20:000 Ecin 1

00:00:60:000 Min 1

00:02:30:000 Emin 1

Output file GUNNEROUT.out

00:00:20:008 out 1

00:01:00:008 out 1

00:02:30:008 out 1

TEST RESULTS: Test passed. The model works as it is expected to behave.

8- COUPLED LOADER: In order to check if the model behaves like the real Coupled LOADER System, the following test cases are made:

Input file LOADER.ma

[top]

components : Barrel@Barrel  Safety_Loader@Safety_Loader

in : Ain Sin Fin

out : Ammu_out

Link : Ain Ain@Barrel

Link : Fin Fin@Barrel

Link : Sin Sin@Safety_Loader

Link : out@Safety_Loader Sfin@Barrel

Link : Ammu_out@Barrel Ammu_outLink : out@Fire out

Input file LOADER.ev

00:00:50:000 Ain 1

00:01:20:000 Sin 1

00:02:30:000 Fin 1

00:03:25:000 Ain 1

00:03:40:000 Sin 0

00:04:10:000 Fin 1

00:05:25:000 Ain 1

00:05:40:000 Sin 1

00:06:15:000 Fin 1

Output file LOADEROUT.out

00:02:30:002 ammu_out 1

00:05:40:002 ammu_out 1

TEST RESULTS: Test passed. The model works as it is expected to behave.

9- TANK: In order to check if the model behaves like the real TANK System, the following test cases are made:

Input file Tank.ma

[top]

components : LOADER GUNNER

in : Ammu_in Safety_in Elec_in Manu_in Emerg_in

out :  Amm_out

Link : Ammu_in Ain@LOADER

Link : Safety_in Sin@LOADER

Link : Elec_in Ecin@GUNNER

Link : Manu_in Min@GUNNER

Link : Emerg_in Emin@GUNNER

Link : out@GUNNER Fin@LOADER

Link : Ammu_out@LOADER Amm_out

[LOADER]

components : Barrel@Barrel  Safety_Loader@Safety_Loader

in : Ain Sin Fin

out : Ammu_out

Link : Ain Ain@Barrel

Link : Fin Fin@Barrel

Link : Sin Sin@Safety_Loader

Link : out@Safety_Loader Sfin@Barrel

Link : Ammu_out@Barrel Ammu_out

[GUNNER]

components : Electric_Fire@Electric_Fire  Manual_Fire@Manual_Fire Emergency_Fire@Emergency_Fire Fire@Fire

in : Ecin Min Emin

out : out

Link : Ecin in@Electric_Fire

Link : Min in@Manual_Fire

Link : Emin in@Emergency_Fire

Link : out@Electric_Fire Ecin@Fire

Link : out@Manual_Fire Min@Fire

Link : out@Emergency_Fire Emin@Fire

Link : out@Fire out

Input file Tank.ev

00:00:05:000 Ammu_in  1

00:00:10:000 Safety_in 1

00:00:20:000 Elec_in 1

00:00:20:000 Manu_in 0

00:00:20:000 Emerg_in 0

00:00:05:000 Ammu_in  1

00:00:10:000 Safety_in 1

00:00:20:000 Elec_in 1

00:00:20:000 Manu_in 0

00:00:20:000 Emerg_in 0

Output file TankOUT.out

00:00:20:010 amm_out 1

TEST RESULTS: Test passed for the whole system as expected in real system. The model works as it is expected to behave.