CD++ Model Data Form

Title:   Simulation of Large WSN with Cell-DEVS

Type: Cell-DEVS Model:

Acronym/Short name:   WSN

Purpose for which Developed:   Demonstrate the implementation of Large Scale WSN network using Cell-DEVS model

Other Applications for which it is Suitable:

Date Developed/Implemented:   Dec 2008

Domain:

Current Version:   1.0

URL:

Description (including characteristics):   The WSN topology problem to be considered is closely related to the minimum configuration of nodes for fully operational WSN, taking into account sensor node energy limitations for a long lasting - survivable WSN network.  In this problem, the model is structured as a two-dimensional cell space n x n, where every cell represents one sensor node (where cell space is not wrapped). The WSN is considered such as all the sensor nodes have same properties.  Each node can have up to 8 neighbors and there are 3 possible states of each cell (active, stand-by and inactive).

During the active mode of operation node emulates an active sensor node within the WSN (i.e. perform processing tasks, hence uses energy which decreases with time).  In the beginning of simulation, all the sensor nodes deployed, have the same amount of energy which decreases as time progresses while node is in active (maximum energy consumption) or stand-by mode (minimum energy consumption).  During the stand-by mode, sensor node is consuming minimal amount of energy; it wakes up randomly in order to see if some other close by nodes are already sensing/monitoring the predefined neighborhood area. If more two sensor nodes within the neighborhood are active cell goes again into stand-by mode, otherwise it becomes active. After the entire energy of a cell is consumed cell becomes inactive; this process continues until all the sensor nodes’ energy is consumed (i.e. all the cells are inactive).As a result, the Cell-DEVS simulation model, gives an insight into an elegant way of implementation the Topology Control Algorithm for a large WSN; in our particular case, addressing the issue of sensor nodes’ energy conservation in order to achieve a longer lifetime operation of the WSN.

Links to Related Documents

Short Title:

URL:

Description:

Keywords:   WSN, Cell-DEVS model, simulation models, sensor networks, cellular automata

Developer:

| Name: Blerim Qela | Acronym: Blero |
| --- | --- |
| [e-mail] Address 1: |  |
| Address 2: University of Ottawa |  |
| City: Ottawa | Province/State-Country: ON, Canada |
| Zip - | Phone: - - |

Comments:   This Cell-DEVS model has been tested and it works fine.