Simulation of Large Wireless Sensor Networks

Using Cell-DEVS

Blerim Qela

School of Information Technology and Engineering,

University of Ottawa
bqela@site.uottawa.ca

Gabriel Wainer

Department of Systems and Computer Engineering,

Carleton University
gwainer@sce.carleton.ca

ABSTRACT

The advancement of electronic sensing devices, microcomputers and wireless communication devices has lead to creation of new smart sensor devices, which can monitor actuate, compute and communicate.  Typically, sensors are deployed in non-deterministic mode (randomly); when deployed in large numbers, these sensor devices have the capability to self-organize into the so-called Wireless Sensor Networks (WSN).  WSN are ad-hoc networks, consisting of spatially distributed sensing and processing devices. In this paper, the objective is to simulate a Large Wireless Sensor Network (WSN) by implementing the Topology Control Algorithm as presented in [1] using the Cell-DEVS model [2], an extension of DEVS [3] (Discrete Event Systems Specifications) which enables efficient execution of cellular automata models.  Thereafter, observe and evaluate the behavior of sensor nodes and entire WSN from the simulation results obtained, under different test scenarios.

# INTRODUCTION

The WSN topology problem to be considered is closely related to the minimum configuration of nodes for fully operational WSN, taking into account sensor node energy limitations for a long lasting - survivable WSN network.  In this problem, the model is structured as a two-dimensional cell space n x n, where every cell represents one sensor node (where cell space is not wrapped). The WSN considered is such as all the sensor nodes have same properties (homogeneous WSN) and flat (i.e. no hierarchy among nodes).  Each node can have up to 8 neighbors and there are 3 possible states of each cell (active, stand-by and inactive). For this model, Moore’s neighborhood is adopted (refer to Fig.1).

As a result, the Cell-DEVS simulation model, gives an insight into an elegant way of implementation the Topology Control Algorithm for a large WSN; in our particular case, addressing the issue of sensor node energy conservation in order to achieve a longer lifetime operation of the WSN.

During the active mode of operation node emulates an active sensor node within the WSN (i.e. perform processing tasks, hence uses energy which decreases with time).  In the beginning of simulation, all the sensor nodes deployed, have the same amount of energy which decreases as time progresses while node is in active (maximum energy consumption) or stand-by mode (minimum energy consumption).  During the stand-by mode, sensor node is consuming minimal amount of energy; it wakes up randomly in order to see if some other close by nodes are already sensing/monitoring the predefined neighborhood area. If more two sensor nodes within the neighborhood are active cell goes again into stand-by mode, otherwise it becomes active.

| 1 | 2 | 3 |
| --- | --- | --- |
| 8 |  | 4 |
| 7 | 6 | 5 |

Fig.1 – Moore’s neighborhood

After the entire energy of a cell is consumed cell becomes inactive; this process continues until all the sensor nodes’ energy is consumed (i.e. all the cells are inactive).

# AN OVERVEIW OF WSN

The advancement of electronic sensing devices, microcomputers and wireless communication devices has lead to creation of new smart sensor devices, which can monitor actuate, compute and communicate.  Typically, sensors are deployed in non-deterministic mode (randomly); when deployed in large numbers, these sensor devices have the capability to self-organize (refer to Fig.2) into the so-called Wireless Sensor Networks (WSN).  WSN are ad-hoc networks, consisting of spatially distributed sensing and processing devices. Smart sensor devices typically consist of a microcontroller, memory for data and code, transceiver, antenna, analog to digital converter, one or more sensors and energy source (i.e. typically sensors are battery powered).

WSN are used in many different applications, such as military, medicine, transportation and urban monitoring, traffic control, environment and habitat monitoring, energy management, industrial applications, etc.

Fig.2 – Sensor nodes self-organized

Due to the fact that sensor nodes have limited bandwidth, computing power and limited energy resources, one of the constraints in WSN is the energy efficiency of sensor nodes (i.e. power consumption).  There are many different approaches to solving the energy efficiency problem, such as the Topology Control Algorithm [1, 4 and 6].  The objective of WSN Topology Control Algorithm is related to the efficiency of WSN network, in order to increase its lifetime.  The rationale of network topology control is to reduce the number of redundant sensor nodes monitoring a particular region, hence increase the efficiency of WSN network and its lifetime.

# MODEL DEFINITION

Due to the complexity of the model under study the problem is adopted for efficient implementation using Cell-DEVS approach [2] and CD++ toolkit [5], as an efficient tool to model and simulate cellular automata models [7], in our case, the WSN topology problem.

The dimensions of the cell-space considered are n x n and two planes (Plane 0 and Plane 1) reflecting a three dimensional space implemented in order to meet the basic constraints of the defined WSN Topology problem, in order to explore and capture the main tasks of the problem considered.

The Moore’s neighborhood is adopted, while the space is unwrapped (similar to real world examples of deployed sensor networks); it was observed that in order to implement the model successfully the approach of modeling and finding the solution should be suited to the environment and tools being used (in our case CD++ toolkit [4]).

As described briefly in introduction, each sensor node starts to operate with a fixed amount of energy.  In this model the energy levels adopted based on [1, 4] are the following:

- WSN sensor energy (at the beginning of operation) = 0.8 J

- The energy of an WSN sensor node in active mode decreases by 0.0165 J every time step (in our case every 1 second)

- The energy of a WSN sensor node in stand-by mode decreases by 0.00006 J every time step (in our case every 1 second)

The WSN sensor node possible states are:

Plane 0:

2 - WSN sensor node is in active mode within the neighborhood (WSN sensor coverage area)

1 - WSN sensor node is in stand-by mode within the neighborhood (WSN sensor coverage area)

0 – WSN sensor node is passive (i.e. energy of a node is consumed)

Plane 1:

0.8 - initial energy level of WSN sensor nodes

-1 - WSN sensor node is passive (i.e. energy of a node is consumed)

Neighborhood ={ (-1,-1,0) (-1,0,0), (-1,1,0), (0,-1,0), (0,0,0), (0,1,0)( 1,-1,0),(1,0,0),(1,1,0), (0,0,1), (0,0,-1) } (refer to Fig.2a, 2b and 2c)

Plane 0 is the plane where the WSN sensor nodes are deployed and observed throughout the simulation of the model.  Whilst the Plane 1 was used as a memory for keeping track of the energy levels of active and stand-by nodes.  The correct decrease of energy level and node behavior in the Plane 0 is interrelated to Plane 1 which contains the sensors energy information throughout the simulation.

Fig.2a - Plane 0 organization (zones)

TC - Top center rule

BC - Bottom center rule

TL - Top left rule

TR - Top right rule

BL - Bottom left rule

BR - Bottom right rule

Right rule

Left rule

And the rest of cell space is WSN rule - local transition rule of the model.

Improved WSN model

The improvements with respect to the initial WSN model are the following:

Entire cell-space can be populated with sensor nodes (refer to Fig.2b

There are no limitations to functionality of the model (Plane 0 organization)

Cell space is not divided into zones

Enables more flexibility to the initial model (reduces significantly the code size)

Randomness was implemented within the model (it enables us to represent more closely the real-world applications related to WSN)

In addition to the existing states present in the initial WSN model (described earlier), additional state was implemented in Plane 0 of the improved WSN model is:

3 – WSN sensor node is typically in stand-by mode, however, nodes randomly become active if one or two nodes are active within the neighborhood (Moore’s neighborhood coverage area); node goes back to stand-by mode if the condition is true (i.e. one or two neighborhood sensors are active), otherwise it remains in the active mode .

Fig.2b - Plane 0 organization (improved model)

## General description of the WSN rules

In Plane 0, each Moore’s neighborhood (consisting of 9 cells) typically is covered by one active cell (with the value of 2) and the rest of the stand-by cells have a value of 1, whilst the passive cells have a value of 0.

At the initial stage all the WSN sensor nodes deployed within the WSN network area (represented by cell space) are active for several time steps until they configure themselves to active and stand-by mode nodes.

In Plane 1, for every time step, the energy of active cells is decreased; the amount of energy available x, for any active and stand-by mode cells before cell dies is 0 ≤ x ≤ 0.8.  Each cells energy level in Plane 1 serves as a reference (memory) of the Plane 0 active and stand-by mode cells (represented by value of 2 and 1 respectively).  Plane 0 refers to Plane 1 by (0, 0, 1), while Plane 1 refers to Plane 0 by (0, 0,-1) neighborhood coordinate (refer to Fig.2c).

The neighboring cells which are in a stand-by mode are represented by the value of 1 within the cell, and are also decreased in negligible quantities during the stand-by mode;  The active cells become passive after their energy is consumed and are replaced by the neighboring cells which are yet alive (cells currently in stand-by mode).

Fig.2c - Cell space definition

The neighboring cells which are in a stand-by mode are represented by the value of 1 within the cell, and are also decreased in negligible quantities during the stand-by mode;  The active cells become passive after their energy is consumed and are replaced by the neighboring cells which are yet alive (cells currently in stand-by mode).  The cell-space in Plane 0 defines sensor nodes, whilst in Plane 1 energy level corresponding to each node (refer to Fig.2c).

The algorithm steps through each neighborhood and decides on which nodes stays active and which is to be configured as a stand-by nodes; where each neighborhood of cells is checked for present active cells by comparing the present cell’s value with the residual energy (which is check referenced in Plane 1) until they becomes passive (represented by value of 0 in the cell space); Afterwards, one of the stand-by neighboring cell is assigned to becomes active, and this process continues until all the cells become passive (i.e. the energy resources of all sensors is consumed).

Plane 1 updates the energy levels of each cell during the simulation as per the specifications.

## Improvement of the Cell-DEVS WSN Simulation Model

Improved WSN simulation model provides closer approximation to the WSN topology algorithm by implementation of randomness within the WSN model; where randomness, redundancy and configuration of nodes play a significant role in reflecting the key factors in deployed networks, such as a network lifetime, coverage area and ratios of active and stand-by sensors at specific points in time.  Randomness was implemented by adding another rule (refer to rule 3, section 3.5.1) to the model, where stand-by cells randomly become active if only one or two active neighbors are active.

The actual results obtained with the improved model, more closely reflect the real-world scenarios and provide better insight into the WSN topology problem.  As it can be observed in section 3.5.1, after some experimental work with the initial WSN model implementation, the problem can be reduced and coded with less than 30 lines of code utilizing CD++ toolkit and Cell-DEVS approach, whilst the similar problem if is implemented in C/C++ or any other high level programming language it could take several hundreds or maybe thousands of lines of code (based on the implementation approach taken).  This proves how efficiently similar problems can be implemented, simulated and analyzed utilizing CD++ toolkit and Cell-DEVS approach.

### 3.5.1) 	Cell-DEVS Improved Coupled Model Specifications and    Implementation using CD++

[top]

components : WSN

[WSN]

type : cell

dim : (33, 33,2)

delay : transport

defaultDelayTime : 1000

border : nowrapped

neighbors : WSN(-1,-1,0) WSN(-1,0,0) WSN(-1,1,0)

neighbors : WSN(0,-1,0)  WSN(0,0,0)  WSN(0,1,0)

neighbors : WSN(1,-1,0)  WSN(1,0,0)  WSN(1,1,0)

neighbors : WSN(0,0,1) WSN(0,0,-1)

initialvalue : 2

initialCellsValue : WSN33.val

localtransition : WSN-rule

[WSN-rule]

rule : { (0,0,0) - 0.0165 } 1000 {cellpos(2) = 1 and  (0,0,-1) = 2 }

rule : { (0,0,0) - 0.00006 } 1000 {cellpos(2) = 1 and (0,0,-1) =1 }

rule : { (0,0,0) - 0.0165 } 1000 {cellpos(2) = 1 and  (0,0,-1) = 3 }

rule : 3 1000 {cellpos(2)=0 and (0,0,1)>0 and

( (0,0,0)=1 and (statecount(2)=2 or statecount(2)=1) and randInt(30)=11 ) }

rule : 2 1000 {cellpos(2)=0 and (0,0,0) !=0 and (0,0,0) !=3 and (0,0,1) > 0 and (0,-1,0)!=2 and (1,-1,0)!=2 and (1,0,0)!=2 and (1,1,0)!=2 }

rule : 1 1000 {cellpos(2)=0 and (0,0,0) !=0 and (0,0,1)>0 and

( (0,-1,0)=2 or (1,-1,0)=2 or (1,0,0)=2 or (0,1,0)=2 or (1,1,0)=2 or (cellpos(2)=0 and (0,0,0)=3) or (-1,-1,0)=2 or (-1,0,0)=2 or (-1,1,0)=2 ) }

rule : 0 1000 {cellpos(2)=0  and ((0,0,0)=1 or (0,0,0)=2 or (0,0,0)=0)  and (0,0,1) <= 0 }

rule : -1 1000 { t }

# SIMULATION RESULTS

For the initial WSN model, the following is the graphical representation of the sensor node states defined in the simulation model:

Table 1 – Possible sensor states of the initial WSN model

For the improved WSN model, the following is the graphical representation of the sensor node states defined within the simulation model:

Table 3- Possible sensor states of the improved WSN model

## Simulation Results of the Initial WSN Model

The first stage of the WSN sensor network is shown in Fig.3 below.  Based on the specification, the sensor nodes are active at the beginning (red cells with value of 2).  While the light blue cells have a value of 0 and represent the unpopulated zones within the cell space. Taking into account that the cell space is constituted by a 11 rows and 11 columns the total number of cells is 121, from which 99 cells represent the deployed WSN sensor network model within the cell space.

Fig.3- Plane 0 (left, red) and Plane 1 (right, green) prior to execution

The WSN network during the process of reconfiguration, where nodes are trying to form a structure as described in section 1.0 and 3.0, where every neighborhood is trying to set some sensors in active mode while others remain in stand-by mode.  And in the Plane 1 data, energy level of each sensor node can be monitored.

Fig.4 – Snapshot of simulations results after 6 time steps

At the beginning of simulation (prior to step 15) WSN network is in the process of reconfiguration, where nodes are trying to form a structure as described in section 1.0 and 3.0.  Every neighborhood is trying to set some sensors in active mode while others remain in stand-by mode.  And in the Plane 1 data, energy level of each sensor node can be monitored.

From the Fig. 5 results it can be observed that the WSN sensors are reconfigured as per the specifications and each neighborhood is covered by typically one active node while others remain in stand-by mode (energy of each note is decreasing during each step based on the model specification).

Fig.5 – Snapshot of simulations results after 15 time units

In Fig.6 we can observe that several active nodes (from previous step) are now passive (died cells - Plane 1 on the right, gray cells). While cells which were previously in stand-by mode are taking over by becoming active.

Fig.7 – Snapshot of simulations results after 62 time units

### Discussion of the Results Obtained with the Initial WSN model

All the sensor nodes become passive after 197 time steps. By comparing the end result of our simulation and the results obtained in [1] in particular number of active sensors after 200 time units  it can be observed (refer to Fig.8) that after approximately 200 time steps is when all the active sensors become passive (energy of all nodes is consumed); hence simulation results obtained by our model very closely reflect the same behavior (i.e. after 197 time units the active sensors become passive).

Fig. 8 - Graphical representation of the Initial WSN simulation results using Cell-DEVS

### Simulation Results of the Improved WSN models

Improved WSN simulation model provides closer approximation to the WSN topology algorithm by implementation of the randomness within the WSN deployed.  The actual results obtained, more closely reflect the real-world scenarios and provide better insight into the WSN topology problem and how efficiently similar problems can be implemented utilizing Cell-DEVS and CD++ toolkit. The improved model is quite easy to modify in order to simulate different sizes of WSN networks (it requires to change only n and m values (i.e. one line of code i.e. dim : (n, m, 2) where n is the number of columns and m is the number of rows in the model and provide the desired initial energy levels for the sensor nodes). The model below (refer to Fig. 9) represent the simulation model for WSN33, where the cell space is constituted by 33 rows and 33 column, and the total number of cells (sensor nodes) is 1089.

Fig.9 – Snapshot of Plane 0 (left, red) and

Plane 1 (right, green) prior to execution

The WSN network 33 x 33 during the process of reconfiguration, where nodes are trying to form a structure as described in section 1.0 and 3.0, where every neighborhood is trying to set some sensors in active while others remain in stand-by mode and some stand-by node are randomly awaken (refer to Fig.10 left, cells in blue, Plane 0) to check the neighborhood.  In case that only one or two sensor nodes are active, stand-by nodes randomly become active and return to stand-by mode only if one or two  more sensor nodes are currently active within the neighborhood).  From the Plane 1 data (refer to Fig.10 right), we can see the energy level of each sensor node up to this time step (see in green, indicating that the energy levels x of sensor nodes are: x ≥ 0.05J).

Fig.10 – Snapshot of simulations results after 40 time units

From the simulation results of WSN33 Cell-DEVS model, from the snapshots taken at different time steps of network activity (referring to Fig. 11 and Fig 12) it can be seen that the coverage area by sensors is mostly covered after 134 time steps and as the time progresses, more and more nodes become passive (their energy is consumed) leading to less covered area of the WSN cell-space, and finally after 193 time units the WSN cell-space becomes entirely passive.  Similar, results were obtained when WSN22 and WSN11 were simulated.

Fig.11 – Snapshot of simulations results after 134 time units

Fig.12 – Snapshot of simulations results after 164 time units

### Discussion of the Results Obtained with the Improved WSN model

Fig.13 below shows the number of active sensors versus time within the WSN network; evaluated for WSN deployment scenarios within two dimensional cell-spaces, using Cell-DEVS:

- WSN11 - representing 11x11 cell-space, with 121 sensor nodes

- WSN22 – representing 22x22 cell-space, with 484 sensor nodes

- WSN33 – representing 33x33 cell-space, with 1089 sensor nodes

Fig.13 – Number of active sensors with the algorithm using Cell-DEVS

The number of active sensors decreases after the configuration of sensor nodes, following the deployment.  After which, redundancy is reduced by having only one active node within the Moore’s neighborhood, while other nodes are in stand-by mode.  In addition, within each Moore’s neighborhood, stand-by nodes become active randomly and remain active if no sensor is active or return to stand-by mode if any sensor node is still active.

Results obtained, provide clear indication that network lifetime is increase approximately by 4, which was shown also in [1].

Fig.14 – Number of stand-by sensors with the algorithm using Cell-DEVS

From Fig. 14 it can be observed that the number of stand-by sensors increases while the WSN cell-space is being configured, and starts to decrease as the active sensors’ energy is consumed (refer to Fig. 13) hence stand-by sensors become active.  Similarly, in Fig. 15 it can be observed that number of sensors alive is at it maximum when the simulation starts (i.e. all the WSN sensor nodes are active) and starts slowly decreasing as the time progresses.

Fig.15 – Number of sensors alive with the algorithm using Cell-DEVS

An important aspect considered in WSN network is the coverage area, which is closely related to the active sensors within the cell-space.  When the redundancy of sensor nodes within the WSN is controlled, the network lifetime is prolonged; hence wider coverage area is maintained for a longer time, as shown by the simulation results in Fig. 16.

Fig.16- Sensor network coverage area (%) using Cell-DEVS

Fig. 17 shows the number of active sensors in the worst case scenario when WSN sensor nodes are active the entire time until the energy of sensors is consumed just before time 50; the network lifetime is approximately 4 time less in comparison to the implementation of WSN topology control algorithm using Cell-DEVS (refer to Fig.13, Fig.15 and Fig. 16).

# CONCLUSIONS

The objective of this paper was to simulate a large Wireless Sensor Network (WSN) using Cell-DEVS [2], by implementing the Topology Control Algorithm as presented in [1].  By observing and evaluating the behavior of WSN simulation model, under different test scenarios, it was proven the effectiveness of Cell-DEVS and CD++ toolkit [5], as an elegant approach to model, simulate and analyze, in this case the WSN topology problem.

The initial WSN model (refer to section 3.4) was further enhanced (refer to section 3.5) in order to provide closer approximation to the WSN topology algorithm by implementation of the randomness within the WSN deployed.  In addition, modification and simplification of model was done, where cell-space is not divided into zones; allowing more flexibility to model any possible WSN cell-space configuration (entire or partial cell-space populated with sensor node).

The actual results obtained with the improved model, more closely reflect the real-world scenarios and provide a better insight into the WSN topology problem, in particular.  As it was observed in section 3.5.1, the complexity of the problem can be simplified and coded with less than 30 lines of code, utilizing CD++ toolkit and Cell-DEVS approach; whilst similar problem if implemented in C/C++ (or any other high level programming language) could possibly take up to several hundreds or perhaps thousands of lines of code (based on the implementation approach taken).  Thus, proving how efficiently complex problems of similar nature can be implemented simulated and analyzed utilizing CD++ toolkit and Cell-DEVS approach.

# ACKNOWLEDGMENTS

I would like express my appreciation to Dr. Wainer for his guidance, encouragements and advice during this work.

# REFERENCES

[1] R.Cunha; A.Silva; A.Loreiro; L. Ruiz: “Simulating Large Wireless Sensor Networks Using Cellular Automata”, Proceeding of the 38th Annual Simulation Symposium (ANSS’05).

[2] G. Wainer; N. Giambiasi: “Application of the Cell-DEVS Paradigm for Cell Spaces Modeling and Simulation”, Simulation, VOl.71, No. 1, pp.22-39, January 2001.

[3] B. Zeigler; T. Kim; H. Praehofer: Theory of Modeling and Simulation: Integrating Discrete Event and Continuous Complex Dynamic Systems, Academic Press, 2000.

[4] F.Ye; G.Zhong; S.Lu, L.Zhang:  “Energy Efficient Robust Sensing Coverage in Large Sensor Networks”, Technical Report, 2002.

[5] G.Wainer:  “CD++:  a Toolkit to Define Discrete Event Models”, Software, Practice and Experience, Wiley, VOl.32, No. 3. pp. 121-1306, November 2002.

# [6] Y. Shen; B. Guo: “Dynamic Power Management (DPM) based on Wavelet Neural Networks in Wireless Sensor Networks”, 2007 IFIP International Conference on Network and Parallel Computing.

[7] B.Chopard; M.Droz: “Cellular Automata Modeling of Physical Systems”. New York, Cambridge Univ. Press, 1998

Fig.17- Number of active sensors with the algorithm