Cable-Anchor Robot Implementation using Embedded CD++

Keith Holman, Jeremy Kuzub

Dept. of Systems and Computer Engineering

Carleton University, Ottawa, Canada

keith@sce.carleton.ca, jkuzub@sce.carleton.ca

# Abstract

Using the Eclipsed-based DEVS++ discrete event toolkit and simulator, a robot controller with a unique locomotion system is designed, implemented and tested. Rather than wheels, the robot is designed to translate against a vertical surface using cables fixed at one end that can wind on motor-controlled spools attached to the robot. This architecture was implemented first as a regressively tested simulation within CD++ then ported to Real-time DEVS. Using the NXT++ interface library, a hardware implementation of the robot using Lego® Mindstorms™ was shown to be controllable using event files.

Unique to this project was control of Mindstorms hardware via a DEVS atomic model within a complex coupled model. This demonstrated that any Mindstorms sensor or actuator can be effectively modeled as an atomic model within DEVS++ then the associated Real-time DEVS atomic model can be substituted in the E-CD++ framework. Additional improvements to the code and build workflow allowed this port to proceeded with little additional modification. A workflow was thus successfully demonstrated from simulation to hardware implementation with DEVS, Real-Time DEVS, and Mindstorms.

# Part I: High-Level Design

## Introduction: Cable-Anchored Robots

In applications where terrain is too difficult to traverse using legs or wheels, other forms of robot locomotion must be found. Examples include disaster areas, such as building collapse, or environmentally sensitive locations where no disturbance can be tolerated. One form of locomotion that could allow 2D and 3D locomotion involves a cable-anchored robot.

Rather than wheels, a cable-anchored robot is designed to hang from two or more points fixed above and around the desired area of movement connected by cables. The ends of the cables meet at the robot and are each wrapped around motor-driven spools which the robot can rotate to let out cable or take it in. This effectively allows motion through a space or across a plane.

### Design Challenges

The challenge with such a system developing a controller that can accurately translate target position coordinates into corresponding cable lengths. A Translator component is required to perform this geometric conversion.

A second challenge is controlling the path between from one target coordinate to the next. For example, if a linear translation from point A to point B is desired, the actual path will vary depending on cable lengths and differential spooling motor speeds. It becomes necessary to generate and move to intermediate “waypoints” between the start and finish coordinates of a robot movement. (See Figure 1 and Figure 2)

Figure 1: Robot path between start and target coordinates

With a cable anchor system the path is never linear and varies with robot position, anchor position, cable lengths, and relative spool motor speeds. This unpredictability in the movement path is undesirable.

Figure 2: Robot path between start and target coordinates using a Path Planner

The path planner “pins” waypoints during robot travel. This removes a large part of the variability that the geometry of the anchor points and cables introduces. Increasing the number waypoints increases the linearity of the path.  This acts as a smoothing operation on the final path of the robot.

It is also desirable to be able to preload a sequence of target coordinates into the robot controller, so that a new destination does not have to be loaded only when the current destination is reached.

### Chosen implementation

A subset of this problem using two cables and a planar surface for movement was used as a real-world design specification. The robot would translate against a vertical surface using fixed cables that can wind on motor controlled spools attached to the top of the robot (Figure 3). The attachment points to the surface (in this case instantiated as a Chalk Board) can be arbitrary, and the controller is modeled in such a way that target (x, y) Cartesian coordinates can be translated to desired cable take-up and incremental motor movements. The Robot model is implemented so that the path between the current Robot position and the desired position is calculated in steps of defined resolution. This allows linear Robot movement to the target position regardless of the geometry of the robot and cable attachment points.

This controller is designed in such a way that it can be implemented using the Lego Mindstorms™ robotics construction toolkit and DEVS-Mindstorms programming interface.

Figure 3: Chosen system design

The robot can translate on a vertical surface, in this case represented by a chalk board. Two cables fixed to anchor point above the robot allow movement by spooling in or out excess cable from two motor on the robot chassis.

### Robot Controller Processing System

Developing architecture for the robot controller involved breaking down the behaviour of the Robot into sequential, event-driven steps. These each had a unique function and could be more easily translated into Atomic models for CD++ implementation, as seen in Figure 4. The corresponding CD++ atomic models can then be matched to each step of the process.

Queue Atomic Model

Event File with coordinates

Path Generator Atomic Model

Translator Atomic Model

Sequential list of target coordinates for Robot movement

Store  these waypoints for use by the Robot

Plan a linear path of intermediate waypoints between Robot and next target coordinate

Translate next intermediate waypoint to cable lengths

Current target coordinate reached, request next target.

Current intermediate waypoint reached. Request next intermediate waypoint

Done movement

(sent by each motor)

Move motors to spool in our appropriate out cable length

Motor Atomic Model

Figure 4: Process flow for the robot controller

### Overall Model Structure

out

Event File

Motor 2

out

Motor 1

out

Generator

in:

{X, Y, D}

done

out

Path Generator

in

done

out

Translator

in:

next(ΔX, ΔY)

done

motorOut1

out

Robot

nextPosition

Chalk Engager

in

engaged

in

motorOut2

in

Queue

Planner

out

in

ready

done

in

engaged

motorOut1

motorOut2

out

Figure 5: Overall Model of Robot System

## Componentized Breakdown

### Robot

The Robot is the ‘top’ level coupled model.  This represents a cable-stay robot.  Each robot is composed of a Generator and Chalk Engager and two motors.  This simulates how a real cable-stay robot would work.  A robot would contain controllers, in this case a chalk engager, and two motors that communicate with Mindstorms.

A robot would also require a software portion, in our case the Generator, which translates input into a format understandable by each controller.   The robot can be in either passive or active state.  When in passive the robot is waiting for the next event from the event file when in active the robot is handling an event from the event file or interpreting a response from the subsystem.

### Generator

A Generator is a coupled model coupling a path generator and a translator.   The purpose of this model is to combine the translation of an x and y position to a format that the controllers (two motors and a chalk engager) would be able to understand.    The purpose of the path generator is to handle multiple concurrent inputs and to perform a pseudo smoothing operation by splitting large requests by into smaller segments that can be sent into the translator.

The generator can be in either active or passive state.  Initially in passive mode the generator is waiting for an in event from the top model or a response from the internal system.  When in active mode the generator is processing input from the top model or interpreting a response from the subsystem.

### Path Generator

The path generator is a coupled model that combines a queuing feature of points and a planner which splits large user requests into manageable portions that will smooth the final output of the robot.

The path generator can be in either a passive or active state.  When passive the path generator is waiting for input from other component.  When in active the path generator is in the process of handling input.

### Queue

The Queue is atomic model that buffers any input received.  It contains two inputs one that input may coming into that is buffered and a ready port which when set active will output the next item at the beginning of the buffer.   The queue is either in an active or passive state.  In passive mode the queue is waiting for new data or for a ready signal that will output the next item.  When in the active state the queue stores the next item if an input or if a ready signal it will output the next item in the queue.  If the queue is empty and it receives a ready signal the queue goes into a passive state to wait (which is internally controlled by a ‘waiting’ flag).  Upon start up the first item received is immediately pushed to the output.

### Planner

This model divides an X, Y destination coordinate into discrete movement steps.  This serves to perform a basic smoothing operation based resolution (or STEP_SIZE).   For an example of why smoothing is required see the overall design requirements in Figure 1 and Figure 2.

Each interim destination coordinates are sent via the nextPosition port.   The planner will wait until input is received on done port before sending out the next position.  This ensures that the motor completes a translation before sending the next position.   If an error occurs the translation is aborted.  The final output indicating success or failure is sent via the out port.

The planner contains two states passive and active.  It is in passive state while waiting for a new position to translate to, and while processing a plan for the next position it is in the active state.

### Translator

The translator is an atomic model which converts a point in Cartesian space to discrete cable lengths that each motor must expelled in order to draw at that point.  The translator contains the position of each motor which is required to calculate the new lengths required.   The translator waits until a response is received by both motors before reporting a success or failure condition.  Failure is considered the dominate condition, i.e. if any motor fails the entire translation is considered to have failed.

The translator may be either a passive or active state.  In a passive state the translator is waiting for input of either a new position to calculate and adjust the motors to or waiting for a response from each of the motors.    Once a response is received from both motors the translation reports the status to the ‘out’ port.

### Chalk Engager

The Chalk Engager is an atomic model that represents the motor that controls the engagement of the chalk for drawing.  The in port accepts a Boolean that if set to true will move the chalk so that it will draw on the chalkboard.  The current state of the chalk is stored, so that a request to put the chalk into the state that it is currently in will be ignored.

The chalk engager can be either in a passive or active state.  In passive, the chalk engager is waiting for a command that will change the visibility flag of the motor.  In active the chalk engager is in the process of moving the motor controlling the chalk engagement.

### Motor

The Motor is an atomic model that is provided with a new length for the amount of cable to expel.  This model contains the amount of cable that is on the spool and the amount that is requested to be expelled.  If the request is larger then the amount of cable on the spool then the motor moves to the maximum length but is considered to have failed and will report a failure condition to the out port.   Otherwise a ‘success’ condition will be sent to the out port when the moving is complete.

Figure 6: Model of Chalk Engager

The motor in Mindstorms can be provided with an angular position to rotate to. This functionality is wrapped within a model which also contains the motor cables length (total), amount of cable expelled and the motor spool’s radius (in order to relate angular rotation with cable length change).

The motor can be either in a passive or active state.  In the passive state the motor is waiting for a command that will change the amount expelled.  In the active state the motor is in the process of moving the Mindstorms motor changing the amount of cable currently expelled.

# Part II: Detailed Design & Testing Strategy

## Atomic Models

For all atomic models, the internal structure in Figure 7 applies.

in

in

in

out

P

A

out

Figure 7: DEVS atomic model internal structure

### Chalk Engager

### Chalk Engager

_engaged

in

out

For a description of the Chalk Engager see section 2.7.

Internally the chalk engager contains the _engaged attribute which stores whether the chalk is currently engaged to prevent the motor from requesting a change to the current state.

Though not documented used in the final coupled model the chalk engager contains an output port indicating when the chalk motor is engaged that is used for testing purposes.

#### Formal Specification

The formal specification <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in}

Y = {out}

δint (active) = passive

δext (in, *)

{

if ( msg.value != _engaged )

_engaged  msg.value

return active

else

return passive

end

}

λ(active) = _engaged

ta(passive) = INFINITY

ta(active) = preparation time specified in ma file

Motor

_expelled

_target

_total

_success

out

in

For a description of the Motor see section 2.8.  Internally the motor contains the _expelled, _target, _total and _success attributes, representing the total amount of cable expelled the target amount to be expelled and the total amount of cable on the spool and whether the motor movement completed with success.

Figure 8: Model of Motor

The _total and _expelled attributes are initially set to values read in from the .ma file.  The _target is adjusted each time a request is made via the ‘in’ port.  The amount _expelled is guaranteed to be less than or equal to the _target amount.   If the motor is requested to move to an impossible location, the _success flag is set to false and this status is outputted to the out port after moving the motor the most extreme position possible.  If the requested movement succeeds a ‘true’ value is outputted to the out port upon completion of the movement.    The amount of time required for the movement is a linear time depending on the change in distance that the motor must move.

#### Formal Specification

The formal specification <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in}

Y = {out}

δint (active) = passive

δext (in, passive) =

{

_success  msg.value > 0 AND msg.value < _total

if ( _success )

_target  msg.value

else

if ( msg.value < 0 )

_target  0

else

_target  _total

end

end

Δdistance  abs( _target - _expelled )

return active

}

δext (in, active) = active

λ(active) = _success

ta(passive) = INFINITY

ta(active) = Minutes: Δdistance / 60; Seconds: Δdistance mod 60

### Planner

_destination

_current

_visible

_success

out

in

done

engaged

nextPosition

ready

Figure 9: Model of Planner

For a description of the Planner see section 2.5. Internally the planner contains the _destination, _position, _visible and _success attributes, representing the requested destination, the current position, whether this line is to be drawn and whether the translation completed with success.

The _destination and _visible attributes with every triple input received on the in port.   After each translation the _current position is updated until the planner reaches are point where _current is equal to _destination as long as _success is set to true.  On failure the planner quits outputting this failure case to the out port.   The engaged port is outputted at the beginning of a translation indicating whether the chalk should be set visible or not.

#### Formal Specification

The formal specification <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in, done}

Y = {engaged, nextPosition, out, ready}

δint (active) = passive

δext (in, *) =

{

_input[index]  msg.value

if ( index == 3 )

_visible  input[0]

_destination  Point( input[1], input[2] )

_success  true

index  0

else

Index  index + 1

end

return active

}

δext (done, *) =

{

if ( _success )

_success  msg.value

end

Return active

}

λ(active) =

{

if ( _postion != _destination )

output( engaged, _visible )

Δposition  _destination - _position

if ( Δposition.x != 0 && Δposition.y != 0 )

if abs( Δposition.x ) > abs( Δposition.y )

Δposition.x  STEP_SIZE

Δposition.y  slope * STEP_SIZE

else

Δposition.x  (1/slope) * STEP_SIZE

Δposition.y  STEP_SIZE

end if

else if ( Δposition.x != 0 )

if ( Δposition.x < 0 )

Δposition.x  -STEP_SIZE

else

Δposition.x  STEP_SIZE

end

else if ( Δposition.y != 0 )

if ( Δposition.y < 0 )

Δposition.y  -STEP_SIZE

else

Δposition.y  STEP_SIZE

end

end

_position  _posittion + Δposition

output( nextPosition, _position.y )

output( nextPosition, _position.y )

end
}

ta(passive) = INFINITY

ta(active) = 0

### Translator

_complete

_success

_anchorPoints[2]

motorOut1

in

done

out

motorOut2

For a description of the Translator see section 2.6.  Internally the translator contains the _complete, _success attributes, representing whether the movement has been completed, and whether it completed with success, respectfully.  The _anchorPoints attribute is an array containing the anchor points of each motor that must be specified in the .ma file.

#### Formal Specification

The formal specifications <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in, done}

Y = {motorOut1, motorOut2, out}

δint (active) = passive

δext (in, *) =

{

_input[index]  msg.value

if ( index == 2 )

for each motor ( I: 0 to …)

x  _input[0] – anchorPoint[I].x

y  _input[1] – anchorPoint[I].y

angle  atan2( y, x )

if ( cos( angle ) == 0 )

_targetLength[I]  y

else

_targetLength[I]  x / cos( angle )

end

end

return active

else

index  index + 1

return passive

end

}

δext (done, *) =

{

if ( _success )

_success = msg.value

_complete  count >= NUMBER_OF_MOTORS

if ( _complete )

return passive

else

return active

}

λ(active) =

{

if ( _complete )

output( out, _success )

else

output( motorOut1, _targetLength[0] )

output( motorOut2, _targetLength[1] )

end
}

ta(passive) = INFINITY

ta(active) = 0

Queue

Figure 10: Model of Translator

_queue

_waiting

out

in

ready

For a description of the Queue see section 2.4.  Internally the queue contains a _queue attribute for buffering input and a _waiting attribute to indicate that it is waiting for a response before forwarding the next item in the queue.

Figure 11: Model of Queue

The formal specification <S, X, Y, δint, δext, λ, ta> for the atomic model is defined as follows:

S = {passive, active}

X = {in, ready}

Y = {out}

δint (active) = passive

δext (in, passive) = active

δext (in, active) = active

δext (ready, passive) = active

δext (ready, active) = active

λ(active)

{

if ( !_queue.empty )

output( out, _queue.pop )

}

ta(passive) = INFINITY

ta(active) = 0.0

## Coupled Models

### Generator

out

in

done

engaged

motorOut2

motorOut1

For a description of the Path Generator model see section 2.2.

Figure 12: Model of Generator

The formal specification <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT> for the coupled model is defined as follows:

X = {in, done};

Y = {out, engaged, motorOut1, motorOut2};

D = {pathGenerator1, translator1};

M(pathGenerator1) = PathGenerator;

M(translator1) = Translator;

I(pathGenerator1) = {translator1};

I(translator1) = {pathGenerator1, self};

Z(pathGenerator1) = {translator1};

Z(translator1) = {pathGenerator1, self};

SELECT: 	({pathGenerator1, translator1}) = pathGenerator1;

### Path Generator

out

in

done

engaged

nextPosition

For a description of the Path Generator model see section 2.3.

Figure 13: Model of PathGenerator

The formal specification <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT> for the coupled model is defined as follows:

X = {in, done};

Y = {out, engaged, nextPosition};

D = {queue1, planner1};

M(queue1) = Queue;

M(planner1) = Planner;

I(queue1) = {planner1};

I(planner1) = {queue1, self};

Z(queue1) = {planner1};

Z(planner1) = {queue1, self};

SELECT: 	({queue1, planner2}) = queue1;

### Robot

Robot

out

Event File

Figure 14: Model of Robot

For a description of the Robot model see section 2.1.

The formal specification <X, Y, D, {Mi}, {Ii}, {Zij}, SELECT> for the coupled model is defined as follows:

X = {Event File};

Y = {out};

D = {generator1, engager1, motor1, motor2};

M(generator1) = Generator;

M(engager1) = Engager;

M(motor1) = Motor;

M(motor2) = Motor;

I(generator1) = {engager1, motor1, motor2};

I(engager1) = {self};

I(motor1) = {generator1, self};

I(motor2) = {generator1, self};

Z(generator1) = {motor1, motor2};

Z(engager1) = {engager1};

Z(motor1) = {generator1, motor1};

Z(motor2) = {generator1, motor2};

SELECT:

({generator1, engager1, motor1, motor2}) = generator1,

({generator1, motor1, motor2}) = generator1,

({generator1, engager1, motor1}) = generator1,

({generator1, engager1, motor2}) = generator1,

({engager1, motor1, motor2}) = engager1,

({generator1, engager1}) = generator1,

({generator1, motor1}) = generator1,

({generator1, motor2}) = generator1,

({engager1, motor1}) = engager1,

({engager1, motor2}) = engager1,

({motor1, motor2}) = motor1;

## Testing Strategy

### Test Plan: Tools

Integration and Unit testing was straightforward using the Eclipse-based CD++ development tool chain. Each atomic and coupled model was tested using a dedicated .ma architecture file with that model defined as [top] and one or more event files. Resulting log files were viewed and interpreted using CD++ Modeller trajectory plots. Additional debugging was done using text outputs within the CPP model files that could be viewed in the simu.exe execution window.

### Test Plan: Procedure

The testing procedure was a full bottom-up unit test of all atomic and coupled models, finishing with the final coupled Robot model. This would allow early detection of bugs and algorithm errors resulting. The result was a smoother development process with error sources quickly narrowed to specific atomic models and applicable code. Where appropriate, up to four types of unit tests were conducted on models:

Test model with valid, expected data. This tests soundness of algorithms and event-driven behaviour and can be checked against pencil and paper calculations.

Test model valid, unexpected data. This could include redundant or repeat signal events. Robot should not enter an inappropriate state.

Test model with invalid and malformed inputs. This could include negative numbers for cable lengths or incomplete coordinate sets. This represents malformed event files or other input sources to the robot.

Test model on boundary conditions.  To see if the robot and path generator handle cases ensure that internal boundary conditions are correctly implemented.

Upon successful completion of all the integration tests a full system test will be performed that will serve as an integration test of all parts and a regression test to ensure that none of the existing functionality is broken by other parts of the system.

### Boundary Condition Test Description

#### Robot Model Boundary Test

This tested Robot model behaviour when the target coordinates were no longer between and below the two anchor points, but outside and to the right of the rightmost anchor point. Although this is physically impossible (unless the cables could have compression applied to them) it is algorithmically and geometrically valid. The Robot model should successfully plan a path to this coordinate and simulate movement and associated cable lengths. The actual testing of this boundary condition was carried out during integration testing of the Robot coupled model but is included here for completeness.

#### Path Generator Boundary Test

This was a boundary test in which the robot was vertically in-line with the anchors and to the outside-right of the rightmost anchor. Although physically impossible, it is algorithmically and geometrically valid with a boundary condition that cable angles are equal to zero. This test would verify that the simulation checks for divide-by-zero conditions during calculations, and return the appropriate cable length boundary values. A similar case occurs when the robot is directly below anchor 1 or 2. The actual testing of this boundary condition was carried out during integration testing of the Robot coupled model but is included here for completeness.

### Full System Test with a Complex Path

This set of tests, engages the model with valid inputs which are physically impossible for the system. This could include attempts to move the robot outside the physically possible movement area between and below the anchor points or requesting that a motor spool out more cable that it has available.

A “round trip” regression test was designed to verify that the final Robot coupled model would perform as expected with a more complex path to follow. The test plan consisted of 4 parts:

Pen and paper design of a set of waypoints. In this case a 22 waypoint path was designed so that the robot would write the word “Lego” on the chalkboard. This test would cover some key performance points of the robot architecture, including:

Paths that would require intermediate waypoints to be generated by the Planner

Paths in all four possible slope quadrants (∆x, ∆y), (-∆x, ∆y), (∆x, -∆y), (-∆x, -∆y), as well as special cases where ∆x or ∆y were equal to zero.

Paths that would cover the same point multiple times – this retracing would verify that the planner is not “collapsing” the set of input waypoints and that each waypoint in the input sequence would be considered ordered and unique. It would also test for any accumulated error in the path Planner and the Translator, which would show up as non-overlapping output points that were identical in the input file.

Input waypoint pairs into an event file – a pair of points and the chalk engage/disengage flag are passed to the Robot in triples with a specific sequence in time: 

For each target waypoint:

<t+dt1> in [ 1 | 0 ]		// chalk engaged | disengaged
<t+dt2> in [ Value ]		// Target X coordinate
<t+dt3> in [ Value ]		// Target Y coordinate

Configure the robot with .ma file anchor positions, initial robot position, corresponding cable lengths and maximum cable lengths. Configure Translator to output its cable lengths to a comma-separated variable file.

Execute the simulation and input the resulting cable length sequence pairs (LengthMotor1, LengthMotor2) into Excel-based graphing application that was designed as part of the test to solve for Robot (x, y) coordinates based on input cable lengths using a different method than that used by the robot’s Translator model. Matching the resulting plot visually against the plot from the input waypoint set showed any errors or deviations immediately.

# Part III: Verification & Validation

## Verification of Atomic Model “Queue”

### Event file input

The input file tested behaviour during loading the queue via the in port, emptying of the queue by sending any input to the ready port, reloading, and fully emptying:

00:00:01:00 in 1

00:00:02:00 in 2

00:00:03:00 in 3

00:00:04:00 ready 1

00:00:05:00 in 4

00:00:06:10 in 5

00:00:07:20 in 6

00:00:20:00 ready 2

00:00:21:00 ready 3

00:00:22:00 ready 4

00:00:23:00 ready 5

00:00:24:00 ready 6

00:00:25:00 ready 7

### ma file

[top]

components : queue1@Queue

out : out

in : in

in : ready

Link : in in@queue1

Link : ready ready@queue1

Link : out@queue1 out

### CD++ Modeller Trajectories

Figure 15: Queue Trajectories

Red is the queue loading input, blue is the output, and green shows the ‘ready’ inputs which trigger output on out. An increasing sequence is used for clarity on the ready port to differentiate between signals.

### Analysis

The trajectory plots from CD++ Modeller show that the queue fills and empties as expected. When empty, it immediately passes the first value it receives to the Out port, and then waits for an input at the ready port to send the next value. Additional redundant ‘ready’ inputs were sent to test output behaviour when the queue was empty – as expected, no output was sent in this case.

## Verification of Atomic Model “Planner”

### Event file input

The input file tested behaviour during loading the planner via the in port, simulation the movement of each point by sending any input to the done port:

00:00:01:00 in 1

00:00:02:00 in 3

00:00:03:00 in 6

00:00:06:00 done 1

00:00:08:00 done 2

00:00:10:00 done 3

00:00:12:00 done 4

00:00:14:00 done 5

00:00:16:00 done 6

### ma file

The planner .ma file includes initial conditions include the Robot’s initial coordinates {x, y}

[top]

components : planner1@Planner

out : nextPosition

out : ready

out : engaged

out : out

in : in

in : done

Link : in in@planner1

Link : done done@planner1

Link : nextPosition@planner1 nextPosition

Link : out@planner1 out

Link : ready ready@planner1

Link : engaged@planner1 engaged

[planner1]

intial.x : 0

intial.y : 0

### CD++ Modeller Trajectories

Figure 16: Planner Trajectories

Red shows the input sequence: Chalk Engaged, followed by desired X and desired Y coordinates for the robot. The nextPosition trajectory shows planner’s sub-goals X, Y coordinates, which are sent to the out port as sequential new targets when done input signals are received. In the coupled model, these done inputs are sent from the motors via the translator. The pink trajectory shows the planner generating a ready output once an entire movement sequence goal is achieved.

### Analysis

The Planner is the single most complex atomic model in the system, as it is responsible for creating a series of waypoint from the robot’s current position to the desired target coordinates. The model behaves as expected, effectively linearizing the path using waypoints which are determined by a specified maximum step size (the maximum distance the robot can travel in any one direction before another waypoint must be calculated).

## Verification of Atomic Model “Translator”

### Event file input

A coordinate pair (x, y) of (3, 4) was sent with initial conditions set in the .ma file of anchor point 1 at (0, 0) and anchor point 2 at (5, 0). The resulting cable length was checked against manual calculation.

00:00:01:00 in 3

00:00:02:00 in 4

00:00:10:00 done 1

00:00:15:00 done 2

### ma file

[top]

components : translator1@Translator

out : motorOut1

out : motorOut2

out : out

in : in

in : done

Link : out@translator1 out

Link : motorOut1@translator1 motorOut1

Link : motorOut2@translator1 motorOut2

Link : in in@translator1

Link : done done@translator1

[translator1]

anchor1.x : 0

anchor1.y : 0

anchor2.x : 5

anchor2.y : 0

### CD++ Modeller Trajectories

Figure 17: Translator Trajectories

The red plot shows a target X, Y coordinate pair being input for translation to corresponding cable lengths. Green and yellow plots show the resulting cable 1 and cable 2 target cable lengths calculated by the translator and output to the motors. The blue plot is a simulated signal from each motor signalling that the specified length has been reached. Once the Translator has received done port signals from both motors, it triggers an output on its out port (bottom plot) indicating robot translation is complete.

### Analysis

The Translator atomic behaved as expected and successfully translated a simulated target coordinate pair into corresponding cable lengths. The Translator also successfully parsed the separate done messages from each motor and sent out the correct out message to signal motor motion complete. It is noted that this test plan did not cover the input of multiple coordinate pairs, which led to discovery of an input buffer clearing issue in the testing of the coupled model Generator (see section 12)

## Verification of Atomic Model “Engager”

This atomic model manages the chalk or pen, which can be retracted or deployed. There is a ‘deploy time’ modelled in the Engager. The engager is designed such that requests to ‘engage’ or ‘disengage the chalk’ are sent as Boolean inputs to the in port. Completion of chalk engagement or retraction is sent via the out port.

### Event file input

The event file simulates requests to retract and deploy the chalk. Multiple requests are modelled to test verify that chalk stay in place.

00:00:01:00 in 1

00:00:10:00 in 1

00:00:20:00 in 0

00:00:30:00 in 0

00:00:40:00 in 1

### ma file

The default state of the Engager and its preparation time is set in the .ma file.

[top]

components : engager1@Engager

out : out

in : in

Link : out@engager1 out

Link : in in@engager1

[engager1]

engaged : 0

preparation : 00:00:02:00

### CD++ Modeller Trajectories

Figure 18: Engager Trajectories

The red plot shows input and the blue plot the confirmation output from the model. The preparation time delay is visible in as a time shift between input and output.

### Analysis

This model was relatively straightforward to implement, yet shows the advantages of the DEVS formalism in easily implementing unknowns of real hardware performance. The model behaved as expected.

## Verification of Atomic Model “Motor”

Initial value are set for length, the total length of cable available on the motor spool, and expelled, the total length of cable that has been unwound from the spool. Te actuation time of the motor is modelled as a calculated time advance function duration proportional on the length of cable to be would or unwound. New length port inputs are ignored until actuation is complete.

### Event file input

The event file first spools out cable, ten spools in cable, then tests an error condition in which the requested spool-out length is greater than the total length of cable available.

00:00:01:00 in 7.7

00:02:00:00 in 2.1

00:10:03:00 in 12

### ma file

[top]

components : motor1@Motor

out : out

in : in

Link : out@motor1 out

Link : in in@motor1

[motor1]

length : 10

expelled : 4

### CD++ Modeller Trajectories

Figure 19: Motor Trajectories

### Analysis

The trajectories show that the model behaved as expected, sending success outputs when the desired length is reached, and sending an error code of zero when the cable expelled exceeds the total cable length, seen here after the third and last command from the even file at 10:11:000. Although it is not visible in the trajectories since it is an internal variable, the motor position was verified to be at its maximum of 10 length units spooled out at the time the “out of cable” error signal was to the “out” port. This behaviour is desirable, as it allows robot maximum range of movement even if the exact boundaries of the robots range of motion are not explicitly known when commands are sent. This is a “best effort” failure mode as opposed to a predictive failure mode.

## Verification of Coupled Model “PathGenerator”

This coupled model combines uses the Queue atomic model to buffer data into the Planner atomic model. The overall behaviour of the PathGenereator coupled model should be appearing the same as thee Planner model alone. Initial verification showed that the Planner was not passivating until the translator reported back a motor done condition via the PathGenerator done port, demonstrating the effectiveness of unit testing in detecting logic errors.

### Event file input

The event file tested three key performance aspects of this central part of the robot:

Loading the Queue with multiple sequential target coordinates before the actually movements could be completed, and verifying that motions are carried out sequentially and that no data is lost or out of sequence.

Testing the algorithm for path generation in the planner was performed along positive and negative diagonal axis (covering all four quadrants of movement).

In addition, this test included sending redundant motor done commands in an empty Queue condition.

00:00:01:00 in 1		//Boolean: Chalk engage

00:00:02:00 in 3		//X-coordinate target

00:00:03:00 in 6		//Y-coordinate target, full ‘instruction’ complete

00:00:04:00 in 0		//start of next instruction: chalk disengage

00:00:05:00 in 0		//X-target

00:00:05:500 in 0		//Y-target

00:00:06:00 done 1	//emulated motor done commands,

00:00:08:00 done 2	//incremental for clarity on trajectory trace

00:00:10:00 done 3

00:00:12:00 done 4

00:00:14:00 done 5

00:00:23:00 done 6

00:00:25:00 done 7

00:00:27:00 done 8

00:00:29:00 done 9

00:00:31:00 done 10

00:00:33:00 done 11

00:00:35:00 done 12

### ma file

The .ma file for the PathGenereator coupled model adds a Queue atomic in ahead of a Planner atomic. Initial position for the robot is set through initial.x and initial.y parameters.

[top]

components : queue1@Queue

components : planner1@Planner

out : out

out : engaged

out : nextPosition

in : in

in : done

Link : in in@queue1

Link : out@queue1 in@planner1

Link : ready@planner1 ready@queue1

Link : out@planner1 out

Link : engaged@planner1 engaged

Link : nextPosition@planner1 nextPosition

Link : done done@planner1

[planner1]

intial.x : 0

intial.y : 0

[queue1]

### CD++ Modeller Trajectories

Figure 20: PathGenerator Trajectories

The top trace show the coordinate pair (3, 6) loaded into the queue model, followed by the coordinate pair (0, 0). The blue trace on Ready shows the Planner requesting two different sets of coordinates from the Queue model as motions are completed. The Red trace shows the Planner’s incremental position output coordinate pairs first in six steps up to the target (3, 6) then six further steps to the next target (0, 0).

### Analysis

The test plan for the PathGenerator composite model focussed on the interaction of the Queue with the Planner in accepting and sequential sets of targets and outputting appropriate incremental coordinate targets. Testing showed that the second coordinate target, in which the lanner makes incremental steps in the negative X direction, showed problems with slopes calculation in the Planner, which were corrected. These were clearly visible in the red nextPosition race as a lack of stair-step output back to (0, 0). The test plan also covered a Queue containing multiple and partial coordinate sets. The final path generator behaved as expected.

## Verification of Coupled Model “Generator”

### Event file input

For verification purposes, this event file sends the same pair of sequential target coordinates that were used to test the PathGenerator: (3, 6) followed by (0, 0). Compared to the PathGenerator test, twice as many inputs to the done port need to be injected, as the translator waits for a done input from each of the two connected motors. At the end of the event file an out of bounds condition is tested in which a coordinate outside the capability of the robot is set (-1, -1).

00:00:01:00 in 1

00:00:02:00 in 3

00:00:03:00 in 6

00:00:04:00 in 0

00:00:05:00 in 0

00:00:05:500 in 0

00:00:06:00 done 1

00:00:07:00 done 2

00:00:09:00 done 3

00:00:10:00 done 4

00:00:12:00 done 5

00:00:13:00 done 6

00:00:15:00 done 7

00:00:16:00 done 8

00:00:18:00 done 9

00:00:19:00 done 10

00:00:26:00 done 11

00:00:27:00 done 12

00:00:29:00 done 13

00:00:31:00 done 14

00:00:33:00 done 15

00:00:34:00 done 16

00:00:36:00 done 17

00:00:37:00 done 18

00:00:39:00 done 19

00:00:40:00 done 20

00:00:41:00 done 19

00:00:42:00 done 20

00:00:50:00 in 1

00:00:51:00 in -1

00:00:52:00 in -1

### ma file

The Generator instance is composed of three atomics, two of which are contained in the coupled model pathGenerator1, which was tested previously (see section 11). The initial position of the Robot is specified for the Planner as initial.X and initial.Y. The positions of the two cable anchor points are specified for translator1 as (0, 0) and (0, 5).

[top]

components : pathGenerator1

components : translator1@Translator

in : in

in : done

out : out

out : engaged

out : motorOut1

out : motorOut2

Link : in in@pathGenerator1

Link : done done@translator1

Link : nextPosition@pathGenerator1 in@translator1

Link : out@translator1 done@pathGenerator1

Link : out@pathGenerator1 out

Link : engaged@pathGenerator1 engaged

Link : motorOut1@translator1 motorOut1

Link : motorOut2@translator1 motorOut2

[pathGenerator1]

components : queue1@Queue

components : planner1@Planner

in : in

in : done

out : nextPosition

out : engaged

out : out

Link : in in@queue1

Link : out@queue1 in@planner1

Link : ready@planner1 ready@queue1

Link : done done@planner1

Link : out@planner1 out

Link : nextPosition@planner1 nextPosition

Link : engaged@planner1 engaged

[planner1]

intial.x : 0

intial.y : 0

[queue1]

[translator1]

anchor1.x : 0

anchor1.y : 0

anchor2.x : 5

anchor2.y : 0

### CD++ Modeller Trajectories

Figure 21: Generator Trajectories

Note that CD++ Modeller plots the in and done port of the Generator over top of the similarly named ports of the PathGenerator and Translator. Target coordinates of (3, 6) are loaded on the in port (start of top trace), followed by a second target of (0, 0) immediately after, which was shown to be successfully queued in previous tests. These results in intermediate coordinate steps are seen again in the top trace – these are the inputs to the Translator. The translator converts each of these intermediate target coordinates to corresponding target cable lengths for motor1 and motor2, which are seen in traces motorOut1 and motorOut2. These were cross checked against a pen-and-paper simulation of the robot and anchor point geometry and found to be accurate. The pink trace shows stepped inputs to the done port of the Generator that emulate the motor models’ signals when they have wound or unwound the desired cable length. The traces show that the Generator sends the next set of motor commands only when both motors have reached the desired position, and that the generator sends the correct output to from its out port (blue trace) when a target coordinate is reached.

Figure 22: Additional Trajectories for Generator

Generator trajectories for simple boundary condition: The robot starts at (2, 5) and attempts to move beyond anchor2.x position of (0.5) towards (10, 0). The PathGenerator does not check for this boundary condition and generates an erroneous path plan, which then is accurately converted to motor cable lengths by the Translator. This boundary test demonstrates the desirability of boundary checking at the atomic model level if possible.

### Analysis

The Generator initially did not behave as expected, despite successful verification of each of its component atomics. This showed a flaw in the verification of the Translator atomic model, in that only a single coordinate was translated in any one test. Had multiple coordinates been translated sequentially, an input buffer clearing bug would have been discovered. Because of the coupled structure of the overall model, previous testing and CD++ trajectory visual analysis, it was relatively straightforward to determine the root cause and correct it. The Generator behaves as expected when given in-bounds coordinates (those that are below and between the two anchor points). However, the Translator was shown to not check for positive X out of bounds conditions where the target robot coordinates are not between the anchor points

## Verification of Coupled Model “Robot”

### Event file input

As testing moves to more integrated coupled models, the event file decreases in complexity, as only target coordinate pairs and chalk engagement Booleans need to be loaded into the model. In this case the robot is instructed to move from its current position of (0, 0) to (3, 6) with the chalk engaged, then to return to the original position with the chalk disengaged.

00:00:01:00 in 1

00:00:02:00 in 3

00:00:03:00 in 6

00:00:04:00 in 0

00:00:05:00 in 0

00:00:05:500 in 0

### .ma file

[top]

components : engager1@Engager

components : motor1@Motor

components : motor2@Motor

components : robotGenerator1

in : in

out : out

Link : in in@robotGenerator1

Link : out@robotGenerator1 out

Link : out@motor1 done@robotGenerator1

Link : out@motor2 done@robotGenerator1

Link : engaged@robotGenerator1 in@engager1

Link : motorOut1@robotGenerator1 in@motor1

Link : motorOut2@robotGenerator1 in@motor2

[robotGenerator1]

components : pathGenerator1

components : translator1@Translator

in : in

in : done

out : out

out : engaged

out : motorOut1

out : motorOut2

Link : in in@pathGenerator1

Link : done done@translator1

Link : nextPosition@pathGenerator1 in@translator1

Link : out@translator1 done@pathGenerator1

Link : out@pathGenerator1 out

Link : engaged@pathGenerator1 engaged

Link : motorOut1@translator1 motorOut1

Link : motorOut2@translator1 motorOut2

[pathGenerator1]

components : queue1@Queue

components : planner1@Planner

in : in

in : done

out : nextPosition

out : engaged

out : out

Link : in in@queue1

Link : out@queue1 in@planner1

Link : ready@planner1 ready@queue1

Link : done done@planner1

Link : out@planner1 out

Link : nextPosition@planner1 nextPosition

Link : engaged@planner1 engaged

[planner1]

intial.x : 0		// initial robot position is adjacent to anchor point 1

intial.y : 0

[translator1]

anchor1.x : 0

anchor1.y : 0

anchor2.x : 5		// anchor point two is 5 distance units to the

anchor2.y : 0		// right of Anchor point 1

[motor1]

length : 10		// initial conditions chould be physically consistent

expelled : 0		// since the Robot is adjacent to motor 1, 0 cable has been

// expelled from that motor’s spool

[motor2]

length : 10

expelled : 5		// physically consistent: robot is 5 units away from

// anchor point 2 and therefore must have expelled

// 5 units of cable

[engager1]

engaged : 0 				// initial condition: chalk is off of surface

preparation : 00:00:00:500		// time for chalk actuator to engage

### CD++ Modeller Trajectories

Figure 23: Robot coupled model trajectories.

The robotGenerator sub-model is shown here since the Robot model trajectories are simple the target coordinate inputs and an overall done signal when the target coordinate storage queue has been emptied. The behaviour shown here matches the earlier robotGenrator test results, the only change being that the motor models and Chalk Engager sent the appropriate done signals, rather than the Event File. The notable traces are Motor1out and Motor2out which show that appropriate cable lengths were spooled out to move the robot between desired target’s coordinates using the waypoints generated by the PathGenerator. The Engaged trace shows the chalk movement matches desired behaviour, drawing a line for the first movement and retracting the chalk for the return movement. Cable lengths for a subset of intermediate waypoints were verified by pencil-and-paper calculation as accurate.

### Simulation Analysis

The Robot coupled model was expected to have similar behaviour to the earlier robotGenerator test, since the only change was the addition of the motor and [chalk] Engager atomics which simulated delay time for actuation. The model behaved as expected, planning and following a physically feasible set of paths. Additional testing was conducted in which the target robot position extended beyond the available lengths of one or both motors, resulting in robot travel to the maximum of one cable, then stopping, as this motor would no longer send done signals. Because the motors are likely to run out of cable at different times, it is likely in practice that the last motion of the robot will depart from that specified by the pathGenerator. This behaviour may not be desirable, and may point to code changes which verify that the next waypoint in the planned path is achievable before executing the motor commands. This would allow best effort behaviour without unexpected motion.

## Complex Path Regression Test

Figure 24: Full robot regression test

This figure shows a scan of the hand-drawn desired path for robot, converted into a sequence of target coordinates. These were input as an event file to the coupled Robot model and a simulation was run. The output shown in the inset graph is plotted using Microsoft Excel which calculates coordinates from target cable lengths sent to the motor models.  on page 28 and the include Excel spread sheet for full results). This demonstrates a regression (round-trip) test of the robot model successfully planning, converting, and following a path using the anchor cable geometry.

As seen in Figure 25Error: Reference source not found, the resulting path plot matches within rounding error the path specified in the event file. Additional waypoints added by the Planner than 1 grid unit are visible in the output plot as extra data points on the longer paths. This regression test shows that the robot behaves as expected. The Planner generated 10 additional intermediate waypoints to produce linear longer paths between target waypoints, and the regression test of the translator showed no error in the cable length calculations.

This test initially was not completely successful, and indicated an error in the path planning algorithms was not initially detected in the Atomic model tests of the Planner and Translator. Specifically, the path planner failed to generate intermediate waypoints in the special case of vertical and horizontal lines. Initial model tests were done with only diagonal lines, so cases where the slope was infinite or zero were not part of that test plan. This was easily pinpointed and corrected. This shows the value of full-up tests, not just to verify successful integration, but also to allow more opportunities to test the System in a larger data space. The greater the level of integration when an error is found, the more difficult it is to pinpoint its source; the lesson is that exhaustive unit testing is necessary, but not sufficient, as some test cases will likely be missed.

This test also showed the value of testing algorithms in regression using two different derivation methods. Inputs and outputs should match if the system is working as expected; errors oversights are more easily spotted. The disadvantage may be in development time as a partial second set of Robot control algorithms must themselves be derived, tested and debugged. The result, however, provides an additional level of confidence on the path to hardware implementation.

## Robot Construction using Mindstorms

Lego kit 8527 was used to build the robot. Two servo motors were used to wind and unwind the cable spools. A third motor actuated the chalk writer. The Mindstorms servo motors were used to implement the Motor and ChalkEngager atomic models in the DEVS++ robot model.

The robot was controlled by a host computer via USB cable. The host computer ran the Embedded DEVS++ Robot Model via robot.bat file.

Figure 25: Mindstroms robot implementation

### Embedded CD++ Development platform Revisions

#### Libraries and Software Version

CMake 2.4

Description
CMake is a makefile and integrated development environment (IDE) project generator.   CMake generates appropriate project files for common IDEs such as Eclipse, KDevelop, Microsoft Visual Studio and many others.

Purpose
Real-time DEVS is implemented on Linux, however future support for NXT is limited (see NXT++ 0.4 library).  Converting the build system to be CMake compatible provides a first step in porting DEVS to the Windows environment.  This also provides the benefit of isolating NXT++ library and the project from Real-time DEVS.   This will make future development easier for students porting from DEVS to Real-time DEVS. It also allows for development in more IDEs than just Eclipse.

NXT 0.4 (Modified)

Description
NXT is an open-source library enabling communication between a C++ program and Mindstorms hardware.

Purpose
NXT++ provided a C++ communication management and wrapper functions for byte-level commands to the Mindstorms hardware.   Since DEVS is coded and compiled in C++, and the Mindstorms language is proprietary (although documented) this provides a suitable bridge between hardware and control software.

GCC 4.3

Description
GCC 4.3 is the latest version of the GNU C++ compiler (at the time of this project).

Purpose
Real-time DEVS was updated to support the latest GCC compiler.   As older GCC versions are becoming harder to obtain, updating the code is an important improvement for continued use of Real-time DEVS.

Additionally, memory leaks and coding errors were fixed as part of this improvement.

libusb 0.1.12

Description
Libusb is an open source library for USB communication with Linux.

Purpose
In order to communicate with NXT over a USB connection this library, with proper configuration is required.  See installation instructions on how to properly configure a USB connection on Linux kernels version 2.6 or above.

Bison 2.3

Description
Bison is a grammar parser to a code generator by GNU.

Purpose
Like DEVS, Real-time DEVS uses description files to describe the format of .ma files.  Bison is a grammar parser for Linux that converts this format to C++ code.  This code is then compiled in Real-time DEVS.

ncurses 5.7

Description
Ncurses is an open-source console text formatting library.

Purpose
Real-time DEVS internally makes some requires the curses library to format output text.

#### Installation Procedure

- Install required libraries and applications as per operating system and/or developer instructions

- cmake 2.4 (or higher)

- GCC 4.3 (or higher)

- ncurses 5.7 (or higher)

- bison 2.3 (or higher)

- libusb (0.1.12 or higher)

Note: NXT 0.4 in included with the source code and will compile as part of the compilation procedure.  There is no need to separately install this library.

- Extract code to a directory.  
Example: tar -xvf robot.tar

- In a terminal window, change to this directory.  Example: cd robot

- The USB interface in the Linux kernel has to be set up to communicate with the NXT microcontroller. This must be performed by a user with administrative privileges (root user) with the following configuration files.  On some Linux setups the commands can be executed by appending the sudo command in front of the actual command to run the command as the <root> user.

Locate and change to the directory containing the lego.sh and 45-lego.rules file extracted from the compressed project file
Example: cd cfg
Copy the lego.sh file into the /etc/udev/ directory
Example: cp lego.sh /etc/udev/
Give execution permission of the lego.sh file
Example: chmod 755 /etc/udev/lego.sh

Contents of the lego.sh file:

#!/bin/bash
#GROUP=lego
if [ "${ACTION}" == "add" ] && [ -f "${DEVICE}" ]then
	chmod o-rwx "${DEVICE}“
	chgrp "${GROUP}" "{$DEVICE}“
	chmod g+rw "${DEVICE}“
fi

Copy the 45-lego.rules file into the /etc/udev/rules.d/ directory.
Example: cp 45-lego.rules /etc/udev/rules.d/
Give execution permission to the 45-lego.rules file
Example: chmod 755 /etc/udev/rules.d/45-lego.rules

Contents of the 45-lego.rules file:
SUBSYSTEM="usb_device", ACTION="add", SYSFS{idVendor}=="0694",SYSFS{idProduct}=="0002", SYMLINK+="lego-%k", GROUP="lego", MODE="0660", RUN+="/etc/udev/lego.sh"

- Run cmake to generate the project. 
Example 1:  cmake .
To generate a project for an IDE use the following command:
	Example 2: cmake -G KDevelop3 .

The cmake command (without a period), will indicate which IDEs are available according to your system's current setup.

#### Compilation Workflow

- Build the required libraries and executable
Example: make

- Change into the “robot” directory and execute the simulation script:
cd robot
./run.sh

- Robot will execute motions based on the robot.ev file in the robot/ directory

#### Separation of Project and NXT library code from Real-time DEVS kernel code

In previous work [3] all project code was embedded directly into Real-time DEVS kernel code, which was subsequently modified to support this code injection.  For this project we wanted to abstract project code from the kernel code by placing project code in a separate component then the Real-time DEVS kernel. This streamlines future development as code development need only affect the project directory, leaving the Real-Time DEVs code-base identical between projects.   This also allows for easier portability between different versions of DEVS. The register functions were modified and placed in a header file; allowing the DEVS code to be directly used with Real-time DEVS without further modification. To test this new portability between DEVs and Real-Time DEVs, assignment 1 robot code was successfully recompiled within Real-Time DEVs.

Steps in isolating the project code, Real-time DEVS kernel code and NXT++ code were:

- Real-time DEVS code was put into the rtdevs directory, which generates the libsimu and simulat executable

- Project specific code was put into the robot directory, which generates the librobot library

- NXT++ was put into the nxt directory, which generates the libnxt library

#### Real-time DEVS Upgrade to support GCC 4.0

Real-time DEVS was originally developed to work with GCC 3.44.  New Linux distributions now use, GCC version 4.0 and obtaining the old version of GCC is becoming increasingly difficult.   Real-time DEVS and NXT libraries were upgraded by removing errors and fixing most of the warnings.  Some memory leaks within Real-time DEVS were also removed.

#### Changing of Build system to use CMake

The changing of Real-time DEVS into separate components, as mentioned previously, made this an opportune time to update the build system.  A new build system was suggested for the following reasons:

- Ability to develop in a modern user decidable IDE

- Future work may require porting to Windows as the newest versions of NXT++ don't support Linux

- Separating user-code into a library requires the user to define the name of the component into the build system.  The new build system uses easy to understand text files.   The user must edit the main CMakeLists.txt in the home directory and create a CMakeLists.txt file in their project's component directory, listing the source files prior to running cmake.  These changes will include the developer’s project in the resulting executable.

The new build system required changing the simulator executable’s name from simu to simulat.  This is required because simu previously was both the name of the library generated containing real-time DEVS and the final executable built.  The new build system doesn't support a library and executable with the same name as this isn't cross-compliant amongst various supported IDEs.

### NXT++ Library Modifications

The older version of the NXT++ library required to be used defined all functions in the header file.  This caused problems if multiple atomic models within the project required communication with the NXT brick.  The application would not link due to the duplicate definition of methods.  The library was heavily modified by separating the declaration and definition of functions.  This fix allows separate atomic models to make calls to NXT without errors.

#### Interface Considerations

The Mindstorms servo motors can be controlled via the NXT++ library using the GoTo() function, which is called in the external transition function of the Motor.cpp and ChalkEngager.cpp atomic models. The motorPower variable specified in motor.ma and chalkEngager.ma files affects both the speed and torque of the Mindstorms servomotors. The spoolDiameter variable is used in relating motor rotations cable length. The RotationsPerSecond variable related the arbitrary motorPower variable value to real-world motor rotations per second. Since GoTo() allows specification of motor power as an integer (range 0 through 100), calculations are made to determine the motor rotation time needed to spool in or out the desired length cable. This is carried out using the spoolDiameter value in the external transition function and the time advance function delay length is set based on this calculated value.

## Considerations for using Mindstorms Hardware

### Interfacing with Mindstorms

#### Interface: Bluetooth and USB

The USB interface was used for robot control in these tests. E-DEVS ran on a host computer and sent commands in real-time to the robot via its USB port –it does not reside in the NXT microcontroller’s memory.

A Bluetooth connection was tested unsuccessfully, as the Bluetooth support available in the host computers were not compatible with the Bluetooth transceiver in the NXT microcontroller. Support for a Bluetooth connection is provided in the NXT++ library.

#### Motors

The Mindstorms motors are gear-reduced, belt driven servo motors. Maximum rotation speed is approximately 3 revolutions per second, and angular resolution for individual motions is approximately 2 degrees. Maximum torque was measured to be less than 2 Newton-meters.

Motor speed is coupled to motor torque in the Mindstorms controller. Faster motor speeds (in NXT++ values approaching ‘100’) produced much more torque than lower speeds. This was considered a limitation, as maximum torque with a lower motor speed was desirable for this application. A ‘worm-gear’ speed reduction system was built to address this limitation; however the reduction ratio was close to 50:1, which would have made movement too slow. It was also noted that moderate off-axis loads on the motors created skipping or erratic rotation behaviour. An example would be a wheel on a long axle, which would set up vibrations that interfered with smooth motor rotation.

For this application ‘servo’ behaviour was desirable, i.e. specifying the motor rotation target in terms of degrees of rotation. A servo-motor is by definition a closed-loop system, meaning the servo-controller would continue rotating the motor until the target angular position was reached. Changes in motor load, and subsequent motor speed, would not affect the accuracy of a servo-motor implementation because of this closed loop behaviour, a highly desirable feature for robot motion control.

Servo calls were possible using the NXT++ interface function GoTo(targetAngle), however it was found that this command would not work properly within the real-time DEVS architecture. Positional reset between motions would not function as desired, as the NXT microcontroller would store as a state variable the rotational position of each motor. It was found that this could be successfully cleared with appropriate NXT++ function call to resetRotationCount().

A fundamental issue with the GoTo() function call and servo the operation mode was the inability to simultaneously move multiple motors using closed-loop servo control. This limitation was traced through the NXT++ library code to be an implementation limitation within the NXT microcontroller operating system itself. It is suspected that servo control is accomplished using a polling mechanism of a given motor’s rotation position sensor, and this polling requires significant bus and/or processor resources. This limitation prevented the use of the NXT motors in closed-loop servo mode.

As an alternate approach, an open-loop motor run time method was implemented. This would allow for both spool motors to operate simultaneously, circumventing the above problem. Motor speed for a given motor power setting was experimentally determined so that motor operation time could be correlated to a given rotation angle. This allowed motor rotations to be specified in terms of motor operation durations, with the potential disadvantage of inaccuracies resulting from slower motor rotation due to high spool-cable loads. Modifications to the DEVS Motor Model

The original DEVS atomic model simulated a motor operation identical to the open-loop method described above. The motor model set the time interval of its time advance function: holdIn (active, time) equal to the motor rotation duration. Additional calculations were made in the external() function based on spool diameter, motor power, and motor rotations per second in order to calculate a time advance value that would spool in or out the desired length of cable.

Calls to the NXT++ library to control NXT motor hardware were made in the external() function. To implement this functionality, the NXT++ functions setForward(motorPower) and setReverse(motorPower) were used. After the time advance interval had expired, Real-time DEVS called the output() function, which stopped and parked the NXT motors using the stop() command.

### Additions To The Robot Model .ma File

The robot.ma file was expanded to include new parameters for the hardware motor implementation. An example of the modifications can also be seen below with an atomic model test file, motor.ma:

[top]

components : motor1@Motor

out : out

in : in

link : out@motor1 out

link : in in@motor1

[motor1]

diameter : 1

length : 100

expelled : 0

motorPower : 80

rotationsPerSecond : 2.13

port : A

New parameters added for hardware support include:

- diameter: Defining the cable spool diameter

- motorPower: The fixed motor speed/torque value. In NXT the power is an arbitrary scale of 0 to 100, with 100 being full speed.

- rotationsPerSecond: The empirically  determined rotation speed of the unloaded motor at the power value specified in motorPower

- port: The NXT microcontroller port to which the motor is connected

## Testing

Video of a sample test robot test using Real-time DEVS, USB control, simulated gravity, and the robot.ma model of can be viewed online at http://jufaintermedia.com/robot/video.html

### Workflow verification using Motor Atomic model Testing

Testing was carried out with the Mindstorms hardware controlled by a host Linux computer via USB. Initial testing involved the “motor” atomic model and interface with the NXT microcontroller via the NXT++ library. This verified the workflow described previously and allowed characterization of the Mindstorms motor component. The GoTo(), stop(), setForward() and setReverse() functions were tested within the DEVS model and the compilation and control workflow was successfully verified. It was during this test that Linux USB interface scripts were implemented due to connectivity issues (see Error: Reference source not found). Putting mechanical and motor behaviour parameters in the motor.ma file simplified testing and made the Real-time DEVS code more portable.

### Full Robot Testing

The motor atomic model was the only model within the coupled robot model that was updated for Real-time DEVS implementation. All-up testing could proceed once its behaviour was verified. Bench-top testing was initially done to verify simultaneous operation of both spool motors. It was discovered at this point that servo control was not feasible, so the open-loop motor control method described in 16.1.2 was used.

The robot.ev file was incrementally expanded to include more elaborate paths to verify motor behaviour and timing:

- Single motion without any intermediate waypoints: single simultaneous rotation of both motors in forward then in reverse.

- A straight line motion requiring interpolation waypoints. This required the motors to make several distinct rotation movements with stops in between.

- A basic rectangular path requiring four straight line segments with interpolated waypoints. This required the motors to move in the same direction and in opposite directions simultaneously.

- All-up test with robot attached to spool cables. Because of the low motor torque, it was necessary to adapt the vertical hanging robot test to a horizontal test. This was accomplished by installing a supplementary unpowered bogey wheels to support the robot body, and loaded, elastic line to the bottom of the robot that substituted for gravity in order to add tension to the two spool-cables. (see video for this test at:

http://jufaintermedia.com/robot/video.html

### Results

Mindstorms hardware control via NXT++ and Real-time DEVS was successful. Motor commands were properly issued and acted upon by the hardware. The behaviour of the “robot” coupled model described and simulated in section 13.

The realities of hardware implementation are that positional accuracies of the simulation were not reflected in real-world testing. This was due to several factors:

Cable elasticity affected positional accuracy – this affected absolute robot position as well as spool-winding, especially during robot starts and stops. A slower robot motion or a non-elastic cable material would alleviate this to some degree.

Motor speed varied with motor load – this in turn varied with the position of the robot relative to the cable anchor points. This is an inherit limitation of the NXT hardware which could be alleviated somewhat by using a worm-gear reduction system as described previously. Because this implementation was open-loop a consistent motor speed with no appreciable acceleration or deceleration time is ideal; therefore, a gear-reduction system is desirable.

Motor ‘brake’ action – this stopped the motor and prevented motor creep under load but tended to introduce a slight positional error whenever it was triggered at waypoints.

Motors were under-powered for this application – they could not reliably support the robot and the NXT microcontroller without overloading. To continue testing, the horizontal setup described above was implemented.

Robot positional accuracy varied with position relative to anchor points – because of the geometry of the cable-stay system, motor position errors are amplified as the robot moves away (down) from the anchor points. As the angle between the two cables decreases, small changes in motor position have a greater effect on robot position. This can be seen in the linked video as a difference between the lengths of the top line segment of the drawn rectangle compared to the bottom line segment. This could be alleviated by using a gear-reduction system to increase motor rotations required for a robot movement or by using cable spools with a smaller diameter.

Coordinate system reversal – the physical coordinate system had a reversed vertical axis compared to the simulated coordinate system in section 13. This was corrected, however motor rotation direction, cable winding direction needed to be carefully documented to ensure no coordinate mirroring or reversal took place

Initial robot starting point – this needed to match closely the expected starting coordinates defined in the robot.ma file. In practice this was difficult to achieve, and a calibration routine or entry of initial cable lengths into the .ma file would result in less positional error.

Lack of visibility into NXT microcontroller behaviour – this made some system behaviour difficult to analyze, as motor states and other ‘hidden’ parameters changed observed performance. For example, motor commands required a wait period after motor motion has stopped of several hundred milliseconds before subsequent command could be sent successfully. Other factors within the firmware seemed to have a cumulative effect on motor command interpretation that could not be traced or quantified reliably because of lack of visibility into the firmware state.

Motor acceleration and deceleration – because an open-loop motor control system was used, these became important if motor movements were relatively short in duration, as they affected positional accuracy. One solution is to use a closed-loop servo system (not feasible with this implementation) or to gear-reduce the motor-spool connection.

### Discussion

Although closed-loop motor control is preferable to open-loop control, Real-time DEVS allowed accurate timing of motor movements; with the motors properly characterized it was possible to provide good accuracy. This accuracy was shown mechanically to be further enhanced with a worm-gear reduction system on the drive motors.

This implementation control of Mindstorms hardware via a DEVS atomic model within a complex coupled model. This demonstrates that any Mindstorms sensor or actuator can be effectively modeled as an atomic model within DEVS++ then the associated Real-time DEVS atomic model can be substituted in the E-CD++ framework with little additional modification required. A workflow was thus successfully demonstrated from simulation to hardware implementation with DEVS, Real-time DEVS, and Mindstorms.

## CELL-DEVS WIKI Contribution

A contribution to the CELL-DEVS wiki was made based on our finding, specifically with reference to the application of Mindstorms with Real-time DEVS.

## Next Steps and Future Development

Expanding the movement of the Robot from 2D to 3D is a second design goal that will require addition of a Z coordinate and an addition motor model and cable. The algorithm for the path planner will also have to be extended to include vectors in three dimensions.

Bluetooth connectivity with Mindstorms would allow for wireless control.

Support sensor readings with Real-time DEVS.  This requires separation of hardware initialization from the kernel of Real-time DEVS, as done in previous works [3]. This initialization procedure defines the connection between ports and sensors.  Additional design should take into account wrapping this communication within an atomic model.

A method implementing closed loop control simultaneously across multiple motors would allow more accurate robot actuation.

Porting Real-time DEVS to the Microsoft Windows platform enables the use of future releases of the NXT++ library. This will allow new features and defect corrections within NXT++ to be used in future Real-time DEVS projects.

Ability to output Mindstorms NXT code, which can be loaded directly on the Mindstorms NXT brick. This would allow the robot to be independent of an external computer.  Though, this would be dependent on the capabilities of the NXT microcontroller and firmware, as well as the code size.

## Conclusion

Using a bottom-up development and testing process resulted in a complex robot controller system that met desired performance goals. The flexibility of the anchor-cable locomotion system is offset by its geometric complexity; however, this was shown to be successfully addressed with a path planner that could linearize robot motions with controllable fidelity.

This architecture was implemented first as a regressively tested simulation within CD++ then ported to Real-time DEVS using the NXT++ interface library.  A hardware implementation of the robot using Lego Mindstorms was shown to be controllable using event files. Open-loop motor control was demonstrated using the fundamentals of a simulated robot in the real world.

Unique to this project was control of Mindstorms hardware via a DEVS atomic model within a complex coupled model. This demonstrates that any Mindstorms sensor or actuator can be effectively modeled as an atomic model within DEVS++ then the associated Real-time DEVS atomic model can be substituted in the E-CD++ framework with little additional modification required. A workflow was thus successfully demonstrated from simulation to hardware implementation with DEVS, Real-time DEVS, and Mindstorms.

# References

All robot controller concepts, algorithms and implementation were derived from first principles in with the goal of discovering and testing solutions within the DEVS framework.

The Lego Mindstorms motor’s servo and position tracking capability was essential in designing the robot controller. References include:

- Wainer, G., "CD++: a toolkit to define discrete-event models". Software, Practice and Experience. Vol. 32, No.3. pp. 1261-1306. November 2002.

- Wainer, G. et al. "CD++ A tool for DEVS and Cell-DEVS Modeling and Simulation. User's Guide". Draft. August 2004.

- Moallemi, Mohammad. “Embedded CD++ on a Robot”. Department of Systems and Computer Engineering, Carleton University Centre of Visualization and Simulation (V-Sim)

- Lego Mindstorms definition at Wikipedia. http://en.wikipedia.org/wiki/Mindstorms_NXT, accessed November 23, 2008.

- Lego Mindstorms NXT Kit. Website available at http://Mindstorms.lego.com.

- “Main Page - NXT++” http://nxtpp.sourceforge.net/index.php, accessed November 23, 2008

[6]	“NXT® motor internals” http://www.philohome.com/nxtmotor/nxtmotor.htm, accessed October 2, 2008.

[7]	“Lego Mindstorms Official Site” http://Mindstorms.lego.com/Overview/Interactive_Servo_Motors.aspx, accessed October 2, 2008

[8]	“CD++ Wiki” http://cell-devs.sce.carleton.ca/mediawiki/index.php/Main_Page, accessed September 18 through October 24, 2008

[9]	USB Interface setup for NXT and Ubuntu at http://forums.nxtasy.org/index.php?showtopic=2143, accessed 24 November 2008

# APPENDIX I: Full Robot Test Output

This table is the output from the full robot test described in Complex Path Regression Test In section 14, given the inputs points described in the test, the simulation produced the lengths for each line in each L1, L2 column.  Using some basic trigonometry identities these are converted to the corresponding x & y values.   These were the x & y values that were charted with Microsoft Excel to produce the output graph seen in Figure 25Error: Reference source not found.

| Anchor1 | x | 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | y | 0 |  |  |  |  |
| Anchor2 | x | 20 |  |  |  |  |
|  | y | 0 |  |  |  |  |
| L0 |  | 20 |  |  |  |  |
|  | L1 | L2 | delta1 | delat2 | x | y |
|  | 1.11803 | 19.0066 | 0.026311 | 0.463683 | 0.999979 | 0.500034 |
|  | 2.23607 | 18.0278 | 0.055503 | 0.463689 | 1.999961 | 1.000083 |
|  | 2.82843 | 18.1108 | 0.110659 | 0.785412 | 1.999973 | 2.000031 |
|  | 3.60555 | 18.2483 | 0.165149 | 0.982797 | 1.999988 | 3.000006 |
|  | 4.47214 | 18.4391 | 0.218669 | 1.107151 | 1.999991 | 4.000009 |
|  | 5.38516 | 18.6815 | 0.270946 | 1.190282 | 2.000038 | 4.99998 |
|  | 5.83095 | 17.72 | 0.286051 | 1.030369 | 3.000039 | 4.999974 |
|  | 6.40312 | 16.7631 | 0.302886 | 0.896063 | 3.999961 | 5.000026 |
|  | 7.07107 | 15.8114 | 0.321751 | 0.7854 | 4.999992 | 5.000012 |
|  | 7.81025 | 14.8661 | 0.343025 | 0.694743 | 5.999977 | 5.000028 |
|  | 8.60233 | 13.9284 | 0.367175 | 0.620251 | 6.999994 | 5.000017 |
|  | 9.43398 | 13 | 0.394791 | 0.558599 | 7.999999 | 4.999999 |
|  | 8.5 | 13.1244 | 0.309703 | 0.489957 | 7.500003 | 3.999994 |
|  | 7.61577 | 13.3417 | 0.226802 | 0.404899 | 6.999975 | 3.000051 |
|  | 6.7082 | 14.3178 | 0.211091 | 0.463642 | 6.000014 | 2.999964 |
|  | 6.80074 | 15.0416 | 0.269167 | 0.628795 | 5.500008 | 3.999997 |
|  | 7.07107 | 15.8114 | 0.321751 | 0.7854 | 4.999992 | 5.000012 |
|  | 8.13941 | 15.6924 | 0.392341 | 0.828855 | 5.499964 | 6.000032 |
|  | 9.21954 | 15.6525 | 0.463648 | 0.862173 | 5.999979 | 7.000012 |
|  | 9.89949 | 14.7648 | 0.493941 | 0.785396 | 7.000015 | 6.999978 |
|  | 10.6301 | 13.8924 | 0.52807 | 0.718824 | 8.000006 | 6.999923 |
|  | 11.4018 | 13.0384 | 0.566733 | 0.661044 | 9.000029 | 7.000037 |
|  | 12.2066 | 12.2066 | 0.610731 | 0.610731 | 10 | 7.000077 |
|  | 13.0384 | 11.4018 | 0.661044 | 0.566733 | 10.99997 | 7.000037 |
|  | 12.0934 | 11.2361 | 0.563317 | 0.519146 | 10.50001 | 6.00001 |
|  | 11.1803 | 11.1803 | 0.46364 | 0.46364 | 10 | 4.999911 |
|  | 10.8167 | 12.53 | 0.499352 | 0.588009 | 9.000002 | 6.00008 |
|  | 10.6301 | 13.8924 | 0.52807 | 0.718824 | 8.000006 | 6.999923 |
|  | 11.4018 | 13.0384 | 0.566733 | 0.661044 | 9.000029 | 7.000037 |
|  | 12.2066 | 12.2066 | 0.610731 | 0.610731 | 10 | 7.000077 |
|  | 13.0384 | 11.4018 | 0.661044 | 0.566733 | 10.99997 | 7.000037 |
|  | 13.4001 | 12.2296 | 0.713062 | 0.639772 | 10.74999 | 8.000026 |
|  | 13.8293 | 13.0863 | 0.758377 | 0.70863 | 10.49996 | 9.000024 |
|  | 14.32 | 13.9665 | 0.798055 | 0.773055 | 10.24998 | 10.00001 |
|  | 14.8661 | 14.8661 | 0.832983 | 0.832983 | 10 | 11.00004 |
|  | 14.2127 | 15.5563 | 0.7854 | 0.885063 | 9.000059 | 10.99999 |
|  | 13.4536 | 14.8661 | 0.737813 | 0.837984 | 8.999961 | 10 |
|  | 12.7279 | 14.2127 | 0.685728 | 0.7854 | 8.999965 | 9.000004 |
|  | 12.8062 | 12.8062 | 0.674736 | 0.674736 | 10 | 7.999922 |
|  | 13.0384 | 11.4018 | 0.661044 | 0.566733 | 10.99997 | 7.000037 |
|  | 13.4164 | 10 | 0.6435 | 0.463647 | 11.99999 | 5.999993 |
|  | 13.9284 | 8.60233 | 0.620251 | 0.367175 | 13.00001 | 5.000017 |
|  | 14.8661 | 7.81025 | 0.694743 | 0.343025 | 14.00002 | 5.000028 |
|  | 15.8114 | 7.07107 | 0.7854 | 0.321751 | 15.00001 | 5.000012 |
|  | 16.1555 | 7.81025 | 0.876059 | 0.380507 | 15 | 6.000004 |
|  | 16.5529 | 8.60233 | 0.950542 | 0.436627 | 14.99996 | 6.999978 |
|  | 15.6525 | 9.21954 | 0.862173 | 0.463648 | 14.00002 | 7.000012 |
|  | 14.7648 | 9.89949 | 0.785396 | 0.493941 | 12.99999 | 6.999978 |
|  | 14.3178 | 9.21954 | 0.708624 | 0.432407 | 12.99999 | 5.999978 |
|  | 13.9284 | 8.60233 | 0.620251 | 0.367175 | 13.00001 | 5.000017 |

Table 1: Output from the Full Robot Test

# APPENDIX II: DEVS++ install on Windows Vista 32 bit

Install on Vista 32 SP1 worked as per the instructions provided on the CELL-DEVS wiki, the only requirement was to make sure full permissions were set for the c:/cygwin folder as below:

## Cygwin folder permissions

(This may also apply to the c:/tmp folder and the c:/eclipse folder. I did this process to all three to be sure)

Right click on c:/cygwin folder, select Properties. In the popup, select the Security tab and highlight your appropriate user group. Click the Edit button under Group or user names. Make sure Full Control is checked. If a popup to apply this to all subfolders appears, click OK.

Also, I had to follow the manual download and install procedure for adding c:/cygwin/bin/cat.exe. The link specified in the installation instructions required the use of a free OSS tar.bz2 archive tool called 7-zip, which handled the extraction without issue.

I would consider this information “debug” if the regular process does not work.

CD++ modeller seems to work as well at least to the point I have tested it.

Sequential screen shots of the permissions process are shown in Figure 26.

## Other debug attempts

If Cygwin setup file, setup.exe, does not run properly, try setting it to run in compatibility mode by right-clicking, selecting Properties, clicking the Compatibility tab, checking “Run this program in compatibility mode for…” and selecting “Windows XP [(Service Pack 2)]”. It was also found that old installs of Java had to be removed and the latest Java 6 installed.

Figure 26: Vista O/S setting screens in sequence for Cygwin application.

LEGO MINDSTORMS is a registered trademark of The LEGO Group.