JK-Clock Model

SYS5807 Assignment 1

Fall 2003

Student Name:	Khaldoon Al-Zoubi

Student #:		100 275 273

Table of Contents

1	Part I: JK flip-flop with a Clock Conceptual Model	3

2	Part II: Formal Specification and Testing Strategy	5

2.1	Models Structure	5

2.1.1	JK – Clock Model (Top Model)	5

2.1.2	JK Flip-Flop structure (coupled Model)	5

2.1.3	SR Flip-Flop Structure (coupled Model)	6

2.2	Coupled Models Formal Specification	6

2.1.4	JK-Clock Model (Top Model)	6

2.1.5	JK Model	7

2.1.6	SR Model	8

2.3	Atomic Models Formal Specification	8

2.1.7	Clock Model	8

2.1.8	And Gate Model	9

2.1.9	Nor Gate Model	11

2.4	Integration Testing Strategy	12

3	Part III: Simulation Analysis	13

3.1	JK-Clock Model Testing	13

3.2	JK Model Testing	13

2.1.10	Test Cases	14

2.1.11	Output	14

3.3	SR Model Testing	15

2.1.12	Test Cases	15

2.1.13	Output	16

3.4	Clock Testing	17

3.5	And Gate Testing	17

2.1.14	Test Cases	17

2.1.15	Output	18

3.6	Nor Gate	19

2.1.16	Test Cases	19

2.1.17	Output	19

# Part I: JK flip-flop with a Clock Conceptual Model

The clock and the J-K flip-flop can be used as primitive elements to build a complex logic circuit.

The above figures show the structures of the JK flip-flop and the SR flip-flop. The JK flip-flop is structured with two AND gates and one SR flip-flop. The SR flip-flop is structured with two NOR gates.

The model will be build as the following:

- Clock (Atomic model):

- It is active all the time and it has two states, S = {0,1}.

- It toggles its state every clock cycle.

- Output the value of the new state when it changes, Y = {0,1}.

- J-K flip-flop (Structural Model)

- Structured with two AND gates and one SR flip-flop.

- It produces an output within a clock cycle.

- Truth Table:

- Q = J 		when J K;

- Q = Q_bar 		when J=K= 1;

- No change to Q 	when J=K=0.

- SR flip-flop (Structural Model)

- Structured with two NOR gates.

- Truth Table

- Q = 1		when S=1, R=0

- Q = 0		when S=0, R=1

- No change to Q	when S=R=0

- Undefined		when S=R=1  (JK fixes this problem)

- NOR Gate (Atomic model)

- It has two state {busy, free}, It goes into busy state for fixed delay time when it is triggered before it produces an output and go into free state again.

- It has two inputs and one output.

- It ignores the inputs unless it is in the free state.

- AND Gate (Atomic model)

- It has two state {busy, free}, It goes into busy state for fixed delay time when it is triggered before it produces an output and go into free state again.

- It has three inputs and one output. One of the inputs allows the gate to be enabled or disabled.

- It ignores the inputs unless it is free and triggered by the controlled input (e.g. CLK = 1).

# Part II: Formal Specification and Testing Strategy

## Models Structure

The JK-Clock model contains three atomic models: Two AND Gates, two NOR Gates and Clock. It also contains two coupled models: The JK flip-flop and the SR flip-flop. The following sections describe the structure of the JK-Clock model.

### JK – Clock Model (Top Model)

The following figure shows the top model, which contains the clock (atomic model) and the JK flip-flop (coupled model).

JK Type

J           _

Q

Enable

K         Q

Clock

Out

_

Q

Q

J

K

### JK Flip-Flop structure (coupled Model)

The following figure shows the JK flip-flop model, which contains two And Gates (atomic model) and One SR flip-flop (coupled model).

### SR Flip-Flop Structure (coupled Model)

The following figure shows the SR flip-flop model, which contains two NOR Gates (atomic model).

## Coupled Models Formal Specification

Notes:

- All of the links are wires. Therefore, They always have a value of 0 or 1.

- All inputs and outputs should always be either 0 or 1.

### JK-Clock Model (Top Model)

#### Specification

JK-Clock = {X, Y, {Clock, JK}, EIC, EOC, IC, Select}

X = {J, K}.

Y = {Q, Q_bar}.

EIC = {(JK_Clock.J, JK.J), (JK_Clock.K, JK.K)}.

EOC = {(JK.Q_bar, JK_Clock.Q_bar), (JK.Q, JK_Clock.Q)}.

IC = {(Clock.out, JK.Enable)}

Select: ({Clock, JK}) = Either one. Note that picking either one at an instant time shouldn’t affect the model behavior.

#### Unit Testing

Note: The code should initialize all inputs and outputs to have valid feedback. The code will start with following values: Q = 0, Q_bar = 1, J = 0, K = 1, S = 1, R = 0 and Clock (clk) = 0.

- The final model will be tested against the JK flip-flop truth table when clock transit from low (0) to high (1). The clock shouldn’t have an effect on the JK flip-flop when the clock transit from high to low.

- The JK flip-flop should produce an output after a certain delay.

- The clock should toggle every clock cycle.

### JK Model

#### Specification

JK = {X, Y, {And_A, And_B, SR}, EIC, EOC, IC, IC, Select}.

X = {J, K, Enable}.

Y = {Q_bar, Q}.

EIC = {(JK.J, AND_A.in_a), (JK.Enable, AND_A.in_enable), (JK.K, AND_B.in_b, (JK.Enable, And_B.in_enable)}.

EOC = {(SR.Q_bar, JK.Q_bar), (SR.Q, JK.Q)}

IC = {(SR.Q_bar, And_A.in_b), (SR.Q, And_B.in_a), (And_A.out, SR.S), (And_B.out, SR.R)}.

Select: ({And_A, And_B}) = Either one. ({And_A, SR}) = Either one. ({And_B, SR}) = Either one. ({AND_A, And_B, SR}) = Either one.

#### Unit Testing

Note: The code should initialize all inputs and outputs to have valid feedback. The code will start with following values: Q = 0, Q_bar = 1, J = 0, K = 1, S = 1, R = 0.

- The model will be tested against the JK flip-flop truth table when high (1) is received at the Enable. Receiving low (0) on port Enable shouldn’t have an effect on the JK flip-flop.

- The JK flip-flop should produce an output after a certain delay.

- The model will be fed test data on the Enable port.

### SR Model

#### Specification

SR = {X, Y, {Nor_A, Nor_B}, EIC, EOC, Select}.

X = {S, R}.

Y = {Q, Q_bar}.

EIC = {(SR.S, Nor_A.in_a), (SR.R, Nor_B.in_b)}.

EOC = {(Nor.A.out, SR.Q_bar), (Nor_B.out, SR.Q)}.

IC = {(Nor_A.out, Nor_B.in_a), (Nor_B.out, Nor_A.in_b)}.

Select: ({Nor_A, Nor_B}) = Either one.

#### Unit Testing

Note: The code should initialize all inputs and outputs to have valid feedback. The code will start with following values: Q = 0, Q_bar = 1, S = 1, R = 0.

- The model will be tested against the SR flip-flop truth table.

- The SR flip-flop should produce an output after a certain delay.

- The model will run continuously. Actually, the SR flip-flop may take two to three feedbacks for the two NORs to stabilize the output.

## Atomic Models Formal Specification

### Clock Model

#### Specification

Clock = {X, Y, S, Internal Function (IF), External Function (EF), Output Function (OF), TA}

X = None.

Y = {out}.

S = {Active}.

EF = None.

IF = IF (Active) = Active.

OF (Active) = out.

TA (Active) = clock_cycle.

#### Implementation

Note that the clock cycle is configurable and is defaulted to 50 ms.

initFunction:

{

Sets clock out to zero.

Trigger the internal function.

}

externalFunction: not used.

internalFunction

{

If state is active then

Toggle the clock’s output and schedule it after one clock cycle time.

}

outputFunction:

{

Send the value of the clock’s output on the “out” port.

}

#### Unit Testing

- Run simulation for a specific time (e.g. 425 ms).

- The clock should toggle its output every clock cycle.

- The clock should be tested for different clock cycles.

### And Gate Model

#### Specification

And = {X, Y, S, Internal Function (IF), External Function (EF), Output Function (OF), TA}

X = {in_a, in_b, in_enable}.

Y = {out}.

S = {Passive, Active}.

EF: 	EF (Passive, in_a) = Passive.

EF (Passive, in_b) = Passive.

EF (Passive, in_enable=1) = Active.

EF (Passive, in_enable=0) = Passive.

EF (Active, in_a || in_b || in_enable) = Ignored.

IF = IF (Active) = Passive.

OF (Active) = out.

TA (Active) = delay.

#### Implementation

Note that the delay time is configurable and is defaulted to 2 ms.

initFunction:

{

Sets in_a to zero and in_b to 1.  // This allows initializing the feedbacks to upper models

}

externalFunction:

{

If state is Passive then

{

If value is received from port in_a stores it.

Else if value is received from port in_b stores it.

Else if value is received from in_enable port and it is 1, trigger the internal function after the delay time.

}

}

internalFunction

{

Go to Passive state.

}

outputFunction:

{

And the last received values on ports in_a and in_b and send the result on the “out” port.

}

#### Unit Testing

- Run the test against the truth table of the And Gate. The And Gate should produce an output after the delay time when it receives one on in_enable port. Receiving zero on the in_enable port should not trigger the And Gate.

- Test when “And” Gate receives inputs during the delay time. Inputs should be ignored in this case.

### Nor Gate Model

#### Specification

Nor = {X, Y, S, Internal Function (IF), External Function (EF), Output Function (OF), TA}

X = {in_a, in_b}.

Y = {out}.

S = {Passive, Active}.

EF: 	EF (Passive, in_a) = Active.

EF (Passive, in_b) = Active.

EF (Active, in_a) = Active.

EF (Active, in_b) = Active.

IF = IF (Active) = Passive.

OF (Active) = out.

TA (Active) = delay.

#### Implementation

Note that the delay time is configurable and is defaulted to 2 ms.

initFunction:

{

Sets in_a to 1 and in_b to 0.  // This allows feedback initializing to the upper models

}

externalFunction:

{

If value is received on port in_a or in_b, store it.

Schedule the internal function after a certain delay.

}

internalFunction

{

Go to Passive state.

}

outputFunction:

{

Nor last received values on ports in_a and in_b and send the result on the “out” port.

}

#### Unit Testing

- Run the test against the truth table of the Nor Gate. The Nor Gate should produce an output after the delay time when it receives a value on either of its input ports.

- Test when “Nor” Gate receives inputs during the delay time. The Gate should take inputs during the delay time, but re-schedule the output for the new delay.

## Integration Testing Strategy

A testing was performed for each model as explained in the above Unit Testing sections in the following sequence:

- Clock

- And Gate.

- Nor Gate.

- SR flop-flop.

- JK flop-flop

- JK-Clock model (Final model)

Each model of the above has its test cases in “modelName.ev” file and its structure in “modelName.ma” file. Each model has a script (modelName.bat) that can be run to test it, using command (./modeName.bat).

# Part III: Simulation Analysis

## JK-Clock Model Testing

Simulation Time: 1 second.

Clock cycle = 50 ms. Clock’s output starts 0 and toggles every 50 ms.

Each of And Gate delay = 2 ms.

Each of Nor Gate delay = 2 ms

Initial values: Q = 0, Q_bar = 1, J = 0, K = 1.

| Test cases | Inputs/Time received (See JK_Clock.ev) | Expected output | Comments |
| --- | --- | --- | --- |
| 1 | 11 ms (J = K = 0) | Q = 0 Q_bar = 1 (No change) | Clock changed from 0 to 1 at 50 ms. Output as expected. |
| 2 | 120 ms (J=1, K = 0) | Q = 1 Q_bar = 0 | Clock changed from 0 to 1 at 150 ms. Output as expected after a delay. |
| 3 | 180 ms (J=0) 185 ms (K =1) | Q = 0 Q_bar = 1 | Clock changed from 0 to 1 at 250 ms. Output as expected after a delay. |
| 4 | 260 ms (J=K = 1) | Toggle Q and Q_bar | Toggles Q and Q_bar at 350 ms, 450 ms, 550 ms and 650 ms. Output as expected after a delay. |
| 5 | 725 ms (J =0, K = 1) | Q = 0 Q_bar = 1 | It will stay the same until the end of the simulation at 1 second. |

- The JK flip flop produced the desired output at the clock transition from low to high.

- The simulation showed that the JK delay is SR delay + the “And” delay.

The simulation showed that the flip-flop’s output is unstable during its delay (in this case 6 ms to 8ms). Therefore, it is important to reduce the SR and the two And Gates delays to avoid instability for a long time.

The specification assumed zero or one will be received on the input ports, but what if we receive different value than zero or one. The model was tested against negative or positive values; the test showed that any other value than zero is treated as one.

## JK Model Testing

JK was tested against its truth table with different delays. The JK should only produces input when it receives 1 on the Enable port.

### Test Cases

And Gate delay = 2 ms.

SR delay = 4 ms – 6 ms.

Simulation = 120 ms.

Test case 1:  Test that receiving 0 on the Enable doesn’t have any affect.

00:00:00:010 J 1

00:00:00:010 K 1

00:00:00:030 enable 0

Test Case 2: Test the truth table of JK’s flip-flop. The output will happen at the time “Enable” port receives 1 plus the delay time (6 ms to 8 ms in this case).

Case 2.1:  J = 1 and K = 1 (Toggle Q and Q_bar)

00:00:00:040 enable 1

Case 2.2:  J = 1 and K = 1 (Toggle Q and Q_bar)

00:00:00:050 enable 1

Case 2.2:  J = 0 and K = 0 (No change for Q and Q_bar)

00:00:00:060 J 0

00:00:00:060 K 0

00:00:00:070 enable 1

Case 2.3:  J = 1 and K = 0 (Q should change to 1and Q_bar to 0)

00:00:00:080 J 1

00:00:00:080 K 0

00:00:00:081 enable 1

Case 2.4:  J = 0 and K = 1 (Q should change to 0and Q_bar to 1)

00:00:00:090 J 0

00:00:00:090 K 1

00:00:00:091 enable 1

### Output

00:00:00:044 q_bar 0

00:00:00:046 q 1

00:00:00:048 q_bar 0

00:00:00:050 q 1

00:00:00:052 q_bar 0

00:00:00:054 q_bar 0

00:00:00:056 q 0

00:00:00:058 q_bar 1

00:00:00:060 q 0

00:00:00:062 q_bar 1

00:00:00:064 q 0

00:00:00:066 q_bar 1

00:00:00:068 q 0

00:00:00:070 q_bar 1

00:00:00:072 q 0

00:00:00:074 q_bar 1

00:00:00:076 q 0

00:00:00:078 q_bar 1

00:00:00:080 q 0

00:00:00:082 q_bar 1

00:00:00:085 q_bar 0

00:00:00:087 q 1

00:00:00:089 q_bar 0

00:00:00:091 q 1

00:00:00:093 q_bar 0

00:00:00:095 q_bar 0

00:00:00:097 q 0

00:00:00:099 q_bar 1

00:00:00:101 q 0

00:00:00:103 q_bar 1

00:00:00:105 q 0

00:00:00:107 q_bar 1

00:00:00:109 q 0

00:00:00:111 q_bar 1

00:00:00:113 q 0

00:00:00:115 q_bar 1

00:00:00:117 q 0

00:00:00:119 q_bar 1

## SR Model Testing

SR was tested against its truth table with different delays.

### Test Cases

Simulation time = 120 ms.

Nor Gate delay = 2 ms.

Case one:  S = 0 and R = 0 (No change to Q and Q_bar)

00:00:00:000 S 0

00:00:00:000 R 0

Case two: S = 1 and R = 0 (Q should change to 1 and Q_bar to 0 at time 26 ms to 28 ms)

00:00:00:020 S 1

00:00:00:020 R 0

Case three: S = 0 and R = 1 (Q should change to 0 and Q_bar to 1 at time 56 ms to 58 ms)

00:00:00:050 S 0

00:00:00:050 R 1

Case four: S = 1 and R = 1 (Undefined state)

00:00:00:080 S 1

00:00:00:080 R 1

### Output

The output showed the correct truth table after a certain delay.

00:00:00:003 q_bar 1

00:00:00:006 q 0

00:00:00:009 q_bar 1

00:00:00:012 q 0

00:00:00:015 q_bar 1

00:00:00:018 q 0

00:00:00:023 q_bar 0

00:00:00:026 q 1

00:00:00:029 q_bar 0

00:00:00:032 q 1

00:00:00:035 q_bar 0

00:00:00:038 q 1

00:00:00:041 q_bar 0

00:00:00:044 q 1

00:00:00:047 q_bar 0

00:00:00:053 q_bar 0

00:00:00:056 q 0

00:00:00:059 q_bar 1

00:00:00:062 q 0

00:00:00:065 q_bar 1

00:00:00:068 q 0

00:00:00:071 q_bar 1

00:00:00:074 q 0

00:00:00:077 q_bar 1

00:00:00:083 q_bar 0

00:00:00:086 q 0

00:00:00:089 q_bar 0

00:00:00:092 q 0

00:00:00:095 q_bar 0

00:00:00:098 q 0

00:00:00:101 q_bar 0

00:00:00:104 q 0

00:00:00:107 q_bar 0

00:00:00:110 q 0

00:00:00:113 q_bar 0

00:00:00:116 q 0

00:00:00:119 q_bar 0

## Clock Testing

The clock was run different clock cycles.

The following output shows the result when the clock ran for 430 ms with cycle of 60 ms.

00:00:00:000 out_clk 0

00:00:00:060 out_clk 1

00:00:00:120 out_clk 0

00:00:00:180 out_clk 1

00:00:00:240 out_clk 0

00:00:00:300 out_clk 1

00:00:00:360 out_clk 0

00:00:00:420 out_clk 1

## And Gate Testing

The And Gate was tested against its truth table with different delays as the following

### Test Cases

Case 1: Test truth table when in_enable is zero. It shouldn’t produce output.

00:00:00:05 in_a 1

00:00:00:05 in_b 1

00:00:00:06 in_enable 0

00:00:00:10 in_a 0

00:00:00:10 in_b 0

00:00:00:10 in_enable 0

00:00:00:13 in_a 1

00:00:00:15 in_b 0

00:00:00:16 in_enable 0

00:00:00:20 in_a 0

00:00:00:20 in_b 1

00:00:00:20 in_enable 0

Case 2: Test truth table when in_enable is one. It should produce output.

00:00:00:25 in_a 1

00:00:00:25 in_b 1

00:00:00:26 in_enable 1

00:00:00:30 in_a 0

00:00:00:30 in_b 0

00:00:00:30 in_enable 1

00:00:00:33 in_a 1

00:00:00:35 in_b 0

00:00:00:40 in_enable 1

00:00:00:45 in_a 0

00:00:00:45 in_b 1

00:00:00:45 in_enable 1

Test Case 3: Test when input is received during the delay

00:00:00:46 in_a 1

00:00:00:46 in_b 1

00:00:00:46 in_enable 1

00:00:00:50 in_a 1

00:00:00:50 in_b 1

00:00:00:50 in_enable 1

### Output

The output showed the correct truth table after a certain delay (2 ms in this case).

00:00:00:028 out 1

00:00:00:032 out 0

00:00:00:042 out 0

00:00:00:047 out 0

00:00:00:052 out 1

## Nor Gate

The NOR Gate was tested against its truth table with different delays as the following

### Test Cases

Test case 1: test truth table

00:00:00:000 in_a 1

00:00:00:000 in_b 1

00:00:00:005 in_a 0

00:00:00:005 in_b 0

00:00:00:010 in_a 1

00:00:00:010 in_b 0

00:00:00:015 in_a 0

00:00:00:015 in_b 1

00:00:00:015 in_a 0

00:00:00:015 in_b 1

Test case 1: Test when input is received during the delay (2 ms in this case)

00:00:00:020 in_a 1

00:00:00:020 in_b 1

00:00:00:021 in_a 0

00:00:00:022 in_b 0

### Output

00:00:00:002 out 0

00:00:00:007 out 1

00:00:00:012 out 0

00:00:00:017 out 0

00:00:00:024 out 1