# SYSC-5807 METHODOLOGICAL ASPECTS OF MODELLING AND SIMULATION

## Assignment 1

## Pengfei Wu   Student#:100258560

## Xiuping Wu   Student#:100258863

## {pfwu, xpwu}@sce.carleton.ca

## October 20, 2003

## Systems and Computer Engineering, Carleton University

## Part I: Description of the Conceptual Model

## System under Analysis: Microwave Oven

Conceptual Model Description: In this assignment, we will model a programmable microwave oven system using DEVS formalism. The microwave oven described here is very similar to the actual ones used in daily life. It has a bunch of buttons which can be used to set the cooking time and power level for a heating job, or adjust the system time that is displayed on the panel when the oven is free. A heating job is started by pushing the START button. It can be stopped by pushing the STOP button or CLEAR button or the time set for the job has elapsed. When the CLEAR button is pressed, the time duration and the power level set for the heating job will be reset to ZERO. The time duration is set by pressing the PHASE_SET button followed by minutes and seconds input. The number of minutes is set by pressing the MINUTES button which increases one minute per press. The number of seconds is set by pressing the SECONDS button which increases one second per press. There is a panel that displays the current time when the oven is free, or the remaining time of a heating job when a job starts. The model structure is illustrated in Figure 1.

Figure 1 The conceptual model of a programmable microwave oven system

This model has four levels. The top level is the Microwave Oven System. It has 9 inputs that correspond to the buttons on the control panel of the microwave oven.

- TIME_SET is used to set or adjust current time that is displayed.

- PHASE_SET is used to set the program for a heating job.

- The POWER_LEVEL is used to set the level of power such as high, medium or low. The value of POWER_LEVEL is set by pressing the Power button on the panel.

- The HOURS is used to set the hour of the current time.

- The MINUTES is used to set the number of minutes that the oven is going to heat the stuff inside or the minute of the current time.

- The SECONDS is used to set the number of seconds that the oven is going to heat.

- The START represents the Start button on the panel which starts the oven.

- The STOP represents the Stop button on the panel which stops the oven.

- The CLEAR represents the Clear button on the panel which clears current set values such the time, power level as well as the phases which will be introduced shortly.

The outputs of the model are: CURRENT_TIME, REMAINING_TIME, HEAT and BEEP. The meaning of these outputs is as follows:

- CURRENT_TIME represents the current time that is displayed.

- REMAINING_TIME represents the remaining time that is displayed which means that the oven is going to heat the inside stuff for that time long.

- HEAT represents the power level that the oven is giving. The possible values are high, medium and low.

- BEEP represents the sound that the oven makes when it finishes a heating job.

The top model has three state variables: T (for time displayed), H (for heating level) and B (for the beep sound).

T = CurTime or RemainTime indicates whether the current time of the day (by CurTime) or the remaining time of a heating job (by RemainTime) is displayed.

H = NONE, LOW, MEDIUM or HIGH indicates the power level at which the oven is heating.

B = Beep_ON or Beep_OFF indicates the beeping sound is on or off.

The model is decomposed into the following components and these components are at the second level.

PhaseRegister records the information associated with a heating job. In this case, we assume that a heating job has only one phase that includes a time duration and a power level. It communicates with PowerDriver and TimeCounter.

PowerDriver takes the power level information from PhaseRegister and controls the heating level such as low, medium or high.

Beeper controls whether the system needs to make a beeping sound. It communicates with the TimeCounter to see if the remaining time is zero. If it is, the beep sound is made. The beep sound is made for about 3 seconds after a job is finished.

TimeRegister records the current time of the day.

Displayer displays the remaining time of a job or the current time of the day. It communicates with TimeCounter and TimeRegister. If the job is finished, it displays the current time. Otherwise, it displays the remaining time of the job for the particular phase.

The component PhaseRegister is decomposed to the following components and these components are at the third level.

Timer records the information of the time duration that has been set for a job.

PowerLevel records the heating level for the job.

The sub-model Timer consists of the following components and these components are at the fourth level. In this case, we assume that it only records number of minutes and seconds.

MinuteRegister records how many minutes set for this job.

SecondRegister records how many seconds set for this job.

TimeCounter counts the remaining time for this job. The remaining time is displayed on the panel.

Part II: Structure and Specification of Coupled Models and Atomic Models

The structure of the system model has been illustrated in Figure 1 in Part I. It has two coupled models and eight atomic models. The coupled models are PhaseRegister and Timer. The atomic models are MinuteRegister, SecondRegister, TimeCounter, Beeper, PowerLevel, PowerDriver, TimeRegister and Displayer.

The formal specifications for atomic models are as follows.

Formal Specification for MinuteRegister:

X={Minute_in<N>, PhaseSet_in<N>};

Y={Minute_out<N>};

S={{Phase, sigma, minute}}

ext ( s, e, x )

{

case port

PhaseSet_in:

if value<>0

phase=active;

sigma=Infinity;

else

phase=passive;

sigma=Infinity;

Minute_in:

if (phase= =active)

minute++;

if (minute= =60)

minute=0.

phase=active;

sigma=0;

output minute;

}

int ( s, e )

{

hold current state;

}

 (s)

{

send minutes to the port Minute_out;

}

Formal Specification for SecondRegister:

X={Second_in<N>, PhaseSet_in<N>};

Y={Second_out<N>};

S={{Phase, sigma, second}}

ext ( s, e, x )

{

case port

PhaseSet_in:

if value<>0

phase=active;

sigma=Infinity;

else

phase=passive;

sigma=Infinity;

Second_in:

if (phase= =active)

second++;

if (second= =60)

second=0.

phase=active;

sigma=0;

output second;

}

int ( s, e )

{

hold current state;

}

 (s)

{

send second to the port Second_out;

}

Formal Specification for TimeRegister:

X={Time_Set<N>, Hours_in<N>, Minutes_in<N> };

Y={Time_out<N>};

S={{Phase, sigma, hour, minute}}

ext ( s, e, x )

{

case port

Time_Set:

if value<>0

phase=active;

sigma=Infinity;

else

phase=passive;

sigma=Infinity;

Hours_in:

if (phase= =active)

hour++;

if (hour= =24)

hour=0;

phase=active;

sigma=0;

Minutes_in:

if (phase= =active)

minute++;

if (minute= =60)

minute=0;

phase=active;

sigma=0;

}

int ( s, e )

{

case phase

passive:

increment minute by one;

wrap the time appropriately;

schedule the next increment for one minute;

active:

still stay in active phase;

sigma=Infinity;

}

 (s)

{

send time(hour*100+minute) to the port Time_out;

}

Formal Specification for Displayer:

X={Day_Time<N>, Counter_Time<N>};

Y={Cur_Time<N>, Remain_Time<N>};

S={{Phase, sigma, time}}

ext ( s, e, x )

{

case port

Day_Time:

If (phase==passive)

Send time to the port Cur_Time;

Counter_Time:

If (time>0)

if (phase= =passive)

Phase=busy;

Sigma = Infinity;

Send time to the port Remain_Time;

Else  /*time= =0*/

Phase = passive;

Sigma=10 seconds ;

}

int ( s, e )

{

hold current state;

sigma=Infinity;

}

 (s)

{

send daytime to the port Cur_Time;

send remaining time to the port Remain_Time;

}

Formal Specification for Beeper:

X={Beep_in<N, R>};

Y={Beep_out<N>};

S={{Phase, sigma, Beep, preparation time}}

ext ( s, e, x )

{

case phase

passive:

sigma=preparation time;

phase=busy;

Beep=1;

busy:

continue;

}

int ( s, e )

{

case phase

busy:

passivate;

passive: /*never happens*/

}

 (s)

{

send Beep to the port Beep_out;

}

Formal Specification for PowerLevel:

X={PowerLevel_in<N>, PhaseSet_in<N>};

Y={ PowerLevel_out<N>};

S={{Phase, sigma, powerlevel }}

ext ( s, e, x )

{

case port

PhaseSet_in:

if value<>0

phase=active;

sigma=Infinity;

else

phase=passive;

sigma=Infinity;

PowerLevel_in:

if (phase= =active)

powerlevel++;

if (powerlevel = = 4)  //there are only three levels represented by 1, 2 and 3

powerlevel=1;

output powerlevel;

}

int ( s, e )

{   hold current state;

}

 (s)

{

send powerlevel to the port PowerLevel_out;

}

Formal Specification for PowerDriver:

X={PowerLevel_in<N>, CounterTime_in<N> };

Y={ Power_out<N>};

S={{Phase, sigma, power }}

ext ( s, e, x )

{

case port

PowerLevel_in:

If (phase = = busy)

Send power to the port Power_out;

Phase = passive;

Sigma = Infinity;

Else /*phase = =passive*/

Continue;

CounterTime_in:

If ( remain_time>0)

Phase=busy;

Sigma=Infinity;

Else /*remain_time=0*/

Phase = passive;

Sigma=Infinity;

}

int ( s, e )

{

case phase

busy:

phase=passive;

sigma=Infinity;

passive: /*never happens*/

}

 (s)

{

send power to the port Power_out;

}

Formal Specification for TimeCounter:

X={Minutes_in<N>, Seconds_in<N>, Start<N>, Stop<N>, Clear<N>, PhaseSet_in<N> };

Y={ RemainTime_Beeper<N>, RemainTime_Power, RemainTime_Display};

S={{Phase, sigma, RemainTime, status }}

ext ( s, e, x )

{

case port

Seconds_in:

If (status = = ready)

Get the input seconds;

Calculate the RemainTime in total seconds;

Output the RemainTime;

Minutes_in:

If (status = = ready)

Get the input minutes;

Calculate the RemainTime in total seconds;

Output the RemainTime;

Start:

If (status= =ready && RemainTime>0)

Status=busy;

Phase= active;

Send output RemainTime ;

Stop:

If (status = = busy && RemainTime >0)

Status=ready;

Phase=passive;

Send output 0 to RemainTime_Power;

Else

Status=idle;

Clear:

RemainTime=0;

Phase=passive;

Status = idle;

Send output 0 to all output ports;

PhaseSet_in:

If (value>0)

If (status = = idle)

Status = ready;

RemainTime=0;

Else  /*value=0*/

RemainTime=0;

}

int ( s, e )

{

if (status= = busy)

if (RemainTime>0)

Keep state for one second;

RemainTime =  RemainTime –1;

If (RemainTime = =0)

Passivate;

Status=idle;

Sigma=0;

Else

Sigma= Infinity;

}

 (s)

{

send RemainTime to the port RemainTime_Beeper;

send RemainTime to the port RemainTime_Power;

send RemainTime to the port RemainTime_Display;

}

The specifications of coupled models are as follows:

# Formal specification for Timer

Timer = < X, Y, { MinuteRegister, SecondRegister}, EIC, EOC, IC, SELECT >

X = { Minutes_in , Seconds_in, PhaseSet_in }

Y = {Time_Minute, Time_Second }

EIC = { ( PhaseReg.Minutes, Minutes_in), (PhaseReg.Seconds, Seconds_in), (PhaseReg.PhaseSet, PhaseSet_in) }

EOC = { (Time_Minute, PhaseReg.Time_Minute), (Time_Second, PhaseReg.Time_Second) }

IC = 

SELECT : none

# Formal specification for PhaseRegister

Timer = < X, Y, { Timer, PowerLevel}, EIC, EOC, IC, SELECT >

X = { Minutes_in , Seconds_in, PhaseSet_in, PowerLevel_in}

Y = {Time_Minute, Time_Second, PowerLevel_out }

EIC = { ( Minutes, Minutes_in), (Seconds, Seconds_in), (PhaseSet, PhaseSet_in), (PowerLevel, PowerLevel_in}

EOC = { (Time_Minute, TimeCounter.Time_Minute), (Time_Second, TimeCounter.Time_Second),  (PowerLevel_out, PowerDriver.PowerLevel) }

IC = 

SELECT : none

Testing Strategy:

Testing of the TimeRegister should check the following cases:

- The initial output should 0.

- Without any external events, the time should increase by one minute automatically according to the simulation time elapsed.

- Input an external event other than time_set, the model should not react to this event. E.g., Input an event Hours, the model should not change the value of hours because of this; Input an event Minutes, the model should not change the value of minutes because of this. The model should work as in case 2).

- Input external event time_set, input external event Hours later, the output should increase the value of hours by one. Same with Minutes input. After the time_set event is released (by sending value 0), the model should resume to work as described in 2).

- In all cases, the hours and minutes should wrap appropriately. In the case of adjusting time, hour of 24 should change to 0; minute of 60 should change to 0. Otherwise, when the value of minute reaches 60, then the value of hour should increase by one and minute change to 0. The value of hour should change to 0 if it reaches 24. The output is in the format of 100*hours + minutes. That is, 10:23 will be represented by 1023.

Testing of PowerLevel should check the following cases:

- The initial output should be the default one level 1.

- Without Phase_Set input, the model should not react to any input event of Power_Level.

- Input Phase_Set by value 1, the model should be able to react to the input event of Power_Level. The value of output power level should increase 1 per input and wrap to 1 when it reaches 4. That is, there are only three levels of power 1, 2, 3 respectively.

Testing of Displayer should check the following cases:

- The initial output should be 0.

- If there is not external event, no further output should be generated then.

- Input the time of the day, it should output that time.

- Input a non-zero remaining time of a heating job, it should output that time. The time of the day won’t be displayed until it receives a Zero remaining time.

- Input a Zero remaining time of heating job, after 10 seconds, it will accept the time of the day events if there are any.

The model of MinuteRegister reacts to the number of minutes set for a cooking job. Testing of MinuteRegister should check the following cases:

- The initial output should be 0.

- If the external event Phase_Set has not been input, the model shouldn’t react to any input of minutes event.

- Input event Phase_Set, the model should be able to react to any further Minutes input by increasing the value of minute by one per Minute input. It wraps to 0 when it reaches 60.

The model of SecondRegister reacts to the number of seconds set for a cooking job. Testing of SecondRegister should check the following cases:

- The initial output should be 0.

- If the external event Phase_Set has not been input, the model shouldn’t react to any input of seconds event.

- Input event Phase_Set, the model should be able to react to any further Seconds input by increasing the value of second by one per Second input. It wraps to 0 when it reaches 60.

The model of TimeCounter accepts the amount of time set for a cooking job. It calculates the remaining time and sends it to Beeper, PowerDriver and Displayer. The testing of this model should check the following cases:

- The initial output is 1.

- If there is no input event Phase_Set, it should not react to any other input events such as CookMinutes and CookSeconds.

- Input event Phase_Set, input event of Minutes or Seconds, it should output them to the port for Displayer.

- Input event Start, it should calculate the elapsed time and send the remaining time to the ports for PowerDriver and Displayer.

- Input event Stop, it will stop calculating the elapsed time and no further output should be sent then.

- Input event Clear whether after event Start or Stop, it will reset the remaining time to Zero and send zero to the port of Beeper, PowerDriver and Displayer.

Testing of model Beeper should check the following cases:

- The initial output should be 0 (Beep_OFF).

- It only reacts to the event Remain_Time. It should output 1 (Beep_ON) only if the Remain_Time changes from positive to zero.

Testing of PowerDriver should check the following cases:

- The initial output is 0.

- If no input event of Remain_Time, it should not react to the input event PowerLevel.

- Input event Remain_Time, input event PowerLevel. The order of them does not matter. If the Remain_Time is positive, it should output the level of power. Otherwise, it should output zero (Heat_OFF).

Testing of the coupled model Timer should check the following cases:

- The initial output is zero.

- If no input event of Phase_Set, it should not react to the events of Minutes and Seconds.

- Input event of Phase_Set, it should react to the event of Minutes by increasing the value of Minutes by 1 and send this output. It should react to the event of Seconds by increasing the value of Seconds by 1 and send this output. The values of minutes and seconds will be wrapped to zero when they reach 60.

Testing of coupled model PhaseRegister should check the following cases:

- The initial output is zero.

- If no input event of Phase_Set, it should not react to the events of Minutes, Seconds and PowerLevel.

- Input event of Phase_Set, it should react to the event of Minutes by increasing the value of Minutes by 1 and send this output to the port for Time_Minutes. It should react to the event of Seconds by increasing the value of Seconds by 1 and send this output to the port for Time_Seconds. The values of minutes and seconds will be wrapped to zero when they reach 60. It should also be able to react to the event of PowerLevel by increasing the value of power level by one and send this output to the port for PowerLevel_out. The value of power level should be less than 4.

Testing of the system model Microwave Oven should check the following cases:

- The initial output should be Current_Time is 0, Heat is 0, Beep is 0.

- Without any external event, the model should output the current time per minute.

- Input events of Hours or Minutes or Seconds or Start or Stop or Clear or Power_Level, the model should have no reaction.

- Input event Time_Set, then input event Hours or Minutes, the model should output the hours by increasing the value by 1; or output minutes by increasing the value by 1. The hours and minutes should be wrapped appropriately as mentioned in the testing of TimeRegister. It resumes to work naturally as in 2) after the Time_Set is released (by value 0).

- Input event Phase_Set, input event Minutes or Seconds, the model should output the remaining time combined with the value of minutes and seconds. Input event PowerLevel, it should output the value of power level. Input event Start, it should output the remaining time of a cooking job by decreasing it by one per second. When it reaches zero, then it should output Heat as zero, Beep as 1 (Beep_ON), display the current time of the day about 10 seconds later (it will display the time of the day from the TimeRegister).

- Input event Phase_Set, input event Minutes or Seconds, the model should output the configuring time combined with the value of minutes and seconds. Input event PowerLevel, it should output the value of power level. Input event Start, it should output the remaining time of a cooking job by decreasing it by one per second. Input an event Clear, then it should output Heat as zero, Beep as 1 (Beep_ON), display the current time of the day about 10 seconds later (it will display the time of the day from the TimeRegister).

- Input event Phase_Set, input event Minutes or Seconds, the model should output the remaining time combined with the value of minutes and seconds. Input event PowerLevel, it should output the value of power level. Input event Start, it should output the remaining time of a cooking job by decreasing it by one per second. Input an event Stop, then it should output Heat as zero.

Part III Implementation and Testing

All the atomic models and coupled models have been implemented and attached in the zip file. A variety of tests have been carried out against the atomic models and coupled models as well as the system model. The results are exactly the same as expected from those proposed in the testing strategy. The detailed test results can be referred to the output files by running the models in the zip file. Some sample results are shown and explained below:

- Tests again the atomic model Displayer

Input events (displayTest.ev)

00:05:30:000	c_day_time	1023           #current time 10:23

00:06:00:100	c_counter_time  250      #Remaining time of a job: 250

00:06:01:100	c_counter_time  249      #Remaining time of a job: 249

00:06:02:100	c_counter_time  248      #Remaining time of a job: 248

00:06:03:000	c_counter_time	0      #Remaining time is reset to zero

01:07:30:000	c_day_time	1025          #current time 10:25

Output (displayTest.out)

00:05:30:000 c_cur_time 1023          #output current time 1023 (10:23)

00:06:00:100 c_rem_time 250           #output remaining time 250

00:06:01:100 c_rem_time 249           #output remaining time 249

00:06:02:100 c_rem_time 248           #output remaining time 248

01:07:30:000 c_cur_time 1025          #output current time 1025

When the remaining time of a job is positive, it outputs the remaining time. It outputs current time otherwise. These results are exactly the expected ones.

2) Tests against the system model Microwave Oven system.

Input Events (microwaveoven.ev)

00:00:30:000	c_seconds	?                  #input seconds

00:00:30:001	c_minutes	?                  #input minutes

00:00:30:002	c_powerlevel_in	?           #input PowerLevel

00:00:35:002	c_time_set	1                  #input time_set

00:00:36:001	c_minutes	?                  #input minutes

00:00:38:001	c_hours		?               #input hours

00:00:39:002	c_time_set	0                   #release time_set

00:00:40:001	c_phase_set	1               #input phase_set

00:00:42:001	c_seconds	?                   #input seconds

00:00:42:002	c_powerlevel_in	?            #input PowerLevel

00:00:42:003	c_minutes	?                  #input minutes

00:00:42:004	c_phase_set	0              #release phase_set

00:00:42:008	c_seconds	?                  #input seconds

00:00:42:008	c_minutes	?                 #input minutes

00:00:45:100	c_phase_set	1

00:00:45:102	c_seconds	?

00:00:45:103	c_seconds	?

00:00:45:104	c_seconds	?

00:00:45:105	c_seconds	?

00:00:45:106	c_powerlevel_in	?

00:00:46:102	c_minutes	?

00:00:46:103	c_minutes	?

00:00:46:104	c_powerlevel_in	?

00:00:46:105	c_minutes	?

00:00:46:106	c_minutes	?

00:00:48:002	c_start	?              #press start

00:00:56:003	c_stop	?                  #press stop

00:00:57:004	c_clear	?              #press clear

00:01:01:000 c_phase_set		1

00:01:05:002 c_seconds		?

00:01:05:003 c_seconds		?

00:01:05:004 c_seconds		?

00:01:05:005 c_seconds		?

00:01:05:006 c_powerlevel_in		?

00:01:05:007 c_seconds		?

00:01:05:008 c_seconds		?

00:01:05:009 c_seconds		?

00:01:07:000 c_seconds	?

00:01:11:000 c_seconds	?

00:01:11:006 c_seconds	?

00:01:11:007 c_powerlevel_in	?

00:01:11:008 c_seconds	?

00:02:11:003 c_seconds	?

00:02:11:004 c_seconds	?

00:02:11:016 c_seconds	?

00:02:11:017 c_seconds	?

00:02:11:018 c_powerlevel_in	?

00:03:11:003 c_minutes	?

00:03:11:004 c_minutes	?

0:03:11:017 c_minutes	?

00:03:48:002	c_start	?

00:04:56:003	c_stop	?

00:05:30:004	c_start	?

Output (microwaveoven.out)

00:00:36:001 c_cur_time 2               #display current time 2

00:00:38:001 c_cur_time 102             # display current time 102 (1:02)

00:00:42:001 c_rem_time 1               #display remaining time 1 sec

00:00:42:003 c_rem_time 101             #display remaining time 101 (61 sec)

00:00:45:102 c_rem_time 1

00:00:45:103 c_rem_time 2

00:00:45:104 c_rem_time 3

00:00:45:105 c_rem_time 4

00:00:46:102 c_rem_time 104

00:00:46:103 c_rem_time 204

00:00:46:105 c_rem_time 304

00:00:46:106 c_rem_time 404

00:00:48:002 c_rem_time 404

00:00:48:002 c_powerlevel_out 3        #output powerlevel 3, start cooking

00:00:49:002 c_rem_time 403

00:00:50:002 c_rem_time 402

00:00:51:002 c_rem_time 401

00:00:52:002 c_rem_time 400

00:00:53:002 c_rem_time 359

00:00:54:002 c_rem_time 358

00:00:55:002 c_rem_time 357

00:00:56:002 c_rem_time 356

00:00:56:003 c_rem_time 355

00:00:56:003 c_powerlevel_out 0       #stop cooking

00:00:57:004 beep_out 1               #clear and beep

00:00:57:004 c_powerlevel_out 0

00:00:59:004 beep_out 0               #beep ends

00:01:05:002 c_rem_time 1

00:01:05:003 c_rem_time 2

00:01:05:004 c_rem_time 3

00:01:05:005 c_rem_time 4

00:01:05:007 c_rem_time 5

00:01:05:008 c_rem_time 6

00:01:05:009 c_rem_time 7

00:01:07:000 c_rem_time 8

00:01:11:000 c_rem_time 9

00:01:11:006 c_rem_time 10

00:01:11:008 c_rem_time 11

00:02:11:003 c_rem_time 12

00:02:11:004 c_rem_time 13

00:02:11:016 c_rem_time 14

00:02:11:017 c_rem_time 15

00:03:11:003 c_rem_time 115

00:03:11:004 c_rem_time 215

00:03:11:017 c_rem_time 315

00:03:48:002 c_rem_time 315

00:03:48:002 c_powerlevel_out 1    #start a new cooking job

00:03:49:002 c_rem_time 314

00:03:50:002 c_rem_time 313

00:03:51:002 c_rem_time 312

00:03:52:002 c_rem_time 311

00:03:53:002 c_rem_time 310

00:03:54:002 c_rem_time 309

00:03:55:002 c_rem_time 308

00:03:56:002 c_rem_time 307

00:03:57:002 c_rem_time 306

00:03:58:002 c_rem_time 305

00:03:59:002 c_rem_time 304

00:04:00:002 c_rem_time 303

00:04:01:002 c_rem_time 302

00:04:02:002 c_rem_time 301

00:04:03:002 c_rem_time 300

00:04:04:002 c_rem_time 259

00:04:05:002 c_rem_time 258

00:04:06:002 c_rem_time 257

00:04:07:002 c_rem_time 256

00:04:08:002 c_rem_time 255

00:04:09:002 c_rem_time 254

00:04:10:002 c_rem_time 253

00:04:11:002 c_rem_time 252

00:04:12:002 c_rem_time 251

00:04:13:002 c_rem_time 250

00:04:14:002 c_rem_time 249

00:04:15:002 c_rem_time 248

00:04:16:002 c_rem_time 247

00:04:17:002 c_rem_time 246

00:04:18:002 c_rem_time 245

00:04:19:002 c_rem_time 244

00:04:20:002 c_rem_time 243

00:04:21:002 c_rem_time 242

00:04:22:002 c_rem_time 241

00:04:23:002 c_rem_time 240

00:04:24:002 c_rem_time 239

00:04:25:002 c_rem_time 238

00:04:26:002 c_rem_time 237

00:04:27:002 c_rem_time 236

00:04:28:002 c_rem_time 235

00:04:29:002 c_rem_time 234

00:04:30:002 c_rem_time 233

00:04:31:002 c_rem_time 232

00:04:32:002 c_rem_time 231

00:04:33:002 c_rem_time 230

00:04:34:002 c_rem_time 229

00:04:35:002 c_rem_time 228

00:04:36:002 c_rem_time 227

00:04:37:002 c_rem_time 226

00:04:38:002 c_rem_time 225

00:04:39:002 c_rem_time 224

00:04:40:002 c_rem_time 223

00:04:41:002 c_rem_time 222

00:04:42:002 c_rem_time 221

00:04:43:002 c_rem_time 220

00:04:44:002 c_rem_time 219

00:04:45:002 c_rem_time 218

00:04:46:002 c_rem_time 217

00:04:47:002 c_rem_time 216

00:04:48:002 c_rem_time 215

00:04:49:002 c_rem_time 214

00:04:50:002 c_rem_time 213

00:04:51:002 c_rem_time 212

00:04:52:002 c_rem_time 211

00:04:53:002 c_rem_time 210

00:04:54:002 c_rem_time 209

00:04:55:002 c_rem_time 208

00:04:56:002 c_rem_time 207

00:04:56:003 c_rem_time 206

00:04:56:003 c_powerlevel_out 0

00:05:30:004 c_rem_time 206

00:05:30:004 c_powerlevel_out 1

00:05:31:004 c_rem_time 205

00:05:32:004 c_rem_time 204

00:05:33:004 c_rem_time 203

00:05:34:004 c_rem_time 202

00:05:35:004 c_rem_time 201

00:05:36:004 c_rem_time 200

00:05:37:004 c_rem_time 159

00:05:38:004 c_rem_time 158

00:05:39:004 c_rem_time 157

00:05:40:004 c_rem_time 156

00:05:41:004 c_rem_time 155

00:05:42:004 c_rem_time 154

00:05:43:004 c_rem_time 153

00:05:44:004 c_rem_time 152

00:05:45:004 c_rem_time 151

00:05:46:004 c_rem_time 150

00:05:47:004 c_rem_time 149

00:05:48:004 c_rem_time 148

00:05:49:004 c_rem_time 147

00:05:50:004 c_rem_time 146

00:05:51:004 c_rem_time 145

00:05:52:004 c_rem_time 144

00:05:53:004 c_rem_time 143

00:05:54:004 c_rem_time 142

00:05:55:004 c_rem_time 141

00:05:56:004 c_rem_time 140

00:05:57:004 c_rem_time 139

00:05:58:004 c_rem_time 138

00:05:59:004 c_rem_time 137

00:06:00:004 c_rem_time 136

00:06:01:004 c_rem_time 135

00:06:02:004 c_rem_time 134

00:06:03:004 c_rem_time 133

00:06:04:004 c_rem_time 132

00:06:05:004 c_rem_time 131

00:06:06:004 c_rem_time 130

00:06:07:004 c_rem_time 129

00:06:08:004 c_rem_time 128

00:06:09:004 c_rem_time 127

00:06:10:004 c_rem_time 126

00:06:11:004 c_rem_time 125

00:06:12:004 c_rem_time 124

00:06:13:004 c_rem_time 123

00:06:14:004 c_rem_time 122

00:06:15:004 c_rem_time 121

00:06:16:004 c_rem_time 120

00:06:17:004 c_rem_time 119

00:06:18:004 c_rem_time 118

00:06:19:004 c_rem_time 117

00:06:20:004 c_rem_time 116

00:06:21:004 c_rem_time 115

00:06:22:004 c_rem_time 114

00:06:23:004 c_rem_time 113

00:06:24:004 c_rem_time 112

00:06:25:004 c_rem_time 111

00:06:26:004 c_rem_time 110

00:06:27:004 c_rem_time 109

00:06:28:004 c_rem_time 108

00:06:29:004 c_rem_time 107

00:06:30:004 c_rem_time 106

00:06:31:004 c_rem_time 105

00:06:32:004 c_rem_time 104

00:06:33:004 c_rem_time 103

00:06:34:004 c_rem_time 102

00:06:35:004 c_rem_time 101

00:06:36:004 c_rem_time 100

00:06:37:004 c_rem_time 59

00:06:38:004 c_rem_time 58

00:06:39:004 c_rem_time 57

00:06:40:004 c_rem_time 56

00:06:41:004 c_rem_time 55

00:06:42:004 c_rem_time 54

00:06:43:004 c_rem_time 53

00:06:44:004 c_rem_time 52

00:06:45:004 c_rem_time 51

00:06:46:004 c_rem_time 50

00:06:47:004 c_rem_time 49

00:06:48:004 c_rem_time 48

00:06:49:004 c_rem_time 47

00:06:50:004 c_rem_time 46

00:06:51:004 c_rem_time 45

00:06:52:004 c_rem_time 44

00:06:53:004 c_rem_time 43

00:06:54:004 c_rem_time 42

00:06:55:004 c_rem_time 41

00:06:56:004 c_rem_time 40

00:06:57:004 c_rem_time 39

00:06:58:004 c_rem_time 38

00:06:59:004 c_rem_time 37

00:07:00:004 c_rem_time 36

00:07:01:004 c_rem_time 35

00:07:02:004 c_rem_time 34

00:07:03:004 c_rem_time 33

00:07:04:004 c_rem_time 32

00:07:05:004 c_rem_time 31

00:07:06:004 c_rem_time 30

00:07:07:004 c_rem_time 29

00:07:08:004 c_rem_time 28

00:07:09:004 c_rem_time 27

00:07:10:004 c_rem_time 26

00:07:11:004 c_rem_time 25

00:07:12:004 c_rem_time 24

00:07:13:004 c_rem_time 23

00:07:14:004 c_rem_time 22

00:07:15:004 c_rem_time 21

00:07:16:004 c_rem_time 20

00:07:17:004 c_rem_time 19

00:07:18:004 c_rem_time 18

00:07:19:004 c_rem_time 17

00:07:20:004 c_rem_time 16

00:07:21:004 c_rem_time 15

00:07:22:004 c_rem_time 14

00:07:23:004 c_rem_time 13

00:07:24:004 c_rem_time 12

00:07:25:004 c_rem_time 11

00:07:26:004 c_rem_time 10

00:07:27:004 c_rem_time 9

00:07:28:004 c_rem_time 8

00:07:29:004 c_rem_time 7

00:07:30:004 c_rem_time 6

00:07:31:004 c_rem_time 5

00:07:32:004 c_rem_time 4

00:07:33:004 c_rem_time 3

00:07:34:004 c_rem_time 2

00:07:35:004 c_rem_time 1

00:07:35:004 beep_out 1      # cooking job is finished without interruption; beep

00:07:35:004 c_powerlevel_out 0

00:07:37:004 beep_out 0

00:07:39:002 c_cur_time 108

00:08:39:002 c_cur_time 109

00:09:39:002 c_cur_time 110

00:10:39:002 c_cur_time 111

00:11:39:002 c_cur_time 112

00:12:39:002 c_cur_time 113

00:13:39:002 c_cur_time 114

00:14:39:002 c_cur_time 115

00:15:39:002 c_cur_time 116

00:16:39:002 c_cur_time 117

00:17:39:002 c_cur_time 118

00:18:39:002 c_cur_time 119

00:19:39:002 c_cur_time 120

00:20:39:002 c_cur_time 121

00:21:39:002 c_cur_time 122

00:22:39:002 c_cur_time 123

00:23:39:002 c_cur_time 124

00:24:39:002 c_cur_time 125

00:25:39:002 c_cur_time 126

00:26:39:002 c_cur_time 127

00:27:39:002 c_cur_time 128

00:28:39:002 c_cur_time 129

00:29:39:002 c_cur_time 130

00:30:39:002 c_cur_time 131

00:31:39:002 c_cur_time 132

00:32:39:002 c_cur_time 133

00:33:39:002 c_cur_time 134

00:34:39:002 c_cur_time 135

00:35:39:002 c_cur_time 136

00:36:39:002 c_cur_time 137

00:37:39:002 c_cur_time 138

00:38:39:002 c_cur_time 139

00:39:39:002 c_cur_time 140

00:40:39:002 c_cur_time 141

00:41:39:002 c_cur_time 142

00:42:39:002 c_cur_time 143

00:43:39:002 c_cur_time 144

00:44:39:002 c_cur_time 145

00:45:39:002 c_cur_time 146

00:46:39:002 c_cur_time 147

00:47:39:002 c_cur_time 148

00:48:39:002 c_cur_time 149

00:49:39:002 c_cur_time 150

00:50:39:002 c_cur_time 151

00:51:39:002 c_cur_time 152

00:52:39:002 c_cur_time 153

00:53:39:002 c_cur_time 154

00:54:39:002 c_cur_time 155

00:55:39:002 c_cur_time 156

00:56:39:002 c_cur_time 157

00:57:39:002 c_cur_time 158

00:58:39:002 c_cur_time 159

00:59:39:002 c_cur_time 200

01:00:39:002 c_cur_time 201

01:01:39:002 c_cur_time 202

01:02:39:002 c_cur_time 203

01:03:39:002 c_cur_time 204

01:04:39:002 c_cur_time 205

01:05:39:002 c_cur_time 206

01:06:39:002 c_cur_time 207

01:07:39:002 c_cur_time 208

01:08:39:002 c_cur_time 209

01:09:39:002 c_cur_time 210

01:10:39:002 c_cur_time 211

01:11:39:002 c_cur_time 212

01:12:39:002 c_cur_time 213

01:13:39:002 c_cur_time 214

01:14:39:002 c_cur_time 215

01:15:39:002 c_cur_time 216

01:16:39:002 c_cur_time 217

01:17:39:002 c_cur_time 218

01:18:39:002 c_cur_time 219

01:19:39:002 c_cur_time 220

01:20:39:002 c_cur_time 221

01:21:39:002 c_cur_time 222

01:22:39:002 c_cur_time 223

01:23:39:002 c_cur_time 224

01:24:39:002 c_cur_time 225

01:25:39:002 c_cur_time 226

01:26:39:002 c_cur_time 227

01:27:39:002 c_cur_time 228

01:28:39:002 c_cur_time 229

01:29:39:002 c_cur_time 230

01:30:39:002 c_cur_time 231

01:31:39:002 c_cur_time 232

01:32:39:002 c_cur_time 233

01:33:39:002 c_cur_time 234

01:34:39:002 c_cur_time 235

01:35:39:002 c_cur_time 236

01:36:39:002 c_cur_time 237

01:37:39:002 c_cur_time 238

01:38:39:002 c_cur_time 239

01:39:39:002 c_cur_time 240

01:40:39:002 c_cur_time 241

01:41:39:002 c_cur_time 242

01:42:39:002 c_cur_time 243

01:43:39:002 c_cur_time 244

01:44:39:002 c_cur_time 245

01:45:39:002 c_cur_time 246

01:46:39:002 c_cur_time 247

01:47:39:002 c_cur_time 248

01:48:39:002 c_cur_time 249

01:49:39:002 c_cur_time 250

01:50:39:002 c_cur_time 251

01:51:39:002 c_cur_time 252

01:52:39:002 c_cur_time 253

01:53:39:002 c_cur_time 254

01:54:39:002 c_cur_time 255

01:55:39:002 c_cur_time 256

01:56:39:002 c_cur_time 257

01:57:39:002 c_cur_time 258

01:58:39:002 c_cur_time 259

01:59:39:002 c_cur_time 300

02:00:39:002 c_cur_time 301

02:01:39:002 c_cur_time 302

02:02:39:002 c_cur_time 303

02:03:39:002 c_cur_time 304

02:04:39:002 c_cur_time 305

02:05:39:002 c_cur_time 306

02:06:39:002 c_cur_time 307

02:07:39:002 c_cur_time 308

02:08:39:002 c_cur_time 309

02:09:39:002 c_cur_time 310

02:10:39:002 c_cur_time 311

02:11:39:002 c_cur_time 312

02:12:39:002 c_cur_time 313

02:13:39:002 c_cur_time 314

02:14:39:002 c_cur_time 315

02:15:39:002 c_cur_time 316

02:16:39:002 c_cur_time 317

02:17:39:002 c_cur_time 318

02:18:39:002 c_cur_time 319

02:19:39:002 c_cur_time 320

02:20:39:002 c_cur_time 321

02:21:39:002 c_cur_time 322

02:22:39:002 c_cur_time 323

02:23:39:002 c_cur_time 324

02:24:39:002 c_cur_time 325

02:25:39:002 c_cur_time 326

02:26:39:002 c_cur_time 327

02:27:39:002 c_cur_time 328

02:28:39:002 c_cur_time 329

02:29:39:002 c_cur_time 330

02:30:39:002 c_cur_time 331

02:31:39:002 c_cur_time 332

02:32:39:002 c_cur_time 333

02:33:39:002 c_cur_time 334

02:34:39:002 c_cur_time 335

02:35:39:002 c_cur_time 336

02:36:39:002 c_cur_time 337

02:37:39:002 c_cur_time 338

02:38:39:002 c_cur_time 339

02:39:39:002 c_cur_time 340

02:40:39:002 c_cur_time 341

02:41:39:002 c_cur_time 342

02:42:39:002 c_cur_time 343

02:43:39:002 c_cur_time 344

02:44:39:002 c_cur_time 345

02:45:39:002 c_cur_time 346

02:46:39:002 c_cur_time 347

02:47:39:002 c_cur_time 348

02:48:39:002 c_cur_time 349

02:49:39:002 c_cur_time 350

02:50:39:002 c_cur_time 351

02:51:39:002 c_cur_time 352

02:52:39:002 c_cur_time 353

02:53:39:002 c_cur_time 354

02:54:39:002 c_cur_time 355

02:55:39:002 c_cur_time 356

02:56:39:002 c_cur_time 357

02:57:39:002 c_cur_time 358

02:58:39:002 c_cur_time 359

02:59:39:002 c_cur_time 400

03:00:39:002 c_cur_time 401

03:01:39:002 c_cur_time 402

03:02:39:002 c_cur_time 403

03:03:39:002 c_cur_time 404

03:04:39:002 c_cur_time 405

The behaviors of all the atomic models and coupled models are consistent with the specifications.

Variations: No variations have ever been made.