SITE / ÉITI

University of Ottawa / Université d’Ottawa

### Department of Systems and Computer Engineering

Carleton University

# SYS 5807

## Methodological Aspects of Modeling and Simulation

###### Complete Assignment 1

Presented to / Présenté à :

Gabriel A. Wainer

| ### Name / Nom | ### Student  / ###  d’étudiant | ### email |
| --- | --- | --- |
| Jean-Christian Delannoy | 2062895 | jcdelannoy@videotron.ca |

October 22, 2003

Part 1

Sensor-Based 4-Way Traffic Control

My proposal consists of modeling a 4-way traffic control system with a system of 4 sensors to detect approaching vehicles from every direction of the intersection. The system will include many different parameters to test the efficiency of the system in various situations: from lone vehicles showing up at 2a.m. to rush-hour traffic in which many vehicles approach from every direction. Configuration of the system will consist of varying values such as the minimal time a light may stay in a state, the maximum time it may stay in a state, the amount of time before switching the lights when a vehicle is detected and the distance at which a vehicle can be detected from the intersection.

General Model Structure

sensor

Light

W

r

y

g

Light

E

r

y

g

Light

N

r

y

g

Light

S

r

y

g

sensor processing unit

lights controller

sensor

sensor

sensor

Controller

- Light variables

LightEnabled: a value which represents whether the red, green or yellow light is currently enabled

- Sensor variables

DetectionDistance: a value which represents how far a vehicle must be from the sensor before it is detected.

VehicleDetected: a boolean which represents whether a vehicle has been detected or not.

- Controller variables

- lights controller variables

RedTime: amount of time a light will usually stay in the red state

GreenTime: amount of time a light will usually stay in the green state

YellowTime: amout of time a light will stay in the yellow state

- sensor processing unit variables

MininimumTime: minimum time in which a vehicle arriving at the intersection will be serviced.

MaximumTime: maximum time in which a vehicle arriving at the intersection will be serviced.

PriorityN: determines the priority of vehicles arriving from the north direction

PriorityS: determines the priority of vehicles arriving from the south direction

PriorityE: determines the priority of vehicles arriving from the east direction

PriorityW: determines the priority of vehicles arriving from the west direction

Part 2

Revised General Model Structure

### TrafficLight

### TrafficLight

GoChangeColor

GoChangeColor

LightController

ToLightN

ChangeWay

ToLightW

ToLightS

ToLightE

SensorProcUnit

SensorS

ChangeLights

SensorN

SensorW

SensorE

### TrafficLight

### TrafficLight

GoChangeColor

GoChangeColor

Coupled Models

# Controller

### LightController

ToLightN

ChangeWay

ToLightW

ToLightS

ToLightE

### SensorProcUnit

SensorS

ChangeLights

SensorN

SensorW

SensorE

Controller = < X , Y , { LightController , SensorProcUnit } , { I } , { Z } >

- X = { SensorN , SensorS , SensorE , SensorW }

- Y = { ToLightN , ToLightS , ToLightE , ToLightW }

- I( LightController ) = self.

- I( SensorProcUnit ) = LightController

- Z( LightController ) = self.

- Z( SensorProcUnit ) = LightController

Testing Strategy

- Send an external event  signal to lights MaxIntervalTime seconds later.

- Send an external event less that MinIntervalTime seconds after a previous external event  signal to lights (MaxIntervalTime-MinIntervalTime) seconds later.

- Send an external event between MinIntervalTime and MaxIntervalTime seconds after a previous external event  signal to lights immediately.

Atomic Models

# LightController

LightController

ToLightN

ChangeWay

ToLightW

ToLightS

ToLightE

LightController = < X ,  Y ,  S ,  ,  >

- X = { ChangeWay }

- Y = { ToLightW, ToLightN, ToLightS, ToLightE }

- S = {NorthSouth , EastWest }

-  = X * S  S

(ChangeWay, NorthSount) = EastWest

(ChangeWay, EastWest) = NorthSouth

- : S  Y

(EastWest) = ToLightE (off), ToLightW (off), ToLightN (on), ToLightS(on)

(NorthSouth) = ToLightE (on), ToLightW (on), ToLightN (off), ToLightS(off)

Testing Strategy

- Send an external signal  signal on ToLightW , ToLightE , signal off ToLightN, ToLightS

- Send an external signal  signal off ToLightW , ToLightE , signal on ToLightN, ToLightS

- Send an external signal  signal on ToLightW , ToLightE , signal off ToLightN, ToLightS

- and so forth …

# SensorProcUnit

SensorProcUnit

SensorS

ChangeLights

SensorN

SensorW

SensorE

SensorProcUnit = < X ,  Y ,  S ,  ,  >

- X = { SensorN, SensorS, SensorE, SensorW }

- Y = { ChangeLights }

- S = {ChangeAtMin, ChangeAtMax }

-  = X * S  S

(ChangeAtMin, SensorN | SensorS | SensorE | SensorW) = ChangeAtMin

(ChangeAtMax, SensorN | SensorS | SensorE | SensorW) = ChangeAtMin

- : S  Y

(ChangeAtMin | ChangeAtMax) = ChangeLights

Pseudocode

When an external event is triggered:

- if (currentTime < lastChangeTime + minChangeInterval)

nextChangeTime=lastChangeTime + minChangeInterval

- else trigger signal on ChangeLights

When triggering a signal on ChangeLights:

- send signal on ChangeLights

- nextChangeTime=currentTime + maxChangeInterval

Testing Strategy

- Send an external event  signal to ChangeLights MaxIntervalTime seconds later.

- Send an external event less that MinIntervalTime seconds after a previous external event  signal to ChangeLights (MaxIntervalTime-MinIntervalTime) seconds later.

- Send an external event between MinIntervalTime and MaxIntervalTime seconds after a previous external event  signal to ChangeLights immediately.

# TrafficLight

TrafficLight

GoChangeColor

TrafficLight = < X ,  Y ,  S ,  ,  >

- X = { GoChangeColor }

- Y = 

- S = {red, green, yellow }

-  = X * S  S

(red , GoChangeColor) = green

(green, GoChangeColor) = yellow

- : 

Pseudocode

When an external event is triggered:

- if( red ) set a timer to become green in YellowTime seconds.

- if( green ) become yellow immediately, become red in YellowTime seconds.

- if( yellow )  impossible case

Testing Strategy

- send an external event when red  becomes green YellowTime seconds later

- send an external event when green  becomre yellow immediately, becomes red YellowTime seconds later

Part 3

Execution Results and Analysis

- light controller

[top]

components : lightcontroller@LightController

out : MyOutN MyOutS MyOutE MyOutW

in : MyIn

Link : ToLightN@lightcontroller MyOutN

Link : ToLightS@lightcontroller MyOutS

Link : ToLightE@lightcontroller MyOutE

Link : ToLightW@lightcontroller MyOutW

Link : MyIn ChangeWay@lightcontroller

| Events: 00:00:10:00 MyIn 1 00:00:15:00 MyIn 1 00:00:20:00 MyIn 1 | Output: 00:00:10:000 myoutn 2 00:00:10:000 myouts 2 00:00:10:000 myoute 1 00:00:10:000 myoutw 1 00:00:15:000 myoutn 1 00:00:15:000 myouts 1 00:00:15:000 myoute 2 00:00:15:000 myoutw 2 00:00:20:000 myoutn 2 00:00:20:000 myouts 2 00:00:20:000 myoute 1 00:00:20:000 myoutw 1 |
| --- | --- |

The LightController model runs perfectly. When it is triggered it will send on (1) messages to the north and south ports and off (2) to the east and west ports. When it is triggered again it will flip its messages around.

- SensorProcUnit

[top]

components : sensorProcUnit@SensorProcUnit

in : MyInN MyInS MyInW MyInE

out : MyChangeLights

Link : ChangeLights@sensorProcUnit MyChangeLights

Link : MyInN SensorN@sensorProcUnit

Link : MyInS SensorS@sensorProcUnit

Link : MyInE SensorE@sensorProcUnit

Link : MyInW SensorW@sensorProcUnit

[sensorProcUnit]

MinIntervalTime : 00:00:10:000

MaxIntervalTime : 00:00:20:000

| Events: 00:00:15:00 MyInN 1 00:00:20:00 MyInE 1 00:00:39:00 MyInS 1 | Output: 00:00:15:000 mychangelights 1 00:00:25:000 mychangelights 1 00:00:39:000 mychangelights 1 00:00:59:000 mychangelights 1 00:01:19:000 mychangelights 1 00:01:39:000 mychangelights 1 00:01:59:000 mychangelights 1 00:02:19:000 mychangelights 1 00:02:39:000 mychangelights 1 00:02:59:000 mychangelights 1 00:03:19:000 mychangelights 1 |
| --- | --- |

The SensorProcUnit also runs perfectly.  (note that the MinIntervalTime and MaxIntervalTime have been initialized to 10 and 20 seconds respectively). When an external event is triggered between the two time intervals (in this case: 15), a signal to change the lights is sent immediately. When an external event is triggered before MinIntervalTime is elapsed, the signal to change the lights is sent as soon as this minimum time interval is reached. In our case the external event is at 20, or 5 seconds after the last change, therefore the light change can only occur at 25 seconds, or 10 seconds after the previous change. At the end, since we are not sending any external events, the change lights signal is being sent every MaxIntervalTime seconds. Note that the program continues indefinitely, I did not find a function to make the simulation stop except ctrl-c.

- Controller

[top]

components : lightN@TrafficLight lightS@TrafficLight lightW@TrafficLight lightE@TrafficLight Controller

out : LightsChange

in : MySensorN MySensorS MySensorE MySensorW

Link : LightN@Controller GoChangeColor@lightN

Link : LightS@Controller GoChangeColor@lightS

Link : LightE@Controller GoChangeColor@lightE

Link : LightW@Controller GoChangeColor@lightW

Link : MySensorN SensorN@Controller

Link : MySensorS SensorS@Controller

Link : MySensorE SensorE@Controller

Link : MySensorW SensorW@Controller

Link : LightN@Controller LightsChange

[Controller]

components : lightController@LightController sensorProcUnit@SensorProcUnit

in : SensorN SensorS SensorE SensorW

out : LightN LightS LightE LightW

Link : ChangeLights@sensorProcUnit ChangeWay@lightController

Link : ToLightN@lightController LightN

Link : ToLightS@lightController LightS

Link : ToLightE@lightController LightE

Link : ToLightW@lightController LightW

Link : SensorN SensorN@sensorProcUnit

Link : SensorS SensorS@sensorProcUnit

Link : SensorE SensorE@sensorProcUnit

Link : SensorW SensorW@sensorProcUnit

[lightN]

YellowTime : 00:00:01:000

[lightS]

YellowTime : 00:00:01:000

[lightE]

YellowTime : 00:00:01:000

[lightW]

YellowTime : 00:00:01:000

[sensorProcUnit]

MinIntervalTime : 00:00:10:000

MaxIntervalTime : 00:00:20:000

| Events: 00:00:15:00 MySensorN 1 00:00:20:00 MySensorE 1 00:00:39:00 MySensorS 1 | Output: 00:00:15:000 lightschange 2 00:00:25:000 lightschange 1 00:00:39:000 lightschange 2 00:00:59:000 lightschange 1 00:01:19:000 lightschange 2 00:01:39:000 lightschange 1 00:01:59:000 lightschange 2 00:02:19:000 lightschange 1 00:02:39:000 lightschange 2 00:02:59:000 lightschange 1 00:03:19:000 lightschange 2 |
| --- | --- |

Finally, the Controller model also functions according to the specified requirements. First, a car arrives at the north end between MinIntervalTime and MaxIntervalTime and therefore sees his request serviced immediately, of course the time for the other green light to go from yellow to red must also be factored in. Five seconds later, a car arrives at the east intersection but must wait until MinIntervalTime is elapsed before his request is serviced. At the end, no cars are arriving at any of the intersections (maybe the super bowl is on television or something) therefore the lights controller simply switches every MaxIntervalTime seconds. Note that the simulation will continue indefinitely. To kill it, hit ctrl-c.