SYSC 5104 - Methodologies for Discrete-Event Modelling and Simulation

Fall 2005

Assignment 2

WireWorld

## Al-Sanad, Abdulelah

## ID# 3521674

Email: aalsa079@site.uottawa.ca

November 9, 2005

1. Conceptual Model Description

1.2 Problem description

WireWorld is a two-dimensional four-color cellular automaton that was introduced by Brian Silverman in 1987. It can be used to build and simulate logical circuits and electronic logic elements or gates based on its state changes. Using its rules, it is possible build a complete computer within Wireworld. Moreover, the system of WireWorld is used to draw wires and electrons, with electrons traveling as time steps pass.

Cells in WireWorld have one of four possible states:

- (0) - Background.

- (1) - Electron head.

- (2) - Electron tail.

- (3) - Wire cell.

1.2 The rules

The rule of WireWorld to find the new value of a cell is from its old value and the values of its 8 neighbors. The system can roughly model the flow of currents in wires according to the following rules:

- The color 0 (Black, considered as background), stays as background.

- The color 1 (Red, considered as an electron head), turns into an electron tail.

- The color 2 (Blue, considered as electron tail), turns into wire.

- The color 3 (Yellow, considered as wire), turns to an electron head if exactly one or two of the surrounding eight neighbouring cells are electron heads, otherwise remains wire.

Time proceeds in discrete steps and cells change their states based on the rules of the system. Using these rules, digital logic (such as OR, XOR, and AND gates) and fairly complicated logic circuits can be constructed. Therefore, WireWorld can be configured to perform and demonstrates universal computations very simply.

| (-1,-1) | (-1,0) | (-1,1) |
| --- | --- | --- |
| (0,-1) | (0,0) | (0,1) |
| (1,-1) | (1,0) | (1,1) |

2 Model Specifications

2.1 Formal specifications for the Cell-DEVS models

The formal specification for the wireworld model is as follows:

M = < I, X, Y, Xlist, Ylist, , N, {m, n}, C, B, Z, select>

Xlist = {  };

Ylist = {  }.

 = 9;

I = < Px, Py>, with Px = {  }; Py = {  };

N = { (-1,-1), (-1,0), (-1,1), (0,-1), (0,0), (0,1), (1,-1), (1,0), (1,1)};

X =  {0, 1, 2, 3};

Y = {0, 1, 2, 3};

m = 20;

n = 20;

B = {};

D = 100

C = {CAij / i  [1,20], j  [1,20]}

Z :

Pij Y1  Pi-1,j-1 X1         		 Pi,j X1   Pi+1,j+1 Y1

Pij Y2  Pi-1,j X2                  	 Pi,j X2    Pi+1,j Y2

Pij Y3  Pi-1,j+1 X3       		 Pi,j X3    Pi+1,j-1 Y3

Pij Y4  Pi,j-1 X4        		 Pi,j X4   Pi,j+! Y4

Pij Y5  Pi,j X5		             Pij X5   Pi,j Y5

Pij Y6  Pi,j+1 X1         		 Pi,j X6   Pi,j-1 Y6

Pij Y7  Pi+1,j-1 X2                 	 Pi,j X7    Pi-1,j+1 Y7

Pij Y8  Pi+1,j X3       		 Pi,j X8    Pi-1,j Y8

Pij Y9  Pi+1,j+1 X4        		 Pi,j X9   Pi-1,j-1 Y9

select = { (-1,-1), (-1,0), (-1,1), (0,-1), (0,0), (0,1), (1,-1), (1,0), (1,1)};

2.1 Building the models using CD++.

The following CD++ code was written to implement the WireWorld:

[top] components : wireworld [wireworld] type : cell width : 20 height : 20 delay : transport defaultDelayTime : 100 border : wrapped neighbors : wireworld(-1,-1) wireworld(-1,0) wireworld(-1,1) neighbors : wireworld(0,-1) wireworld(0,0) wireworld(0,1) neighbors : wireworld(1,-1) wireworld(1,0) wireworld(1,1) initialvalue : 0 initialCellsValue : OR.val % The file that has the input values for the model to show different example localtransition : wireworld-rule [wireworld-rule] rule : 0 100 { (0,0) = 0 } % a background cell always stays background rule : 2 100 { (0,0) = 1 } % an electron head always becomes an electron tail rule : 3 100 { (0,0) = 2 } % an electron tail always becomes a wire % wire stays as wire unless it has just one or two neighbours that are electron heads, in which case it becomes an electron head rule : 1 100 { (0,0) = 3 and (statecount(1) = 1 or statecount(1) = 2)} rule : 3 100 { t }

3 Model Execution Analyses

Wireworld consists of colored square cells laid out onto a two dimensional rectangular grid. The cells change color according to what their immediate neighbors look like.

After building the models using CD++, any desired initial state on the grid can be set and it can be easily monitoring its evolution as time elapse. For example, the figure below shows a single row of wire cells with an electron tail followed by electronic head as a pair that moves over the wire.

Figure 1:  An electron head-tail pair moves along a row of wire cells

3.1 Testing the model with some basic elements

3.1.1 Implementing a diode

Diodes block the data flow in one direction only that flows in the opposite direction to the way it is facing the diode. A diode can be constructed in wireworld from its rules. If three neighbors are electron heads, the cell will remain wire since a wire cell becomes an electron head if it has just one or two neighbors that are electron heads. The following initial values construct a diode:

(7,0) = 0 (7,1) = 2 (7,2) = 1 (7,3) = 3 (7,4) = 3 (7,5) = 3 (7,6) = 3 (7,7) = 3 (7,8) = 3 (7,9) = 0 (7,10) = 3 (7,11) = 3 (7,12) = 3 (7,13) = 3 (7,14) = 3 (7,15) = 3 (7,16) = 3 (7,17) = 3 (7,18) = 3 (7,19) = 0 (6,8) = 3 (6,9) = 3 (8,8) = 3 (8,9) = 3

Figure 2 shows a diode that let the electron propagate to right side of the wire.  Figure 3 shows the execution of the model when the electron propagates through the diode.

Figure 2: A diode that let the electron propagate to right side of the wire.

Figure 3: The execution of the model when the electron propagates through the diode.

Figure 4 shows a different diode in the opposite direction that blocks the flow of electron. The output of this model is shown in figure 5 where the diode stops the electron movement.

Figure 4: Different diode in the opposite direction that blocks the flow of electron

Figure 5: The diode stops the electron movement.

3.1.2 Implementing OR gate

The OR gate consists of five wire cells that are in a plus shape as shown in Figure 6. This Figure shows as an OR gate with two input signal of values 11. The following initial values construct OR gate with two inputs.

(5,0) = 0 (5,1) = 2 (5,2) = 1 (5,3) = 3 (5,4) = 3 (5,5) = 3 (5,6) = 3 (5,7) = 3 (6,8) = 3 (7,7) = 3 (7,8) = 3 (7,9) = 3 (7,10) = 3 (7,11) = 3 (7,12) = 3 (7,13) = 3 (7,14) = 3 (7,15) = 3 (7,16) = 3 (7,17) = 3 (7,18) = 3 (7,19) = 0 (8,8) = 3 (9,0) = 0 (9,1) = 3 (9,2) = 3 (9,3) = 3 (9,4) = 3 (9,5) = 3 (9,6) = 3 (9,7) = 3

Figure 6: The OR gate consists of five wire cells that are in a plus shap

Figure 7 shows the execution of this example and its output.

Figure 7: The output of the OR gate when the two input has values '11'

Figure 8 shows another example of the OR gate that has different input combination of 10 and its result.

Figure 8: Another example of the OR gate that has different input combination of '10' and its result.

4. References:

- Eric W. Weisstein et al. "WireWorld." From MathWorld--A Wolfram Web Resource. <http://mathworld.wolfram.com/WireWorld.html>

- Nyles Heise, World Wide WireWorld, <http://pages.prodigy.net/nylesheise/wireworld.html>

- "Wireworlds" the classical WireWorld cellular automaton and examples of logical gates created in WireWorld. < http://www.zillions-of-games.com/cgi-bin/zilligames/submissions.cgi/5295?do=show;id=2 >

- "A Wireworld Computer", the construction of a full-scale computer using WireWorld. < http://www.quinapalus.com/wi-index.html >